3GPP TSG-RAN WG3 Meeting #129	R3-255352

Bengaluru, India, 25th – 29th August, 2025

Title:	Further discussion on UAV altitude information reporting

Agenda Item:	8.3

Source:	Huawei, Nokia

Document for:	Discussion

# 1	Introduction

This contribution further discusses RAN aspects and impacts to enable the UAV altitude reporting based on the latest version of TS 23.256 and the status in RAN3 following the discussions in RAN3#128 meeting.

# 2	Discussion

## 2.1 Background

In RAN3#127 meeting (Athens 2025) RAN3 discussed further the network-based mechanism for identifying, detecting and reporting the flight status information associated to Rel-18 UAV UEs based on the feedback from SA2 in [2], included in the reply LS to the LS sent by RAN3 to SA2 (Cc: RAN, SA, SA1) in [3] where RAN3 asked SA2 for some guidance on whether there was a requirement for the NG-RAN node to provide flight information to CN for both Rel-18 UAV UEs and pre Rel-18 non-UAV devices.

As part of its feedback, SA2 indicated in [2] that, although no requirements exist, the flight status information reporting from NG-RAN to the CN pertaining to Rel-18 UAV UEs could be supported based on the current specification work that SA2 is conducting as part of the Rel-19 WI “Uncrewed Aerial System Phase 3” (UAS_Ph3) [4]. More specifically, SA2 specified new features in Rel-19, in particular the “NEF Assisted In-flight Monitoring” which is detailed in clause 5.12.3 of TS 23.256 [5]. For these features, SA2 also specified means to instruct the UAV UEs to perform altitude reporting in clause 5.16 of TS 23.256, which can be done via application layer or via 3GPP-defined signalling.

The typical scenario where the network-based mechanism mentioned above may be needed can refer to the “NEF Assisted In-flight Monitoring” feature specified in Rel-19 by SA2 based on the 3GPP-defined signalling solution in TS 23.256, for which SA2 already foresaw the NG-RAN to be required to report flight status information to the CN/AMF which, in turn, will report such information to the UAV monitoring platform via the NEF. The general assumption for the 3GPP-defined signalling solution is that the NG-RAN relies on the existing mobility measurements in RRC to trigger a UE report when the altitude has reached a certain threshold, i.e., events H1 and H2 as specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [6].

In RAN3#127 meeting it was agreed to reuse the existing Location Report Control and Location Report procedures defined in TS 38.413 [8] to configure and perform the UAV altitude reporting, respectively – meaning that it is feasible from RAN3 perspective that the NG-RAN can provide the UAV UE altitude information to CN upon configuration – and that the UAV altitude reporting can be configured to be periodic, threshold-based or event-based based on NG-RAN implementation [7]:

The existing Location Report Control and Location Report procedures can be reused to configure and perform altitude reporting. Three report types i.e., periodical, threshold-based, event-based, can be supported to report altitude information based on NG-RAN node implementation.

The agreements above have been captured in a new (reply) LS in [1] to make SA2 aware of the discussion status in RAN3 so to update TS 23.256 accordingly, and to ask for further feedback from SA2 on whether the CN needs to indicate derived altitude thresholds and reporting periodicity for the UAV UE's altitude reporting.

In SA2#168 meeting (April 2025) SA2 discussed the LS from RAN3 in [1] and, based on this LS, SA2 aligned the description in clause 5.16.2 of TS 23.256 to clarify that the existing NGAP’s LOCATION REPORTING CONTROL message sent from AMF to NG-RAN node is used for the UAV altitude reporting configuration. Moreover, for the actual reporting from NG-RAN to AMF described in clause 5.16.3 of TS 23.256, it is clarified that the N2 message used to transfer the UAV altitude information is the existing NGAP’s LOCATION REPORT message.

On top of that, SA2 also agreed to introduce a new clause (5.16.4) in TS 23.256 pertaining a new procedure to instruct a UAV UE to stop and/or update the altitude reporting due to a trigger from NG-RAN and/or a request from the UAV monitoring platform. The main motivation for this is that it is not clear how the UAV UE’s altitude reporting is stopped and/or updated, e.g., when the UAV UE reaches the destination point or when the 5GC needs to change the derived altitude reporting thresholds. Also in this case SA2 specified means to address this scenario via either application layer or 3GPP-defined signalling, and only the latter one may have RAN3 impacts.

As part of this new procedure for instructing a UAV UE to stop/update the altitude reporting, for the 3GPP-defined signalling option, it is stated in TS 23.256 that the existing NGAP’s LOCATION INFORMATION FAILURE INDICATION message is used to provide the AMF with information about the configured altitude reporting. In our understanding, the 5GC would be able to assess whether to stop and/or update the UAV’s altitude reporting based on the content of the LOCATION REPORTING FAILURE INDICATION message. As a consequence of the 5GC decision, the procedure includes a step where the AMF sends a LOCATION REPORTING CONTROL message to the NG-RAN node which, in turn, will trigger the release of the configured altitude reporting for the UAV UE (hence stopping the reporting).

SA2 sent the reply LS in [9] to make RAN3 aware of the SA2 updates to TS 23.256, also asking RAN3 to provide feedback on whether the NG-RAN node can add a timestamp when it receives the UAV UE’s altitude report, indicating when the report has been received from the UE.

In the last meeting RAN3 discussed based on the reply LS from SA2 in [9] and agreed the following [10]:

The timestamp of the altitude report refers to Time Stamp IE. Introduce a new mandatory IE in User Location Information IE for Timestamp. Whether to introduce Aerial UE Reporting Reference ID? Failure indication of UAV UE flight information reporting Option 1: Add a new cause value to indicate the NG-RAN node cannot initiate aerial UE altitude reporting in the existing LOCATION REPORTING FAILURE INDICATION message. Add a new IE in the existing LOCATION REPORT message to indicate that the ongoing altitude reporting is stopped. Option 2: Add a new indication to indicate the NG-RAN node cannot initiate aerial UE altitude reporting and the ongoing altitude reporting is stopped in the existing LOCATION REPORTING FAILURE INDICATION message.

Moreover, a bunch of CRs to TS 38.300, TS 38.410, TS 38.413 and TS 38.423 have been endorsed to reflect the agreements so far. However, it still needs to be discussed and agreed which option to go for – either Option 1 or Option 2 above – so that the NG-RAN can provide the AMF with information needed to assess whether to stop and/or update the UAV’s altitude reporting.

Another reply LS to SA2 was agreed in [11] as well, informing SA2 about the provision of the timestamp information along with the UAV UE’s altitude information.

In this contribution we will discuss the open issues from the last RAN3 meeting.

## 2.2 On the Aerial UE Reporting Reference ID

In the last meeting there was a discussion on whether the Aerial UE Reporting Reference ID IE in TS 38.413 needs to be included in the Aerial UE Flight Information Reporting Control IE to identify each location report procedure for aerial UE, as described in the SoD related to the UAV offline discussion in [12] from RAN3#128 meeting. The Aerial UE Reporting Reference ID IE refers to existing Location Reporting Reference ID IE in TS 38.413 and, by checking TS 38.413, our understanding is that the Location Reporting Reference ID IE is used to identify an area of interest (within a list of areas of interest) where the NG-RAN may be requested to check the UE presence.

In the context of the discussion on the aerial UE altitude reporting, the proposed Aerial UE Reporting Reference ID IE should be used to identify multiple altitude reporting configurations provided by the AMF/5GC, but from the latest TS 23.256, it seems that the scenario where the AMF/5GC instructs the NG-RAN for altitude reporting by providing multiple altitude reporting configurations (with each configuration containing minimum and maximum altitude thresholds, as well as reporting periodicity) for the same UAV UE is not addressed.

Observation 1: The scenario where the AMF/5GC instructs the NG-RAN for the UAV UE altitude reporting by providing multiple altitude reporting configurations (with each configuration containing minimum and maximum altitude thresholds, as well as reporting periodicity) is not addressed in TS 23.256.

Moreover, after checking with our SA2 delegates – as per the conclusion within the SoD in [12] – SA2 has no requirement to cover the scenario of multiple altitude reporting configurations for the same UAV UE.

Observation 2: SA2 has no requirements to address the scenario of multiple altitude reporting configurations for the same UAV UE.

Based on the above observations we think that the Aerial UE Reporting Reference ID IE in TS 38.413 does not need to be introduced.

Proposal 1: RAN3 to agree that the Aerial UE Reporting Reference ID IE does not need to be introduced in TS 38.413.

## 2.3 On the failure indication of UAV UE flight information reporting

In the latest version of TS 23.256 [5] a new clause pertaining a procedure to instruct a UAV UE to stop and/or update the altitude reporting due to a trigger from NG-RAN and/or a request from the UAV monitoring platform was introduced. Such procedure can be via either application layer or 3GPP-defined signalling, and only the latter one may have RAN3 impacts.

To be specific, for the 3GPP-defined signalling option, it is stated in TS 23.256 that the existing NGAP’s LOCATION INFORMATION FAILURE INDICATION message is used to provide the AMF with information about the configured altitude reporting. In our understanding, the 5GC would be able to assess whether to stop and/or update the UAV’s altitude reporting based on the content of the LOCATION REPORTING FAILURE INDICATION. As a consequence of the 5GC decision, the procedure includes a step where the AMF sends a LOCATION REPORTING CONTROL message to the NG-RAN node which, in turn, will trigger the release of the configured altitude reporting for the UAV UE (hence stopping the reporting) – this is reflected in step 2b. and step 7b-8b. of clause 5.16.4 Stop and update altitude reporting for aerial UEs in TS 23.256, respectively:

### 5.16.4 Stop and update altitude reporting for aerial UEs Procedure shown in Figure 5.16.4-1 specifies how the 5GS instructs the aerial UEs to stop and/or update their altitude reporting due to a trigger from NG-RAN and/or a request from the USS. Figure 5.16.4-1: Procedure for instructing aerial UEs to stop/update the altitude reporting 0. The UAV UE is requested to perform altitude information reporting as specified in clause 5.16.2, and the UAV UE performs its altitude information reporting as specified in clause 5.16.3. 1. If the UAV UE fails to report the altitude information and/or the NG-RAN to receive the altitude information, the UAS NF/NEF or the USS may decide to stop/update the UAV UE's altitude reporting. NOTE 1: The UAS NF/ NEF or the serving USS can also decide to stop UAV UE's altitude reporting in cases when the UAV UE deviates from the assigned flight trajectory, which can be detected during the in-flight monitoring (clause 5.12.3) or the USS changeover (clause 5.13.2), or when the DAA procedure is triggered to avoid a collision (clause 5.14). *** skip non-relevant text *** Option B: 3GPP-defined signalling. 2b. The NG-RAN node sends an N2 message(LOCATION REPORTING FAILURE INDICATION message, see clause 8.12.2.2 of TS 38.413 [24]) to the AMF with information about the configured altitude information reporting. *** skip non-relevant text *** 7b-8b. The AMF sends the corresponding N2 message (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to the NG-RAN node and the NG-RAN triggers the release of the configured altitude reporting for the UAV UE, thereby stop the altitude reporting, as specified in TS 38.331 [23]. NOTE 3: Aspects related to AMF informing NG-RAN regarding start/stop of UAV UE's altitude reporting are specified in TS 38.413 [24].

In step 2b. of the procedure above, the NGAP’s LOCATION REPORTING FAILURE INDICATION message is mentioned. Such message is defined in TS 38.413 [8] as part of the Location Reporting Failure Indication procedure, whose purpose is to allow the NG-RAN node to inform the AMF of a failed location reporting request contained in the Location Reporting Control procedure (among others) – see highlighted text in yellow below:

### 8.12.2 Location Reporting Failure Indication #### 8.12.2.1 General The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the location reporting request contained in the Location Reporting Control procedure, the Handover Resource Allocation procedure or the Initial Context Setup procedure has failed. The procedure uses UE-associated signalling. *** skip non-relevant text ***

In the last RAN3 meeting there was a discussion on whether the SA2 decision to rely on the Location Reporting Failure Indication procedure needs to be revised. In fact, it is our understanding that the intention in SA2 is to address the scenario where the NG-RAN triggers the 5GC to stop/update an ongoing measurement (UAV UE’s altitude) reporting via an appropriate triggering condition included in the LOCATION REPORTING FAILURE INDICATION message. But, according to the current NGAP text excerpted above, the Location Reporting Failure Indication procedure is initiated by the NG-RAN node when a location reporting request fails, meaning that there cannot be an ongoing measurement reporting prior to the sending of the LOCATION REPORTING FAILURE INDICATION message. However, other companies in RAN3 had a different understanding on the scenario to address, that is, not only to stop/update an ongoing measurement reporting, but also to indicate a failure in the initiation of a measurement reporting. This led to the open issue from the last meeting as follows [10]:

Failure indication of UAV UE flight information reporting Option 1: Add a new cause value to indicate the NG-RAN node cannot initiate aerial UE altitude reporting in the existing LOCATION REPORTING FAILURE INDICATION message. Add a new IE in the existing LOCATION REPORT message to indicate that the ongoing altitude reporting is stopped. Option 2: Add a new indication to indicate the NG-RAN node cannot initiate aerial UE altitude reporting and the ongoing altitude reporting is stopped in the existing LOCATION REPORTING FAILURE INDICATION message.

As can be seen above, two options need to be evaluated. In Option 1, the failure indication is included in both the LOCATION REPORTING FAILURE INDICATION message and the LOCATION REPORT message to cover the cases of failure in the initiation of a measurement reporting and failure of an ongoing measurement reporting, respectively. While, in Option 2, the failure indication is included in the LOCATION REPORTING FAILURE INDICATION message to address both cases.

It is worth noting to mention that the options above listed have both been captured in the NGAP CR in [13] endorsed in the last meeting, but with reversed definitions, i.e., Option 1 (Option 2) in the NGAP CR is actually Option 2 (Option 1) in the Chair’s meeting minutes from RAN3#128 [10].

Observation 3: There is a mismatch between the definitions of Option 1 and Option 2, as captured in the RAN3#128 Chair’s minutes for the handling of the UAV UE altitude reporting failure scenario, and the way these options have been implemented in the NGAP CR in R3-255135 endorsed at RAN3#128.

In the following of this paper, as well as in our revised version of the NGAP CR in [14], we will refer to Option 1 and Option 2 as defined in the Chair’s meeting minutes from RAN3#128 [10].

In our view, Option 1 should be selected because it allows to keep the purpose of existing NGAP procedures (Location Reporting Failure Indication and Location Report) unchanged. That is, the Location Reporting Failure Indication procedure should be enhanced to indicate a failure in the initiation of a UAV UE altitude measurement reporting, while the Location Report procedure should be enhanced to indicate a failure of an ongoing UAV UE altitude measurement reporting.

Observation 4: The purpose of existing NGAP procedures (Location Reporting Failure Indication and Location Report) should be kept unchanged.

According to TS 38.413 [8], the Location Reporting Failure Indication procedure allows the NG-RAN node to inform the AMF of a failed location reporting request contained not only in the Location Reporting Control procedure, but also in the Handover Resource Allocation procedure or the Initial Context Setup procedure:

### 8.12.2 Location Reporting Failure Indication #### 8.12.2.1 General The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the location reporting request contained in the Location Reporting Control procedure, the Handover Resource Allocation procedure or the Initial Context Setup procedure has failed. The procedure uses UE-associated signalling. *** skip non-relevant text ***

While a failure indication following a Location Reporting Control procedure clearly addresses the case of failure in the initiation of a measurement reporting, a failure indication following the NGAP’s Handover Resource Allocation or Initial Context Setup procedures seems to indicate a failure of an ongoing measurement reporting.

Take the handover as an example: let’s assume that the UAV UE is handed over from source gNB to target gNB via a NG-based handover, and the UAV UE was performing altitude reporting while being served by the source gNB (which previously configured the UAV UE to do so, based on 5GC/AMF configuration parameters); as per clause 8.4.2.1 in NGAP, the AMF sends the HANDOVER REQUEST to the target gNB which includes a Location Reporting Request to perform altitude reporting for the handed over UAV UE, according to the same configuration (altitude thresholds and reporting periodicity) as provided by the source gNB; after the UAV UE successfully accessed the target cell, the target gNB re-configures the UAV UE for altitude reporting following the same procedure as when it was served by the source gNB. In our understanding this means that a new UAV UE altitude information reporting session is initiated at the target gNB, which is independent from the previous reporting session that was ongoing at the source gNB side.

Therefore, in our view the failure scenarios other than the one following the Location Reporting Control procedure still refer to a failure of the measurement initiation and not of an ongoing measurement session, and hence it makes sense to rely on the NGAP Location Reporting Failure Indication procedure.

Observation 5: Scenarios where the location reporting request contained in the Handover Resource Allocation or Initial Context Setup procedures fails still refer to a failure of the measurement initiation.

Based in the above observations, we propose the following:

Proposal 2: RAN3 to agree to select Option 1

- Add a new cause value to indicate the NG-RAN node cannot initiate aerial UE altitude reporting in the existing LOCATION REPORTING FAILURE INDICATION message, and

- Add a new IE in the existing LOCATION REPORT message to indicate that the ongoing altitude reporting is stopped.

A NGAP CR reflecting the above Proposal 2 (built on top of the endorsed NGAP CR from RAN3#128 in [13]) can be found in [14].

A draft reply LS to SA2 in [15] is proposed to be agreed to include an update of the RAN3 work concerning the UAV UE altitude reporting and ask to update TS 23.256 accordingly to address RAN3 decisions on the failure indication of UAV UE altitude reporting.

Proposal 3: RAN3 to agree to send a LS to SA2 to provide an update of the RAN3 work concerning the UAV UE altitude reporting and ask to update TS 23.256 accordingly to address RAN3 decisions on the failure indication of UAV UE altitude reporting.

# 3	Conclusion

This document proposes the following observation and proposals:

On the Aerial UE Reporting Reference ID

Observation 1: The scenario where the AMF/5GC instructs the NG-RAN for the UAV UE altitude reporting by providing multiple altitude reporting configurations (with each configuration containing minimum and maximum altitude thresholds, as well as reporting periodicity) is not addressed in TS 23.256.

Observation 2: SA2 has no requirements to address the scenario of multiple altitude reporting configurations for the same UAV UE.

Proposal 1: RAN3 to agree that the Aerial UE Reporting Reference ID IE does not need to be introduced in TS 38.413.

On the failure indication of UAV UE flight information reporting

Observation 3: There is a mismatch between the definitions of Option 1 and Option 2, as captured in the RAN3#128 Chair’s minutes for the handling of the UAV UE altitude reporting failure scenario, and the way these options have been implemented in the NGAP CR in R3-255135 endorsed at RAN3#128.

Observation 4: The purpose of existing NGAP procedures (Location Reporting Failure Indication and Location Report) should be kept unchanged.

Observation 5: Scenarios where the location reporting request contained in the Handover Resource Allocation or Initial Context Setup procedures fails still refer to a failure of the measurement initiation.

Proposal 2: RAN3 to agree to select Option 1

- Add a new cause value to indicate the NG-RAN node cannot initiate aerial UE altitude reporting in the existing LOCATION REPORTING FAILURE INDICATION message, and

- Add a new IE in the existing LOCATION REPORT message to indicate that the ongoing altitude reporting is stopped.

Proposal 3: RAN3 to agree to send a LS to SA2 to provide an update of the RAN3 work concerning the UAV UE altitude reporting and ask to update TS 23.256 accordingly to address RAN3 decisions on the failure indication of UAV UE altitude reporting.

A NGAP CR reflecting the above Proposal 2 is in [14].

A draft reply LS to SA2 is in [15].

# 4	References

- R3-250876, LS on UAV regulation from RAN3 (Response to R3-250022/S2-2413011), Rel-19, RAN3 (CMCC)

- R3-250022 (S2-2413011), Reply LS on UAV regulation, Samsung

- R3-245815, LS on UAV regulation, CMCC

- SP-230518, Uncrewed Aerial System Phase 3, Rel-19

- 3GPP TS 23.256 “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”, Rel-19, V19.3.0

- 3GPP TS 38.331 “NR; Radio Resource Control (RRC); Protocol specification”, Rel-18

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM.doc)

- 3GPP TS 38.413 “NG-RAN; NG Application Protocol (NGAP)”, Rel-18

- R3-253020 (S2-2504471), Reply LS on clarification of UAV regulation from RAN3 (Response to R3-250876), Rel-19, SA2 (CMCC)

- 3GPP RAN3#128 Chairman’s notes (RAN3_128_agenda_20250523_1530_EOM.doc)

- R3-253866, Reply LS on clarification of UAV regulation from SA2 (Response to R3-253020/S2-2504471), Rel-19, RAN3 (CMCC)

- R3-253756, Sod for CB: # 4_UAV, CMCC

- R3-255135, Support Aerial UE Flight Information Reporting to CN, Rel-19 NGAP CR, Ericsson, CMCC, ZTE, CATT, Nokia, Huawei

- R3-255353, Support Aerial UE Flight Information Reporting to CN, Rel-19 NGAP CR, Huawei, Nokia

- R3-255191, [DRAFT] Reply LS on clarification of UAV regulation from SA2, Rel-19, Nokia, Huawei