3GPP TSG-RAN WG3 Meeting #129	R3-255356

Bengaluru, India, 25th – 29th August, 2025

Title: 	(TP for AI/ML BLCR to TS 37.340) Capturing agreements in Stage 2 RAN specifications

Source: 	Huawei, Jio Platforms, Ofinno, Qualcomm, InterDigital, Deutsche 	Telekom

Agenda item:	11.2

Document Type:	Discussion and decision

# 1. Introduction

RAN3 is discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

In the last meeting RAN3 agreed a Stage 2 TP for TS 37.340 in [7], which became the BLCR for that specification, introducing a new section for AI/ML. This contribution provides a TP for the AI/ML BLCR to TS 37.340 in [8] capturing RAN3 agreements which are relevant for Stage 2 description of Rel-19 AI/ML for NG-RAN.

# 2. Discussion

## 2.1 RAN3 agreements relevant for the AI/ML BLCR to TS 37.340

***** RAN3#125bis ***** [4] Rel-18 leftovers: Mobility Optimization for NR-DC As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance. ***** RAN3#126 ***** [5] Rel-18 leftovers: Mobility Optimization for NR-DC Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side. Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context. ***** RAN3#127bis ***** [6] Rel-18 leftovers: Mobility Optimization for NR-DC The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked. The following list of exit conditions to stop the collection of UE performance at SN side: - the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE; - the NR-DC with the S-NG-RAN node for the UE is released.

It is proposed to agree the TP to AI/ML BLCR to TS 37.340 in Annex 1.

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 37.340 in Annex 1.

# 3. Conclusion

We make the following proposal:

Proposal: RAN3 to agree the TP to AI/ML BLCR to TS 37.340 in Annex 1.

# 4. Reference

- RP-250326, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#107

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_EOM1.doc)

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- R3-253954, TP to BLCR 37.340 AIML for NR-DC, Lenovo, CATT, ZTE Corporation, Samsung, Ericsson, Nokia, Huawei, Ofinno, FiberCop, LG Electronics

- R3-255047, (BL CR to 37.340) AIML for NR-DC, Huawei, Lenovo, CATT, ZTE Corporation, Samsung, Ericsson, Nokia, Ofinno, FiberCop, LG Electronics, Jio Platforms

# 5. Annex 1 – TP for AI/ML BLCR to TS 37.340 (based on 	R3-255047)

<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1], TS 36.300 [2] and TS 38.300 [3].

AI	Artificial Intelligence

BFD	Beam Failure Detection

CHO	Conditional Handover

CLI	Cross Link Interference

CPA	Conditional PSCell Addition

CPAC	Conditional PSCell Addition or Change

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DC	Intra-E-UTRA Dual Connectivity

DCP	DCI with CRC scrambled by PS-RNTI

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

IDC	In-Device Coexistence

LTM	L1/L2 Triggered Mobility

MCG	Master Cell Group

ML	Machine Learning

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MUSIM	Multi-Universal Subscriber Identity Module

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR-DC	NR-NR Dual Connectivity

QMC	QoE Measurement Collection

QoE	Quality of Experience

RLM	Radio Link Monitoring

SCG	Secondary Cell Group

SMTC	SS/PBCH block Measurement Timing Configuration

SN	Secondary Node

SPR	Successful PSCell Addition/Change Report

V2X	Vehicle-to-Everything

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>

## 10.2	Secondary Node Addition

### 10.2.1	EN-DC

*** skip unmodified text ***

### 10.2.2	MR-DC with 5GC

The Secondary Node (SN) Addition procedure is initiated by the MN and is used to establish a UE context at the SN in order to provide resources from the SN to the UE. For bearers requiring SCG radio resources, this procedure is used to add at least the initial SCG serving cell of the SCG. This procedure can also be used to configure an SN terminated MCG bearer (where no SCG configuration is needed). In case of CPA, the Conditional Secondary Node Addition procedure can be used for CPA configuration and CPA execution. This procedure can also be used to support coordination between the MN and the SN for managing the configuration and reporting of QoE measurements and/or RAN visible QoE measurements in NR-DC.

Secondary

Node Addition

Figure 10.2.2-1 shows the SN Addition procedure.

Figure 10.2.2-1: SN Addition procedure

1.	The MN decides to request the target SN to allocate resources for one or more specific PDU Sessions/QoS Flows, indicating QoS Flows characteristics (QoS Flow Level QoS parameters, PDU session level TNL address information, and PDU session level Network Slice info). In addition, for bearers requiring SCG radio resources, MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the latest measurement results for SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. In NGEN-DC and NR-DC, the MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision. The MN may request the SCG to be activated or deactivated.

For MN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides Xn-U UL TNL address information. For SN terminated bearers, the MN provides a list of available DRB IDs. The S-NG-RAN node shall store this information and use it when establishing SN terminated bearers. The SN may reject the request.

For SN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides in step 1 a list of QoS flows per PDU Sessions for which SCG resources are requested to be setup upon which the SN decides how to map QoS flows to DRB.

In case of coordination between the MN and the SN on QoE and/or RAN visible QoE measurement configuration and reporting, the SN Addition Request message may contain the QMC Coordination Request IE.

For a MN that supports AI/ML for NG-RAN as specified in TS 38.300 [3], the SN Addition Request message may contain the Data Collection ID IE to configure the collection and reporting of UE performance measured at the SN side as specified in TS 38.423 [5].

NOTE 1:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective QoS Flow is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MN decision is reflected in step 1 by the QoS Flow parameters signalled to the SN, which may differ from QoS Flow parameters received over NG.

NOTE 2:	For a specific QoS flow, the MN may request the direct establishment of SCG and/or split bearers, i.e. without first having to establish MCG bearers. It is also allowed that all QoS flows can be mapped to SN terminated bearers, i.e. there is no QoS flow mapped to an MN terminated bearer.

*** skip unmodified text ***

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>

## 10.5	Secondary Node Change (MN/SN initiated)

### 10.5.1	EN-DC

*** skip unmodified text ***

### 10.5.2	MR-DC with 5GC

MN initiated SN Change

The MN initiated SN change procedure is used to transfer a UE context from the source SN to a target SN and to change the SCG configuration in UE from one SN to another.

The Secondary Node Change procedure always involves signalling over MCG SRB towards the UE.

Figure 10.5.2-1: SN change procedure - MN initiated

Figure 10.5.2-1 shows an example signalling flow for the SN Change initiated by the MN:

1/2.	The MN initiates the SN change by requesting the target SN to allocate resources for the UE by means of the SN Addition procedure. The MN may include measurement results related to the target SN. If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration. For a MN that supports AI/ML for NG-RAN as specified in TS 38.300 [3], the SN Addition Request message may contain the Data Collection ID IE to configure the collection and reporting of UE performance measured at the target SN side as specified in TS 38.423 [5].

NOTE 1:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the 					current SCG configuration and the QMC configuration information managed by the SN, and to allow 					provision of data forwarding related information before step 1.

*** skip unmodified text ***

<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>

## 13.x	AI/ML for NR-DC

In NR-DC, an MN that supports AI/ML for NG-RAN as specified in TS 38.300 [3], can configure the collection and reporting of UE performance measurements from the SN after successful Secondary Node Addition or Secondary Node Change (MN initiated), if supported.

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>