3GPP TSG-RAN WG3 Meeting #129	R3-255357

Bengaluru, India, 25th – 29th August, 2025

Title: 	Further discussion on per-UE per-slice UE performance in AI/ML-based Network Slicing

Source: 	Huawei, Jio Platforms, FiberCop, BT

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split and split architectures) for AI/ML-based Network Slicing.

This contribution analyzes further the technical enablers for the AI/ML-based Network Slicing use case to be introduced over the Xn interface, and provides proposals for discussion based on the outcome of the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting the TP for the XnAP BLCR in [4] was agreed and then reflected in the latest version of the XnAP BLCR for Rel-19 AI/ML for NG-RAN in [5]. The TP introduced a new IE for the slice-based UE performance, rather than relying on the existing UE Performance IE specified in Rel-18:

#### 9.2.3.x4 Slice Based UE Performance This IE represents UE performance metrics per S-NSSAI. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Average Slice UE Throughput DL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.1.. Average Slice UE Throughput UL; O; Bit Rate 9.2.3.4; Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.2.. Average Slice Packet Delay; O; Average Packet Delay 9.2.3.187; Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [XX], clause 6.3.1.1.. Average Slice Packet Loss DL; O; INTEGER (0..10000000, ...) [FFS on the range]; Corresponds to DL PDCP SDU Drop Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.2.1.. Average Slice Packet Loss UL; O; INTEGER (0..10000000, ...) [FFS on the range]; Corresponds to • UL PDCP SDU Loss Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.1.1.. Editor’s note: the semantics description of Average Slice Packet Loss to be further checked based on potential LS from SA5.

From the above it can be noticed take, differently from the Average Slice UE Throughput DL/UL and Average Slice Packet Delay DL/UL, the Average Slice Packet Loss DL/UL has some open points that need to be addressed further.

For the Average Slice UE Throughput DL/UL, instead, the semantics description is aligned with the Rel-18 XnAP CR in [6] agreed in the last meeting and reflected in the latest version of TS 38.423 [7], where references to existing measurements in SA5’s TS 28.558 (clauses 6.3.1.4.1 and 6.3.1.4.2) are added by considering that such measurements can be provided per-UE (as explained by SA5 in [8]) and per supported S-NSSAI, i.e., per-slice [9]:

#### 6.3.1.4 UE throughput ##### 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. *** skip unmodified text *** ##### 6.3.1.4.2 Average UL UE throughput in gNB a) This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M5 in TS 37.320[9].

Observation 1: The semantics description of the Average Slice UE Throughput DL/UL refers to existing measurements in TS 28.558 (clauses 6.3.1.4.1 and 6.3.1.4.2) which are defined per-UE and that could also be provided per supported S-NSSAI (per-slice).

Similar conclusion was also drawn by RAN3 for the Average Slice Packet Delay DL/UL, where the packet delay measurement components listed in TS 38.314 [10] (clause 4.2.1.2.1) and referenced in TS 28.558 [9] (clause 6.3.1.1) are measured with a per-DRB per-UE granularity, as well as per supported S-NSSAI, i.e., per-slice:

--- Excerpt from TS 38.314 --- #### 4.2.1.2 Packet delay ##### 4.2.1.2.1 General Packet delay includes RAN part of delay and CN part of delay. The RAN part of DL packet delay measurement comprises: - D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1. - D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3. - D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2. - D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1. The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE. *** skip non-relevant text ***

--- Excerpt from TS 28.558 --- #### 6.3.1.1 Packet delay ##### 6.3.1.1.1 Average delay DL air-interface a) This measurement provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction. The measurement is calculated per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as D1 (see TS 38.314 [8]) as part of DL M6 measurement (see TS 37.320 [9]). *** skip non-relevant text ***

Similar statements as excerpted above from TS 28.558 are also included in clauses 6.3.1.1.2, 6.3.1.1.2, 6.3.1.1.3, and 6.3.1.1.4 for the other DL packet delay components.

--- Excerpt from TS 38.314 --- #### 4.2.1.2 Packet delay ##### 4.2.1.2.1 General ##### *** skip non-relevant text *** The RAN part (including UE) of UL packet delay measurement comprises: - D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1). - D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2). - D2.2 (average RLC packet delay, as defined in 4.2.1.2.3). - D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2). - D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4). The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms. *** skip non-relevant text ***

--- Excerpt from TS 28.558 --- ##### 6.3.1.1.5 UL PDCP packet average delay a) This measurement provides the average (arithmetic mean) UL PDCP packet average delay. The measurement is calculated per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as D1 (see TS 38.314 [8]) as part of UL M6 measurement (see TS 37.320 [9]). *** skip unmodified text ***

Similar statements as excerpted above from TS 28.558 are also included in clauses 6.3.1.1.6, 6.3.1.1.7, and 6.3.1.1.8 for the other UL packet delay components.

Hence, based on the above-mentioned clauses in TS 28.558, the DL/UL packet delay components could be defined per-DRB per-UE per-slice, but not per-UE per-slice as per the granularity agreed in RAN3 for the AI/ML-based Network Slicing use case. The decision in RAN3 from the last meeting to introduce a reference to TS 28.558 in the semantics description of the Average Slice Packet Delay IE is based on the underlying assumption that the packet delay measurement components listed in TS 38.314 which are measured with a per-DRB per-UE granularity can be aggregated in such a way to derive corresponding per-UE metrics. This issue already exists in Rel-18 UE performance, that is the reason why RAN3 agreed in the last meeting to send a reply LS to SA5 in [11] to ask clarifications about whether per-DRB per-UE aggregation of packet delay measurements is already supported or needs to be specified (if feasible):

1. Overall Description: RAN3 thanks SA5 for their reply LS on per-UE UE performance metrics. RAN3 would further like to ask for clarifications about the granularity of Average Packet Delay UL/DL and Average Packet Loss UL/DL. RAN3 assumes that UE level performance is derived from DRB level measurements of the concerned UE. Average packet delay measurements: RAN3 has noticed that Average Packet Delay UL/DL measurements comprise packet delay measurement components listed in TS 38.314 clause 4.2.1.2.1 which are measured with a per-DRB per-UE granularity and referenced to corresponding measurements’ definitions in TS 28.558 clause 6.3.1.1. RAN3 assumes that the UL and DL Average Packet Delay per-UE measurements are derived from per-DRB measurements across all configured DRBs for the concerned UE. RAN3 would like to ask SA5 whether the measurements are aggregated per-DRB per-UE to reflect corresponding per-UE metrics. If such per UE aggregation is not supported and if such UE aggregation is feasible, RAN3 would like to ask SA5 to specify these measurements. *** skip non-relevant text *** If the above is feasible then RAN3 would like to ask SA5 to carry out specification work accordingly. *** skip non-relevant text ***

If the above aggregation issue (i.e., how to derive per-UE metrics from per-DRB measurements of the concerned UE) is clarified by SA5, then it should be straightforward to follow to same approach as adopted for the Average Slice UE Throughput DL/UL to be made available with per-UE per-slice granularity.

Observation 2: The semantics description of the Average Slice Packet Delay refers to existing measurements in TS 28.558 (clause 6.3.1.1 and associated sub-clauses) which are defined per-DRB per-UE and that could also be provided per supported S-NSSAI (per-slice).

Observation 3: RAN3 assumes that per-DRB per-UE packet delay measurements in TS 28.558 can be aggregated to derive corresponding per-UE metrics. SA5 clarifications are expected to confirm (or not) such assumption.

Observation 4: If SA5 confirms that the aggregation of per-DRB per-UE packet delay measurements is supported, then corresponding per-UE per-slice measurements can be correctly referenced from TS 28.558 as already agreed for the Average Slice UE Throughput DL/UL.

The Average Slice Packet Loss DL/UL, instead, has two different aspects to be discussed jointly:

- the semantics description of the corresponding IEs in the Slice based UE Performance IE, and

- the range of both Average Slice Packet Loss DL and Average Slice Packet Loss UL integer IEs IE.

On the semantics description of the Average Slice Packet Loss DL and Average Slice Packet Loss UL integer IEs

As can be noticed from the new Slice Based UE Performance IE in the XnAP BLCR for the Rel-19 RAN AI/ML, the semantics description of the Average Slice Packet Loss DL and Average Slice Packet Loss UL IEs refers to corresponding PDCP-level measurements defined by SA5 in TS 28.552 [12] clauses 5.1.3.2.1 and 5.1.3.1.1, respectively. Such decision originates from the feedback provided by SA5 in the LS in [13] – which is the reply LS to the LS RAN3 sent in [14] pertaining the feasibility of specifying PDCP-level UE performance metrics:

# 1 Overall description SA5 would like to thank RAN3 for their LS in R3-252492 on clarification on UE performance measurement at PDCP level. SA5 has discussed the LS from RAN3 and would like to provide the following clarifications: *** skip non-relevant text *** Packet Loss Rate at PDCP Level (UL and DL) SA5 recognises the importance of monitoring packet loss at the PDCP level in both UL and DL directions. The following measurements are already defined: - UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1 - DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1 These metrics allow packet loss monitoring in UL and packet drop monitoring in DL for PDCP-layer traffic. *** skip non-relevant text ***

Referencing to TS 28.552 for the PDCP-level measurements as suggested by SA5 to reflect DL/UL Average Slice Packet Loss metrics is not appropriate because:

- TS 28.552 does not list UE level measurements (as TS 28.558 does) but it encompasses NG-RAN level performance measurements, with a per-DRB granularity and, optionally, per-DRB per supported S-NSSAI (i.e., per-DRB per-slice);

- the PDCP-level measurements mentioned in the SA5 reply LS are valid only for the split gNB-scenario. Hence they are not applicable in case of non-split gNB.

### 5.1.3 Performance measurements valid for split gNB deployment scenario #### 5.1.3.1 Packet Loss Rate ##### 5.1.3.1.1 UL PDCP SDU Loss Rate a) This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI. *** skip non-relevant text *** e) The measurement name has the form DRB.PacketLossRateUl and optionally DRB.PacketLossRateUl.QOS where QOS identifies the target quality of service class, and DRB.PacketLossRateUl.SNSSAI where SNSSAI identifies the S-NSSAI. *** skip non-relevant text *** ##### 5.1.3.2.1 DL PDCP SDU Drop rate in gNB-CU-UP a) This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI. *** skip non-relevant text *** e) The measurement name has the form DRB.PdcpPacketDropRateDl and optionally DRB.PdcpPacketDropRateDl.QOS where QOS identifies the target quality of service class, and DRB.PdcpPacketDropRateDl.SNSSAI where SNSSAI identifies the S-NSSAI. *** skip non-relevant text ***

Observation 5: Referencing to TS 28.552 for the PDCP-level measurements to reflect DL/UL Average Slice Packet Loss metrics is not appropriate because TS 28.552 does not specify UE level measurements. Moreover, the PDCP-level measurements suggested by SA5 are valid only for the split gNB-scenario.

Since RAN3 is investigating whether PDCP-level measurements for DL/UL Average Packet Loss (as well as DL/UL Average Packet Delay) could also be specified by SA5 on a per-DRB per-UE basis (hence being potentially included in TS 28.558) and being applicable to Rel-18 RAN AI/ML use cases, RAN3 agreed in the last meeting to include a question in the reply LS to SA5 in [11] to ask whether packet loss measurements could be specified with a per-DRB per-UE granularity in the UL and DL at PDCP layer, that could also be aggregated to reflect a per-UE metric:

1. Overall Description: RAN3 thanks SA5 for their reply LS on per-UE UE performance metrics. RAN3 would further like to ask for clarifications about the granularity of Average Packet Delay UL/DL and Average Packet Loss UL/DL. RAN3 assumes that UE level performance is derived from DRB level measurements of the concerned UE. *** skip non-relevant text *** Average packet loss measurements: In RAN3#127-bis, RAN3 sent an LS to SA5 (R3-252436) to ask SA5 if it is feasible to measure Packet Loss Rate in UL and DL at PDCP level for both split and non-split architecture and, if feasible, RAN3 asked SA5 to specify these measurements for both split and non-split architecture. In the LS from SA5 R3-253824, SA5 has confirmed that the following metrics can be used for UE Packet Loss: - UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1 - DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1 RAN3 would like to further ask whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity in the UL and DL at PDCP layer, which may also be aggregated to reflect a per-UE Packet Loss Rate metric. If the above is feasible then RAN3 would like to ask SA5 to carry out specification work accordingly. *** skip non-relevant text ***

Based on the above we think that, for solving the open point on the semantics description of the DL/UL Average Slice Packet Loss IEs, the SA5 input is essential.

Proposal 1: To remove the Editor’s Note on the semantics description of the DL/UL Average Slice Packet Loss, RAN3 to wait for the reply LS from SA5 on the feasibility of specifying PDCP-level packet loss metrics in DL and UL with a per-DRB per-UE granularity (and optionally per-DRB per-UE per-slice), that could be aggregated to reflect a per-UE (and optionally per-UE per-slice) metric.

Considering that this is the last meeting for the Rel-19 RAN AI/ML WI, in case SA5 would not be able to send a reply LS during the meeting week, RAN3 should also discuss whether considering RLC-level packet loss currently specified in TS 28.558 for the DL only (clause 6.3.1.2.1) with a per-DRB per-UE granularity (and optionally also per supported S-NSSAI) is acceptable in order to conclude the WI on time.

Proposal 2: RAN3 to discuss whether considering the RLC-level packet loss currently specified in TS 28.558 for the DL only is acceptable in order to conclude the WI on time.

On the range of the Average Slice Packet Loss DL and Average Slice Packet Loss UL integer IEs

In the latest version of the XnAP BLCR for Rel-19 RAN AI/ML [4] the DL/UL Average Slice Packet Loss metrics are encoded as integers within a range from 0 to 107, but the actual range is not agreed yet and needs further discussion.

This tentative encoding originates from the contribution in [15], where the shortcomings of the Rel-18 encoding of the Average Packet Loss DL IE in XnAP are detailed, and by considering the RAN3 agreement from RAN3#127bis [16]:

A Rel18 correction to the Average Packet Loss DL IE may be made when agreements on whether and how to specify Average Packet Loss in Rel19 are taken.

We think that the encoding currently captured in the XnAP BLCR needs to be completed by also clarifying the granularity associated to the signalled packet loss values, so to derive the actual packet loss experienced by the concerned UE after an AI/ML-driven mobility event (i.e., handover). Still in [15], it is proposed to adopt a granularity of 10-8, and for us this makes sense in order to provide accurate and reliable packet loss measurements reflecting the actual measured packet loss.

Proposal 3: RAN3 to agree to remove the FFS on the range associated to the “IE type and reference” of the Average Slice Packet Loss DL/UL, and to indicate the measurement granularity of 10-8 in the semantics description.

A TP for the XnAP BLCR reflecting the above proposals is provided in [17], assuming to re-use the RLC-level packet loss currently specified in TS 28.558 for the DL only.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Observation 1: The semantics description of the Average Slice UE Throughput DL/UL refers to existing measurements in TS 28.558 (clauses 6.3.1.4.1 and 6.3.1.4.2) which are defined per-UE and that could also be provided per supported S-NSSAI (per-slice).

Observation 2: The semantics description of the Average Slice Packet Delay refers to existing measurements in TS 28.558 (clause 6.3.1.1 and associated sub-clauses) which are defined per-DRB per-UE and that could also be provided per supported S-NSSAI (per-slice).

Observation 3: RAN3 assumes that per-DRB per-UE packet delay measurements in TS 28.558 can be aggregated to derive corresponding per-UE metrics. SA5 clarifications are expected to confirm (or not) such assumption.

Observation 4: If SA5 confirms that the aggregation of per-DRB per-UE packet delay measurements is supported, then corresponding per-UE per-slice measurements can be correctly referenced from TS 28.558 as already agreed for the Average Slice UE Throughput DL/UL.

Observation 5: Referencing to TS 28.552 for the PDCP-level measurements to reflect DL/UL Average Slice Packet Loss metrics is not appropriate because TS 28.552 does not specify UE level measurements. Moreover, the PDCP-level measurements suggested by SA5 are valid only for the split gNB-scenario.

Proposal 1: To remove the Editor’s Note on the semantics description of the DL/UL Average Slice Packet Loss, RAN3 to wait for the reply LS from SA5 on the feasibility of specifying PDCP-level packet loss metrics in DL and UL with a per-DRB per-UE granularity (and optionally per-DRB per-UE per-slice), that could be aggregated to reflect a per-UE (and optionally per-UE per-slice) metric.

Proposal 2: RAN3 to discuss whether considering the RLC-level packet loss currently specified in TS 28.558 for the DL only is acceptable in order to conclude the WI on time.

Proposal 3: RAN3 to agree to remove the FFS on the range associated to the “IE type and reference” of the Average Slice Packet Loss DL/UL, and to indicate the measurement granularity of 10-8 in the semantics description.

A TP for the XnAP BLCR reflecting the above proposals is provided in [17], assuming to re-use the RLC-level packet loss currently specified in TS 28.558 for the DL only.

# 4. Reference

- RP-251245, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#108

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- R3-253952, (TP for BLCR to 38.423) Support of AI/ML assisted Network Slicing, ZTE Corporation, Qualcomm, China Unicom, LG Electronics Inc., Ericsson, CATT, NEC, Ofinno

- R3-253986, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno

- R3-253814, Correction on UE performance feedback, Rel-18 Cat. F CR# 1495 to TS 38.423, Huawei, Lenovo, Deutsche Telekom, BT, Ericsson, ZTE, CATT, FiberCop, NEC

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), Rel-18, V18.6.0

- R3-253023 (S5-252056), Reply LS on per-UE UE performance metrics (Response to R3-247775), Rel-19, SA5 (NEC, Intel, Ericsson)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, Rel-19, V19.3.0

- 3GPP TS 38.314, NR; Layer 2 measurements, Rel-18, V18.0.0

- R3-253816, Reply LS on per-UE UE performance metrics (Response to R3-253023 / S5-252056), Rel-18, RAN3 (Nokia)

- 3GPP TS 28.552, Management and orchestration; 5G performance measurements, Rel-19, V19.3.0

- R3-253824 (S5-252839), Reply LS on clarification on UE performance measurement at PDCP level (Response to R3-252492), Rel-19, SA5 (NEC, Intel, Ericsson)

- R3-252492, LS on clarification on UE performance measurement at PDCP level, Rel-19, RAN3 (ZTE Corporation)

- R3-253431, On packet loss metric in UE performance, Ericsson, Jio Platforms, Deutsche Telekom, InterDigital

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- R3-255358, (TP for AI/ML BLCR to TS 38.423) Per-UE per-slice UE performance in AI/ML-based Network Slicing, Huawei, Ofinno, Jio Platforms, FiberCop, BT