3GPP TSG-RAN WG3 Meeting #129	R3-255367

Bengaluru, India, 25th – 29th August, 2025

Title: 	(TP for AI/ML BLCR to TS 38.423) Discussion on UE performance feedback for AI/ML-based Mobility Optimization in NR-DC

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 kept on discussing the objectives of the Rel-19 WI “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to the mobility optimization for NR-DC.

This contribution analyzes the technical enablers for specifying support over the Xn interface for mobility optimization in NR-DC, and provides proposals for discussion based on the outcome of previous RAN3 meetings.

# 2. Discussion

RAN3 did not reach further agreements in the last meeting concerning the mobility optimization in NR-DC, and only agreed the Stage 2 TP for the TS 37.340 BLCR in [4] (which became the new BLCR in [5]) to introduce support for AI/ML in NR-DC. However, there are some open issues from the RAN3#127bis meeting which are reported below along with the agreements achieved back then [6]:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked. The following list of exit conditions to stop the collection of UE performance at SN side: - the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE; - the NR-DC with the S-NG-RAN node for the UE is released. UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture? Whether to introduce assistance information (predicted PSCell ID) from MN to SN? Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID? Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In the next sections we discuss the open issues listed above related to UE performance.

## 2.1 On UE performance metrics in NR-DC

The UE performance metrics to be reported as part of the feedback information after an AI/ML-based action as well as the UE performance collection configuration have been specified in Rel-18 for XnAP [7] clause 9.2.3.179 and clause 9.2.3.186, respectively. Still in Rel-18, UE performance is only for the basic single connectivity case, where the collection and reporting is done at the target node side towards the source node on a per-UE granularity. For the case of NR-DC, RAN3 needs to investigate which UE performance metrics are triggered to be collected by the SN after successful SN Addition / Change (MN-initiated) for the concerned UE as a function of the bearer type. Even though most of the metrics in TS 28.558 [8] are specified to be measured at RLC (i.e., in the gNB-DU), in RAN3#127bis meeting and as part of the discussion on split architecture support for Rel-18 use cases RAN3 eventually considered the “gNB-CU-UP-centric” approach, based on which UE performance metrics are measured (and aggregated, for the case of packet delay) at PDCP level (and hence in the gNB-CU-UP) and reported to the gNB-CU-CP over the E1 interface. It was however not clear yet whether PDCP-level measurements for Average UE Throughput UL/DL and Average Packet Loss DL/UL can be made available in TS 28.558, therefore RAN3 agreed to send a LS to SA5 in [9] to check whether it is feasible to define PDCP-level measurements for throughput and packet loss.

In the last meeting RAN3 received the corresponding reply LS from SA5 in [10] where, for the throughput measurements in DL and UL, RAN3 concluded that, based on the SA5 feedback, it would be better to refer to existing measurements in TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2 performed at RLC. While, for packet loss, SA5 indicated that measurements in TS 28.552 [11] clauses 5.1.3.1.1 and 5.1.3.2.1 could be referred for measuring packet loss at PDCP in UL and DL directions, respectively. However, we observe the following shortcomings:

- it was highlighted in contribution [12] that, in case of NR-DC with split bearers, measurements performed in the gNB-DU (at RLC level) are per leg and hence they cannot be aggregated to derive per-DRB measurements (even though it is possible to collect the individual MCG and SCG leg level measurements), due to the fact that there is no common RLC SDU point for MCG and SCG legs and neither MN nor SN knows the transmission/buffer status of data in the other leg. The issue may be solved by relying on PDCP-level measurements performed by the node hosting the PDCP entity (either MN or SN), which allow to collect per-DRB information in all NR-DC scenarios regardless of the bearer type (MCG, SCG, or split). But for throughput, according to the outcome of the last meeting where it was agreed to rely on existing RLC-level measurements in TS 28.558, our understanding is that it is not possible for the MN to aggregate throughput measurements reported by the SN with the ones performed by the MN directly, so to derive the overall throughput of the UE configured in NR-DC. Therefore, the MN may request the SN to report the UE performance (including UE throughput) only if there is at least a SN-terminated SCG bearer configured at SN Addition or MN-initiated SN Change;

- referencing to TS 28.552 for the PDCP-level measurements to reflect DL/UL Average Packet Loss metrics is not appropriate because TS 28.552 specifies NG-RAN level performance measurements, and not UE level measurements. Moreover, the PDCP-level measurements suggested by SA5 are valid only for the split gNB-scenario. As a consequence, RAN3 agreed in the last meeting to send a reply LS to SA5 in [13] to ask whether packet loss measurements could be specified with a per-DRB per-UE granularity in the UL and DL at PDCP layer, and that could also be aggregated to reflect a per-UE metric. Considering that this is the last meeting for the Rel-19 RAN AI/ML WI, in case SA5 would not be able to send a reply LS during the meeting week, RAN3 should also discuss whether considering RLC-level packet loss currently specified in TS 28.558 for the DL only (clause 6.3.1.2.1) with a per-DRB per-UE granularity is acceptable in order to conclude the WI on time, taking the limitation above described for throughput that the MN may request the SN to report the UE performance (including DL packet loss) only if there is at least a SN-terminated SCG bearer configured at SN Addition or MN-initiated SN Change.

Observation 1: PDCP-level measurements performed by the node hosting the PDCP entity (either MN or SN) allow to collect per-DRB information in all NR-DC scenarios regardless of the bearer type (MCG, SCG, or split).

Observation 2: Referring to existing RLC-level measurements in TS 28.558 for DL/UL Average UE Throughput and for DL Average Packet Loss precludes the MN to request the SN to report such measurements if SN-terminated split or MCG bearers are configured at SN Addition and/or MN-initiated SN Change.

Proposal 1: RAN3 to agree that MN requests the SN to report UE performance (DL/UL Average Throughput, DL/UL Average Packet Delay and DL Average Packet Loss) only if there is at least a SN-terminated SCG bearer configured for the UE at SN Addition or MN-initiated SN Change.

An important issue to discuss is the granularity of the UE performance metrics that the SN needs to report. Our understanding is that UE performance metrics in TS 28.558 [8] are measured on a per-DRB per-UE granularity, other than the Average UE Throughput DL/UL measurements which are already defined with a per-UE granularity. However, the UE performance metrics in clause 9.2.3.179 of TS 38.423 [7] are intended to be reported on a per-UE granularity.

As a starting point, we can consider UE performance to be reported on a per-UE granularity as done in Rel-18. This because the intended use of such UE performance should be to evaluate whether the AI/ML model decision to setup NR-DC with a certain SN for the UE was beneficial with respect to keeping the concerned UE in single connectivity. And single connectivity is addressed in Rel-18 where only per-UE UE performance are reported. How to derive per-UE UE performance metrics (Average Packet Delay and Average Packet Loss) is currently under investigation in SA5 following the reply LS from RAN3 in [13] starting from the corresponding per-DRB per-UE measurements.

Proposal 2: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per-UE granularity, pending SA5 confirmation on how to derive per-UE metrics from the corresponding per-DRB per-UE measurements.

## 2.2 Conditions governing collection of UE performance at SN side

Conditions to start the UE performance collection in NR-DC

The following FFS from RAN3#127bis meeting remains to be discussed [6]:

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In RAN3#127, contribution [15] proposed to capture an additional condition, i.e., SCG Activation, to be evaluated by the SN – along with the condition related to the successful S-NG-RAN Node Addition, as per the latest version of the XnAP BLCR – in order to understand when to start the collection of UE performance to be then reported to the MN. More in detail, the current version of the XnAP BLCR captures the following text [16]:

### 8.3.1 S-NG-RAN node Addition Preparation #### 8.3.1.1 General The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same. *** skip unmodified text *** Interaction with the Activity Notification procedure Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level. Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures: If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful S-NG-RAN node addition, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

As per TS 37.340 [17] clause 7.13, an activation/deactivation mechanism of SCG is supported to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. Moreover, the MN can configure the SCG as (de-)activated upon PSCell addition (among other procedures such as, PSCell change, RRC Resume or handover), and it is also allowed that the SN independently (de-)activates the SCG. By taking Proposal 2 into account (for which UE performance metrics should be requested to be reported by the SN only if there is at least a SN-terminated SCG bearer configured for the UE at SN Addition or MN-initiated SN Change) we think that for the case of SCG bearer the SCG should be activated so that there will be data transmission via SCG RLC bearers, meaning that the improvement in UE performance enabled by data transmission over the SCG can be observed. And the activation of the SCG may be requested by the MN when sending the XnAP S-NODE ADDITION REQUEST message to the SN (via the SCG Activation Request IE included therein). As a consequence, RAN3 should discuss whether UE performance at the SN can be collected only after SCG resources are activated for the case of SN-terminated SCG bearer.

Proposal 3: RAN3 to agree whether UE performance at the SN side can be collected after successful S-NG-RAN node addition and after SCG activation in case of SN-terminated SCG bearer.

Conditions to stop the UE performance collection in NR-DC

Current TS 38.423 clause 8.4.13.2 captures, for the case of UE in single connectivity, the exit conditions to be considered by the target node in order to evaluate when to stop the collection of UE performance [7]:

### 8.4.13 Data Collection Reporting Initiation ### **** Skip unmodified text **** #### 8.4.13.2 Successful Operation **** Skip unmodified text **** If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message. **** Skip unmodified text ****

In RAN3#127bis meeting there was a discussion about the exit conditions to be evaluated by the SN in order to stop the collection of UE performance at the SN side, and the following was agreed [6]:

The following list of exit conditions to stop the collection of UE performance at SN side: - the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE; - the NR-DC with the S-NG-RAN node for the UE is released.

The above agreements have also been properly captured in the latest version of the XnAP BLCR in [16]:

### 8.4.13 Data Collection Reporting Initiation #### 8.4.13.1 General This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN. The procedure uses non UE-associated signalling. *** skip non-relevant text *** If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled: - the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE; - the time since S-NG-RAN node addition successfully completed is equal to the value of the Collection Time Duration for UE Performance IE; - UE moves to RRC_INACTIVE or RRC_IDLE state; - UE is handed over to another cell; - the NR-DC with the S-NG-RAN node for the UE is released. The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

In light of Proposal 4, we think that the exit condition agreed in RAN3#127bis should be enhanced to also consider the SCG activation in case SN-terminated SCG bearers are configured. To be specific:

- for SN-terminated SCG bearers, the time since NR-DC was successfully setup with the S-NG-RAN node with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE.

In Rel-18 and for the single connectivity case, UE performance can be collected and reported periodically during the collection time duration (the latter identified via the Collection Time Duration for UE Performance IE in XnAP) which start at successful handover. Based on this, for the AI/ML-based mobility in NR-DC, it should be discussed what is the SN behaviour in case SN-terminated SCG bearers are configured but the SCG is deactivated during the periodic collection and reporting of UE performance within the configured collection time duration. For the UE performance collection, since Rel-18, there is the common understanding in RAN3 that UE performance metrics are collected and reported within a time interval which is typically very short, therefore, in case the SCG is deactivated during this short collection time duration, we believe that the SN should simply stop the collection of UE performance and report the collected measurements to the MN at the first available DATA COLLECTION UPDATE message, without the need to resume the collection process in case the SCG is re-activated during the residual collection time duration which is supposed to be extremely short and close to its expiration, hence without adding further complexity to the UE performance collection and reporting process.

Proposal 4: RAN3 to agree the following exit conditions to stop the collection of UE performance at the SN side:

- for SN-terminated SCG bearers, the time since NR-DC was successfully setup with the S-NG-RAN node with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE;

- for SN-terminated SCG bearers, the SCG is deactivated during the collection time duration.

The TP for the XnAP BLCR for AI/ML for NG-RAN reflecting the above proposals can be found in the Annex of this paper.

# 3. Conclusion

Based on the above discussion, we have the following proposals:

On UE performance metrics in NR-DC

Observation 1: PDCP-level measurements performed by the node hosting the PDCP entity (either MN or SN) allow to collect per-DRB information in all NR-DC scenarios regardless of the bearer type (MCG, SCG, or split).

Observation 2: Referring to existing RLC-level measurements in TS 28.558 for DL/UL Average UE Throughput and for DL Average Packet Loss precludes the MN to request the SN to report such measurements if SN-terminated split or MCG bearers are configured at SN Addition and/or MN-initiated SN Change.

Proposal 1: RAN3 to agree that MN requests the SN to report UE performance (DL/UL Average Throughput, DL/UL Average Packet Delay and DL Average Packet Loss) only if there is at least a SN-terminated SCG bearer configured for the UE at SN Addition or MN-initiated SN Change.

Proposal 2: RAN3 to agree that, as a starting point, UE performance is reported from the SN to the MN on a per-UE granularity, pending SA5 confirmation on how to derive per-UE metrics from the corresponding per-DRB per-UE measurements.

Conditions governing collection of UE performance at SN side

Proposal 3: RAN3 to agree whether UE performance at the SN side can be collected after successful S-NG-RAN node addition and after SCG activation in case of SN-terminated SCG bearer.

Proposal 4: RAN3 to agree the following exit conditions to stop the collection of UE performance at the SN side:

- for SN-terminated SCG bearers, the time since NR-DC was successfully setup with the S-NG-RAN node with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE;

- for SN-terminated SCG bearers, the SCG is deactivated during the collection time duration.

The TP for the XnAP BLCR for AI/ML for NG-RAN reflecting the above proposals can be found in the Annex of this paper.

# 4. Reference

- RP-251245, “Revised WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#108

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- R3-253954, TP to BLCR 37.340 AIML for NR-DC, Lenovo, CATT, ZTE Corporation, Samsung, Ericsson, Nokia, Huawei, Ofinno, FiberCop, LG Electronics

- R3-255047, (BL CR to 37.340) AIML for NR-DC, Huawei, Lenovo, CATT, ZTE Corporation, Samsung, Ericsson, Nokia, Ofinno, FiberCop, LG Electronics, Jio Platforms

- 3GPP RAN3#127bis Chairman’s notes (RAN3_127bis_agenda_20250415_EOM1.doc)

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP), V18.6.0 (2025-06)

- 3GPP TS 28.558, Management and orchestration; UE level measurements for 5G system, V19.4.0 (2025-06)

- R3-252492, LS on clarification on UE performance measurement at PDCP level, Rel-19, RAN3 (ZTE)

- R3-253824 (S5-252839), Reply LS on clarification on UE performance measurement at PDCP level (Response to R3-252492), Rel-19, NEC, Intel, Ericsson

- 3GPP TS 28.552, Management and orchestration; 5G performance measurements, Rel-19, V19.4.0

- R3-250575, Discussion on Rel-18 UE Performance measurements, Nokia

- R3-253816, Reply LS on per-UE UE performance metrics (Response to R3-253023 / S5-252056), Rel-18, RAN3 (Nokia)

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1)

- R3-250573, Procedures for AI/ML Mobility Optimization in NR-DC, Nokia

- R3-255044, (BL CR to 38.423) Support of enhancements on AI/ML for NG-RAN, ZTE Corporation, Qualcomm, China Unicom, Ericsson, Samsung, Nokia, Lenovo, CATT, Huawei, NEC, CMCC, China Telecom, Telecom Italia, Deutsche Telekom, Verizon Wireless, LGE, Jio Platforms (JPL), Rakuten, Orange, FiberCop, InterDigital, Ofinno

- 3GPP TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Overall Description; Stage-2, V18.6.0 (2025-06)

# 5. Annex - TP for BLCR to TS 38.423 for AI/ML NR-DC (based on R3-255044)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

Interaction with the Data Collection Reporting Initiation and the Data Collection Reporting procedures:

If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message and if SN terminated SCG bearers are established, the S-NG-RAN node shall, if supported, report via the Data Collection Reporting procedure to the M-NG-RAN node after successful S-NG-RAN node addition and SCG activation the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the slice-specific information reporting is requested, the Slice To Report for Data Collection List IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	for SN terminated SCG bearers, the time since S-NG-RAN node addition successfully completed with activated SCG is equal to the value of the Collection Time Duration for UE Performance IE;

-	for SN terminated SCG bearers, the SCG of the S-NG-RAN node is deactivated during the time interval equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the NR-DC with the S-NG-RAN node for the UE is released.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.186	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. For SN terminated SCG bearers, it also indicates the time duration starting at successful S-NG-RAN node addition with activated SCG within which the UE performance measurements are collected when the UE operates in NR-DC. Unit: millisecond |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>