3GPP TSG-RAN WG3 Meeting #129       	   				    R3-255382

Bengaluru, IN, 25th – 29th Aug, 2025

Agenda Item:		14.3

Source:			China Telecom

Title:	NG transmission suspend resume procedure

Document for:	 	Discussion

- Introduction

The Rel-19 NTN enhancements WI contains the following objective[1]:

Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we will continue to discuss the potential impact of supporting NTN regeneration payloads from RAN3 perspective.

- Discussion

In case of hard feeder link switch over, a satellite/gNB can only be connected to one NTN-GW at the same time. If 5GC does not know the hard feeder link switch over, the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW. The old NTN-GW has no way to deliver it to the gNB or forward it to the new NTN-GW, which will result in packet loss. Therefore, at the previous RAN3 meeting, we agreed to introduce the Downlink NG Transmission Control IE to suspend and resume data and signaling procedures in the RAN Configuration Update procedure[2].

Observation: It is beneficial for the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB before and after hard FLSO.

Some companies have raised concerns regarding this approach. These include the view that hard FLSO should be avoided in favor of soft FLSO, and the argument that, due to the predictability of satellite operations, the CN could anticipate the feederlink switch timing of the on-board gNB in advance.

From our perspective, since the standards support both hard FLSO and soft FLSO, we need to support differentiated solutions. Regarding the second point, the division of responsibilities between OAM  and explicit signaling has been under discussion for NTN since Rel-17. Both RAN3 and SA2 have expressed significant concerns about whether the timeliness of OAM configuration can be guaranteed. With OAM based configurations, CN may obtain an approximate timeframe for hard FLSO occurring at the on-board gNB — typically ranging from hundreds of milliseconds to a few seconds. However, it is challenging to acquire the precise timing required to store and suspend NG interface contexts. Therefore, we propose introducing the Downlink NG Transport Control IE. SA2 has been made aware of this discussion and will update its specifications subject to RAN3 progress [3].

Proposal 1: Turn WA to agreement: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.

On the other hand, some companies suggest further check the functionality relationship between Downlink NG Transmission Control IE and NG suspend/resume procedure. In some cases, support for NG Suspend/Resume could make the NG connection recovery faster, no need to frequently remove and setup the NG interfaces between on board gNB and the AMFs and the two mechanisms can be used flexibly in combination.

Due to different application scenarios, so we slightly prefer explicitly describe the two cases in Stage-2.

- Conclusions

In this paper, we have the following proposal:

Observation: It is beneficial for the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB before and after hard FLSO.

Proposal 1: Turn WA to agreement: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.

- References

[1]	RP-241667, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.

[2]	R3-252409, (TP to BL CR for TS 38.413) Enhancement to support hard FLSO (Nokia, Nokia Shanghai Bell, Qualcomm, Xiaomi, China Telecom)

[3]	S2-2505707, N2 interface enhancement for downlink data suspension/resumption, ZTE.