3GPP TSG-RAN WG3 Meeting #129	R3-255401

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	12.2

Source: 	Lenovo

Title: 	(TP to BL CR 38.413) Discussion on remaining issues for WAB-node

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) focus on RAN3.

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management.

In this contribution, the remaining issues for WAB are discussed, at least including WAB-node NTN backhaul, mulit-hop topology and some other aspects.

# Introduction

NTN backhaul for WAB

If non-terrestrial link is used between WAB MT and BH gNB and/or between BH gNB and BH CN, the WAB-gNB informs UE’s CN that the BH includes a non-terrestrial link. FFS how a WAB node know the BH-gNB is using a non-terrestrial link. Possible options include BH-gNB informs WAB-gNB via Xn.

In the legacy NTN scenario, according to the TS 38.413, the gNB will provide the RAT Information IE which indicates the NR satellite RAT Type to AMF during the NG Setup procedure or the RAN Configuration Update procedure. And then the AMF sends the Satellite backhaul category to SMF and further to PCF for some enhancements on UPF selection.

For WAB architecture, the WAB-MT and WAB-gNB can connects different 5GCs. Based on the legacy NTN mechanism above, the following two cases of reporting can be already supported.

- Case 1: the BH gNB reports the RAT type to BH-5GC that the BH gNB adopt NTN architecture for backhaul link (e.g., WAB-MT accesses to the BH gNB via a satellite)

- Case 2: the WAB-gNB indicates the RAT type to UE’s 5GC that the WAB-gNB adopt NTN for access link (e.g., UE accesses to the WAB-gNB via a satellite). (This case is not supported by WAB, WAB only support TN access link)

However, WAB-gNB’s NG traffic (including traffic of served UE) is transported to UE’s 5GC via Backhaul PDU session. Thus, for example, the end-to-end delay should consider both access link and backhaul link of the WAB node. Based on the legacy NTN mechanism, the UE’s 5GC can only be aware of that UE accesses to the WAB-gNB via a satellite. But the UE’s 5GC cannot know whether the BH link is TN or NTN, or whether WAB-MT accesses to the BH gNB via a satellite. Without the information, the UE’s 5GC cannot perform UPF selection properly due to larger time delay of NTN backhaul.

In order for the WAB-gNB to report the BH link RAT type of WAB-MT to UE's AMF, the WAB-node needs to get the BH link RAT type of WAB-MT from the BH-gNB or BH-5GC. For example, BH-gNB indicates WAB-gNB with the BH link RAT type of WAB-MT via XnAP, but in this case, the BH-gNB needs to know the co-location of the WAB-MT and the WAB-gNB. Or BH-gNB indicates WAB-MT with the BH link RAT type of WAB-MT via RRC, or BH-5GC indicates WAB-MT with the BH link RAT type of WAB-MT via NAS.

Proposal 1: WAB-node gets the BH link RAT type of WAB-MT from the BH-gNB or BH-5GC.

- BH-gNB indicates WAB-MT with the BH link RAT type of WAB-MT via RRC (prioritized)

- BH-5GC indicates WAB-MT with the BH link RAT type of WAB-MT via NAS

- BH-gNB indicates WAB-gNB with the BH link RAT type of WAB-MT via XnAP

After the WAB-node being aware of the BH link RAT type of WAB-MT, the WAB-gNB reports it to UE's AMF via NGAP NG Setup Request message or the RAN Configuration Update message. Because WAB only support TN access link, then the WAB-node doesn’t need to report the RAT Information IE for the access link during the NG Setup procedure or the RAN Configuration Update procedure. And the legacy RAT Information IE can be reused for the BH link RAT type of the WAB-MT. For example, upon the UE’s AMF receiving the RAT Information IE from a WAB-gNB, the IE is applied for BH link of WAB-node.

Proposal 2: Reuse the existing NGAP RAT Information IE and existing codepoints of this IE for indicating the BH link RAT type of WAB-MT to UE’s AMF.

Proposal 3: Upon the UE’s AMF receiving the RAT Information IE from a WAB-gNB, the IE is applied for BH link of WAB-node.

Multi-hop WAB topology

To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected 1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells. 3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator. Solution 1 (dedicated existing OTA parameters) is used for preventing multihop WAB topology. Solution 3 For multi-hop avoidance during WAB-MT initial access, RAN3 to agree that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT. Solution 1 is implementation specific and it is not subject to specifications changes. There is no consensus in RAN3 on an agreement for solution 3 and make the decision in next meeting. To be continued...

The case a) raised in SA2’s LS [2] had not reached consensus on RAN3, where two camps share different preference on Solution 1 and Solution 3.

As for Solution 1, we had achieved common understanding that the legacy OTA parameter may be used to avoid WAB-MT selecting to other WAB-gNB cells, and this solution is a pure implementation of network and doesn’t introduce any impacts on our specification.

However, Solution 1 may introduce the complexity on the network planning based on the many operators’ comments. And as for Solution 3, this is always workable, and only simple impact is expected to be introduced in SIB1. Then we prefer to use Solution 3 supplemented with Solution 1 in case of WAB-node initial access.

Proposal 4: In case of WAB-node initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

WAB dual connectivity

It is FFS whether a WAB gNB can act as an MN or SN in Dual Connectivity.

As defined in the objectives of the WID, we need to avoid setting up Xn connection between WAB-gNBs. That is, due to the mobility of the WAB-node, the Xn relation between WAB-gNBs need to be frequently updated. Similarity, as for the dual connectivity of WAB, once a WAB-gNB becomes a MN or a SN of served UE, both UE and another RAN node need to frequently maintain the relation with the mobile WAB-node.

In addition for R18 mobile IAB, dual-connectivity is also down-prioritized when considering mobility of the mobile IAB node. Then we also prefer to deprioritize the combination of WAB and dual-connectivity.

Proposal 5: The combination of WAB and dual-connectivity is deprioritized in this release.

WAB-gNB indication

FFS whether to Introduce a “WAB-gNB” indication in the NG SETUP REQUEST message.

In last meeting, some companies proposed to introduce a WAB-gNB indication in the NG SETUP REQUEST message, in order for the CN to be aware of the RAN node being a WAB-gNB. However, the motivation and benefit for the CN is ambiguous on the awareness of WAB-gNB. And anyway, additional ULI is supported by the WAB, the CN can always recognize the RAN node as a WAB-gNB based on the received additional ULI.

Proposal 6: No need to introduce a WAB-gNB indication in the NG SETUP REQUEST message.

WAB-gNB migration

It is FFS whether the single-gNB solution with two logical cells can be included in the description in TS38.401

As for the single logical WAB-gNB solutions, an LS was sent to SA2 (and CC RAN2) in last meeting to check whether these single-gNB solutions can be achieved without CN impact. Based on the Reply LS, there is no consensus on the support of a single-gNB solution in SA2. From RAN3 point of view, the discussion on the single-gNB solution is suspended in RAN3 and we only need to revisit the solution based on further inputs from other WG.

Proposal 7: Single-gNB solution is deprioritized in this release unless further inputs from other WGs.

# Conclusion

This contribution aims to discuss remaining issues for WAB. And following observations and proposals are concluded.

Proposal 1: WAB-node gets the BH link RAT type of WAB-MT from the BH-gNB or BH-5GC.

- BH-gNB indicates WAB-MT with the BH link RAT type of WAB-MT via RRC (prioritized)

- BH-5GC indicates WAB-MT with the BH link RAT type of WAB-MT via NAS

- BH-gNB indicates WAB-gNB with the BH link RAT type of WAB-MT via XnAP

Proposal 2: Reuse the existing NGAP RAT Information IE and existing codepoints of this IE for indicating the BH link RAT type of WAB-MT to UE’s AMF.

Proposal 3: Upon the UE’s AMF receiving the RAT Information IE from a WAB-gNB, the IE is applied for BH link of WAB-node.

Proposal 4: In case of WAB-node initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Proposal 5: The combination of WAB and dual-connectivity is deprioritized in this release.

Proposal 6: No need to introduce a WAB-gNB indication in the NG SETUP REQUEST message.

Proposal 7: Single-gNB solution is deprioritized in this release unless further inputs from other WGs.

And a TP is drafted to reflect WAB NTN backhaul in the Appendix.

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- R3-244019. LS on questions regarding FS_VMR_Ph2. SA2.

- Chairman notes of RAN3 127-128 meetings.

# TP to BL CR 38.413

Start of Change

#### 9.3.1.125	RAT Information

This IE provides RAT related information associated with a TAC, used as described in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Information | M |  | ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT) | In case the RAT Information IE is received from a WAB-gNB, the nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT codepoints are applied for BH link of WAB-node. |

End of Change