3GPP TSG-RAN WG3 Meeting #129	R3-255412

Bengaluru, India, 25 – 29 August, 2025

Agenda Item:	12.2

Source:	Huawei

Title:	 (TP for WAB BL CRs) Radio Resource Multiplexing Coordination for WAB-node

Document for:	Discussion

1	Introduction

At the RAN3 #127bis meeting, the resource coordination has been discussed and the following agreements were minuted in the Chair notes [1]:

When access and backhaul links of a WAB node are operated out-of-band, there is no need for WAB resource coordination.

RAN3 assumes that WAB deployments use out-of-band operation in case access and backhaul use different PLMNs.

In this release, In-band WAB operation is only considered for intra PLMN scenarios where BH gNB is upgraded with WAB-specific enhancements.

For WAB resource coordination, introduce a new class-1 Xn procedure between WAB-gNB and BH gNB. The message content can be based on relevant parts of the XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423.

After RAN3 has finished specifying WAB resource coordination, RAN3 to send an LS to inform RAN1 and RAN2 about conclusions.

In RAN3#128 meeting, the progress on the resource multiplexing is quite limited, only the following agreements was captured: The specifications shall not define any priority between the WAB-gNB or the BH-gNB on how to split resources.

This contribution will dicusses the remaining issues of resource multiplexing coordination for WAB.

2	Discussion

2.1	Resource multiplexing coordination for WAB-node

2.1.1 General case

- For the IAB-DU to donor-CU resource reporting, the IAB-DU includes the IAB Info IAB-DU IE which contains the multiplexing info and the IAB STC info. The multiplexing info represents the multiplexing capabilities between the IAB-DU’s cell and the cells configured on the co-located IAB-MT. The IAB STC info contains cell SSB Transmission Configurations (STC) information of the IAB-DU.

- In addition, for the donor-CU to IAB-DU messages, e.g., the GNB-DU RESOURCE CONFIGURATION message, can provide the following information:

  - the resource configuration of the cells served by the IAB-DU;

  - the child node’s cell resource configuration and multiplexing capabilities;

  - the neighbour nodes’ cell resource configuration;

  - the NA resource configuration of the serving cell of the co-located IAB-MT.

To facilitate the resource coordination between access link and backhaul link of IAB, there are some procedures related to the resouce multiplexing coordination over F1AP since R16.

For inband WAB deployment, it would be helpful if the BH-gNB can know the information of WAB-node to avoid scheduling confliction between WAB-MT’s backhaul link and the WAB-gNB’s access link, similar as the parent node to know the child IAB node’s configuration in IAB case. So, the WAB-gNB should report its cell resource configuration (including cell specific signalling) and multiplexing capabilities to the BH-gNB. After investigate the existing IEs in TS 38.423, the content of the IAB Cell Information IE defined in clause 9.2.2.94 contains all the necessary information, and can be reused as much as possible with some wording update. It is worth noting that the RACH Config Common IAB IE in this clause is not applicable for WAB since there is no WAB specidic RACH configuration defined by RAN1. Moreover, the gNB-DU Cell Resource Configuration IE defined in clause 9.2.2.25 of TS 38.423 contains the sub-IE NA cell resource configuration List, which indicates the list of unavailable resources of the cell for the dual-connected boundary IAB-node. Such sub-IE is not useful for the WAB.

However, in the reverse direction, i.e., from the BH-gNB to WAB-gNB, the XnAP signalling is not needed, because the WAB-MT already know the cell resource configuration of the BH-gNB via the air interface signalling, then the WAB-gNB can share such information based on internal interaction, which is out of scope.

- The WAB-gNB sends the multiplexing capabilities and the resource configuration information of its served cells to the BH-gNB via XnAP signalling, the content of the signalling can be same as the IAB Cell Information IE, but the RACH Config Common IAB IE and the NA cell resource configuration List IE in 9.2.2.25 of TS 38.423 should be excluded.

- The WAB-node get the resource configuration of BH-gNB’s cell using legacy signalling over backhaul Uu interface.

Since the WAB-gNB is a full gNB, the relation between WAB-gNB and BH-gNB is somehow different from the relation between IAB-DU and donor-CU, it is not sure whether the BH-gNB can configure the WAB-gNB’s cell resource like the donor-CU provides resource configuration to IAB-DU. In the previous releases, as decided by RAN1, the IAB resource coordination between the F1-terminating donor and non-F1 terminating donor only allow the two gNBs exchange information on the cell configuration. There is no dominant node who can decide the configuration for another one, and how to adjust the cell configuration is up to the node implementation. Similarly, in last RAN3 #128 meeting, the principle “The specifications shall not define any priority between the WAB-gNB or the BH-gNB on how to split resources.” has been agreed. Following such principle, the BH-gNB should not decide cell resource configuration for the WAB-node connects to it.

- BH-gNB cannot configure the resource of the WAB-gNB cells.

Furthermore, it would be helpful if the WAB-gNB is informed with the neighbour nodes’ cell resource configuration, for the interference avoidance. But considering the limited time for the WI, such enhancement can be considered in future release.

For the child node and serving cell configurations, since the WAB-gNB should not have any child node, the “child node” related configurations in IAB is not needed for WAB.

The “NA resource configuration of the serving cell of the co-located WAB-MT” is also not needed, since the parent node should not be an IAB-node, and there is no “NA resource” for a non-IAB-DU.

In summary, it is proposed:

- It is not recommended to purse that the WAB-node should be aware of the neighbour nodes’ cell resource configuration in Rel-19.

2.1.2 Resource multiplexing coordination during WAB-node mobility

During WAB mobility, the WAB-node resource multiplexing coordination with the target BH-gNB may be untimely, causing the performance degradation. This issue can be categorized into two cases:

- Case 1: If the Xn interface between the WAB-gNB and the target BH-gNB does not exist, the resource multiplexing related signalling cannot be exchanged between the WAB-gNB and the target BH-gNB.

- Case 2: Even if the Xn interface is established, the performance degradation may still occur in this case, since the WAB-node may only perform resource multiplexing coordination with the serving BH-gNB but not the candidate target BH-gNBs before the WAB-MT handover to the target gNB.

Therefore, before the WAB-MT handover execution, it will be beneficial if the target BH-gNB and the WAB-gNB can be aware of each other, and implement resource multiplexing coordination timely. If the Xn interface does not exist, the Xn interface setup can be triggered at that time, and then the resource multiplexing can be implemented. To address this issue, two solutions are provided here:

- Solution 1. The target BH-gNB triggers the resource multiplexing coordination towards the WAB-gNB during the co-located WAB-MT’s handover. To support this solution, the target BH-gNB can be aware of the co-location between the WAB-gNB and the WAB-MT during the handover preparation phase, and establishes the Xn interface towards the WAB-gNB if needed. Then the resource multiplexing is implemented.

- Solution 2. The WAB-gNB triggers the resource multiplexing coordination towards the target BH-gNB during the co-located WAB-MT’s handover. To support this solution, the WAB-gNB should know the target BH-gNB’s ID from the source BH-gNB after the handover decision, and provide information for co-location discovery at the target BH-gNB. Then the resource multiplexing can be implemented.

We think both solutions can be supported, and then it is up to WAB-gNB/target BH-gNB to trigger the resource multiplexing coordination as needed.

It is proposed:

- For WAB-node mobility, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.

2.1.3 How to handle if no consensus on resource multiplexing.

In RAN3 #128 meeting. The discussion on the resourece multiplexing is time-consuming, but the progress is quite limited. Because companies cannot reach consensus on which part of IAB resource multiplexing mechanism can be applied to the WAB deployment. In our view, it should be RAN1 rather than RAN3 to discuss and decide which part of the IAB resource multiplexing coordination mechanism can be reused for WAB. So, we propose the following two choices to solve the resource multiplexing coordination issue for WAB:

Option 1: RAN3 decides to reuse the whole IAB resource multiplexing coordination mechanism in XnAP, only except some specific IEs (e.g., RACH Config Common IAB IE and the NA cell resource configuration List IE), and send LS to RAN1 and RAN2 for coordination. A draft LS to RAN1 and RAN2 is provided in the Annex.

Option 2: RAN3 sends LS to RAN1 to discuss and decide which part of the IAB resource multiplexing coordination mechanism can be reused for WAB.

If none of the above options can be agreed, RAN3 may not be able to specify any solutions on the resource multiplexing coordination. To complete the WI, another way is to remove the objective from the WID accordingly, since this feature is only useful for inband deployment which is not quite interested by many operators.

- To complete the WI, the following options can be discussed for resource multiplexing coordination:

Option 1: RAN3 decides to reuse the whole IAB resource multiplexing coordination mechanism in XnAP, only except some inappropriate IEs (e.g., RACH Config Common IAB IE and the NA cell resource configuration List IE), and send LS to RAN1 and RAN2 for coordination.

Option 2: RAN3 sends LS to RAN1 to discuss and decide which part of the IAB resource multiplexing coordination mechanism can be reused for WAB.

Option 3: If RAN3 is not able to specify any solutions on the resource multiplexing coordination, the related objective can be removed from the WID.

3	Conclusion

This paper provides analysis about the resource multiplexing coordination for WAB-node. The proposals are as follows:

- The WAB-gNB sends the multiplexing capabilities and the resource configuration information of its served cells to the BH-gNB via XnAP signalling, the content of the signalling can be same as the IAB Cell Information IE, but the RACH Config Common IAB IE and the NA cell resource configuration List IE in 9.2.2.25 of TS 38.423 should be excluded.

- The WAB-node get the resource configuration of BH-gNB’s cell using legacy signalling over backhaul Uu interface.

- BH-gNB cannot configure the resource of the WAB-gNB cells.

- It is not recommended to purse that the WAB-node should be aware of the neighbour nodes’ cell resource configuration in Rel-19.

- For WAB-node mobility, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.

- To complete the WI, the following options can be discussed for resource multiplexing coordination:

Option 1: RAN3 decides to reuse the whole IAB resource multiplexing coordination mechanism in XnAP, only except some inappropriate IEs (e.g., RACH Config Common IAB IE and the NA cell resource configuration List IE), and send LS to RAN1 and RAN2 for coordination.

Option 2: RAN3 sends LS to RAN1 to discuss and decide which part of the IAB resource multiplexing coordination mechanism can be reused for WAB.

Option 3: If RAN3 is not able to specify any solutions on the resource multiplexing coordination, the related objective can be removed from the WID.

4. Reference

- Chair notes of RAN3 #127bis.

Annex A:	LS to RAN1 and RAN2 on resource coordination

3GPP TSG-RAN WG3 Meeting #129	R3-XXXXXX

Bengaluru, India, 25th – 29th August, 2025

Title:	[DRAFT] LS on resource multiplexing of WAB

Response to:	xxxxxx

Release:	Rel-19

Work Item:	NR_WAB_5GFemto-Core

Source:	Huawei [to be: RAN3]

To:	RAN1, RAN2

Cc:

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 TP for XnAP

1. Overall Description:

RAN3 is currently working on additional topological enhancements for NR, including the Rel-19 WAB (Wireless Access Backhaul). The WAB-node comprises a gNB component (WAB-gNB) and an MT component (WAB-MT), The WAB-MT is served by a BH-gNB via an NR-Uu backhaul link, and the WAB-gNB provides NR-Uu access link to UE. Both inband and outband WAB deployment are supported in this WI.

For inband WAB-noe deployment, the resource multiplexing coordination between the access link and the backhaul link of WAB-node is needed. RAN3 assumes that the resource multiplexing framework defined for IAB (e.g., the HSNA attribute, the multiplexing capability of access and backhaul, the backhaul link scheduling should avoid hard resource in access link, etc.) can be reused, and take this as baseline for the signalling design in WAB topic.

With that in mind, RAN3 agreed the XnAP signalling between WAB-gNB and BH-gNB for resource coordination, and agreed the TP in R3-25xxxx.

RAN3 would like to request RAN1 and RAN2 to take the above progress into consideration, and if needed, to update specifications to ensure that the mechnisms defined for IAB resource multiplexing coordination between access link and backhaul link are also applicable for WAB.

2. Actions:

To RAN1 and RAN2:

ACTION: 	RAN3 requests RAN1 and RAN2 to update specifications to ensure that the mechnisms defined for IAB resource multiplexing coordination between access link and backhaul link are also applicable for WAB, and provide feedback if needed.

3. Date of Next RAN3 Meetings:

RAN3#129-bis		13th – 17th October, 2025					Prague, CZ

RAN3#130			17th – 21st November, 2025				Dallas, US

Annex B:	TP for WAB BL CR for TS 38.401

CHANGES START

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<

## X.8 	Coordination of WAB-gNB and WAB-MT radio resources

Coordination of access link and backhaul link radio resources is needed when the access link and the backhaul link of a WAB-node are operated in the same band (i.e., referred to as the in-band operation). To facilitate resource coordination, the BH-gNB discovers the co-location of the WAB-MT and the WAB-gNB based on a WAB-MT’s ID which the WAB-gNB includes in an XnAP message it sends to the BH-gNB. The resource coordination procedure is conducted between the WAB-gNB and the BH-gNB serving the WAB-gNB’s co-located WAB-MT.

During the mobility of the WAB-node, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.

NOTE: 	In-band WAB operation is only considered in scenarios where a WAB-gNB and its co-located WAB-MT connect to the same PLMN. When a WAB-gNB and its co-located WAB-MT connect to different PLMNs, it is assumed that the access link and the backhaul link of a WAB-node are operated in different bands.

End of Change

Annex C:	TP for WAB BL CR for TS 38.423

Start of Change

8	XnAP procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | XN REMOVAL REQUEST | XN REMOVAL RESPONSE | XN REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |
| WAB Resource Coordination | WAB RESOURCE COORDINATION REQUEST | WAB RESOURCE COORDINATION RESPONSE |  |

Next Change

8.X	WAB Procedures

8.X.1	WAB Resource Coordination

8.X.1.1	General

The purpose of the WAB Resource Coordination procedure is to exchange the semi-static radio resource configuration pertaining to WAB-gNB, between the WAB-gNB and the BH-gNB, for the purpose of resource multiplexing between the WAB-MT and the WAB-gNB. The procedure can be initiated by the WAB-gNB.

The procedure uses UE-associated signalling.

8.X.1.2	Successful Operation

Figure 8.X.1.2-1: WAB Resource Coordination, successful operation

The WAB-gNB initiates the procedure by sending the WAB RESOURCE COORDINATION REQUEST message to the BH-gNB. The BH-gNB shall respond with the WAB RESOURCE COORDINATION RESPONSE message to the WAB-gNB.

Upon receiving the WAB-gNB Cells List IE included in the WAB RESOURCE COORDINATION REQUEST message, the BH-gNB should take this information into account for resource multiplexing coordination towards the WAB-MT which co-locates with the sending WAB-gNB.

8.X.1.3	Unsuccessful Operation

Not applicable.

8.X.1.4	Abnormal Conditions

Not applicable.

Next Change

9.1.X	Messages for WAB Procedures

9.1.X.1	WAB RESOURCE COORDINATION REQUEST

This message is sent by a WAB-gNB to its BH-gNB, for the purpose of resource multiplexing coordination between the WAB-gNB’s access link and backhaul link.

Direction: WAB-gNB BH-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| WAB-gNB Cells List |  | 1 |  | List of cells served by the WAB-gNB. | YES | reject |
| >WAB-gNB Cells List Item |  | 1 .. <maxnoofServedCellsWAB > |  |  | – |  |
| >>WAB-gNB Cell Information | M |  | IAB Cell Information 9.2.2.94 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsWAB | Maximum number of cells served by an WAB-gNB. Value is 512. |

9.1.X.2	WAB RESOURCE COORDINATION RESPONSE

This message is sent by a BH-gNB to a WAB-gNB, as a response to a WAB RESOURCE COORDINATION REQUEST message.

Direction: BH-gNB WAB-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

Next Change

9.2.2.79	Multiplexing Info

This IE contains information about the multiplexing capabilities between the IAB-DU’s cell and the cells configured on the co-located IAB-MT, or between the WAB-gNB’s cell and the cells configured on the co-located WAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB-MT Cell List |  | 1 |  |  |
| >IAB-MT Cell Item |  | 1 .. <maxnoofServingCells> |  |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identity of a serving cell configured for a co-located IAB-MT or WAB-MT. |
| >>DU_RX/MT_RX | M |  | ENUMERATED (supported, not supported, supported and FDM required, …) | An indication of whether the IAB-node supports simultaneous reception at its DU and MT side. For WAB-node, this IE indicates whether the WAB-node supports simultaneous reception at its gNB and MT side. |
| >>DU_TX/MT_TX | M |  | ENUMERATED (supported, not supported, supported and FDM required, …) | An indication of whether the IAB-node supports simultaneous transmission at its DU and MT side. For WAB-node, this IE indicates whether the WAB-node supports simultaneous transmission at its gNB and MT side. |
| >>DU_TX/MT_RX | M |  | ENUMERATED (supported, not supported, supported and FDM required, …) | An indication of whether the IAB-node supports simultaneous transmission at its DU and reception at its MT side. For WAB-node, this IE indicates whether the WAB-node supports simultaneous transmission at its gNB and reception at its MT side. |
| >>DU_RX/MT_TX | M |  | ENUMERATED (supported, not supported, supported and FDM required, …) | An indication of whether the IAB-node supports simultaneous reception at its DU and transmission at its MT side. For WAB-node, this IE indicates whether the WAB-node supports simultaneous reception at its gNB and transmission at its MT side. |

| Range bound | Explanation |
| --- | --- |
| maxnoofServingCells | Maximum no. of serving cells for an IAB-MT or a WAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

Next Change

9.2.2.94	IAB Cell Information

This IE contains IAB-DU cell resource configuration, cell specific signal/channel configuration and multiplexing info of the cell of an IAB-node or IAB-donor-DU or a WAB-gNB. If the cell is served by WAB-gNB, the gNB-DU/IAB-DU refers to the WAB-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.2.7 |  |
| CHOICE IAB-DU Cell Resource Configuration-Mode-Info | O |  |  |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>gNB-DU Cell Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains TDD resource configuration of gNB-DU’s cell. |
| >>>Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD DL resource configuration of gNB-DU’s cell. |
| >>>UL Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>DL Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>UL Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>DL Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. |
| IAB STC Info | O |  | 9.2.2.96 | STC configuration of this gNB-DU’s cell. |
| RACH Config Common | O |  | OCTET STRING | Includes the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [10]. |
| RACH Config Common IAB | O |  | OCTET STRING | Includes the IAB-specific rach-ConfigCommonIAB-r16 as defined in subclause 6.3.2 of TS 38.331 [10]. This IE shall be ignored when the node is a WAB. |
| CSI-RS Configuration | O |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SR Configuration | O |  | OCTET STRING | Includes the SchedulingRequestResourceConfig IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| PDCCH Configuration SIB1 | O |  | OCTET STRING | Includes the PDCCH-ConfigSIB1 IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SCS Common | O |  | OCTET STRING | Includes the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [10]. |
| Multiplexing Info | O |  | 9.2.2.79 | Contains information on multiplexing with cells configured for collocated IAB-MT or WAB-MT, if applicable. |

Next Change

9.2.2.95	gNB-DU Cell Resource Configuration

This IE contains the resource configuration of the cells served by a gNB-DU or a WAB-gNB, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [40], clause 11.1.1). For IAB, this IE is only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. For WAB, the gNB-DU referes to the WAB-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. |
| DUF Transmission Periodicity | O |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  |
| DUF Slot Configuration List |  | 0..1 |  |  |
| >DUF Slot Configuration Item |  | 1..<maxnoofDUFSlots> |  |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  |
| >>>Explicit Format |  |  |  |  |
| >>>>Permutation | M |  | ENUMERATED (DFU, UFD, …) |  |
| >>>>Number of Downlink Symbols | O |  | INTEGER (0..14) |  |
| >>>>Number of Uplink Symbols | O |  | INTEGER (0..14) |  |
| >>>Implicit Format |  |  |  |  |
| >>>>DUF Slot Format Index | M |  | INTEGER (0..254) | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [40], excluding the last row in Table 14-2. |
| HSNA Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, …) |  |
| HSNA Slot Configuration List |  | 0..1 |  |  |
| >HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  |
| >>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot. |
| >>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot. |
| >>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot. |
| RB Set Configuration | O |  | 9.2.2.97 | Indicates the configuration for up to M non-overlapping RB sets for a given DU cell, used for frequency domain resource allocation. The maximum value of M is 8. |
| Frequency-domain HSNA Configuration List |  | 0..1 |  |  |
| >Frequency-domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell1, …) | Refers to an RB set defined by RB Set Configuration. The RB set indexes are consecutive (and increasing) starting at 0. |
| >>Frequency-domain HSNA Slot Configuration List |  | 1 |  |  |
| >>>Frequency-domain HSNA Slot Configuration item |  | 1..<maxnoofHSNASlots> |  |  |
| >>>>Slot Index | M |  | INTEGER (.. maxnoofHSNASlots) | Index to a slot within the HSNA Transmission Periodicity. * |
| >>>>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. |
| >>>>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. |
| >>>>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. |
| NA cell resource configuration List |  | 0..1 |  | List of unavailable resources of this cell for the dual-connected boundary IAB-node. This IE shall be ignored when the node is a WAB. |
| >NA cell resource configuration Item |  | 1..<maxnoofHSNASlots> |  |  |
| >>NA Downlink | O |  | ENUMERATED (true, false, …) | Indicates whether downlink symbols, in a slot, are unavailable to serve the boundary IAB-node. |
| >>NA Uplink | O |  | ENUMERATED (true, false, …) | Indicates whether uplink symbols, in a slot, are unavailable to serve the boundary IAB-node. |
| >>NA Flexible | O |  | ENUMERATED (true, false, …) | Indicates whether flexible symbols, in a slot, are unavailable to serve the boundary IAB-node. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. Corresponds to the maxNrofSlots defined in TS 38.331 [10]. |
| maxnoofSymbols | Maximum no. of symbols in a slot. Value is 14. |
| maxnoofHSNASlots | Maximum no of "Hard", "Soft" or "Not available" slots in 160ms. Value is 5120. |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per DU cell. Value is 8. |
| maxnoofChildIABNodes | Maximum number of child nodes served by an IAB-DU or IAB-donor-DU. Value is 1024. |
| maxnoofRBsetsPerCell1 | Maximum no. of RB sets per DU cell minus 1. Value is 7. |

Next Change

9.2.2.96	IAB STC Info

This IE contains cell SSB Transmission Configuration (STC) information of an IAB-DU or an IAB-donor-DU or a WAB-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB STC-Info List |  | 1 |  |  |
| >IAB STC-Info Item |  | 1 ..<maxnoofIABSTCInfo> |  |  |
| >>SSB Frequency Info | M |  | INTEGER (0.. maxNRARFCN) | The SSB central frequency. |
| >>SSB Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …) | The SSB subcarrier spacing. |
| >>SSB Transmission Periodicity | M |  | ENUMERATED (sf10, sf20, sf40, sf80, sf160, sf320, sf640, …, sf5) |  |
| >>SSB Transmission Timing Offset | M |  | INTEGER (0.. 127, …) | SSB transmission timing offset in number of half-frames. |
| >>CHOICE SSB Transmission Bitmap | M |  |  | Corresponds to information provided in the SSB-ToMeasure IE defined in TS 38.331 [10]. |
| >>>short bitmap |  |  |  |  |
| >>>>Short Bitmap | M |  | BIT STRING (SIZE (4)) |  |
| >>>medium bitmap |  |  |  |  |
| >>>>Medium Bitmap | M |  | BIT STRING (SIZE (8)) |  |
| >>>long bitmap |  |  |  |  |
| >>>>Long Bitmap | M |  | BIT STRING (SIZE (64)) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIABSTCInfo | Maximum no. of STC configurations. Value is 5. This includes 1 STC configuration for access and 4 STC configurations for backhaul. |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |

Next Change

9.2.2.97	RB Set Configuration

This IE contains the configuration for up to M non-overlapping RB sets for a given gNB-DU cell or a given WAB-gNB cell, used for frequency domain resource allocation. For IAB, this IE is only applicable if the gNB-DU is an IAB-DU. The maximum value of M is 8. For WAB, the gNB-DU/IAB-DU refers to the WAB-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |
| RB Set Size | M |  | ENUMERATED (rb2, rb4, rb8, rb16, rb32, rb64) | Number of PRBs in each RB set. If the RB sets of IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth, the remaining RBs not part of an RB set configuration are considered as included in the last RB set. |
| Number of RB Sets | M |  | INTEGER(1.. maxnoofRBsetsPerCell) | Number of configured RB sets. The RB sets are contiguous and non-overlapping. If the NR Carrier List IE(9.2.2.63) is provided, the start RB index of the first RB set is the RB index of the lowest common RB with the SCS provided by RB Set Configuration IE, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for the IAB-DU cell. Otherwise, the lowest subcarrier of the start RB of the first RB set is aligned with point A for the IAB-DU cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per IAB-DU cell. Value is 8. |

End of Change