3GPP TSG-RAN WG3 Meeting #129	R3-255414

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	12.3

Source:	Huawei

Title:	(TP for Femto BL CRs for TS 38.300) Discussion on NR Femto awareness at CN

Document for:	Approval

1	Introduction

In RAN3 #128 meeting, the following issue which is related to the NR femto awareness at CN was marked as to be continued [1]:

To be further discussed: whether to send a specific Femto indication in the NG: Initial UE message, from NR Femto to enable control of sending Allowed PNI NPN List or not.

This contribution further discusses the necessity of NR femto awareness at CN.

2	Discussion

In RAN3# 128 meeting, the contribution [2] brought an issue that the AMF should know the node type of a NR femto node. As stated in [2], the motivation for such awareness at AMF is to avoid sending the allowed PNI-NPN list to the NR femto node considering that it is not trustable. The solution in [2] requires that the NR femto node to add the NR femto indication in Initial UE MESSAGE. But other companies were not convinced by such motivation. Because RAN3 has agreed to reuse the CAG functionality for NR femto, and there was no conclusion in SA3 to indicate that the mobility restriction list should not be sent to the NR femto.

------------Excerption from BL CR for TS38.300 (R3-254017) -----------

4.X.2.2	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto node directly connected to an AMF:

-	At NG Setup, verifying, as defined in TS 33.545 [xx], that the identity used by the NR Femto node is valid;

-	At Initial UE Message and NG Handover procedures, verifying, as defined in TS 33.545 [xx], that the reported CAG ID list is valid for the indicated cell of the NR Femto node;

-	At NG PWS Restart Indication and PWS Failure Indication, verifying, as defined in TS 33.545 [xx], that the indicated cell identity is valid.

According to the conclusion in SA3, the AMF and the NR femto GW need to perform verifiction to the identity of NR femto node, the cell identity of the cell served by the NR femto node, and the reported CAG ID, etc. Some contents regarding the verification at AMF captured in the RAN3 38.300 BL CR [3] are shown below.

However, for the normal gNB, the AMF does not need to perform such security related verification. So, if the NR femto node is directly connected to the AMF, AMF should aware that this gNB is an NR femto node rather than a normal gNB. In case the NR femto node connects to an NR femto GW, the NR femto GW can know that the connected gNB is a NR femto node directly because normal gNB will not setup NG interface towards an NR femto GW.

Observation 1: If the NR femto node connects to AMF directly, the AMF should know the node type is NR femto node, to perform special security related verification on some parameters provided by the NR femto node.

Observation 2: If the NR femto node connects to the NR femto GW, the GW can directly know the node type is NR femto node.

To help the AMF be aware of the node type is NR femto node, a simplest way is adding NR femto indication in the NG SETUP REQUEST message from the NR femto node.

Proposal 1:  If the NR femto node connects to AMF directly, it includes a Femto indication in the NG SETUP REQUEST message.

In the Annex part, we also provide the TP for TS 38.300, and the TP for TS 38.413 is provided in [4] to capture the above proposal.

3	Conclusion

This paper focuses on the issue about NR femto awareness at CN , and provides the TP for TS 38.300. The following proposals and observations are derived.

Observation 1: If the NR femto node connects to AMF directly, the AMF should know the node type is NR femto node, to perform special security related verification on some parameters provided by the NR femto node.

Observation 2: If the NR femto node connects to the NR femto GW, the GW can directly know the node type is NR femto node.

Proposal 1:  If the NR femto node connects to AMF directly, it includes a Femto indication in the NG SETUP REQUEST message.

4	Reference

- RAN3 #128 chair notes.

- R3-253225, Nokia, Completion of other open points of NR Femto, RAN3 #128 meeting.

- R3-254017, Ericsson, etc., (BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects.

- R3-255415, Huawei, (TP for Femto BL CRs for TS 38.413) NR Femto awareness at CN.

Annex ——TP for BL CR on TS 38.300

Start of Change

4.X.2.2	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto node directly connected to an AMF:

-	At NG Setup, verifying, as defined in TS 33.545 [xx], that the identity used by the NR Femto node is valid, if the NG SETUP REQUEST contains NR Femto Indication IE;

-	At Initial UE Message and NG Handover procedures, verifying, as defined in TS 33.545 [xx], that the reported CAG ID list is valid for the indicated cell of the NR Femto node;

-	At NG PWS Restart Indication and PWS Failure Indication, verifying, as defined in TS 33.545 [xx], that the indicated cell identity is valid.

End of Change