3GPP TSG-RAN WG3 Meeting #129	R3-255416

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	21.3

Source: 	Huawei

Title: 	Discussion on UL rate control

Document for:	Discussion and Decision

# 1. Introduction

CU sends the uplink rate control indication per QoS flow over F1 to DU.

Other additional assistance information from CU to DU?

- Recommended UL bit rate infor per QoS flow

- Measured bit rate per QoS flow

- No additional information

RAN3 has agreed to introduce F1AP indication to support UL rate control during the last meeting. A remaining issue to whether to introduce any other information from the gNB-CU to the gNB-DU for UL rate control. The related FFS issue captured in RAN3#128 meeting Chair notes are copied below.

We will provide our views for the FFS issue in this contribution. Some other miscellaneous issues will also be discussed.

# 2. Discussion

## 2.1 UL rate control

As introduced in section 1, RAN3 left an open issue on whether to introduce some assistance information from the gNB-CU to the gNB-DU. Some options are listed as below:

    - Recommended UL bit rate info per QoS flow

    - Measured bit rate per QoS flow

    - No additional information

The key point for this remaining issue is whether the gNB-CU or the gNB-DU should determine the bitrate for UL QoS flows.

First, it is more efficient to determine the recommended rate at the gNB-DU since the recommended bitrate is signalled to the UE via MAC CE, and the gNB-DU is more aware of the radio link status and the resource scheduling so that it can react to the congestion timely. Otherwise, if the gNB-CU is responsible for determining the recommended bit rate, it has to obtain the congestion information from the gNB-DU first, and provide the recommended bit rate to the gNB-DU for sending the MAC CE, which introduces additional interactions between the gNB-CU and the gNB-DU and may make the uplink rate control been executed with unexpected hysteresis.

Observation 1: It is more efficient and with less spec impacts to determine the recommended bit rate at the gNB-DU.

Besides, as analysed in our previous paper R3-253348, there is no technical obstacles for the gNB-DU to decide the recommended bitrate for a QoS flow. On one hand, the gNB-DU can obtain the QoS flow level QoS parameters via F1AP, so that it can derive the available rate for each QoS flow based on the available resource and the QoS requirements. For example, assuming there are two QoS flows with their GFBR equal to 10Mbps (QoS flow 1) and 20Mbps (QoS flow 2) respectively, if the gNB-DU can provide up to 45Mbps for the two QoS flows in total, then it can determine the recommended bit rate as 15Mbps for QoS flow 1 and 30Mbps for QoS flow 2.

Observation 2: The gNB-DU can obtain the QoS flow level QoS requirements.

On the other hand, the gNB-DU is also capable to measure the actual bitrate of each QoS flow by taking statistics of the received UL packets. Specifically, if there is only one QoS flow mapping to the DRB, the gNB-DU can measure the bitrate for the DRB, which equals to the rate of the QoS flow. If there are multiple QoS flows mapping to the DRB, the UL SDAP header should be configured, because the gNB needs to know the QFI and fill it into the NG-U header when forwarding the packets to the UPF (the QFI must be included in the UL PDU SESSION INFORMATION Format according to the TS 38.415 [1]). The gNB-DU can distinguish the packets belong to different QoS flows via the SDAP header so that the rate of each QoS flow can be measured. With monitoring the actual rate for the QoS flows, the gNB-DU can determine the recommended rate for each QoS flow by taking the available resource, measured bitrate per QoS flow and the QoS requirement of each Qos flow into account.

Observation 3: The gNB-DU can measure the bitrate per QoS flow without assistance information from the gNB-CU.

In previous discussion, some companies raised a concern on whether the gNB-DU can interpret the SDAP header because the PDCP layer and SDAP layer are located at the gNB-CU. From technical point of view, we do not see any obstacles. First, according to TS 38.323 [2], the SDAP header will not be ciphered, so the gNB-DU can read the information in the SDAP header. Besides, the gNB-DU is already implemented to read the upper layer headers actually. According to TS 38.425 [3], the gNB-DU can send the DL DATA DELIVERY STATUS frame to the gNB-CU, which includes the Highest successfully delivered NR PDCP Sequence Number and the Highest transmitted NR PDCP Sequence Number. This means the gNB-DU is able to interpret the DL PDCP header. With such capability, there should be no additional difficulty for the gNB-DU to interpret the UL SDAP header.

------ Excerption from TS 38.323 ------ ## 5.8 Ciphering and deciphering The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the MAC-I (see clause 6.3.4) and the data part of the PDCP Data PDU (see clause 6.3.3) except the SDAP header and the SDAP Control PDU if included in the PDCP SDU. The ciphering is not applicable to PDCP Control PDUs.

Observation 4: It is technical feasible for the gNB-DU to measure per QoS flow UL bitrate due to the following reasons:

- UL SDAP header must be included if considering multiple QoS flows are mapped to one DRB.

- The SDAP header is not ciphered, and the QFI is visible to gNB-DU.

- The gNB-DU can read information from upper layer header.

With the above observations, the gNB-DU is capable to determine the recommended bitrate per QoS flow and there is no need to have any additional assistance information from the gNB-CU. To minimize the specification impact, it is more suitable to let the gNB-DU determine the QoS flow level recommended bit rate.

Proposal 1: To support the UL rate control, no additional information is needed from CU to DU. It is up to gNB-DU’s implementation to derive the recommended UL bitrate.

# 3. Conclusion

In this contribution, we analyzed the remaining issues for UL rate control and QNC. The following observations and proposals were made:

Observation 1: It is more efficient and with less spec impacts to determine the recommended bit rate at the gNB-DU.

Observation 2: The gNB-DU can obtain the QoS flow level QoS requirements.

Observation 3: The gNB-DU can measure the bitrate per QoS flow without assistance information from the gNB-CU.

Observation 4: It is technical feasible for the gNB-DU to measure per QoS flow UL bitrate due to the following reasons:

- UL SDAP header must be included if considering multiple QoS flows are mapped to one DRB.

- The SDAP header is not ciphered, and the QFI is visible to gNB-DU.

- The gNB-DU can read information from upper layer header.

Proposal 1: To support the UL rate control, no additional information is needed from CU to DU. It is up to gNB-DU’s implementation to derive the recommended UL bitrate.

# 4. Reference

[1]. 3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[2]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification ".

[3]. 3GPP TS 38.425: “NR user plane protocol”.