3GPP TSG-RAN WG3 Meeting #129	R3-255417

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	21.3

Source: 	Huawei

Title: 	(TP for XR BL CRs) Discussion on RLC enhancements for XR

Document for:	Discussion and Decision

# 1. Introduction

Reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS on how to capture this agreement.

For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

During the R19 XR discussions, RAN2 has introduced some enhancements for the RLC AM mode, including to avoid unnecessary retransmission and to perform timely retransmission. RAN3 has discussed the potential RAN3 impacts during the last meeting, and the following agreements were captured.

In this contribution, we will further discuss the remaining issues and give our views.

# 2. Discussion

### 2.1 Avoid unnecessary retransmission

To avoid unnecessary retransmission for the packets with stringent delay budget, RAN2 agreed to stop transmission and retransmission for an SDU when the TX RLC entity receives a discard indication of the SDU from the PDCP. For DL transmission in case of CU-DU split, RAN3 agreed to reuse the PDCP discard indication in F1-U to indicate the gNB-DU to stop transmission/retransmission of an RLC SDU or SDU segment.

Currently, the F1-U indication is used to indicate the gNB-DU to discard the corresponding SDUs, as specified in TS 38.425 [1]. According to the RLC specification in TS 38.322 [2], the RLC entity on the gNB-DU will discard the indicated SDUs if they haven’t been submitted to lower layers. If the RLC SDU or a segment thereof has already been submitted to lower layers, it will not be affected. For simplicity, we can call such discard behaviour as “legacy discard procedure”.

------ Excerption from TS 38.425 ------ ### 5.4.1 Transfer of Downlink User Data ------ unrelated part skipped ------ The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs. ------ unrelated part skipped ------ #### 5.5.2.1 DL USER DATA (PDU Type 0) This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU. The following shows the respective DL USER DATA frame. NOTE 1: All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=0); Spare; DL Discard Blocks; DL Flush; Report polling; 1. Spare; Request OutofSeq Report; Report Delivered; User data existence flag; Assistance Info. Report Polling Flag; Retransmission flag; 1. NR-U Sequence Number; 3. DL discard NR PDCP PDU SN; 0 or 3. DL discard Number of blocks; 0 or 1. DL discard NR PDCP PDU SN start (first block); 0 or 3. Discarded Block size (first block); 0 or 1. …. DL discard NR PDCP PDU SN start (last block); 0 or 3. Discarded Block size (last block); 0 or 1. DL report NR PDCP PDU SN; 0 or 3. Padding; 0-3. Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format ------ unrelated part skipped ------ #### 5.5.3.8 DL discard NR PDCP PDU SN Description: This parameter indicates the downlink NR discard PDCP PDU sequence number up to and including which all the NR PDCP PDUs should be discarded. Value range: {0..218-1}. Field length: 3 octets. #### 5.5.3.9 DL Discard Blocks Description: This parameter indicates the presence of DL discard Number of blocks, DL discard NR PDCP PDU SN start and Discarded Block size. Value range: {0= DL discard Number of blocks, DL discard NR PDCP PDU SN start and Discarded Block size not present, 1= DL discard Number of blocks, DL discard NR PDCP PDU SN start and Discarded Block size present}. Field length: 1 bit. #### 5.5.3.10 DL discard NR PDCP PDU SN start Description: This parameter indicates the starting SN of a downlink NR PDCP PDU block to be discarded. Value range: {0..218-1}. Field length: 3 octets. #### 5.5.3.11 DL discard Number of blocks Description: This parameter indicates the number of NR PDCP PDU blocks to be discarded. Value range: {1..244}. Field length: 1 octet. #### 5.5.3.12 Discarded Block size Description: This parameter indicates the number of NR PDCP PDUs counted from the starting SN to be discarded. Value range: {1..255}. Field length: 1 octet.

------ Excerption from TS 38.322 ------ ## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

To support DL unnecessary retransmission avoidance, the gNB-DU’s behaviour upon receiving the F1-U indication should be re-defined. The functionality should be configurable. When it is configured, the gNB-DU adopt the new discard behaviour, i.e., stop the transmission and retransmission for the indicated SDUs regardless of whether they are submitted to lower layers. Otherwise, the legacy discard procedure is applied.

To facilitate the new discard behaviour at gNB-DU, the gNB-CU can provide signalling to the gNB-DU to enable or disable unnecessary RLC retransmission avoidance via F1AP. Specifically, the gNB-CU can carry the configuration signalling in the UE CONTEXT SETUP/MODIFICTION REQUEST messages, to indicate the gNB-DU to stop transmission/retransmission for the SDUs for a DRB when the corresponding discard indication is received.

Proposal 1: The gNB-CU indicates the gNB-DU to stop the transmission/retransmission for the SDUs indicated by the F1-U discard indication via F1AP signalling.

According to the above analysis, RAN3 should specify the configuration signalling in F1AP. Besides, the gNB-DU’s behaviour defined in TS 38.425 should also be updated to support the new mechanism for avoiding unnecessary retransmission. Specifically, the case when the gNB-DU is configured to stop the transmission/retransmission for the indicated SDUs should be added additionally.

Both TPs for F1AP and F1-U are provided in Annex A, which should be adopted.

Proposal 2: Adopt the TPs for F1AP and F1-U specifications in Annex A to support unnecessary retransmission avoidance for downlink.

### 2.2 Timely retransmission

RAN2 also discussed in Rel-19 on how to perform timely RLC retransmission based on the remaining time of the packets to improve the reliability and guarantee the stringent delay budget. RAN3 has considered the potential impacts for both uplink and downlink during the last meeting. The following issues were listed.

- For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB. - For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

As agreed by RAN2, timely retransmission, including autonomous retransmission and enhanced polling, would be triggered when the remaining time of an RLC SDU falls below the specified threshold, and the remaining time of the SDU is determined based on its PDCP discard timer.

For uplink, the remaining time threshold for autonomous retransmission or enhanced polling is configured by the gNB to the UE. One of the remaining issues worries that in case of CU-DU split, whether the gNB-DU configures the thresholds. According to the agreement in RAN2#130 meeting, as excerpted below, the thresholds are configured per PDCP entity. And in the latest running CR for TS 38.331 [3], the thresholds are configured as part of PDCP-Config, which is determined by the gNB-CU. Thus, there is no need to enhance the gNB-DU for uplink timely retransmission configuration.

- The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

Proposal 3: There is no need to enhance the gNB-DU for uplink timely retransmission configuration.

For downlink, timely retransmission can be supported by introducing additional indication from the gNB-CU to the gNB-DU to indicate SDUs when their remaining time falls below the threshold for autonomous retransmission or enhanced polling. For example, the gNB-CU can indicate the PDCP SNs of the SDUs whose remaining time are below some threshold to gNB-DU, and such indications can be included in the DL UP packets. Upon receiving the indication, the gNB-DU can perform autonomous retransmission or enhanced polling accordingly for the corresponding SDUs.

Proposal 4: Introduce indication from the gNB-CU to the gNB-DU to indicate the SDUs with remaining time less than the threshold for autonomous retransmission or enhanced polling.

In addition, to inform the gNB-DU about the existence of the additional indication for autonomous retransmission or enhanced polling, and to enable the gNB-DU to perform timely retransmission accordingly based on the indication in DL UP packets, the gNB-CU can signal the configuration to gNB-DU in advance. The granularity of DL configuration should be the same as UL, i.e., per PDCP entity. To be specific, per DRB configuration for DL timely retransmission can be contained inthe UE CONTEXT SETUP/MODIFICATION REQUEST messages.

Proposal 5: The gNB-CU configures the gNB-DU to perform autonomous retransmission or enhanced polling on per DRB basis.

Furthermore, when the gNB-CU is composed by the gNB-CU-CP and one or more gNB-CU-UPs, the gNB-CU-UP is responsible to maintain the PDCP discard timer and determine the remaining time of the SDUs. In such case, the indication to trigger autonomous retransmission or enhanced polling should be sent by the gNB-CU-UP to the gNB-DU via F1-U. Thus, the gNB-CU-UP has to be aware of the thresholds. They can be provided by the gNB-CU-CP as part of PDCP configuration during the DRB context setup and modification procedures.

Proposal 6: The gNB-CU-CP provides the thresholds for autonomous retransmission and enhanced polling to the gNB-CU-UP.

Correspondingly, we provide TPs for TS 38.473, TS 38.425 and TS 37.483 in Annex B to reflect the above proposals.

Proposal 7: Adopt the TPs in Annex B to support timely retransmission for downlink.

# 3. Conclusion

In this contribution, we analyzed the RAN3 impacts to support the RLC enhancements discussed in R19. The following proposals were made:

Avoid unnecessary retransmission:

Proposal 1: The gNB-CU indicates the gNB-DU to stop the transmission/retransmission for the SDUs indicated by the F1-U discard indication via F1AP signalling.

Proposal 2: Adopt the TPs for F1AP and F1-U specifications in Annex A to support unnecessary retransmission avoidance for downlink.

Timely retransmission:

Proposal 3: There is no need to enhance the gNB-DU for uplink timely retransmission configuration.

Proposal 4: Introduce indication from the gNB-CU to the gNB-DU to indicate the SDUs with remaining time less than the threshold for autonomous retransmission or enhanced polling.

Proposal 5: The gNB-CU configures the gNB-DU to perform autonomous retransmission or enhanced polling on per DRB basis.

Proposal 6: The gNB-CU-CP provides the thresholds for autonomous retransmission and enhanced polling to the gNB-CU-UP.

Proposal 7: Adopt the TPs in Annex B to support timely retransmission for downlink.

# 4. Reference

[1]. 3GPP TS 38.425: "NR user plane protocol".

[2]. 3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[3]. Email Discussion [POST130][506][XR] RRC running CR and open issues: https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BRAN2%23130%5D/%5BPOST130%5D%5B506%5D%5BXR%5D%20RRC%20running%20CR%20and%20open%20issues%20(Huawei)

# Annex A - TPs for unnecessary retransmission avoidance

## Annex A.1 – TP for BL CR for TS 38.473

CHANGES START

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU discarding for the indicated DRB.

If the DL Discarded SDUs Transmission Stop IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, stop transmission or retransmission of the SDUs indicated as discarded, as specified in TS 38.425 [x], for the indicated DRB.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

------ Unchanged part skipped ------

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for the UE’s Ranging and Sidelink Positioning service.

For each GBR QoS flow whose DRB has been successfully established and the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and perform available bitrate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established, if the MMSID IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [21] and TS 38.300 [6].

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

NEXT CHANGE

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU for the indicated DRB.

If the DL Discarded SDUs Transmission Stop IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, stop transmission or retransmission of the SDUs indicated as discarded, as specified in TS 38.425 [x], for the indicated DRB.

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

------ Unchanged part skipped ------

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

For each GBR QoS flow whose DRB has been successfully established or modified and the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and, perform available bitrate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established or modified, if the MMSID IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [21] and TS 38.300 [6].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

NEXT CHANGE

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>>>DL Discarded SDUs Transmission Stop | O |  | ENUMERATED (true, false, …) | Indicates whether the RLC entity should stop transmission and retransmission of the RLC SDUs or its segments when discard indication of the SDUs is received from the PDCP layer. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>>>DL Discarded SDUs Transmission Stop | O |  | ENUMERATED (true, false, …) | Indicates whether the RLC entity should stop transmission and retransmission of the RLC SDUs or its segments when discard indication of the SDUs is received from the PDCP layer. | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>>>DL Discarded SDUs Transmission Stop | O |  | ENUMERATED (true, false, …) | Indicates whether the RLC entity should stop transmission and retransmission of the RLC SDUs or its segments when discard indication of the SDUs is received from the PDCP layer. | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

CHANGES END

### Annex A.2 – TP for BL CR for TS 38.425

CHANGE STARTS

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs. If configured, the corresponding node shall, if supported, stop transmitting/retransmitting these indicated NR PDCP PDUs.

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

The corresponding node shall detect whether an NR-U packet was lost and memorise the respective sequence number after it has declared the respective NR-U packet as being "lost".

The corresponding node shall transfer the remaining NR PDCP PDUs towards the UE and memorise the highest NR PDCP PDU sequence number of the NR PDCP PDU that was successfully delivered (as defined in TS 36.322 [6] and TS 38.322 [7]) in sequence towards the UE (in case RLC AM is used) and the highest NR PDCP PDU sequence number of the NR PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered. The DL DATA DELIVERY STATUS sent as a response to a specific DL report NR PDCP PDU SN shall be sent only when all PDCP PDU SNs up to this DL report NR PDCP PDU have been successfully delivered in-sequence.

If the Request OutOfSeq Report is set to 1, the corresponding node shall, if supported, include the NR PDCP PDU sequence number successfully delivered out of sequence in the DL DATA DELIVERY STATUS to the node hosting the NR PDCP entity.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost NR-U packets assist the node hosting the NR PDCP entity in avoiding NR PDCP HFN de-synchronisation. If a deployment decides to not use the Transfer of Downlink User Data procedure, NR PDCP HFN synchronization should be ensured by other means.

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

CHANGE ENDS

# Annex B - TPs for timely retransmission

## Annex B.1 – TP for BL CR for TS 38.425

CHANGE STARTS

### 5.4.1	Transfer of Downlink User Data

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs.

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

The corresponding node shall detect whether an NR-U packet was lost and memorise the respective sequence number after it has declared the respective NR-U packet as being "lost".

The corresponding node shall transfer the remaining NR PDCP PDUs towards the UE and memorise the highest NR PDCP PDU sequence number of the NR PDCP PDU that was successfully delivered (as defined in TS 36.322 [6] and TS 38.322 [7]) in sequence towards the UE (in case RLC AM is used) and the highest NR PDCP PDU sequence number of the NR PDCP PDU that was transmitted to the lower layers.

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered. The DL DATA DELIVERY STATUS sent as a response to a specific DL report NR PDCP PDU SN shall be sent only when all PDCP PDU SNs up to this DL report NR PDCP PDU have been successfully delivered in-sequence.

If the Request OutOfSeq Report is set to 1, the corresponding node shall, if supported, include the NR PDCP PDU sequence number successfully delivered out of sequence in the DL DATA DELIVERY STATUS to the node hosting the NR PDCP entity.

NOTE:	The Transfer of Downlink User Data procedure and the associated feedback of lost NR-U packets assist the node hosting the NR PDCP entity in avoiding NR PDCP HFN de-synchronisation. If a deployment decides to not use the Transfer of Downlink User Data procedure, NR PDCP HFN synchronization should be ensured by other means.

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

The node hosting the NR PDCP entity can indicate to the corresponding node to either retransmit all NR PDCP PDUs up to and including a defined DL retransmission NR PDCP PDU SN or retransmit one or a number of blocks of downlink NR PDCP PDUs, as specified in TS 38.322 [7].

The node hosting the NR PDCP entity can indicate to the corresponding node to either trigger polling for all NR PDCP PDUs up to and including a defined DL polling NR PDCP PDU SN or trigger polling for one or a number of blocks of downlink NR PDCP PDUs, as specified in TS 38.322 [7].

Figure 5.4.1.1-1: Successful Transfer of Downlink User Data

NEXT CHANGE

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined e.g. to allow the corresponding node to detect lost NR-U packets and may be associated with the transfer of a Downlink PDCP PDU.

The following shows the respective DL USER DATA frame.

NOTE 1:	All information elements defined in Figure 5.5.2.1-1 are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

| Bits |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 |  | 5 |  |  | 4 |  | 3 |  | 2 |  | 1 |  |  | 0 |  |
| PDU Type (=0) |  |  |  |  |  |  |  | Spare |  | DL Discard Blocks |  | DL Flush |  |  | Report polling | 1 |
| New IE Flag | BSSI |  |  | TTNBI |  | Request OutofSeq Report |  | Report Delivered |  | User data existence flag |  | Assistance Info. Report Polling Flag |  |  | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| BSSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| TTNB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 2 |
| New IE Flag 7(E) |  | New IE Flag 6 |  |  | New IE Flag 5 |  | New IE Flag 4 |  | New IE Flag 3 |  | New IE Flag 2 |  | New IE Flag 1 | New IE Flag 0 |  | 0 or 1 New IE Flags Octet |
| DL retransmission NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL retransmission Number of blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL retransmission NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmission Block size (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL retransmission NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmission Block size (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL polling NR PDCP PDU SN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL polling Number of blocks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| DL polling NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Polling Block size (first block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL polling NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Polling Block size (last block) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

The New IE Flag in bit 7 of 2nd octet in DL USER DATA (PDU Type 0) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL retransmission NR PDCP PDU SN is present (1) or not (0).

Bit 1 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL retransmission Number of blocks, DL retransmission NR PDCP PDU SN start and Retransmission Block size are present (1) or not (0).

Bit 2 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL polling NR PDCP PDU SN is present (1) or not (0).

Bit 3 of New IE Flags Octet in DL USER DATA (PDU Type 0) indicates if the DL polling Number of blocks, DL polling NR PDCP PDU SN start and Polling Block size are present (1) or not (0).

NEXT CHANGE

#### 5.5.3.a	DL retransmission NR PDCP PDU SN

Description: This parameter indicates the downlink NR PDCP PDU sequence number up to and including which all the NR PDCP PDUs should be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.b	DL retransmission NR PDCP PDU SN start

Description: This parameter indicates the starting SN of a downlink NR PDCP PDU block to be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.c	DL retransmission Number of blocks

Description: This parameter indicates the number of NR PDCP PDU blocks to be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {1..244}.

Field length: 1 octet.

#### 5.5.3.d	Retransmission Block size

Description: This parameter indicates the number of NR PDCP PDUs counted from the starting SN to be considered for remaining time based retransmission, as specified in TS 38.322 [7].

Value range: {1..255}.

Field length: 1 octet.

#### 5.5.3.e	DL polling NR PDCP PDU SN

Description: This parameter indicates the downlink NR PDCP PDU sequence number up to and including which all the NR PDCP PDUs should be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.f	DL polling NR PDCP PDU SN start

Description: This parameter indicates the starting SN of a downlink NR PDCP PDU block to be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.g	DL polling Number of blocks

Description: This parameter indicates the number of NR PDCP PDU blocks to be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {1..244}.

Field length: 1 octet.

#### 5.5.3.h	Polling Block size

Description: This parameter indicates the number of NR PDCP PDUs counted from the starting SN to be considered for remaining time based polling, as specified in TS 38.322 [7].

Value range: {1..255}.

Field length: 1 octet.

CHANGES END

## Annex B.2 – TP for BL CR for TS 38.473

CHANGES START

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU discarding for the indicated DRB.

If the DL Remaining Time Based Retransmission IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to perform remaining time based retransmission for the indicated DRB as specified in TS 38.322 [x] and TS 38.425 [y].

If the DL Remaining Time Based Polling IE is included in the DRB To Be Setup List IE, the gNB-DU shall, if supported, take it into account to perform remaining time based polling for the indicated DRB as specified in TS 38.322 [x] and TS 38.425 [y].

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

------ Unchanged part skipped ------

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for the UE’s Ranging and Sidelink Positioning service.

For each GBR QoS flow whose DRB has been successfully established and the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and perform available bitrate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established, if the MMSID IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [21] and TS 38.300 [6].

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

NEXT CHANGE

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

------ Unchanged part skipped ------

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the PSI based SDU Discard UL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE and the value is set as "start", the gNB-DU shall, if supported, take it into account to perform UL PSI based SDU discarding activation or deactivation for the indicated DRB as defined in TS 38.321 [16].

If the PSI based SDU Discard DL IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, take it into account to provide suggestion on whether to activate or deactivate DL PSI based SDU for the indicated DRB.

If the DL Remaining Time Based Retransmission IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, take it into account to perform remaining time based retransmission for the indicated DRB as specified in TS 38.322 [x] and TS 38.425 [y].

If the DL Remaining Time Based Polling IE is included in the DRB To Be Setup List IE or the DRB To Be Modified List IE, the gNB-DU shall, if supported, take it into account to perform remaining time based polling for the indicated DRB as specified in TS 38.322 [x] and TS 38.425 [y].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

------ Unchanged part skipped ------

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

For each GBR QoS flow whose DRB has been successfully established or modified and the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and, perform available bitrate monitoring, as specified in TS 23.501 [21].

For each QoS flow whose DRB to be established or modified, if the MMSID IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the QoS flow is related to a multi-modal service, as described in TS 23.501 [21] and TS 38.300 [6].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

NEXT CHANGE

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>>>DL Remaining Time Based Retransmission | O |  | ENUMERATED (configured, not-configured…) | Indicates whether DL remaining time based retransmission is configured or not for the DRB. | YES | ignore |
| >>>>DL Remaining Time Based Polling | O |  | ENUMERATED (configured, not-configured…) | Indicates whether DL remaining time based polling is configured or not for the DRB. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

NEXT CHANGE

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>>>DL Remaining Time Based Retransmission | O |  | ENUMERATED (configured, not-configured…) | Indicates whether DL remaining time based retransmission is configured or not for the DRB. | YES | ignore |
| >>>>DL Remaining Time Based Polling | O |  | ENUMERATED (configured, not-configured…) | Indicates whether DL remaining time based polling is configured or not for the DRB. | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>PSI based SDU Discard DL | O |  | ENUMERATED (configured, not-configured, …) | Indicates whether DL PSI based SDU discard is configured or not for the DRB. | YES | ignore |
| >>>>DL Remaining Time Based Retransmission | O |  | ENUMERATED (configured, not-configured…) | Indicates whether DL remaining time based retransmission is configured or not for the DRB. | YES | ignore |
| >>>>DL Remaining Time Based Polling | O |  | ENUMERATED (configured, not-configured…) | Indicates whether DL remaining time based polling is configured or not for the DRB. | YES | ignore |
| ------ Unchanged part skipped ------ |  |  |  |  |  |  |

CHANGES END

## Annex B.3 – TP for BL CR for TS 37.483

CHANGES START

#### 9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP SN UL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN UL size in bits. Corresponds to information provided in the pdcp-SN-SizeUL contained in the PDCP-Config IE as defined in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [33] for eNB CP-UP separation. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - |  |
| PDCP SN DL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN DL size in bits. Corresponds to information provided in the pdcp-SN-SizeDL contained in the PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [33] for eNB CP-UP separation. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - |  |
| RLC mode | M |  | ENUMERATED (RLC-TM, RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, …) | Indicates the RLC mode for the DRB. For more information see PDCP-Config IE in TS 38.331 [10] for gNB or ng-eNB CP-UP separation, or in TS 36.331 [33] for eNB CP-UP separation. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - |  |
| ROHC Parameters | O |  | 9.3.1.40 |  | - |  |
| T-Reordering Timer | O |  | 9.3.1.41 |  | - |  |
| Discard Timer | O |  | 9.3.1.42 | This IE is ignored if the Discard Timer Extended IE is present. | - |  |
| UL Data Split Threshold | O |  | 9.3.1.43 |  | - |  |
| PDCP Duplication | O |  | ENUMERATED (True, …) | Indicates whether PDCP duplication is to be configured for the DRB. This IE is ignored when the “Additional PDCP duplication Information” IE is present. | - |  |
| PDCP Re-establishment | O |  | ENUMERATED (true,…) | Indicates PDCP entity re-establishment to be triggered as defined in TS 38.323 [17] for gNB or ng-eNB CP-UP separation, or in TS 36.323 [34] for eNB CP-UP separation. | - |  |
| PDCP Data Recovery | O |  | ENUMERATED (true,…) | Indicates PDCP data recovery to be triggered as defined in TS 38.323 [17] for gNB or ng-eNB CP-UP separation, or in TS 36.323 [34] for eNB CP-UP separation. | - |  |
| Duplication Activation | O |  | ENUMERATED ( Active, Inactive, …) | Information on the initial state of DL PDCP duplication | - |  |
| Out Of Order Delivery | O |  | ENUMERATED (true,…) | Indicates whether or not outOfOrderDelivery specified in TS 38.323 [17] is configured. Out of order delivery is configured only when the radio bearer is established for gNB or ng-eNB CP-UP separation, or indicates whether or not rlc-OutOfOrderDelivery in TS 36.323 [34] is configured for eNB CP-UP separation. | - |  |
| PDCP Status Report Indication | O |  | ENUMERATED (downlink, uplink, both, …) | For AM DRB, “downlink” indicates that the PDCP entity is configured to send PDCP status report(s) to the UE, and “uplink” indicates that the UE is configured to send PDCP status report(s), as specified in TS 38.323 [17] for gNB or ng-eNB CP-UP separation, or in TS 36.323 [34] for eNB CP-UP separation. “both” indicates that both “downlink” and “uplink” should be applied. | YES | ignore |
| Additional PDCP duplication Information | O |  | ENUMERATED (three, four, …) | Indicates the number of PDCP duplication configured when it is more than 2 for the DRB | YES | ignore |
| EHC Parameters | O |  | 9.3.1.90 |  | YES | ignore |
| UDC Parameters | O |  | 9.3.1.104 |  | YES | ignore |
| Discard Timer Extended | O |  | 9.3.1.128 |  | YES | reject |
| PSI Based Discard Timer | O |  | 9.3.1.146 |  | YES | ignore |
| PDCP SN Gap Report | O |  | ENUMERATED (true, …) | Indicates whether the PDCP entity is configured to send a PDCP SN gap report in the downlink, as specified in TS 38.323 [17]. | YES | ignore |
| Remaining Time Based Retransmission Threshold | O |  | INTEGER (1..64, …) | Indicates the threshold to trigger remaining time based retransmission, as specified in TS 38.322 [x]. Corresponds to information provided in the remainingTimeBasedReTxThreshold contained in the PDCP-Config as defined in TS 38.331 [10]. Unit: millisecond. | YES | ignore |
| Remaining Time Based Polling Threshold | O |  | INTEGER (1..64, …) | Indicates the threshold to trigger remaining time based polling, as specified in TS 38.322 [x]. Corresponds to information provided in the remainingTimeBasedPollingThreshold contained in the PDCP-Config as defined in TS 38.331 [10]. Unit: millisecond. | YES | ignore |

CHANGES END