3GPP TSG-RAN WG3 #129	R3-255420

Bengaluru - India, 25 – 29 August 2025

Agenda Item:	18.2. Support LP-WUS Indicating Paging Monitoring

Source:	NTT DOCOMO INC.

Title:	Further discussion on LP-WUS

Document for:	Discussion, agreement

Introduction

In past meetings, RAN3 achieved following:

[RAN3#125]

- New IEs should be introduced to support LP-WUS on NG, Xn and F1.

[RAN3#125bis]

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

- The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

- The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

[RAN3#126]

- WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

- It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

- NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

- WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

[RAN3#127]

- For the following issues, including Extended UE Identity Index、The maximum value of CN subgroup ID、Emergency call back、the terminology of “LP-WUS”、 the LP-WUS node capability need more inputs from other WGs.

[RAN3#127bis]

- The maximum number of subgroups is decided to 31.

- Turn the working assumption into agreement and remove the FFS on the presence of the LP-WUS Subgrouping Support Indication IE per cell in the F1AP Paging message.

- RAN3 will not send an LS to SA3 regarding this issue.

- Whether Extended UE Identity Index or Hashed UE_ID is used and how many bits are needed for UE_ID is pending on RAN2.

[RAN3#128]

- FFS on the range of LP-WUS CN Subgroup ID IE included in LP-WUSPS Assistance Information pending on RAN1 decision on which subgroup ID(s) would be reserved for common codepoint.

- Whether to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.

- The issue on UE battery drain during LP-WUS monitoring and very long eDRX cycles can be discussed based on companies’ contributions.

This document summaries current situations and discussions.

Discussion

Range of CN based subgroup ID

In the previous meeting, depending on the definition of common codepoint in RAN1, the range of CN based subgroup ID in NG/Xn/F1 interface was FFS. Common codepoint was introduced in RAN1, and this is used to wake up all UEs related to each PO. According to the RAN1#120bis discussion, following agreement was captured, and the definitions of subgroup ID and common codepoint are as follows.

Agreement

For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is

- 7 for the case where 4 POs are mapped to one LO

- 15 for the case where 2 POs are mapped to one LO

Also, the current running RRC CR captures the description of lp-WubgroupsNumPerPO as follows:

LP-SubgroupConfig field descriptions

lp-SubgroupsNumPerPO Total number of subgroups per Paging Occasion (PO) for UE to monitor codepoint from LP-WUS signaling. The field represents the sum of CN-assigned and UEID-based subgroups supported for LP-WUS by the network. When lowPowerConfig is configured, there is always at least one subgroup (UEID-based subgroup or CN-assigned subgroup) configured. The maximum number of subgroups per PO is 31, 15, and 7 when the number of PO(s) associated with one LO for LP-WUS, i.e., lpwus-PoNumPerLo, is configured as 1, 2, and 4, respectively.

In PEI, the subgroupNumPerPO can be configured at maximum 8 at any time. However, based on the above, lp-SubgroupsNumPerPO is dependent on lpwus-PoNumPerLo. This is because common codepoints are defined for each PO per LO, different codepoints are defined for subgroups for each PO per LO, and total number of codepoints should be 32 or less.

Since lpwus-PoNumPerLo depends on the cell which the UE camps on, the CN based subgroup IDs in RAN3 specifications should be able to carry the maximum possible value. Therefore, the range of the CN based subgroup ID should be 0.. 30.

Proposal 1:	The range of CN based subgroup ID in NG/Xn/F1 message is 0.. 30.

Paging capability loss issue

Over the last few meetings, RAN3 has discussed an issue that paging message does not reach UEs which reported UE capability in pre-R19 gNBs. RAN2 sent LS [1] to SA2, RAN3 and CT1 at the last meeting. We also received LS [2] from SA2 regarding this issue.

According to the RAN2 agreement, the capability for R19 LP-WUS will be carried by a new IE within UE Radio Capability for Paging, which requires all gNBs in a specific area to be R19 container capable.

SA2 thinks this restriction on the deployment undesirable and provides an example of potential solution reusing existing mechanisms. Our understanding is that the solution allows the R19 gNB to resolve the UE Radio Capability for Paging mismatch stored in the AMF. However, the resolution of the mismatch can be done only after the Initial Context Setup procedure and the gNB is not aware of this mismatch when it receives the paging message, thus paging loss issues can still occur, as illustrated in the Fig 1.

Fig 1. paging loss scenario and proposed solution by SA2.

If the NW can explicitly activate/deactivate LP-WUS, this problem may be solved. Currently we see no other solution to resolve this issue than relying on NW implementations and/or deployments.

Proposal 2:	the paging loss because of the paging capability loss issue would be resolved based on NW implementation or deployment.

Proposal 3:	the paging capability mismatch would be resolved based on NW implementation or deployment relying on existing functionalities.

LP-WUS handling during emergency PDU session

How to handle LP-WUS functionality when UE/NW is establishing an emergency PDU session was discussed over several meetings. SA2/RAN3 removed the note about the emergency PDU session from the PEI/LP-WUS stage2 specification. Also, at the RAN3#128 meeting, a note was captured regarding eDRX configuration when establishing an emergency PDU session [3]. This is based on RAN3 understanding that gNB can be aware of the emergency PDU session based on ARP value of QoS flow.

Based on the above understanding, the gNB can be aware of the presence of an emergency PDU session, so for example, the NW can decide to let UE remain RRC_CONNECTED. From the operator's point of view, existing solutions such as keeping UE in RRC_CONNECTED state may be sufficient, hence this issue can be solved by NW implementation.

Also, if LP-WUS activation/deactivation is supported, it may be possible to avoid wake up delay while sending the UE to RRC_IDLE state. This is also an alternative, but this solution is up to RAN2.

Proposal 4:	LP-WUS during emergency PDU session between UE and NW would be handled based on NW implementation.

LP-WUS activation/deactivation

According to the previous RAN2 offline discussion on LP-WUS [4], two options for LP-WUS enabling/disabling functions are discussed: RRC based and NAS based solutions. When LP-WUS is enabled/disabled, gNB should decide whether to send WUS accordingly, regardless of whether UE uses CN based or UE_ID based subgroup. For example, when LP-WUS is disabled, UE may need to do RRC_IDLE to CONNECTED transition quickly (e. g. emergency PDU session is established). In this case, gNB should send paging immediately without sending WUS. When LP-WUS is enabled, gNB should send WUS before sending paging. The expected RAN3 impacts for each solution are:

- NAS based solution

According to the summary of RAN2 offline discussion, the LP-WUS enabling/disabling procedure is as shown in Fig 2. In the current NG/Xn/F1 signaling, AMF/gNB can indicate the use of LP-WUS with CN based subgrouping by CN based subgroup ID included in the paging message, however, it cannot notify information about LP-WUS with UE_ID based subgrouping, and gNB cannot decide whether to send WUS properly. Therefore, gNB needs to be aware that LP-WUS is enabled/disabled.

From above, RAN3 impact is to include LP-WUS enabled/disabled indication in NG/Xn/F1 LP-WUSPS Assistance Information.

- RRC based solution

The procedure in this case is shown in Fig 3. In this solution, the gNB which performs RRCRelease knows whether the UE has enabled/disabled LP-WUS, but other gNBs and the AMF do not know this, hence other gNBs cannot transmit WUS appropriately. Therefore, last serving gNB should notify the AMF that the gNB has enabled/disabled LP-WUS by RRCRelease, and other gNBs than the last serving gNB should also be notified that LP-WUS is enabled/disabled.

Expected RAN3 impact from the above is to include the LP-WUS enabled/disabled indication in the LP-WUSPS Assistance Information and the UE Context Release Request.

Fig 2. NAS signaling based LP-WUS enabling/disabling procedure.

Fig 3. RRC signaling based LP-WUS enabling/disabling procedure.

Proposal 5:	If the LP-WUS enabling/disabling feature is supported in RAN2, RAN3 will specify the following enhancements:

      - For NAS solution: introduce LP-WUS enabled/disabled indication within LP-WUSPS Assistance  Information IEs in NG/Xn/F1.

      - For RRC solution: introduce LP-WUS enabled/disabled indication within LP-WUSPS Assistance  Information IEs in NG/Xn/F1 and NGAP UE Context Release Request message.

Further Extended UE Identity Index Value

The use of Further Extended UE Identity Index Value IE introduced in NG/Xn/F1 at the last meeting is unclear. In our understanding, this new IE should be used in case that the UE and the gNB may use LP-WUS, regardless of the use of eDRX/PEI. This way makes receiving node and sending node behavior clear, and the TPs agreed in the last meeting align with this. In addition, sending node has to send both legacy and new UE Identity Index IE if it does not know receiving node’s capability because receiving node may not support LP-WUS or not be able to decode R19 IEs but use eDRX/PEI and in this case new IE cannot be decoded by receiving node but still UE Identity Index may be needed for eDRX/PEI. In case sending node can make it sure that receiving node can decode, only new IE is needed, because, as in the agreed TPs, R19 or later gNBs ignore legacy IE and always use new IE.

Proposal 6:	Further Extended UE Identity Index Value IE is used in case that LP-WUS is used regardless of the use of eDRX/PEI.

Proposal 7:	Further Extended UE Identity Index Value IE is sent with Extended UE Identity Index Value IE if sending node does not know receiving node’s capability of R19 IEs.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	The range of CN based subgroup ID in NG/Xn/F1 message is 0.. 30.

Proposal 2:	the paging loss because of the paging capability loss issue would be resolved based on NW implementation or deployment.

Proposal 3:	the paging capability mismatch would be resolved based on NW implementation or deployment relying on existing functionalities.

Proposal 4:	LP-WUS during emergency PDU session between UE and NW would be handled based on NW implementation.

Proposal 5:	If the LP-WUS enabling/disabling feature is supported in RAN2, RAN3 will specify the following enhancements:

      - For NAS solution: introduce LP-WUS enabled/disabled indication within LP-WUSPS Assistance Information IEs in NG/Xn/F1.

      - For RRC solution: introduce LP-WUS enabled/disabled indication within LP-WUSPS Assistance Information IEs in NG/Xn/F1 and NGAP UE Context Release Request message.

Proposal 6:	Further Extended UE Identity Index Value IE is used in case that LP-WUS is used regardless of the use of eDRX/PEI.

Proposal 7:	Further Extended UE Identity Index Value IE is sent with Extended UE Identity Index Value IE if sending node does not know receiving node’s capability of R19 IEs.

References

- R3-253012, “Reply LS on paging capability loss issue”, RAN2

- R3-255026, “Response to Reply LS on paging capability loss issue”, SA2

- R3-253765, “Correction of Emergency Call and eDRX”, Nokia

- “R2-250xxxx [Post130][222][LPWUS] Enabling Disabling LP-WUS per UE”, Huawei