3GPP TSG-RAN WG3 Meeting #129	R3-255443

Bangalore, India, 25 – 29 August 2025

Agenda Item:	15.2

Source: 				Jio Platforms Limited

Title:			(TP for TS 36.300) IoT‑NTN Switchover (FLSO) & Store‑and‑Forward Principles

Document for:		Agreement

# 1.	Introduction

This document contains the TP for IoT‑NTN Switchover (FLSO) & Store‑and‑Forward Principles for TS 36.300.

# 2.	TP for TS 36.300

This appendix provides the Text proposal for TS 36.300 based on RAN3 agreements.

Reason for change: Introduce LTE/E‑UTRAN anchors for S&F and FLSO with full eNB as regenerative payload; reuse S1/X2 procedures; minimise UE impact.

Affected clauses: Add: 23.21.X Store‑and‑Forward operation.

----------------Start of the First Change---------------

## 23.21.X Store‑and‑Forward operation

### 23.21.X.1 Definitions

Store‑and‑Forward (S&F) operation is a network operation in which, for UEs configured for S&F, the eNB may buffer user plane PDUs per DRB when backhaul towards the EPC is unavailable or degraded.

Store-and-Forward operation may be used in conjunction with feeder link switchover as specified in clause 23.21.5.

### 23.21.X.2 Assumptions

a) S&F operation applies to delay‑tolerant services.

b) Entry to, and exit from, S&F operation may be determined by configuration and/or implementation.

c) Buffer management policy during S&F is implementation‑specific.

d) This subclause does not introduce multiple SCTP associations for IoT‑NTN.

NOTE 1: Prioritisation, retention time and size limits for buffer management may be OAM-configurable; specific values are implementation-dependent.

NOTE 2: Security of buffered data follows existing PDCP/EPC security architecture; no additional security procedures are introduced by this subclause.

This subclause applies to operation with full eNB as regenerative payload for IoT-NTN

### 23.21.X.3 Procedures

Entry to, and exit from, S&F operation for a UE are exposed towards the EPC using S1AP and existing UE‑associated procedures as specified in TS 36.413 (e.g. UE CONTEXT MODIFICATION in clause 8.3.4 and PATH SWITCH REQUEST in clause 8.4.4).

UE configuration aspects to enable or adjust S&F operation are specified in TS 36.331.

-----------------End of the First Change---------------