3GPP TSG-RAN WG3 Meeting #129	R3-255453

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	11.4

Source:	Nokia

Title:	(TP to TS 38.423) Remaining issues for AI/ML Mobility Optimization in NR-DC

Document for:	Text Proposal

# 1	Introduction

In this section we summarize earlier agreements that relate to the AI/ML Mobility Optimization for NR-DC topic. In RAN3 #126 we made the following agreements regarding the procedures to trigger the collection of UE Performance measurements for NR-DC:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

In addition, in RAN3 #127bis we made the following agreements:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following list of exit conditions to stop the collection of UE performance at SN side:

- the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- the NR-DC with the S-NG-RAN node for the UE is released.

In RAN3 #128, and after receiving an LS from SA5 (R3-253824), we had a discussion regarding which entity measures UE Performance. We discussed that UE Performance measured at PDCP layer would simplify Rel-19 discussions and it is also beneficial since it provides service level performance of the UE. Supporting this observation, SA5, in their LS, also recognised the importance of monitoring packet loss at the PDCP level in both UL and DL directions. However, they expressed concern about a UE Throughput measurement at PDCP layer both in UL and DL directions. Based on the input from SA5 in the last meeting we agreed the following:

- Introduce new IE to include DL and UL Packet Loss Rate at PDCP Level in R19 refer to SA5 spec

- DL/UL UE Throughput Measurement will not be supported in split architecture in R19.

Note that when it comes to the packet delay measurement, in RAN3 #127bis we had agreed that the total delay is aggregated in the gNB-CU-UP (following similar principles as the QoS monitoring framework) and reported to the gNB-CU-CP upon request:

Packet delay measured for UE Performance feedback is sent from DU to CU via ASSISTANCE INFORMATION DATA frame over F1-U in the case of CU-DU split architecture.

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

In RAN3 #127bis, the following open points were also captured:

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In this contribution, we provide further views on UE Performance measurements in NR-DC. In addition, we provide our views on the open topic of providing assistance information from MN to SN. Finally, we discuss the effect of SCG activation and deactivation for the UE Performance data collection.

# 2	Discussion

## 2.1 UE performance measurements in NR-DC

Over the last meetings, different options have been discussed on the granularity of UE Performance metrics exchanged over the interfaces for the different use cases discussed in NG-RAN AI/ML. We discussed the granularity of sending UE Performance metrics over Xn for AI/ML Slicing and for AI/ML Mobility Optimization in NR-DC scenarios. We also discussed the granularity of sending UE Performance metrics in split architecture for the support of the Rel-18 use cases. In particular, we had lengthy discussions on several granularity options, such as per UE, per slice, per DRB, or per group of DRBs. In addition, we further discussed which is the entity that measures those metrics, namely whether it is the RLC (gNB-DU) or the PDCP layer (gNB-CU-UP). In the last meeting we agreed that PDCP entity measures UL and DL packet loss rate and that UL/DL throughput is not supported in split architecture. We had earlier agreed that delay is measured at the PDCP layer (gNB-CU-UP), by enhancing QoS monitoring framework and reported to gNB-CU-CP upon request.

In our view, also for the AI/ML Mobility Optimization it is important to measure UE Performance at the PDCP. This is because if the measurements are collected at PDCP layer, then the measurements can always be collected from a single layer and single node, i.e., from PDCP Layer in Master Node in case of MN-Terminated MCG, SCG or Split bearer and from PDCP Layer in Secondary Node in case of SN-Terminated MCG, SCG or Split bearer. Master Node configures Data Collection initiation and reporting to the Secondary Node only if there are one or more SN-Terminated bearers. Otherwise, MN can measure UE Performance measurements from its own PDCP layer.

Therefore, we think that it is essential to measure UE Performance at PDCP layer and we see the discussions and agreements on UE Performance for split architecture also relevant for the use case of Mobility Optimization for NR-DC. Similarly to split architecture, we believe that we should not consider reporting of UL/DL Average Throughput in NR-DC scenarios since it will not be feasible to aggregate the different RLC components of the measurement at the MN in case of split bearer.

Observation 1: It is not possible to aggregate the different RLC components of the UL/DL Average Throughput measurement at the MN in case of split bearer in NR-DC scenarios.

Proposal 1: For the AI/ML Mobility Optimization in NR-DC use case consider reporting only the following measurements, measured at PDCP:

- UL/DL Average Packet Delay

- UL/DL Packet Loss Rate

When it comes to UE Performance granularity in NR-DC, we think that the full PDU session may not be terminated at a single node. It is possible that some DRBs of the PDU session are terminated in the MN PDCP and some in the SN PDCP. Similarly, in case multiple PDU sessions are set up at the UE, some of these sessions may be terminated in the MN and other may be terminated in the SN.

Observation 2: It is possible that some DRBs of the PDU session are terminated in MN PDCP and some in SN PDCP so that it is not possible to report UE Performance per-slice or per UE from the SN to the MN. Also, reporting at a per UE granularity is not possible if MN and SN serve different PDU sessions for the UE.

Therefore, we think that it is necessary to measure UE Performance metrics in NR-DC per DRB.

Proposal 2: In AI/ML Mobility Optimization in NR-DC measure UE Performance per DRB.

Reporting of UE Packet Loss Rate and Average Packet Delay is currently supported per DRB from the gNB-CU-UP to the gNB-CU-CP (E1), and it would therefore be straight-forward for the SN to forward the received per-DRB information to the MN. If usage of this information in the MN requires averaging or aggregation of the per-DRB information into e.g. per-UE information or per-slice information, this can be done in the MN following the same algorithm as used for single connectivity. Initial averaging or aggregation of the per-DRB information in the SN would introduce an additional processing step compared to single connectivity, and the resulting UE performance information would then not be comparable between UEs served in single connectivity and UEs served in dual connectivity.

Proposal 3: For AI/ML Mobility Optimization in NR-DC, report UE Performance measurements per DRB over E1 and from SN to MN over Xn.

We provide the required stage 3 updates in the XnAP TP in the annex of this paper.

## 2.2 SCG Activation and Deactivation impact on Data Collection

In TS 38.423, we have captured the following interaction between the Data Collection Reporting and Data Collection Reporting Initiation procedures considering the S-Node Addition trigger for NR-DC scenario:

While there is only a single procedure controlling the handover (Handover Preparation procedure), in Dual Connectivity there exist two independent processes, namely S-NG-RAN node Addition and SCG Activation/Deactivation (see TS 37.340, Clause 7.13). SCG activation can be requested by the MN, by the SN, and by the UE. SCG deactivation can be requested by the MN and by the SN. For SN-terminated bearers, this allows to establish an endpoint for data transfer in the SN (or the gNB-CU-UP of the SN) immediately as part of QoS flow setup or incoming handover without necessarily requiring the UE to actively monitor the SCG (e.g., PDCCH decoding) in periods without data. Therefore, it is possible to indicate whether the SCG resources are required to be activated or deactivated when S-NODE ADDITION REQUEST is sent from MN to SN. SCG can also be independently activated or deactivated by the SN. Only when the SCG resources are activated, data may actually be transferred over the SCG and corresponding UE Performance measurements can be collected. We therefore believe that UE Performance measurements for bearers handled over the SCG leg should only be collected when SCG resources are activated. Some examples on scenarios are listed next:

- Case #1: MN can add an SN together with an active SCG resources using XnAP: S-NODE ADDITION REQUEST message with SCG Activation Request=activate SCG. In this case the UP data flow and UE performance data collection can start as soon as the SN addition procedure is successful.

- Case #2: MN can add a SN with SCG resources de-activated using XnAP: S-NODE ADDITION REQUEST message with SCG Activation Request=de-activate SCG. In this case the UP data flow using the SCG resources is not started and hence any UE Performance data such as packet delay collected in the SN would provide 0 measurement results.

- Case #3: After a successful SN addition with de-activated SCG resources, MN can subsequently activate the SCG resources using S-Node Modification Request (w/SCG Activation Request=activate SCG). After this, UP data flow starts using SCG resources and the UE Performance data collected after the activation only is meaningful for any evaluation in the MN.

- Case #4: After a successful SN addition with de-activated SCG resources, SN can subsequently activate the SCG resources using S-Node Modification Required (w/SCG Activation Request). After this, UP data flow starts using SCG resources and the UE Performance data collected after the activation only is meaningful for any evaluation in the MN.

So, we see from the above examples that data collection should not start after successful S-NG-RAN node addition but only when SCG is activated. In the last meeting, there were some proposals indicating the need for additional conditions as triggers to start data collection such as SCG activation. Accordingly, an FFS was captured in Chair notes:

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

Another alternative to those additional conditions would be to keep the conditions to start the data collection of UE Performance unchanged, but enable the SN to handle such scenarios by skipping the corresponding UE measurements. This second alternative would allow handling of this issue without additional triggers. We would like to note here that this issue is present irrespectively of whether the measurements are made in PDCP layer or not. If the secondary node continues the reporting under a deactivated SCG, it can only report zero UE Performance since there is no UP traffic. In that case, the MN cannot interpret whether a reported zero measurement is due to the lack of traffic or due to the channel conditions, namely MN may consider that there is 0 packet loss if there were no packets transmitted.

Observation 2: In case of inactive SCG bearer, UE Performance Reporting by the SN will result in useless measurements, e.g., measurements where the reported UE Performance is zero which cannot be interpreted by the MN, namely whether a zero measurement is due to the lack of traffic or due to the channel conditions.

Furthermore, if the SN uses the measurements collected during deactivated SCG for the averaging together with other samples in a reporting period it will impact the accuracy of the reported measurements.

Observation 3: If the SN uses the measurements collected during deactivated SCG for the averaging together with other samples in a reporting period it will impact the accuracy of the reported measurements.

Consider the example in Figure 1 which illustrates a scenario where SN is added but SCG is deactivated during SN addition and is activated later during the second reporting period. If SN reports periodically at T1 and T2 then it can only report measurements with 0 values since there does not exist any UP Traffic while SCG is deactivated. When SCG is activated again reporting can resume. The measurement at T3 may still not have enough samples since SCG is only activated in the middle of the reported period. If the SN determines that it cannot have enough samples for an accurate measurement then it may skip also T3 measurement and resume reporting at T4, T5, …

Figure 1 Example of Data Collection with SCG Deactivation

Observation 4: Measurement reporting from SN to MN for a given UE in NR-DC can be skipped in one or more reporting periods taking into account the amount of useful samples in a reporting period, i.e., samples collected after SCG activation.

Proposal 4: Update the text in TS 38.423 as follows:

UE Performance measurements for a given UE in NR-DC need not be sent from SN to MN in one or more reporting period(s) in case of absence of measurements.

## 2.3 Assistance Information from MN to SN

One topic that was discussed in previous meetings was sending the predicted (or assumed) PSCell ID from MN to SN during Dual Connectivity. Next, we highlight the procedure with which a PSCell is selected. The SN selects the target PSCell among the reported cells by the UE, considering the radio quality, load of the reported cells, UE Capability, etc. The SN consequently further checks based on the band combination, the feature set combination to select one combination that maximizes the throughput. Thus, the PSCell selection is up to target SN and its internal logic.

Observation 5: PSCell selection is up to target SN and its internal logic.

The MN can get a list of visited PSCells for a UE from SCG UE History Information IE. However, it cannot know which were the reasons that led SN to choose those PSCells. For example, when establishing NR-DC for two different UE(s), SN can choose two different PSCell(s) because the actual selection algorithm in SN considers their bearer characteristics and UE Capabilities (e.g., supported band combinations) amongst other considerations (e.g. load). Thus, MN cannot accurately predict a PSCell since it is missing input on the internal logic that led SN into its PSCell decisions at each time.

Observation 6: An MN attempt to predict the PSCell ID chosen by the SN without considering the SN internal logic that resulted in the PSCell selection will result in inaccurate PSCell predictions.

Any attempt to predict by the MN the SN PSCell selection process would mean that that MN needs to be provided along with SCG UE History Information, all the associated criteria taken into consideration by the SN including load and deployment aspects or other implementation specific details. This, makes it very complex if not impossible for the MN to predict or even assume the PSCell choices made by the SN.

Observation 7: It is very complex if not impossible for the MN to predict (or assume) the PSCell choices made by the SN.

For the above reasons, the predicted PSCell ID does not provide assistance information that can help optimize the NR-DC decisions. One related discussed issue was whether to enhance the UE performance IE in the DATA COLLECTION UPDATE message with the actual PSCell ID. Similar to the arguments above, since PSCell is dependent on SN implementation the PSCell ID would not provide useful information to the MN that can help evaluate the decision of triggering Dual Connectivity with the SN.

Observation 8: Enhancing the UE performance IE in the DATA COLLECTION UPDATE message with the actual PSCell ID would not provide the MN with useful information that could help it evaluate the Dual Connectivity decision.

Proposal 5: RAN3 to agree not to consider transferring of assistance information from MN to SN.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: It is not possible to aggregate the different RLC components of the UL/DL Average Throughput measurement at the MN in case of split bearer in NR-DC scenarios.

Proposal 1: For the AI/ML Mobility Optimization in NR-DC use case consider reporting only the following measurements, measured at PDCP:

- UL/DL Average Packet Delay

- UL/DL Packet Loss Rate

Observation 2: It is possible that some DRBs of the PDU session are terminated in MN PDCP and some in SN PDCP so that it is not possible to report UE Performance per-slice or per UE from the SN to the MN. Also, reporting at a per UE granularity is not possible if MN and SN serve different PDU sessions for the UE.

Proposal 2: In AI/ML Mobility Optimization in NR-DC measure UE Performance per DRB.

Proposal 3: For AI/ML Mobility Optimization in NR-DC, report UE Performance measurements per DRB over E1 and from SN to MN over Xn.

Observation 2: In case of inactive SCG bearer, UE Performance Reporting by the SN will result in useless measurements, e.g., measurements where the reported UE Performance is zero which cannot be interpreted by the MN, namely whether a zero measurement is due to the lack of traffic or due to the channel conditions.

Observation 3: If the SN uses the measurements collected during deactivated SCG for the averaging together with other samples in a reporting period it will impact the accuracy of the reported measurements.

Observation 4: Measurement reporting from SN to MN for a given UE in NR-DC can be skipped in one or more reporting periods taking into account the amount of useful samples in a reporting period, i.e., samples collected after SCG activation.

Proposal 4: Update the text in TS 38.423 as follows:

UE Performance measurements for a given UE in NR-DC need not be sent from SN to MN in one or more reporting period(s) in case of absence of measurements.

Observation 5: PSCell selection is up to target SN and its internal logic.

Observation 6: An MN attempt to predict the PSCell ID chosen by the SN without considering the SN internal logic that resulted in the PSCell selection will result in inaccurate PSCell predictions.

Observation 7: It is very complex if not impossible for the MN to predict (or assume) the PSCell choices made by the SN.

Observation 8: Enhancing the UE performance IE in the DATA COLLECTION UPDATE message with the actual PSCell ID would not provide the MN with useful information that could help it evaluate the Dual Connectivity decision.

Proposal 5: RAN3 to agree not to consider transferring of assistance information from MN to SN.

We provide a TP to TS 38.423 in annex A of this paper.

# Annex A			  TP for BL CR to TS 38.423

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.420: "NG-RAN; Xn General Aspects and Principles".

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

[XX]	3GPP TS 28.558: "UE level measurements for 5G system".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Skipped >>>>>>>>>>>>>>>>>>>>

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	the time since S-NG-RAN node addition successfully completed is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the NR-DC with the S-NG-RAN node for the UE is released.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message. The result of UE Performance collection for a given UE in NR-DC need not be sent from SN to MN in one or more reporting period(s) in case of absence of measurements.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2. If the Slice To Report List for Data Collection IE is included for a cell, the Predicted Radio Resource Status IE for such cell shall, if supported, include the Slice Radio Resource Status Item IE.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity or as MN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the DRB Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE as SN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity or as MN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the DRB Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE as SN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Slice Available Capacity Group IE, if the tenth bit, "Predicted Slice Available Capacity" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1"and if

-	the Slice To Report List for Data Collection IE is included for the cell, and

-	the measurement object is admitted by NG-RAN node2.

If the Cell Capacity Class Value Uplink IE and the Cell Capacity Class Value Downlink IE are included within the Predicted Slice Available Capacity Group IE for the cell for which the Predicted Slice Available Capacity IE is reported, these IEs are used to assign weights to the available capacity indicated in the requested predicted Slice Available Capacity Value Uplink IE and predicted Slice Available Capacity Value Downlink IE respectively.

-	the Slice Average UE Throughput DL IE, if the eleventh bit, "Slice Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity and if the measurement object is admitted by NG-RAN node2

-	the Slice Average UE Throughput UL IE, if the twelfth bit, "Slice Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average Packet Delay IE, if the thirteenth bit, "Slice Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity or as MN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the DRB Average Packet Delay IE, if the thirteenth bit, "Slice Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE as SN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average Packet Loss DL IE, if the fourteenth bit, "Slice Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity or as MN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the DRB Average Packet Loss DL IE, if the fourteenth bit, "Slice Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE as SN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the Slice Average Packet Loss UL IE, if the fifteenth bit, "Slice Average Packet Loss UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE in single connectivity or as MN in NR-DC and if the measurement object is admitted by NG-RAN node2.

-	the DRB Average Packet Loss UL IE, if the fifteenth bit, "Slice Average Packet Loss UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1", the NG-RAN node2 serves the UE as SN in NR-DC and if the measurement object is admitted by NG-RAN node2.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity, Eleventh Bit = Slice Average UE Throughput DL, Twelfth Bit = Slice Average UE Throughput UL, Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL, Fifteenth Bit = Slice Average Packet Loss UL. Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory Seventh Bit = Slice Average UE Throughput DL, Eighth Bit = Slice Average UE Throughput UL, Ninth Bit = Slice Average Packet Delay Tenth Bit = Slice Average Packet Loss DL, Eleventh Bit = Slice Average Packet Loss UL. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| >>Slice Measurement Initiation Result | O |  | 9.2.3.x1 | List of measurement objects that failed to be initiated per slice. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.3.x3 |  | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| >>Slice UE Performance | O |  | 9.2.3.x2 |  | YES | ignore |
| >>DRB UE Performance | O |  | 9.2.3.x5 |  | YES | ignore |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x5	DRB UE Performance

This IE indicates per DRB UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB UE Performance List |  | 1 |  |  |
| >DRB UE Performance Item |  | 1..<maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |
| >>DRB Based UE Performance | M |  | 9.2.3.x6 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x6	DRB Based UE Performance

This IE indicates the UE performance measurements metrics reported at per-DRB granularity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Average Packet Delay | O |  | Average Packet Delay 9.2.3.187 |  |
| DRB Average Packet Loss DL | O |  | INTEGER (0..1000000) | Corresponds to DL PDCP SDU Loss Rate per DRB |
| DRB Average Packet Loss UL | O |  | INTEGER (0..1000000) | Corresponds to UL PDCP SDU Loss Rate per DRB as specified in TS 28.558 [XX] clause 6.3.1.3.1 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedReportIntervalMDT,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-LTEA2XUEPC5AggregateMaximumBitRate,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-PDUSession-PairID,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRA2XUEPC5AggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QoSMonitoringDisabled,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-Cause,

id-SecurityIndication,

id-RRCConnReestab-Indicator,

id-SourceDLForwardingIPAddress,

id-SourceNodeDLForwardingIPAddress,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-BeamMeasurementIndicationM1,

id-Supported-MBS-FSA-ID-List,

id-MBS-AssistanceInformation,

id-MBS-SessionAssociatedInformation,

id-MBS-SessionInformation-List,

id-SliceRadioResourceStatus-List,

id-CompositeAvailableCapacitySupplementaryUplink,

id-SSBOffsets-List,

id-NG-RANnode2SSBOffsetsModificationRange,

id-NR-U-Channel-List,

id-NR-U-ChannelInfo-List,

id-MIMOPRBusageInformation,

id-UEAssistantIdentifier,

id-IAB-MT-Cell-List,

id-NoPDUSessionIndication,

id-permutation,

id-UL-GNB-DU-Cell-Resource-Configuration,

id-DL-GNB-DU-Cell-Resource-Configuration,

id-tdd-GNB-DU-Cell-Resource-Configuration,

id-Additional-Measurement-Timing-Configuration-List,

id-SurvivalTime,

id-Local-NG-RAN-Node-Identifier,

id-Neighbour-NG-RAN-Node-List,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-Redcap-Bcast-Information,

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-Full-and-Short-I-RNTI-Profile-List,

id-QosFlowMappingIndication,

id-EquivalentSNPNs,

id-CHOTimeBasedInformation,

id-ChannelOccupancyTimePercentageUL,

id-EnergyDetectionThresholdUL,

id-PSCellListContainer,

id-RadioResourceStatusNR-U,

id-FiveGProSeLayer2Multipath,

id-FiveGProSeLayer2UEtoUERelay,

id-FiveGProSeLayer2UEtoUERemote,

id-ClockQualityReportingControlInfo,

id-CapabilityForBATAdaptation,

id-PNI-NPNBasedMDT,

id-PNI-NPN-AreaScopeofMDT,

id-SNPN-CellBasedMDT,

id-SNPN-TAIBasedMDT,

id-SNPN-BasedMDT,

id-S-CPAC-Request,

id-S-CPAC-Request-Info,

id-S-CPAC-ReferenceConfigRequest,

id-S-CPAC-InterSN-ExecutionNotify,

id-S-CPAC-dataforwardinginfofromSource,

id-CPACcandidatePSCells-wotherInfo-list,

id-eRedcap-Bcast-Information,

id-NRPagingLongeDRXInformationforRRCINACTIVE,

id-MBSCommServiceType,

id-AssistanceInformationQoE-Meas,

id-QoERVQoEReportingPaths,

id-DirectForwardingPathAvailability,

id-CHO-CPAC-Info,

id-CHO-Maxnoof-CondReconfig,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-TAISliceUnavailableCellList,

id-MobileIABCell,

id-XR-Bcast-Information,

id-MaximumDataBurstVolume,

id-CPAC-Preparation-Type,

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-SliceToReportForDataCollection-List,

id-PredictedSliceAvailableCapacity,

id-SliceMeasurementInitiationResult,

id-SliceUEPerformance,

id-DRBUEPerformance,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNIDs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofQoSParaSets,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofExtSliceItems,

maxnoofSNPNIDs,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods,

maxnoofslots,

maxnoofExtTLAs,

maxnoofGTPTLAs,

maxnoofCHOcells,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofRAReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationstate,

maxnoofBluetoothName,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofTAforMDT,

maxnoofWLANName,

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofUEIDIndicesforMBSPaging,

maxnoofMBSFSAs,

maxnoofMBSQoSFlows,

maxnoofMRBs,

maxnoofCellsforMBS,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofAssociatedMBSSessions,

maxnoofMBSSessions,

maxnoofSuccessfulHOReports,

maxnoofPSCellsPerSN,

maxnoofNR-UChannelIDs,

maxnoofCellsinCHO,

maxnoofCHOexecutioncond,

maxnoofServingCells,

maxnoofBHInfo,

maxnoofTLAsIAB,

maxnoofTrafficIndexEntries,

maxnoofBAPControlPDURLCCHs,

maxnoofServedCellsIAB,

maxnoofDUFSlots,

maxnoofSymbols,

maxnoofHSNASlots,

maxnoofRBsetsPerCell,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofPSCellCandidates,

maxnoofTargetSNs,

maxnoofUEAppLayerMeas,

maxnoofSNSSAIforQMC,

maxnoofCellIDforQMC,

maxnoofPLMNforQMC,

maxnoofTAforQMC,

maxnoofMTCItems,

maxnoofCSIRSconfigurations,

maxnoofCSIRSneighbourCells,

maxnoofCSIRSneighbourCellsInMTC,

maxnoofNeighbour-NG-RAN-Nodes,

maxnoofSRBs,

maxnoofSMBR,

maxnoofNSAGs,

maxnoofRBsetsPerCell1,

maxnoofTargetSNsMinusOne,

maxnoofThresholdsForExcessPacketDelay,

maxnoofESNPNs,

maxnoofSuccessfulPSCellChangeReports,

maxnoofUEsforRAReportIndications,

maxnoofPSCellsinCPAC,

maxnoofCPACexecutioncond,

maxnoofLBTFailureInformation,

maxnoofCellsTrajectoryPredict,

maxnoofCellsTrajectory,

maxFailedCellMeasObjects,

maxFailedMeasPerNode,

maxnoofUEReports,

maxnoofCandidateRelayUEs,

maxnoofCAGforMDT,

maxnoofMDTSNPNs,

maxnoofSecurityConfigurations,

maxnoofRSPPQoSFlows,

maxnoofFailedSliceMeasObjects

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- C

CellToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellToReportForDataCollection-Item

CellToReportForDataCollection-Item	::= SEQUENCE {

cell-ID								GlobalNG-RANCell-ID,

iE-Extensions						ProtocolExtensionContainer { { CellToReportForDataCollection-Item-ExtIEs} }	OPTIONAL,

...

}

CellToReportForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceToReportForDataCollection-List		CRITICALITY ignore	EXTENSION SliceToReportForDataCollection-List	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

CellMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementInitiationResult-Item

CellMeasurementInitiationResult-Item ::= SEQUENCE {

cellID											GlobalNG-RANCell-ID,

cellMeasurementFailureCause-List				CellMeasurementFailureCause-List OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { CellMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceMeasurementInitiationResult		CRITICALITY reject	EXTENSION SliceMeasurementInitiationResult	PRESENCE optional},

...

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

CellInfoResultForDataCollection-Item ::= SEQUENCE {

cellID											GlobalNG-RANCell-ID,

predictedRadioResourceStatus					RadioResourceStatus					OPTIONAL,

predictedNumberofActiveUEs						NumberofActiveUEs					OPTIONAL,

predictedRRCConnections							RRCConnections						OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {CellInfoResultForDataCollection-Item-ExtIEs} }	OPTIONAL,

...

}

CellInfoResultForDataCollection-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PredictedSliceAvailableCapacity		CRITICALITY ignore	EXTENSION PredictedSliceAvailableCapacityGroup	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- D

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

DRBToQoSFlowMapping-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSRequestInfo		CRITICALITY ignore		EXTENSION DAPSRequestInfo		PRESENCE optional },

...

}

DRBUEPerformance ::= SEQUENCE {

dRBUEPerformance-List					DRBUEPerformance-List,

iE-Extensions							ProtocolExtensionContainer { { DRBUEPerformance-ExtIEs} } OPTIONAL,

...

}

DRBUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBUEPerformance-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBUEPerformance-Item

DRBUEPerformance-Item	::= SEQUENCE {

dRB_ID								DRB-ID,

dRBBasedUEPerformance				DRBBasedUEPerformance,

iE-Extensions						ProtocolExtensionContainer { { DRBUEPerformance-Item-ExtIEs} }	OPTIONAL,

...

}

DRBUEPerformance-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DRBBasedUEPerformance	::= SEQUENCE {

dRBAverageUEPacketDelay			AveragePacketDelay,					OPTIONAL,

dRBAveragePacketLossDL				INTEGER (0..1000000)				OPTIONAL,

dRBAveragePacketLossUL				INTEGER (0..1000000)				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { DRBBasedUEPerformance-ExtIEs} } OPTIONAL,

...

}

DRBBasedUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

DUF-Slot-Config-List	::= SEQUENCE (SIZE(1..maxnoofDUFSlots)) OF DUF-Slot-Config-Item

DUF-Slot-Config-Item 	::=	CHOICE {

explicitFormat				ExplicitFormat,

implicitFormat				ImplicitFormat,

choice-extension				ProtocolIE-Single-Container { { DUF-Slot-Config-Item-ExtIEs} }

}

DUF-Slot-Config-Item-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- F

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofCellsinNG-RANnode)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

globalNG-RANCell-ID						GlobalCell-ID,

future-cellCoverageState				INTEGER (0..63, ...),

future-SSB-Coverage-Modification-List	Future-SSB-Coverage-Modification-List		OPTIONAL,

predicted-Coverage-Modification-Cause	Predicted-CoverageModificationCause			OPTIONAL,

timeForFutureCoverageState				INTEGER (1..60, ...)						OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Future-SSB-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF Future-SSB-Coverage-Modification-Item

Future-SSB-Coverage-Modification-Item ::= SEQUENCE {

sSBIndex						INTEGER(0..63),

future-SSBCoverageState			INTEGER (0..15, ...),

iE-Extension					ProtocolExtensionContainer { { Future-SSB-Coverage-Modification-Item-ExtIEs} }	OPTIONAL,

...

}

Future-SSB-Coverage-Modification-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- P

Predicted-CoverageModificationCause ::= ENUMERATED {

coverage,

cell-edge-capacity,

...

}

PredictedSliceAvailableCapacityGroup	::= SEQUENCE {

sliceAvailableCapacity				SliceAvailableCapacity,

cellCapacityClassValueDownlink		CellCapacityClassValue			OPTIONAL,

cellCapacityClassValueUplink		CellCapacityClassValue			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PredictedSliceAvailableCapacityGroup-ExtIEs} }	OPTIONAL,

...

}

PredictedSliceAvailableCapacityGroup-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- S

SliceToReportForDataCollection-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceToReportForDataCollection-List-Item

SliceToReportForDataCollection-List-Item	::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions				ProtocolExtensionContainer { { SliceToReportForDataCollection-List-Item-ExtIEs} }	OPTIONAL,

...

}

SliceToReportForDataCollection-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceMeasurementInitiationResult::= SEQUENCE {

sliceMeasurementInitiationResult-List		SliceMeasurementInitiationResult-List,

iE-Extensions					ProtocolExtensionContainer { {SliceMeasurementInitiationResult-ExtIEs} } OPTIONAL,

...

}

SliceMeasurementInitiationResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF SliceMeasurementInitiationResult-Item

SliceMeasurementInitiationResult-Item ::= SEQUENCE {

pLMNIdentity									PLMN-Identity,

sNSSAIMeasurementInitiationResult-List			SNSSAIMeasurementInitiationResult-List,

iE-Extensions									ProtocolExtensionContainer { { SliceMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

SliceMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementInitiationResult-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SNSSAIMeasurementInitiationResult-Item

SNSSAIMeasurementInitiationResult-Item ::= SEQUENCE {

sNSSAI								S-NSSAI,

sNSSAIMeasurementFailureCause-List		SNSSAIMeasurementFailureCause-List		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {SNSSAIMeasurementInitiationResult-Item-ExtIEs} }	OPTIONAL,

...

}

SNSSAIMeasurementInitiationResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIMeasurementFailureCause-List ::=SEQUENCE (SIZE(1..maxnoofFailedSliceMeasObjects)) OF SNSSAIMeasurementFailureCause-Item

SNSSAIMeasurementFailureCause-Item ::= SEQUENCE {

sNSSAIMeasurementFailedReportCharacteristics	BIT STRING(SIZE(32)),

cause											Cause,

iE-Extensions									ProtocolExtensionContainer { {SNSSAIMeasurementFailureCause-Item-ExtIEs} }		OPTIONAL,

...

}

SNSSAIMeasurementFailureCause-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceUEPerformance ::= SEQUENCE {

pLMNIdentity								PLMN-Identity,

s-NSSAIUEPerformance-List					S-NSSAIUEPerformance-List,

iE-Extensions								ProtocolExtensionContainer { { SliceUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

S-NSSAIUEPerformance-List ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF S-NSSAIUEPerformance-List-Item

S-NSSAIUEPerformance-List-Item	::= SEQUENCE {

sNSSAI								S-NSSAI,

sliceBasedUEPerformance				SliceBasedUEPerformance,

iE-Extensions						ProtocolExtensionContainer { { S-NSSAIUEPerformance -List-Item-ExtIEs} }	OPTIONAL,

...

}

S-NSSAIUEPerformance-List-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SliceBasedUEPerformance	::= SEQUENCE {

averageSliceUEThroughputDL			BitRate,		OPTIONAL,

averageSliceUEThroughputUL			BitRate,		OPTIONAL,

averageSliceUEPacketDelay			AveragePacketDelay,					OPTIONAL,

averageSlicePacketLossDL			FFS,								OPTIONAL,

averageSlicePacketLossUL			FFS,								OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { SliceBasedUEPerformance-ExtIEs} } OPTIONAL,

...

}

SliceBasedUEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- U

UEAssociatedInfoResult-List ::= SEQUENCE (SIZE(1..maxnoofUEReports)) OF UEAssociatedInfoResult-Item

UEAssociatedInfoResult-Item ::= SEQUENCE {

uEAssistantIdentifier						NG-RANnodeUEXnAPID,

uEPerformance								UEPerformance			OPTIONAL,

measuredUETrajectory						MeasuredUETrajectory	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEAssociatedInfoResult-Item-ExtIEs} } OPTIONAL,

...

}

UEAssociatedInfoResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceUEPerformance	CRITICALITY ignore	EXTENSION SliceUEPerformance PRESENCE optional } |

{ ID id-DRBUEPerformance	CRITICALITY ignore	EXTENSION DRBUEPerformance PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs						INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRAReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 256

maxnoofUEIDIndicesforMBSPaging				INTEGER ::= 4096

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMRBs									INTEGER ::= 32

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofMBSServiceAreaInformation			INTEGER ::= 256

maxnoofTAIforMBS							INTEGER ::= 1024

maxnoofAssociatedMBSSessions				INTEGER ::= 32

maxnoofMBSSessions							INTEGER ::= 256

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofPSCellsPerSN							INTEGER ::= 8

maxnoofNR-UChannelIDs						INTEGER ::= 16

maxnoofCellsinCHO							INTEGER ::= 8

maxnoofCHOexecutioncond						INTEGER ::= 2

maxnoofServedCellsIAB						INTEGER ::= 512

maxnoofServingCells							INTEGER ::= 32

maxnoofBHInfo								INTEGER ::= 1024

maxnoofTrafficIndexEntries					INTEGER	::=	1024

maxnoofTLAsIAB								INTEGER	::=	1024

maxnoofBAPControlPDURLCCHs					INTEGER ::= 2

maxnoofIABSTCInfo							INTEGER ::= 45

maxnoofSymbols								INTEGER ::= 14

maxnoofDUFSlots								INTEGER ::= 320

maxnoofHSNASlots							INTEGER ::= 5120

maxnoofRBsetsPerCell						INTEGER ::= 8

maxnoofRBsetsPerCell1						INTEGER ::= 7

maxnoofChildIABNodes						INTEGER ::= 1024

maxnoofPSCellCandidates						INTEGER ::= 8

maxnoofTargetSNs							INTEGER ::= 8

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofMTCItems								INTEGER ::= 16

maxnoofCSIRSconfigurations					INTEGER ::= 96

maxnoofCSIRSneighbourCells					INTEGER ::= 16

maxnoofCSIRSneighbourCellsInMTC				INTEGER ::= 16

maxnoofNeighbour-NG-RAN-Nodes				INTEGER ::= 256

maxnoofSRBs									INTEGER ::= 5

maxnoofSMBR									INTEGER ::= 8

maxnoofNSAGs								INTEGER ::= 256

maxnoofTargetSNsMinusOne					INTEGER ::= 7

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofSuccessfulPSCellChangeReports		INTEGER ::= 64

maxnoofUEsforRAReportIndications			INTEGER ::= 64

maxnoofPSCellsinCPAC						INTEGER ::= 8

maxnoofCPACexecutioncond					INTEGER ::= 2

maxnoofLBTFailureInformation				INTEGER ::= 64

maxnoofCellsTrajectoryPredict				INTEGER ::= 16

maxnoofCellsTrajectory						INTEGER	::=	16

maxFailedCellMeasObjects					INTEGER ::= 124

maxFailedMeasPerNode						INTEGER ::= 124

maxnoofUEReports							INTEGER ::= 16

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofFailedSliceMeasObjects				INTEGER ::= 124

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs																					ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort																			ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RAReport																							ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered																						ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																			ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-EUTRAPagingeDRXInformation																		ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-DirectForwardingPathAvailability																	ProtocolIE-ID ::= 253

id-SourceNG-RAN-node-ID																				ProtocolIE-ID ::= 254

id-SourceDLForwardingIPAddress																		ProtocolIE-ID ::= 255

id-SourceNodeDLForwardingIPAddress																	ProtocolIE-ID ::= 256

id-ExtendedReportIntervalMDT																		ProtocolIE-ID ::= 257

id-SecurityIndication																				ProtocolIE-ID ::= 258

id-RRCConnReestab-Indicator																			ProtocolIE-ID ::= 259

id-TargetNodeID																						ProtocolIE-ID ::= 260

id-ManagementBasedMDTPLMNList																		ProtocolIE-ID ::= 261

id-PrivacyIndicator																					ProtocolIE-ID ::= 262

id-TraceCollectionEntityIPAddress																	ProtocolIE-ID ::= 263

id-M4ReportAmount																					ProtocolIE-ID ::= 264

id-M5ReportAmount																					ProtocolIE-ID ::= 265

id-M6ReportAmount																					ProtocolIE-ID ::= 266

id-M7ReportAmount																					ProtocolIE-ID ::= 267

id-BeamMeasurementIndicationM1																		ProtocolIE-ID ::= 268

id-MBS-Session-ID																					ProtocolIE-ID ::= 269

id-UEIdentityIndexList-MBSGroupPaging																	ProtocolIE-ID ::= 270

id-MulticastRANPagingArea																			ProtocolIE-ID ::= 271

id-Supported-MBS-FSA-ID-List																		ProtocolIE-ID ::= 272

id-MBS-SessionInformation-List																		ProtocolIE-ID ::= 273

id-MBS-SessionInformationResponse-List																ProtocolIE-ID ::= 274

id-MBS-SessionAssociatedInformation																	ProtocolIE-ID ::= 275

id-SuccessfulHOReportInformation																	ProtocolIE-ID ::= 276

id-SliceRadioResourceStatus-List																	ProtocolIE-ID ::= 277

id-CompositeAvailableCapacitySupplementaryUplink													ProtocolIE-ID ::= 278

id-SCGUEHistoryInformation																			ProtocolIE-ID ::= 279

id-SSBOffsets-List																					ProtocolIE-ID ::= 280

id-NG-RANnode2SSBOffsetModificationRange															ProtocolIE-ID ::= 281

id-Coverage-Modification-List																		ProtocolIE-ID ::= 282

id-NR-U-Channel-List																				ProtocolIE-ID ::= 283

id-SourcePSCellCGI																					ProtocolIE-ID ::= 284

id-FailedPSCellCGI																					ProtocolIE-ID ::= 285

id-SCGFailureReportContainer																		ProtocolIE-ID ::= 286

id-SNMobilityInformation																			ProtocolIE-ID ::= 287

id-SourcePSCellID																					ProtocolIE-ID ::= 288

id-SuitablePSCellCGI																				ProtocolIE-ID ::= 289

id-PSCellChangeHistory																				ProtocolIE-ID ::= 290

id-CHOConfiguration																					ProtocolIE-ID ::= 291

id-NR-U-ChannelInfo-List																			ProtocolIE-ID ::= 292

id-PSCellHistoryInformationRetrieve																	ProtocolIE-ID ::= 293

id-NG-RANnode2SSBOffsetsModificationRange															ProtocolIE-ID ::= 294

id-MIMOPRBusageInformation																			ProtocolIE-ID ::= 295

id-F1CTrafficContainer																				ProtocolIE-ID ::= 296

id-IAB-MT-Cell-List																					ProtocolIE-ID ::= 297

id-NoPDUSessionIndication																			ProtocolIE-ID ::= 298

id-IAB-TNL-Address-Request																			ProtocolIE-ID ::= 299

id-IAB-TNL-Address-Response																			ProtocolIE-ID ::= 300

id-TrafficToBeAddedList																				ProtocolIE-ID ::= 301

id-TrafficToBeModifiedList																			ProtocolIE-ID ::= 302

id-TrafficToBeReleaseInformation																	ProtocolIE-ID ::= 303

id-TrafficAddedList																					ProtocolIE-ID ::= 304

id-TrafficModifiedList																				ProtocolIE-ID ::= 305

id-TrafficNotAddedList																				ProtocolIE-ID ::= 306

id-TrafficNotModifiedList																			ProtocolIE-ID ::= 307

id-TrafficRequiredToBeModifiedList																	ProtocolIE-ID ::= 308

id-TrafficRequiredModifiedList																		ProtocolIE-ID ::= 309

id-TrafficReleasedList																				ProtocolIE-ID ::= 310

id-IABTNLAddressToBeAdded																			ProtocolIE-ID ::= 311

id-IABTNLAddressToBeReleasedList																	ProtocolIE-ID ::= 312

id-nonF1-Terminating-IAB-DonorUEXnAPID																ProtocolIE-ID ::= 313

id-F1-Terminating-IAB-DonorUEXnAPID																	ProtocolIE-ID ::= 314

id-BoundaryNodeCellsList																			ProtocolIE-ID ::= 315

id-ParentNodeCellsList																				ProtocolIE-ID ::= 316

id-tdd-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 317

id-UL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 318

id-DL-GNB-DU-Cell-Resource-Configuration															ProtocolIE-ID ::= 319

id-permutation																						ProtocolIE-ID ::= 320

id-IABTNLAddressException																			ProtocolIE-ID ::= 321

id-CHOinformation-AddReq																			ProtocolIE-ID ::= 322

id-CHOinformation-ModReq																			ProtocolIE-ID ::= 323

id-SurvivalTime																						ProtocolIE-ID ::= 324

id-TimeSynchronizationAssistanceInformation															ProtocolIE-ID ::= 325

id-SCGActivationRequest																				ProtocolIE-ID ::= 326

id-SCGActivationStatus																				ProtocolIE-ID ::= 327

id-CPAInformationRequest																			ProtocolIE-ID ::= 328

id-CPAInformationAck																				ProtocolIE-ID ::= 329

id-CPCInformationRequired																			ProtocolIE-ID ::= 330

id-CPCInformationConfirm																			ProtocolIE-ID ::= 331

id-CPAInformationModReq																				ProtocolIE-ID ::= 332

id-CPAInformationModReqAck																			ProtocolIE-ID ::= 333

id-CPC-DataForwarding-Indicator																		ProtocolIE-ID ::= 334

id-CPCInformationUpdate																				ProtocolIE-ID ::= 335

id-CPACInformationModRequired																		ProtocolIE-ID ::= 336

id-QMCConfigInfo																					ProtocolIE-ID ::= 337

id-ProtocolIE-ID338-NotToBeUsed																		ProtocolIE-ID ::= 338

id-Additional-Measurement-Timing-Configuration-List													ProtocolIE-ID ::= 339

id-PDUSession-PairID																				ProtocolIE-ID ::= 340

id-Local-NG-RAN-Node-Identifier																		ProtocolIE-ID ::= 341

id-Neighbour-NG-RAN-Node-List																		ProtocolIE-ID ::= 342

id-Local-NG-RAN-Node-Identifier-Removal																ProtocolIE-ID ::= 343

id-FiveGProSeAuthorized																				ProtocolIE-ID ::= 344

id-FiveGProSePC5QoSParameters																		ProtocolIE-ID ::= 345

id-FiveGProSeUEPC5AggregateMaximumBitRate															ProtocolIE-ID ::= 346

id-ServedCellSpecificInfoReq-NR																		ProtocolIE-ID ::= 347

id-NRPagingeDRXInformation																			ProtocolIE-ID ::= 348

id-NRPagingeDRXInformationforRRCINACTIVE															ProtocolIE-ID ::= 349

id-Redcap-Bcast-Information																			ProtocolIE-ID ::= 350

id-SDTSupportRequest																				ProtocolIE-ID ::= 351

id-SDT-SRB-between-NewNode-OldNode																	ProtocolIE-ID ::= 352

id-SDT-Termination-Request																			ProtocolIE-ID ::= 353

id-SDTPartialUEContextInfo																			ProtocolIE-ID ::= 354

id-SDTDataForwardingDRBList																			ProtocolIE-ID ::= 355

id-PagingCause																						ProtocolIE-ID ::= 356

id-PEIPSassistanceInformation																		ProtocolIE-ID ::= 357

id-UESliceMaximumBitRateList																		ProtocolIE-ID ::= 358

id-S-NG-RANnodeUE-Slice-MBR																			ProtocolIE-ID ::= 359

id-PositioningInformation																			ProtocolIE-ID ::= 360

id-UEAssistantIdentifier																			ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																		ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-QosFlowMappingIndication																			ProtocolIE-ID ::= 373

id-Full-and-Short-I-RNTI-Profile-List																ProtocolIE-ID ::= 374

id-MBS-DataForwarding-Indicator																		ProtocolIE-ID ::= 375

id-IABAuthorizationStatus																			ProtocolIE-ID ::= 376

id-EquivalentSNPNs																					ProtocolIE-ID ::= 377

id-SelectedNID																						ProtocolIE-ID ::= 378

id-MT-SDT-Information																				ProtocolIE-ID ::= 379

id-PosPartialUEContextInfo																			ProtocolIE-ID ::= 380

id-SRSConfiguration																					ProtocolIE-ID ::= 381

id-CHOTimeBasedInformation																			ProtocolIE-ID ::= 382

id-ChannelOccupancyTimePercentageUL																	ProtocolIE-ID ::= 383

id-EnergyDetectionThresholdUL																		ProtocolIE-ID ::= 384

id-SuccessfulPSCellChangeReportInformation															ProtocolIE-ID ::= 385

id-PSCellListContainer																				ProtocolIE-ID ::= 386

id-RadioResourceStatusNR-U																			ProtocolIE-ID ::= 387

id-CPACConfiguration																				ProtocolIE-ID ::= 388

id-RaReportIndicationList																			ProtocolIE-ID ::= 389

id-SPRAvailability																					ProtocolIE-ID ::= 390

id-DLLBTFailureInformationRequest																	ProtocolIE-ID ::= 391

id-DLLBTFailureInformationList																		ProtocolIE-ID ::= 392

id-TargetCellCRNTI																					ProtocolIE-ID ::= 393

id-TimeSinceFailure																					ProtocolIE-ID ::= 394

id-AerialUESubscriptionInformation																	ProtocolIE-ID ::= 395

id-LTEA2XServicesAuthorized																			ProtocolIE-ID ::= 396

id-NRA2XServicesAuthorized																			ProtocolIE-ID ::= 397

id-LTEA2XUEPC5AggregateMaximumBitRate																ProtocolIE-ID ::= 398

id-NRA2XUEPC5AggregateMaximumBitRate																ProtocolIE-ID ::= 399

id-A2XPC5QoSParameters																				ProtocolIE-ID ::= 400

id-CellBasedUETrajectoryPrediction																	ProtocolIE-ID ::= 401

id-DataCollectionID																					ProtocolIE-ID ::= 402

id-RequestedPredictionTime																			ProtocolIE-ID ::= 403

id-NodeMeasurementInitiationResult-List																ProtocolIE-ID ::= 404

id-CellMeasurementInitiationResult-List																ProtocolIE-ID ::= 405

id-UEAssociatedInfoResult-List																		ProtocolIE-ID ::= 406

id-ProtocolIE-ID-407-not-to-be-used																	ProtocolIE-ID ::= 407

id-UETrajectoryCollectionConfiguration																ProtocolIE-ID ::= 408

id-UEPerformanceCollectionConfiguration																ProtocolIE-ID ::= 409

id-CellMeasurementResultForDataCollection-List															ProtocolIE-ID ::= 410

id-CellToReportForDataCollection-List																ProtocolIE-ID ::= 411

id-FiveGProSeLayer2Multipath																		ProtocolIE-ID ::= 412

id-FiveGProSeLayer2UEtoUERelay																		ProtocolIE-ID ::= 413

id-FiveGProSeLayer2UEtoUERemote																		ProtocolIE-ID ::= 414

id-CandidateRelayUEInfoList																			ProtocolIE-ID ::= 415

id-NRCellsAndSSBsList																				ProtocolIE-ID ::= 416

id-ActivatedNRCellsAndSSBsList																		ProtocolIE-ID ::= 417

id-ClockQualityReportingControlInfo																	ProtocolIE-ID ::= 418

id-CapabilityForBATAdaptation																		ProtocolIE-ID ::= 419

id-PNI-NPN-AreaScopeofMDT																			ProtocolIE-ID ::= 420

id-PNI-NPNBasedMDT																					ProtocolIE-ID ::= 421

id-SNPN-CellBasedMDT																				ProtocolIE-ID ::= 422

id-SNPN-TAIBasedMDT																					ProtocolIE-ID ::= 423

id-SNPN-BasedMDT																					ProtocolIE-ID ::= 424

id-S-CPAC-Request																					ProtocolIE-ID ::= 425

id-S-CPAC-Request-Info																				ProtocolIE-ID ::= 426

id-S-CPAC-ReferenceConfigRequest																	ProtocolIE-ID ::= 427

id-S-CPAC-InterSN-ExecutionNotify																	ProtocolIE-ID ::= 428

id-S-CPAC-dataforwardinginfofromSource																ProtocolIE-ID ::= 429

id-CPACcandidatePSCells-wotherInfo-list																ProtocolIE-ID ::= 430

id-eRedcap-Bcast-Information																		ProtocolIE-ID ::= 431

id-NRPagingLongeDRXInformationforRRCINACTIVE														ProtocolIE-ID ::= 432

id-MBS-AssistanceInformation																		ProtocolIE-ID ::= 433

id-QMCCoordinationRequest																			ProtocolIE-ID ::= 434

id-QMCCoordinationResponse																			ProtocolIE-ID ::= 435

id-QoE-Measurement-Results																			ProtocolIE-ID ::= 436

id-MBSCommServiceType																				ProtocolIE-ID ::= 437

id-AssistanceInformationQoE-Meas																	ProtocolIE-ID ::= 438

id-ProtocolIE-ID-439-not-to-be-used																	ProtocolIE-ID ::= 439

id-QoERVQoEReportingPaths																			ProtocolIE-ID ::= 440

id-Src-SN-to-Tgt-SNQMCInfoInquiry																	ProtocolIE-ID ::= 441

id-DirectForwardingPathAvailabilityWithSourceMN														ProtocolIE-ID ::= 442

id-CHO-Maxnoof-CondReconfig																			ProtocolIE-ID ::= 443

id-accessed-PSCellID																				ProtocolIE-ID ::= 444

id-conditional-Reconfig-ToCancel-List																ProtocolIE-ID ::= 445

id-CHOinformation-AddReqAck																			ProtocolIE-ID ::= 446

id-CHO-CPAC-Info																					ProtocolIE-ID ::= 447

id-PDUSetQoSParameters																				ProtocolIE-ID ::= 448

id-N6JitterInformation																				ProtocolIE-ID ::= 449

id-ECNMarkingorCongestionInformationReportingRequest												ProtocolIE-ID ::= 450

id-PDUSetbasedHandlingIndicator																		ProtocolIE-ID ::= 451

id-TAISliceUnavailableCellList																		ProtocolIE-ID ::= 452

id-MobileIAB-AuthorizationStatus																	ProtocolIE-ID ::= 453

id-MIAB-MT-BAP-Address																				ProtocolIE-ID ::= 454

id-MobileIABCell																					ProtocolIE-ID ::= 455

id-sk-Counter																						ProtocolIE-ID ::= 456

id-Source-M-NG-RANnodeID																			ProtocolIE-ID ::= 457

id-ProtocolIE-ID458-NotToBeUsed																		ProtocolIE-ID ::= 458

id-SourceSN-to-TargetSN-QMCInfo																		ProtocolIE-ID ::= 459

id-RegistrationRequestForDataCollection																ProtocolIE-ID ::= 460

id-ReportCharacteristicsForDataCollection															ProtocolIE-ID ::= 461

id-ReportingPeriodicityForDataCollection															ProtocolIE-ID ::= 462

id-NodeAssociatedInfoResult																			ProtocolIE-ID ::= 463

id-SLPositioning-Ranging-Services-Info																ProtocolIE-ID ::= 464

id-XR-Bcast-Information					 															ProtocolIE-ID ::= 465

id-PDUSessionsListToBeReleased-UPError																ProtocolIE-ID ::= 466

id-MaximumDataBurstVolume																			ProtocolIE-ID ::= 467

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-Future-Coverage-Modification-List																ProtocolIE-ID ::= xx1

id-SliceToReportForDataCollection-List																ProtocolIE-ID ::= xx2

id-PredictedSliceAvailableCapacity																	ProtocolIE-ID ::= xx3

id-SliceMeasurementInitiationResult																	ProtocolIE-ID ::= xx4

id-SliceUEPerformance																				ProtocolIE-ID ::= xx5

id-DRBUEPerformance																				 ProtocolIE-ID ::= xx5

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>