3GPP TSG-RAN WG3 Meeting #129	R3-255457

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	20.2

Source:	Nokia

Title:	(TP to TS 38.305) Addressing remaining aspects in case 3a and case 3b

Document for:	Text Proposal

# 1	Introduction

In RAN3 #128 good progress was achieved for AI/ML positioning case 3a observed through a number of agreements. In particular, we agreed the following:

Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator.

FFS if UE location can be requested.

The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message.

Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs

Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason.

RAN3 confirms that Performance monitoring is done in the RAN in case 3a.

RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface.

Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion

FFS: if LMF should specifically request for AIML inferred measurement.

In this contribution, we provide our views pertaining to the above open aspects. In addition, we further discuss some other aspects that in our view need to be addressed, such as a) user consent for case 3a and b) stage 2 updates for case 3a and case 3b. Finally, we provide a stage 2 TP to TS 38.305 in the annex of this paper.

# 2	Discussion

## 2.1	Open Issues

When it comes to case 3a, we need to address the FFS points captured in Chairman notes as well as other open aspects in the BLCR to TS 38.455. Those open issues translate to addressing the following:

- Whether UE location can be requested from LMF as part of Part B

- Whether different encoding for Positioning Data Unavailable IE in POSITIONING DATA COLLECTION REPORT message is needed

- Whether the LMF can request inferred measurements from a gNB

- Whether the LoS/NLoS indicator can be inferred and, in that case, whether to flag if it is produced through AI/ML or not

In this paper, we provide our views on the above issues.

### 2.1.1 	UE location can be requested from LMF as part of Part B

In the last meeting, we agreed to define the Positioning Data Collection Needed IE as a new bitmap in which the requested information in the bitmap is timing measurement (UL-RTOA, gNB Rx-Tx Time Difference) together with an optional LoS/NLoS indicator. Some companies mentioned that also UE location could be requested and this was captured as an open point. However, according to the current RAN1 agreements only timing information is requested from a gNB towards an LMF. Request of location information from a gNB was discussed in RAN1, but was controversial due to user privacy concerns and could not be agreed. In particular, in RAN1 #120-bis, RAN1 made the following agreement:

Agreement

For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label,

- The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label.

Note: It is assumed that user data privacy of non-PRU UE is preserved.

As can be seen from the note only timing information is provided by LMF and location information is nowhere mentioned. In addition, given that this is the last meeting of Rel-19, we think that this FFS can be removed.

Proposal 1: gNB does not request UE Location from an LMF in Rel-19.

### 2.1.2	Positioning Data Unavailable IE

Another FFS in the BLCR for TS 38.455 pertains to the Positioning Data Unavailable IE which was introduced to indicate that LMF is not able to provide the requested positioning information. Currently, it is encoded as an ENUMERATED with single codepoint ‘true’, but the final encoding remains FFS. The reason of this FFS is that some companies think that a finer granularity of the encoded codepoints could be beneficial towards the gNB to determine the reason why data is unavailable, e.g. to avoid unnecessary signalling related to repeated requests. We think that we can introduce two codepoints to reflect the reason of data unavailability, namely “no data available” and “user consent not granted”.

Proposal 2: Introduce a finer granularity to the codepoints of Positioning Data Unavailable IE and encode it as ENUMERATED (no data available, user consent not granted, …).

2.1.3	Requesting of inferred measurements from a gNB

In the last meeting we agreed to introduce a new “Inferred measurement” flag in the TRP Measurement Result IE to align with RAN1 conclusion. An open point related to that is whether a gNB provides the flag to the LMF only when LMF requests AI/ML measurements or independently. In our view, it would be reasonable to expect that a gNB indicates to the LMF that a measurement is an outcome of AI/ML operation only if the LMF explicitly asked for AI/ML inference. There are several reasons behind this. This is also in line with the NG-RAN AI/ML framework where a node only sends predictions to a neighbour node upon request. In addition, if a gNB sends a flag towards an LMF that a measurement is produced by AI/ML, without this being requested by the LMF, it may create confusion and interoperability issues. It may create an expectation that the LMF needs to take precautionary actions when receiving AI/ML generated measurements. Or it may create an expectation that once LMF receives measurements obtained through AI/ML it becomes responsible to determine the quality of the measurement and take corrective actions. Since model performance is already agreed to be performed by the gNB this may create interoperability issues. Finally, it may waste gNB’s resources to provide measurements produced by AI/ML when LMF doesn’t need to receive such.

Proposal 3: gNB provides inferred measurements to the LMF upon request.

### 2.1.4	Details of LoS/NLoS Indicator

Another FFS in the BLCR for TS 38.455 pertains to the LoS/NLoS indicator and whether it can be indicated as inferred towards the LMF. In our view, and based on the RAN1 discussions, LoS/NLoS indicator may be provided through AI/ML methods or not. However, we do not think that a gNB needs to indicate to the LMF whether LoS/NLoS is provided through AI/ML methods or not since this will not affect the LMF behavior.

Proposal 4: LoS/NLoS indicator may be inferred by a gNB, but no explicit indication is needed to indicate to LMF how it is calculated (with AI/ML or not).

## 2.2	User consent check

During the discussions of case 3a, user consent was brought up by several companies. User consent is part of user subscription information and is therefore available at the Unified Data Management (UDM). In Rel-19, SA2 introduced support for LMF to check user consent for direct AI/ML positioning methods.

In TS 23.273, SA2 has defined procedures for data collection at LMF for LMF-based AI/ML positioning based on UE measurements and based on NG-RAN measurements. If LMF determines that data collection from the NG-RAN is required, then before any data is received from the NG-RAN it needs to obtain user consent for data collection (including a specific purpose) from UDM (TS 23.502). This is done by the LMF by checking that user consent is provided for each SUPI for which data is collected. LMF additionally may subscribe to changes on subscription data related to user consent so that if user consent is revoked the data collection for this SUPI will be stopped. Figure 1 shows an example flow for data collection including user consent check from clause 6.22.3, TS 23.273 (steps 4, 5 and 9):

Figure 1 Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (Figure 6.22.3-1, TS 23.273)

The SA2 user consent framework did not consider AI/ML assisted positioning with gNB-side AI/ML model, namely case 3a. However, the main assumption by SA2 is that user consent is checked before data is collected for AI/ML Model Training from the NG-RAN (or from a given UE). We therefore think that some aspects of the SA2 user consent framework can be reused for Opportunistic Data Collection.

In the BL CR to TS 38.305 the LMF starts positioning measurements for a given UE as shown in Figure 2:

Figure 2 Positioning Data Collection Procedure (Figure 7.x.2-1, BL CR to TS 38.305)

In step 2, a gNB determines that Data Collection is needed for the UE being measured and indicates to the LMF that it needs positioning data for this UE. User consent must be checked before step 4, when LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message that includes positioning data (e.g., UE location) to the gNB. In that sense, we think that the user consent framework from SA2 can be reused for case 3a.

Proposal 5:	User consent is checked by LMF before LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message, and SA2 framework can be reused.

## 2.3	Stage 2 Open Issues

Since this is the last meeting of Rel-19, we think it is necessary to clarify possible stage 2 issues in the BLCR for TS 38.305. In particular, in TS 38.305 there are two Editor’s Notes to indicate that the current agreed text as well as the provided signalling and IEs can be further refined. In our view, the following clarifications are needed:

-	It is more accurate and generalized to say “UE being measured” rather than “UE being positioned” (e.g., in case of PRU).

-	As discussed earlier in the paper, according to RAN1 discussions Part B does not comprise location information but timing information. Therefore, the text in TS 38.305 referring to Part B as “information related to UE location” is misleading. We think that “information related to UE location” should be replaced with more general terminology such as “positioning data”.

-	Similarly, since we are not yet sure about the details of Part A information (which could also comprise implementation-specific information), we should try to use general terminology such as “local data collected by the gNB”.

In addition, one of the agreements in RAN1 #119 for case 3b is the following:

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

The fact that a gNB may use different Nt', Nt and/or k values, other than the signalled parameters from LMF, for measurement reporting is not currently captured in our specifications. We therefore propose to add a NOTE in TS 38.305, clarifying this.

Proposal 6: Introduce a NOTE in the TP for BLCR to TS 38.305 to indicate that a gNB may use different Nt', Nt and/or k values, other than the signalled parameters for measurement reporting from LMF.

We provide a TP for BLCR to TS 38.305 in the annex of this paper, reflecting the above proposals.

Proposal 7: Agree to the TP for BLCR to TS 38.305 in the annex of this paper.

# 4	Conclusion

In this paper we make the following observations and proposals:

Proposal 1: gNB does not request UE Location from an LMF in Rel-19.

Proposal 2: Introduce a finer granularity to the codepoints of Positioning Data Unavailable IE and encode it as ENUMERATED (no data available, user consent not granted, …).

Proposal 3: gNB provides inferred measurements to the LMF upon request.

Proposal 4: LoS/NLoS indicator may be inferred by a gNB, but no explicit indication is needed to indicate to LMF how it is calculated (with AI/ML or not).

Proposal 5:	User consent is checked by LMF before LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message, and SA2 framework can be reused.

Proposal 6: Introduce a NOTE in the TP for BLCR to TS 38.305 to indicate that a gNB may use different Nt', Nt and/or k values, other than the signalled parameters for measurement reporting from LMF.

Proposal 7:Agree to the TP for BLCR to TS 38.305 in the annex of this paper.

Also, a TP for BLCR to TS 38.305 is provided in the annex of this paper.

# Annex 	TP for BLCR to TS 38.305

## <<< start of changes >>>

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures described in Clause 8.

2.	The gNB(s) determines that data collection is needed for the UE being measured.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that positioning data collection is needed for the UE being measured.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clause 8.

4.	User consent is checked as described in TS 23.273 [35].

NOTE 1:	Step 4 could be performed earlier.

5.	The LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in step 3 that positioning data collection is needed. The message includes the positioning data and information that enables the gNB to correlate the positioning data with local data collected by the gNB which pertains to the same UE.

## <<< Next change >>>

### 8.13.2	Information to be transferred between NG-RAN/5GC Elements

This clause defines the information that may be transferred between LMF and gNB/TRPs.

[…]

#### 8.13.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL-RTOA NOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information for each measurement

LoS/NLoS information for each measurement NOTE 2 (FFS)

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

Time domain channel measurements (FFS)NOTE 3

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the NG-RAN node. NOTE 3: These measurements may also be used by an LMF to directly infer the UE location using a trained AI/ML model hosted by the LMF.

#### 8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

Bandwidth aggregation request indication

Positioning validity area cell list

Validity area specific SRS information

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity and amount

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

Time window information for measurements

Information on time window size and number of channel response samples for channel measurements NOTE

NOTE: The gNB may use different value of channel response number and the list of time domain channel responses reported within the measurement report indicate the value used by gNB.

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger List - Activation time

UL-SRS: - Release all

## <<< End of changes >>>