3GPP TSG-RAN WG3 Meeting #129	R3-255462

Bengaluru, India, August 25th–29th, 2025

Title:	(TP to TS 38.300) AI/ML support

Source:	Ericsson, Deutsche Telekom, Jio Platforms, BT

Agenda item:	11.1

Document Type:	Discussion and agreement

# 1. Introduction

In this paper, we present a TP to TS 38.300 for Support of AI/ML for NG-RAN based on the agreements reached in the previous meetings.

The TP aims at covering support for all use cases developed during Rel-19 and for that reason it does not fall under any specific use case section.

# 2. Discussion

In the following we gather the agreements made in the previous meetings regarding the support for AI/ML for NG-RAN.

From RAN3#125-bis we note the following agreements:

AI/ML enabled Slicing:

Enhance the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure to enable requesting and reporting of the predicted radio resource status per slice and predicted slice available capacity over the Xn interface.

Introduce Slice To Report List for Data Collection IE including PLMN ID and S-NSSAI in DATA COLLECTION REQUEST message.

AI/ML enabled Coverage and Capacity Optimization:

Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.

The Coverage Modification Cause reflects the predicted CCO issue.

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

R18 leftovers Continuous MDT:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

From RAN3#126 we note the following agreements:

AI/ML enabled Slicing:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

AI/ML enabled Coverage and Capacity Optimization:

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

R18 leftovers Mobility Optimization for NR-DC:

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

From RAN3#127 we note the following agreements:

AI/ML enabled Slicing:

Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback

R18 leftovers Continuous MDT:

RAN3 assumes a solution for continuous MDT should minimise impacts on the current Management Based MDT framework.

From RAN3#127-bis we note the following agreements:

AI/ML enabled Slicing:

Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics.

Add Slice UE performance IE in the DATA COLLECTION UPDATE message.

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

Introduce the measured CCCV within the predicted slice available capacity.

AI/ML enabled Coverage and Capacity Optimization:

Legacy UE performance measurement metrics can be reused for CCO.

R18 leftovers Mobility Optimization for NR-DC:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following list of exit conditions to stop the collection of UE performance at SN side:

- the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- the NR-DC with the S-NG-RAN node for the UE is released.

From RAN3#128 we note the following agreements:

AI/ML enabled Coverage and Capacity Optimization:

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

R18 leftovers Continuous MDT:

Information from OAM to participating nodes in the NG-RAN allowing for the identification of a continuous MDT process are provided in the form of specific MDT TR(s).

Solution for continuous MDT shall not have an impact on the 5GC in R19 from RAN3 perspective.

Agree to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem.

Based on the above, we propose RAN3 to agree to the TP for TS 38.300 that can be found in the Annex.

Proposal 1:  RAN3 to agree to the TP for TS 38.300 that can be found in the Annex.

# 3. Conclusions

Based on the discussion in this paper, the following proposal was made.

Proposal 1:  RAN3 to agree to the TP for TS 38.300 that can be found in the Annex.

# 4. Annex

# TP to TS 38.300 - Support of AI/ML for NG-RAN

-------------------------------------- START OF CHANGES --------------------------------------

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information, including predicted radio resource status per slice;

-	UE performance feedback, including per slice UE performance;

-	Measured UE trajectory;

-	Energy Cost (EC);

-	Predicted slice available capacity.

The collection and reporting of the above information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback, including per-slice UE performance feedback, is triggered at successful Handover. In NR-DC scenarios, the MN can configure the collection of UE performance feedback at the SN; the collection is triggered at successful SN addition or SN change.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

An NG-RAN node may derive future coverage states for its cells and SSBs to prevent (or limit at an early stage) the rise of predicted Coverage and Capacity Optimisation (CCO) issues. The NG-RAN node may notify its neighbour NG-RAN nodes, using the NG-RAN node Configuration Update procedure, of the future modification of coverage states for cells and SSBs with cause of modification and their associated time or it may notify its neighbour NG-RAN nodes that a predicted CCO issue is cancelled.

### 16.20.4	OAM Requirements

In relation to the reporting of energy cost, OAM configures the following:

-	The minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area; and

-	The recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

OAM further configures management-based MDT for continuous MDT towards the NG-RAN. OAM provides specific MDT TR(s) so the NG-RAN can identify continuous MDT processes.

-------------------------------------- END OF CHANGES --------------------------------------