3GPP TSG-RAN WG3 Meeting #129	R3-255473

Bengaluru, India, August 25th–29th, 2025

Title:	AI/ML support for NR-DC

Source:	Ericsson, Jio Platforms

Agenda item:	11.4

Document Type:	Discussion and agreement

# Introduction

At the RAN3#127-bis meeting, the following agreements and “to-be-continued” were captured for AI/ML assisted mobility optimization for NR-DC in the meeting minutes:

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following list of exit conditions to stop the collection of UE performance at SN side:

- the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

- the NR-DC with the S-NG-RAN node for the UE is released.

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

However, at RAN3#128, little progress was achieved towards any of the open points above. Given that the RAN3#129 meeting is the last one for the WI “Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, in this contribution, we discuss the best way forward for a successful completion of the work in this use case.

# AI/ML support for mobility optimization for NR-DC

## General

According to the use case description for the AI/ML-based mobility optimization for NR-DC in the TR 38.743 (clause 5.1.1), AI/ML techniques can be used to optimize mobility in NR-DC; furthermore, any inference related to this optimization is located at the MN only. However, it is not described how AI/ML techniques can be used at the MN.

In our view, and considering the inminent conclusion of the WI, RAN3 should focus on providing a robust and future-looking data collection framework for UE performance in NR-DC to assist the AI/ML-based decisions at the MN. We described such enhancements in the sequel.

## Enhancements for data collection in dual connectivity

To assist inference at the MN, it is possible to mostly reuse the Rel-18 signaling design established to support AI/ML-assisted use cases. As already agreed in previous meetings, we can reuse the Data Collection Reporting Initiation procedure and the Data Collection Reporting procedure to allow an MN to request and obtain from a neighbor gNB (acting as SN) the reporting of UE performance.

At the RAN3#127-bis meeting, two additional exit conditions for the collection of UE performance feedback for NR-DC were agreed: (1) the expiration of a timer after NR-DC was successfully setup and (2) the NR-DC with the S-NG-RAN node for the UE is released. The related following open issue was noted:

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

It has been agreed in RAN3 that, for the collection of UE performance in split architecture, the UE performance metrics should be obtained at PDCP level. Consequently, SCG activation cannot be a condition for starting the collection of UE performance. For example, an SN-terminated MCG bearer would see data flowing through the PDCP entity at the SN even if the SCGs are not active. If “SCG activation” is considered as a starting condition for collection of UE performance, it will not be possible to obtain consistent measurements at PDCP level.

Proposal 1:  For AI/ML-assisted mobility optimization for NR-DC, “SCG activation” should not be considered as a starting condition for collection of UE performance.

Another open issue noted at the RAN3#127-bis meeting is the following:

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

In our view, including the PSCell ID in the reported UE performance feedback is a necessary condition for the MN to understand the received information as we explain below.

In the Rel-18 solution, it is specified that the target gNB should stop collecting UE performance when the “UE is handed over to another cell”. UE performance is currently reported from target gNB to source gNB without an explicit indication of the cell they refer to. This is good enough for the Handover case, since the target cell is explicitly signalled in the HANDOVER REQUEST message, and the target gNB knows exactly for which cell it should report the UE performance. Also, since the data collection should stop when the UE is handed over to another cell, there is only one cell the UE performance can refer to. This is not the case in NR-DC.

In NR-DC, the SN has the capability to change the PSCell without notifying the MN. While some companies suggest that the MN can deduce a PSCell change by analyzing received RRC messages, relying on this method is not a good engineering practice. Developing a process that requires decoding RRC messages for the sole purpose of detecting PSCell changes can lead to inefficiencies and increased complexity. Instead, we should strive for a self-contained approach, e.g., by including the PSCell ID in the reported UE performance.

Observation 1:  The MN is not readily aware of the PSCell ID. Forcing the MN to analyze received RRC messages to deduce a PSCell change is not a good engineering practice.

Furthermore, at the RAN3#127-bis meeting, the agreed exit conditions for collecting UE performance feedback do not include an equivalent to the legacy "UE is handed over to another cell," such as a "PSCell change" condition. As a result, UE performance data might be gathered across multiple PSCells of the added SN. In this case, it is essential to include the PSCell ID in the UE Performance IE, which would enable the SN to report the UE performance separately for each PSCell. This can be achieved by including several UE Associated Info Result Item IE for the same UE but with different UE Performance IE, one per PSCell the UE was connected to. This would enable the MN to easily understand that the data was collected from different cells, thus improving the interpretability of the data.

Observation 2:  UE performance data might be gathered across multiple PSCells in NR-DC. Including the PSCell ID in the UE Performance IE would enable the SN to report the UE performance separately for each PSCell.

Finally, some companies argue that including the PSCell ID in the UE Performance IE is unnecessary, claiming that the MN only needs performance information at the SN level, not at the individual cell level. This reasoning stems from the fact that the MN selects the SN, not the PSCell. However, this overlooks a key aspect of the selection process: when a gNB chooses a neighboring gNB as the SN for a UE, it does so based on the availability of good cell coverage for that specific UE. Therefore, understanding the UE performance in each PSCell is essential for making informed and effective SN selection decisions at the MN.

Observation 3:  Including the PSCell ID in the UE Performance is crucial for enabling accurate and effective SN selection based on actual cell-level performance.

Proposal 2:  For AI/ML-assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

Finally, the following open point was discussed for many meetings:

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

We have previously expressed our views, namely that indicating an expected PSCell to the SN, without indicated the expected SCG bearers the SN will serve is of little use.

We note that no progress has been made on these proposals and that substantially there is no consensus in RAN3 to move in any direction with respect to these proposals. We therefore propose to close this open point and not address it in Rel19

Proposal 3: RAN3 to agree that there is no support for signalling of a predicted PSCell ID from MN to SN in Rel19

# Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Proposal 1:  For AI/ML-assisted mobility optimization for NR-DC, “SCG activation” should not be considered as a starting condition for collection of UE performance.

Observation 1:  The MN is not readily aware of the PSCell ID. Forcing the MN to analyze received RRC messages to deduce a PSCell change is not a good engineering practice.

Observation 2:  UE performance data might be gathered across multiple PSCells in NR-DC. Including the PSCell ID in the UE Performance IE would enable the SN to report the UE performance separately for each PSCell.

Observation 3:  Including the PSCell ID in the UE Performance is crucial for enabling accurate and effective SN selection based on actual cell-level performance.

Proposal 2:  For AI/ML-assisted mobility optimization for NR-DC, extend the UE Performance IE to include the PSCell serving the UE in the SN.

Proposal 3: RAN3 to agree that there is no support for signalling of a predicted PSCell ID from MN to SN in Rel19A TP reflecting the proposals above is presented in R3-255474.