3GPP TSG-RAN WG3 Meeting #129	R3-255493

Bengaluru, India, 25th - 29th Aug, 2025

Title:	(TP for 38.401) Multi-hop relay

Agenda Item:	22.2

Source:	Huawei

Document for:	other

# 1	Introduction

In RAN3#128 meeting, RAN3 has good progress on the control plan aspects for supporting multi-hop relay. We have the following agreements and agreed the BL CRs for TS 38.401[1], TS 38.473[2], and TS 38.470[3].

The BL CRs to TS 38.401 and TS 38.473 refer to the Multi-hop relay related terminologies in the RAN2 running CR to TS 38.300. RAN3 agree to change the following Editor’s Notes into the Notes. - Editor’s Note: FFS whether Step 18 can be performed earlier, e.g., via Steps 6-13. - Editor’s Note: FFS whether this step may be performed earlier. Reuse the Peer UE ID IE as counterpart information with the update for the semantics description. WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

Additional, we have the following FFSs.

FFS whether the same PC5 RLC channel ID can be allocated toward the parent UE and child UE. FFS whether upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1. FFS whether to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel is needed can be further discussed in next meeting.

In this paper, we will discuss these FFSs and provide the corresponding TPs in the Annex.

# 2	Discussion

## PC5 RLC channel ID

In RAN3#128 meeting, RAN3 has the following WA.

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

The parent UE and child UE have different have local IDs due to they have the same last relay UE, so the same PC5 RLC channel ID can be allocated toward the parent UE and child UE. During the transmission of SRAP PDU packets, the SRAP packets of the parent UE and the child UE can be accurately transmitted through different local IDs in the packet header. This can be supported by the current spec.

The same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

There is no need to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel.

## Other FFSs

-----------------------------------------------------------TS 38.473 V18.4.0----------------------------------------------------------

#### 9.3.1.267	Remote UE Local ID

This IE uniquely identifies a L2 U2N Remote UE within the connected Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Remote UE Local ID | M |  | INTEGER (0..255, ...) | Corresponds to the sl-LocalIdentity contained in the SL-SRAP-Config IE defined in TS 38.331 [8]. |

--------------------------------------------------------------------------------------------------------------------------------------------

Similarly for multi-hop relay, the gNB-CU needs to identify the remote UE based on the INITIAL UL RRC MESSAGE TRANSFER message.

If the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE, the maximum number of remote UE connected to one last relay UE is 256, the gNB-DU shall indicate the remote UE local ID and the gNB-DU UE F1AP ID of the last relay UE in the INITIAL UL RRC MESSAGE TRANSFER message. This can be supported by the BL CR to 38.401[4].

So, there is no need for gNB-DU to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1 because the gNB-DU UE F1AP ID of the last relay UE has been in the INITIAL UL RRC MESSAGE TRANSFER message

There is no need for the gNB-DU UE F1AP IDs of the U2N First Relay UE to be included in the INITIAL UL RRC MESSAGE TRANSFER message.

According to the above discussion and proposals, we update the BL CRs for TS 38.401 in the Annex 2.

# 3	Conclusion

This document proposes:

- The same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

There is no need to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel.

There is no need for the gNB-DU UE F1AP IDs of the U2N First Relay UE to be included in the INITIAL UL RRC MESSAGE TRANSFER message.

# 4	Reference

[1] R3-253826, (TP to BL CR for TS 38.401) Support on Multi-hop U2N relay, Nokia, Nokia Shanghai Bell, LG Electronics, NIST, 3GPP TSG-RAN WG3 Meeting #128.

[2] R3-253827, (TP to BL CR for TS 38.473) Support on Multi-hop U2N relay, CATT, LG Electronics, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG3 Meeting #128.

[3] R3-253828, (TP to BL CR for TS 38.470) Support on multi-hop U2N relay, ZTE Corporation, LG Electronics, 3GPP TSG-RAN WG3 Meeting #128.

[4] R3-254014, (BL CR to 38.401) Support of Multi-hop relay, LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei, 3GPP TSG-RAN WG3 Meeting #128.

# Annex1	Text Proposal for TS 38.401

The text proposal include the changes based on the BL CR for TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.19.xx	Remote UE initial access for Multi-hop Layer-2 UE-to-Network Relay

The signalling flow for Remote UE Initial access is shown in Figure 8.19.xx-1.

Figure 8.19.xx-1: Overall procedure for Remote UE’s initial access via multi-hop relay

1.	The U2N Remote UE, the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the First U2N Relay UE via PC5 Relay RLC channel.

3.	The First U2N Relay UE withholds the received RRC message. If the First U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the Intermediate U2N Relay UE via PC5 Relay RLC channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the Intermediate U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message from the First U2N Relay UE. If all Relay UEs are in RRC_CONNECTED state, this step could be skipped.

4.	The First U2N Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the Intermediate U2N Relay UE and Last U2N Relay UE.

5.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the First U2N Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

6.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the Last U2N Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) and PC5 Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0. The same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

7.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the Last U2N Relay UE to gNB-CU.

8.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the Last U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration and PC5 Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0.

9.	The gNB-DU sends the RRCReconfiguration message to the Last U2N Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration, PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0.

10.	The Last U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

11.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the Last U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

12.	The gNB-CU configures the Intermediate U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the Intermediate U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

13.	The gNB-CU configures the First U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the First U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

14.	After receiving the local ID of the U2N Remote UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the First U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU via the Intermediate U2N Relay UE and the Last U2N Relay UE. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

15.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the Last U2N Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

16.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

17.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

18.	The gNB-CU configures the Last U2N Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the Last U2N Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

The gNB-CU configures the First U2N Relay UE and the Intermediate U2N Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First U2N Relay UE and the Intermediate U2N Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet.

NOTE 1: Step 18 can be performed earlier, e.g., via Steps 6-13.

19.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

20.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

21.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

22.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

23.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

24.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

25.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The U2N Remote UE responds with the SecurityModeComplete message.

27.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

28.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

29.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

30.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

31.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

32.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

33.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the Last U2N Relay UE, and additional PC5 Relay RLC channels for the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE.

NOTE 2: This step can be performed earlier.