3GPP TSG-RAN WG3 Meeting #129															R3-255514

Bengaluru, India, 25th - 29th Aug, 2025

Agenda Item:	15.2

Source:	Huawei, Ericsson, Jio Platforms

Title:	(TP for TS 36.300) Provision of S&F Mode Indication of neighbour cell list

Document for:	Other

# 1	Introduction

This contribution proposes a TP to TS 36.300 on the neighbor cell list for Store & Forward (S&F) operation according to RAN2 progress.

# 2	Discussion

At the previous RAN2 #130 meeting [1], a Working Assumption (WA) was agreed to include an indication specifying whether a neighboring cell operates in S&F mode, as follows:

WA: In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

The motivation is that neighboring cells may switch between S&F mode and normal mode, the onboard eNB can include the S&F indication in this neighbor cell list to assist the cell reslection for its UEs. For example, if a satellite in S&F mode is not in the NAS list, the UE can avoid accessing it and save power. And a list of satellites in normal mode (or S&F mode) can also help to prioritize the cell reselection.

There are two potential approaches as follows:

- Option 1: Exchange between onboard eNBs via the X2 interface.

- Option 2: Preconfigured by OAM.

Option 1 (inter-eNB exchange over X2) has a key limitation — it relies on the availability and stability of the X2 interface, which may not always be present or maintained in NTN deloyments. Xn also does not take account of NGAP neighbouring relation where the indication is also welcome. In contrast, provisioning the SF mode indication via OAM is more robust and implementation-independent, ensuring that the onboard eNB can obtain neighbor cell information even when X2 connectivity is unavailable. In addition, OAM allows the configuration of multiple neighboring cell entries at once, which is a more efficient way.

Therefore, it is porposed:

- RAN3 to agree to introduce an OAM-provision the S&F mode indication of the neighbour cells pending to RAN2 decision..

We should also note that RAN2 took a WA, on this issue, RAN3 should wait RAN2 final decision, if no decision this week, the topic should be able to handle via correction next meeting, as needed.

# 3	Conclusion

This paper provides analysis for the introduce an indication in the neighbor cell list to specify whether the neighbor cell operates in S&F mode or not, the following proposal is made:

- RAN3 to agree to introduce an OAM-provision the S&F mode indication of the neighbour cells pending to RAN2 decision.

# 4 Reference

- R2-2504673 Report from Break-out session on NR-NTN and IoT-NTN, RAN2#130.

# 5	TP for TS 36.300

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 23.21.8	O&M Requirements

The NTN related parameters shall be provided by O&M to the eNB providing non-terrestrial access, as specified in TS 38.300 [79]. If Store & Forward (S&F) operation is supported, the S&F indication of neighbor cell(s) is configured to the eNB by O&M.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>