3GPP TSG-RAN WG3 Meeting #129	R3-255520

Bengaluru, India, August 25th–29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1296 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Aerial UE Flight Information Failure Reporting in FAILURE INDICATION message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CMCC, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the NGAP CR “Support Aerial UE Flight Information Reporting to CN”, there are FFS on how to indicate to CN the failure and the reporting is stopped by NG-RAN. - ”Aerial UE flight information reporting failed” indication in the Location Reporting Failure message. (FFS Option 1 for Failure Handling) - Aerial UE flight information reporting failed” indication in the Location Report message and new cause value. (FFS Option 2 for Failure Handling) This CR is to implement Option 1 above: Aerial UE flight information reporting failed” indication in the Location Reporting Failure message |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include in the Location Reporting procedure: ”Aerial UE flight information reporting failed” indication in the Location Reporting Failure message. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Aerial UE flight information reporting from NG-RAN to CN for the Aerial UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.2, 9.2.11.2, 9.4.4, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1460 TS 38.300 draft CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Based on R3-255135 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.12.2	Location Reporting Failure Indication

#### 8.12.2.1	General

The purpose of the Location Reporting Failure Indication procedure is to allow the NG-RAN node to inform the AMF that the location reporting request contained in the Location Reporting Control procedure, the Handover Resource Allocation procedure or the Initial Context Setup procedure has failed. The procedure uses UE-associated signalling.

#### 8.12.2.2	Successful Operation

Figure 8.12.2.2-1: Location reporting failure indication

The NG-RAN node initiates the procedure by sending a LOCATION REPORTING FAILURE INDICATION message to the AMF. Upon reception of the LOCATION REPORTING FAILURE INDICATION message the AMF shall, based on the failure reason indicated by the Cause IE, take appropriate action.

If the aerial UE measurement is failed or stopped, the NG-RAN node initiates the procedure by sending a LOCATION REPORTING FAILURE INDICATION message to the AMF. Upon reception of the LOCATION REPORTING FAILURE INDICATION message the AMF shall, based on the Aerial UE flight information reporting failed or stopped IE, take appropriate action.

#### 8.12.2.3	Abnormal Conditions

Void.

****************************** Skip to Next Change *******************************

#### 9.2.11.2	LOCATION REPORTING FAILURE INDICATION

This message is sent by the NG-RAN node and is used to indicate the failure of location reporting.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Aerial UE flight information reporting failed or stopped | O |  | ENUMERATED(true, …) |  | YES | ignore |

****************************** Skip to Next Change *******************************

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

A2X-PC5-QoS-Parameters,

AerialUEsubscriptionInformation,

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

AssociatedSessionID,

AUN3DeviceAccessInfo,

AuthenticatedIndication,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

BroadcastTransportFailureTransfer,

BroadcastTransportRequestTransfer,

BroadcastTransportResponseTransfer,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

CEmodeBrestricted,

****************************** Skip to Next Change *******************************

id-ServedGUAMIList,

id-SliceSupportList,

id-S-NSSAI,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-AMFInformationReroute,

id-SourceToTarget-TransparentContainer,

id-SRVCCOperationPossible,

id-SupportedTAList,

id-Suspend-Request-Indication,

id-Suspend-Response-Indication,

id-TAI,

id-TAIListForPaging,

id-TAIListForRestart,

id-TargetID,

id-TargetNSSAIInformation,

id-TargettoSource-Failure-TransparentContainer,

id-TargetToSource-TransparentContainer,

id-TimeSyncAssistanceInfo,

id-TimeToWait,

id-TNGFIdentityInformation,

id-TraceActivation,

id-TraceCollectionEntityIPAddress,

id-TraceCollectionEntityURI,

id-TWIFIdentityInformation,

id-UEAggregateMaximumBitRate,

id-UE-associatedLogicalNG-connectionList,

id-UECapabilityInfoRequest,

id-UEContextRequest,

id-UE-DifferentiationInfo,

id-UE-NGAP-IDs,

id-UEPagingIdentity,

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapability-EUTRA-Format,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityID,

id-UERetentionInformation,

id-UESecurityCapabilities,

id-UESliceMaximumBitRateList,

id-UE-UP-CIoT-Support,

id-UL-CP-SecurityInformation,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-W-AGFIdentityInformation,

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-WUS-Assistance-Information,

id-XrDeviceWith2Rx,

id-SLPositioningRangingServiceInfo,

id-Aerial-UE-FlightInformationReporting-failed-or-stopped

****************************** Skip to Next Change *******************************

-- **************************************************************

--

##### -- LOCATION REPORTING FAILURE INDICATION

--

-- **************************************************************

LocationReportingFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportingFailureIndicationIEs} },

...

}

LocationReportingFailureIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-Aerial-UE-FlightInformationReporting-failed-or-stopped			CRITICALITY ignore	TYPE Aerial-UE-FlightInformationReporting-failed-or-stopped					PRESENCE optional}|,

...

}

Aerial-UE-FlightInformationReporting-failed-or-stopped::= ENUMERATED {

true,

...

}

****************************** Skip to Next Change *******************************

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

****************************** Skip to Next Change *******************************

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-Aerial-UE-FlightInformationReporting-failed-or-stopped			ProtocolIE-ID ::= 4xx

END

-- ASN1STOP