3GPP TSG-RAN WG3 #129								R3-255523

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	12.2

Source:	China Telecom, CATT, Huawei, DoCoMo, Lenovo, Samsung, NEC

Title: 	Way Forward On Multi-hop Prevention for WAB

Document for:	Discussion

Introduction

The new WI on additional topological enhancements for NR was approved in RAN#105 meeting [1]. The following candidate solutions to avoid multi-hop WAB topology were discussed at RAN3#126 meeting:

- Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters.

- Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed.

- Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

- Solution5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

During the outcome of online discussion, Solution 2 for handover scenario was agreed and the following agreement was achieved:

Agree on Solution 2:

For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

For initial access procedure, the solution 1 and Solution 3 still need to be further discussed. In the last meeting, avoiding multi-hop in case of initial access was discussed. The chairman notes are captured as below:

FFS from previous meeting: To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected 1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells. 3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator. Solution 1 (dedicated existing OTA parameters) is used for preventing multihop WAB topology. Solution 3 For multi-hop avoidance during WAB-MT initial access, RAN3 to agree that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT. Solution 1 is implementation specific and it is not subject to specifications changes. It is FFS whether Solution 3, requiring impacts to SIBs, can be agreed. If there is consensus in RAN3 on an agreement for solution 3, RAN2 involvement is needed.

It can be observed that Solution 1 (dedicated existing OTA parameters) is considered as an implementation specific solution without specification impact, while Solution 3 needs RAN2 involvement on enhancements to SIB and WAB-MT handling. Therefore, a compromise solution is needed to solve the co-existence of Solution 1 and Solution 3.

Discussion

Restrictions on implementation-based solution (Solution 1)

As we known, Solution 1 was introduced in NCR which is a stationary node in deployment. We understand Solution 1 is suitable for stationary scenario or small region. In addition, Solution 1 also implies that WAB is restricted to operation within a dedicated network that possesses exclusive resources. At present, a dedicated network is exclusively deployed for railway applications. However, according to the justification provided in the WID, WAB supports multiple application scenarios, including aircraft, cruise ships, helicopters, and vehicles [1].

Solution 1 claims some potential dedicated resources for WAB, such as PCI, frequency, NR CGI and TAC. The problem of dedicated PCI/frequency is that it is very difficult in mobility or roaming scenario. Because frequency/PCI is precious resource, it should be locally assigned to facilitate the radio resource reusing, that the dedicated frequency/PCI for WAB in one place would not be applicable to another place where the WAB-node moves, which will make the network configuration very complicated. So, this is only applicable for stationary cases such as the operator’s trials or demonstrations.

The problem of dedicated TAC/NR CGI is that it requires reserving a range of values used for WAB from the global arrangement pool. For example, it requires several TAC values reserved for each of the AMFs where the WAB-nodes may move. Normally, all ranges of TAC/NR CGI have already been allocated by operators in their 5G commercial network. Moreover, the change of TAC/CGI may impact on charging system. Therefore, operators believe they have to totally re-frame their network planning if Solution 1 applied, which is currently a large burden for a mature 5G network. During online discussion in last meeting, serval operators express the concerns on the impact on network planning. It is observed that Solution 1 would be workable in some cases (e.g., stationary scenario), but it has many restrictions.

Secondly, the implementation-based solution is assumed out of WI scope, because WIs usually only focus on specification-based solution.

Observation 1: Although implementation-based solution (Solution 1) is feasible in some cases,

- It will cause large burden to operators’ network planning/deployment in reality.

- It is assumed beyond RAN3’s discussion, the WIs usually only focus on specification-based solution.

Feasibility of spec-based solution (Solution 3)

Solution 3 is based on a WAB-specific indicator broadcasted in SIB to bar WAB-MT, RAN2 involvement for introducing the new indicator to SIB and new WAB-MT behaviour. However, it does not require any resource reservation, thus, it can be used in whatever scenario and make the operators deploying WAB easily. Although Solution 3 is subject to RAN2 spec change, we assume it’s simple and the impact to RAN2 is limited. In general, WAB-MT anyway has to read the broadcast information before camping on a cell, reading the WAB-specific indicator will not introduce much complexity. It’s well known that barring UEs via broadcasting information is a widely used solution in RAN2.

Observation 2: Solution 3 is applicable to any scenario, simple to realized, and the impact to RAN2 is assumed limited.

Way Forward

Per analysis above, based on the restrictions of Solution 1, Solution 1 is not sufficient to address multi-hop avoidance. It’s necessary to support Solution 3 as an alternative except solution 1. That means WAB-gNB cells can broadcast a new indicator to bar WAB-MT.

Proposal 1: RAN3 to confirm supporting Solution 3, and send a LS to RAN2 to start the spec work on supporting the spec-based solution.

In last several meeting, some vendors expected the legacy UE (e.g., with only Rel-15~Rel-18 UE capabilities) can also act as WAB-MT. As a compromise solution, both Rel-19 UE and Rel-15~Rel-18 UE can also act as WAB-MT.

The the WAB-MTs with Rel-19 UE capability shall support the capability of decoding the new indicator to avoid (re-)selecting to a WAB cell, while the WAB-MTs with only Rel-15~Rel-18 UE capabilities can avoid multi-hop via the implementation-based solution. This means only the WAB-MTs with this capability need to handle the new indicator broadcasted. But there is no restriction for the WAB-MTs with this capability also using the implementation-based solution.

Proposal 2: RAN3 assumes that supporting cell barring based on the new indicator in SIB is an optional capability for WAB-MT, which means:

- WAB-MTs with Rel-19 UE capability can read the new indicator in SIB to avoid to access WAB node

- WAB-MTs without this UE capability (e.g., with only Rel-15~Rel-18 UE capabilities) can avoid multi-hop based on implementation

Proposal

Based on the above analysis, we have the following observations and proposals:

Observation 1: Although implementation-based solution (Solution 1) is feasible in some cases,

- It will cause large burden to operators’ network planning/deployment in reality.

- It is assumed beyond RAN3’s discussion, the WIs usually only focus on specification-based solution.

Observation 2: Solution 3 is applicable to any scenario, simple to realized, and the impact to RAN2 is assumed limited.

Proposal 1: RAN3 to confirm supporting Solution 3, and send a LS to RAN2 to start the spec work on supporting the spec-based solution.

Proposal 2: RAN3 assumes that supporting cell barring based on the new indicator in SIB is an optional capability for WAB-MT, which means:

- WAB-MTs with Rel-19 UE capability can read the new indicator in SIB to avoid to access WAB node

- WAB-MTs without this UE capability (e.g., with only Rel-15~Rel-18 UE capabilities) can avoid multi-hop based on implementation

Reference

- RP-242395，New WID on additional topological enhancements for NR，NTT DOCOMO, INC., AT&T