3GPP TSG-RAN WG3 Meeting #129														       R3-255535

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	9.2

Source:	Ericsson, CATT, Jio Platforms

Title:	Rel-18 LTM Correction for L2 reset

Document for:	Discussion, Agreement

# Introduction

In this contribution we discuss a correction for inter-gNB-DU LTM in Rel-18, to allow the new serving gNB-DU to take appropriate decision regarding the L2 reset handling.

# Layer 2 reset at Inter-gNB-DU LTM

In Rel-18, during the LTM preparation phase, the CU can send to source/candidate DU the intra-DU L2 reset configuration in the LTM Reset Information IE within the UE CONTEXT MODIFICATION REQUEST message. The DU shall take this into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch:

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

There appears to be some ambiguity in RAN3 regarding whether L2 reset is restricted to intra-DU LTM only. Based on RAN2’s agreements, no such scenario limitation on L2 reset is specified.

RAN2 agreed on the followings.

- L2 reset (RLC reestablishment and PDCP data recovery) is also applicable for the intra-DU LTM case.

- To determine if to reset L2 or not is based on RRC configuration.

Therefore, L2 reset should be supported for both intra-DU and inter-DU cases.

- L2 reset is applicable for both intra-DU and inter-DU LTM in Rel-18.

After an LTM cell switch, the new serving DU needs to understand whether to apply a L2 reset for the UE or not. The new serving DU can deduce this by comparing the L2 reset ID of the old serving cell with the L2 reset ID of the new serving cell.

At intra-DU LTM cell switch execution, the source and target DU are the same, the DU knows the identities of the source cell and target cell. The new serving DU can compare the L2 reset ID of the old serving cell and the L2 reset ID of the new serving cell, it understands whether the UE has performed L2 reset or not, and can take this into account.

At inter-DU LTM cell switch execution, the target DU receives the CU-DU CELL SWITCH NOTIFICATION which only contains the target cell ID, not the old serving cell ID, and because of that, it cannot understand if the UE has performed L2 reset or not. To fix this problem, we propose that the CU-DU CELL SWITCH NOTIFICATION is extended, to include the old serving cell ID. If this information is added, the new serving DU can compare the L2 reset ID of the source and target cell, understand if the UE performed L2 reset, and take this into account.

Based on the above, we think the issue can be resolved with the following impacts in our specification:

1) In F1AP, extend the CU-DU CELL SWITCH NOTIFICATION message adding the source cell ID.

2) update stage 2 (38.401), to change step 20 where the CU sends to the new serving DU a CU-DU CELL SWITCH NOTIFICATION including the old serving cell id.

- Add a new Old Serving Cell ID IE in the CU-DU CELL SWITCH NOTIFICATION message to indicate the ID of the cell serving the UE before LTM cell switch execution.

- Update stage 2 (TS 38.401) to update the Cell Switch Notification message where the CU sends to the new serving DU a CU-DU CELL SWITCH NOTIFICATION message including the ID of the cell serving the UE before LTM cell switch execution.

- Agree to the CRs for TS 38.473 and TS 38.401 in R3-255536 and R3-255537 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made the following observation and proposals:

Observation 1	L2 reset is applicable for both intra-DU and inter-DU LTM in Rel-18.

Proposal 1	Add a new Old Serving Cell ID IE in the CU-DU CELL SWITCH NOTIFICATION message to indicate the ID of the cell serving the UE before LTM cell switch execution.

Proposal 2	Update stage 2 (TS 38.401) to update the Cell Switch Notification message where the CU sends to the new serving DU a CU-DU CELL SWITCH NOTIFICATION message including the ID of the cell serving the UE before LTM cell switch execution.

Proposal 3	Agree to the CRs for TS 38.473 and TS 38.401 in R3-255536 and R3-255537 that capture the above proposals.