3GPP TSG-RAN WG3 Meeting #129		       R3-255539

Bengaluru - India, 25th – 29th August 2025

Title: 	Partial Data Collection Reporting

Source: 	Ericsson, Jio Platforms, BT, LG Electronics

Agenda item:	31.2

Document Type:	Discussion and agreement

# 1.	Introduction

In Rel-18, the Data Collection Reporting Initiation and Data Collection Reporting procedures were introduced. In this contribution, we discuss enhancement for the reporting of UE performance.

# 2.	Discussion

### 2.2	Data collection for AI/ML enabled use cases

The Data Collection Reporting Initiation and Data Collection Reporting procedures were introduced to collect data, e.g., to support AI/ML in RAN. The signalling procedures are generic, i.e., they do not explicitly indicate for which purpose a certain information is being requested or provided.

For AI/ML enabled use cases, the information can be used, for instance, to train an AI/ML model, as input for an AI/ML inference function, or as feedback to collect data that can be used as ground truth corresponding to the inference output of an AI/ML model.

In Rel-18 and Rel-19, extensive discussions have been conducted, for example, in relation to UE performance reporting, under which conditions UE performance is collected, when and how the reporting can occur. In particular, in the configuration phase, a gNB1 can request a gNB2 to report UE performance only once (one-time reporting) or periodically. In this second case, a reporting periodicity is specified (Reporting Periodicity for Data Collection, current range contains discrete values: 500ms, 1000ms, 2000ms, 5000ms, 10000 ms). A common configuration parameter for both one-time and periodic reporting is the Collection Time Duration for UE Performance IE (current range: 1..5000ms), with which gNB1 indicates to gNB2 a maximum time interval, starting from a successful handover (for UE in single connectivity) or from an SN addition (for UE in dual connectivity), until when gNB2 will collect UE performance.

In addition to a specified maximum time interval for data collection, gNB2 shall stop the collection of UE performance when certain exit conditions are fulfilled:

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	the NR-DC with the S-NG-RAN node for the UE is released.

A collected UE performance is reported at the next available DATA COLLECTION UPDATE message.

Considering the current signalling details, a DATA COLLECTION UPDATE message can contain UE reports for multiple UEs (up to 16) and the samples of UE performance for the UEs can be collected over different time intervals. An example for the case of one-time reporting where this can happen is the following:

  - the collection is requested for 5s, and this applies to all UEs handed over from gNB1 to gNB2

  - UE1, UE2 and UE3 are handed over from gNB1 to gNB2, one second after the other. UE4 is configured in dual connectivity.

  - for UE1, the collection of UE performance at gNB2 starts at time T1. After 5s, UE1 is still connected to the same cell and the collection stops at time (T1)+5s.

  - for UE2, the collection of UE performance at gNB2 starts at time T1+1s. After 2s, UE2 moves to RRC_INACTIVE (or RRC_IDLE), and collection stops for UE2.

  - for UE3, the collection of UE performance at gNB2 starts at time T1+2s. After 3s, UE3 is handed over to another cell.

  - for UE4, the collection of UE performance at gNB2 starts at time T1. After 4s, the SN is released.

The “next available DATA COLLECTION UPDATE message” (after data collection is stopped) is at time T1+5s. It is quite clear that, although one of the metrics included in the UE performance for each one of the concerned UEs can be represented by a single value, they are calculated over different time intervals:

- for UE1, one value for the DL throughput, averaged over a collection period of 5 sec

- for UE2, one value for the DL throughput, averaged over a collection period of 2 sec

- for UE3, one value for the DL throughput, averaged over a collection period of 3 sec

- for UE4, one value for the DL throughput, averaged over a collection period of 4 sec

The Rel-18 solution does not make any differentiation between the above samples, i.e., gNB1 receives the 4 above values of DL throughput and has no other choice than consider all of them as equal, in terms of their relevance.

Put in other words, assuming that gNB1 will use the values above as feedback, i.e., as ground truth corresponding to the inference output of an AI/ML model at gNB1, the DL Throughput of UE1 is collected over a period of time that is more than double of the time used to collect the DL Throughput of UE2, but still they are treated in the same way!

In fact, an implementation may decide to have this type of treatment. However, it is important that gNB1 should at least be informed that, although it specified a certain collection time duration, there are samples (e.g., particularly for UE2) for which the collection has been interrupted due to other conditions, so that gNB1 has a clear view and understanding of the UE performance. It will be up to implementation to decide how to handle this type of samples. For example, gNB1 may decide that this partially collected data is not valid and should be ignore. By providing this information, gNB1 gets the ability to decide on the best way forward.

Proposal 1: Enhance the UE Performance IE in the DATA COLLECTION UPDATE message with an IE (e.g., Partial Collection Duration) that indicates whether the UE performance refers to a collection terminated before the configured collection time duration.

Proposal 2: Agree to the CR to TS 38.423 in Appendix.

# 3.	Conclusions and Proposals

In the previous sections we made the following proposals:

Proposal 1: Enhance the UE Performance IE in the DATA COLLECTION UPDATE message with an IE (e.g., Partial Collection Duration) that indicates whether the UE performance refers to a collection terminated before the configured collection time duration.

Proposal 2: Agree to the CR to TS 38.423 in Appendix.

# Appendix - CR to XnAP – Partial Data Collection Reporting

3GPP TSG-RAN WG3 Meeting #129		       R3-25xxxx

Bengaluru - India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | - | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Partial Data Collection Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio Platforms, BT, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core,TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the reporting of UE performance, there is no indication of whether the collection of data terminated before the configured Collection Time Duration for UE Performance. A gNB receiving the UE performance cannot know whether the UE performance relates to a portion of the configured Collection Time or the complete requested time interval, and for example may not be valid. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new Partial Collection Duration IE in the UE Performance IE in the DATA COLLECTION UPDATE message to indicate that UE performance was collected only in a portion of the configured collection time duration. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the Data Collection Reporting procedure. This CR is BC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the NG-RAN node1 receives measured UE performance from NG-RAN node2, the NG-RAN node1 does not understand the relevance of the collected UE performance and it is unclear how to use this information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.179, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------ CHANGES STARTS HERE ------------------------------------------

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| Partial Collection Duration | O |  | ENUMERATED(true, ...) | Indicates that the time of collection of UE performance ended earlier than the configured Collection Time Duration for UE Performance or reporting periodicity. |

------------------------------------------ NEXT CHANGE ------------------------------------------

### 9.3.5	Information Element definitions

(Skip Unchanged)

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-PartialCollectionDuration,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

------------------------------------------ NEXT CHANGE ------------------------------------------

-- P

(Skip Unchanged)

PartialCollectionDuration ::= ENUMERATED (true, ...)

------------------------------------------ NEXT CHANGE ------------------------------------------

-- U

(Skip Unchanged)

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						BitRate					OPTIONAL,

uL-UE-AverageThroughput						BitRate					OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossDL						PacketLossRate			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PartialCollectionDuration					CRITICALITY ignore	EXTENSION PartialCollectionDuration						PRESENCE optional },

...

}

------------------------------------------ NEXT CHANGE ------------------------------------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

(Skip Unchanged)

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

(Skip Unchanged)

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-PartialCollectionDuration																		ProtocolIE-ID ::= 475 – To be assigned by MCC

END

-- ASN1STOP