3GPP TSG RAN WG3 Meeting #129	R3-255570

Bengaluru, India, 25 - 29 August, 2025

Agenda item:	20.2

Source:	Ericsson, Nokia

Title:	(TP to NRPPa/TS 38.305 BL CR): Addressing remaining issues of NW-assisted AI/ML positioning with gNB-sided model

Document for:	Other

# 1	Introduction

This paper addresses the remaining issues of Case 3a NG-RAN node assisted positioning with gNB-side model related to data collection and model inference.

A TP to NRPPa BL CR and TP TS 38.305 BLCR are proposed in the annex of this document.

# 2	Discussion

## Case 3a Data Collection procedure open issues

- Issue#1: Placement of Positioning Data Collection Needed IE in the MEASUREMENT RESPONSE/REPORT messages.

In last meeting, the Positioning Data Collection Needed IE was added in the NRPPa MEASUREMENT RESPONSE and MEASUREMENT REPORT messages as a top-level IE.

When the LMF receives the IE, it sends the new POSITIONING DATA COLLECTION REPORT message containing the positioning data information in a list of TRPs, which indicate the area of interest.

However, it is not clear if the NG-RAN node requests for data collection in a best effort way (i.e., regardless of what are the currently measuring TRPs, leaving it to LMF decision providing what is available as TRP positioning data), or if the NG-RAN should map the request for data collection to a specific list of measuring TRPs.

Observation 1: When the NG-RAN sends the Positioning Data Collection Needed IE, it is not clear if the NG-RAN node requests for data collection in a best effort way, or if instead the NG-RAN should map the request for data collection to a specific list of measuring TRPs.

If the latter, then the Positioning Data Collection Needed IE should be encoded as part of TRP List in the NRPPa MEASUREMENT RESPONSE and MEASUREMENT REPORT messages.

Proposal 1: RAN3 to discuss whether the Positioning Data Collection Needed IE should be re-encoded within a list of TRPs in NRPPa MEASUREMENT RESPONSE and MEASUREMENT REPORT messages.

Based on offline discussion, another understanding is that the LMF provides all the available data collection for the TRPs participating in the positioning session. In that case, no change is required to the NRPPa BL CR. But a clarification in stage 2 is required.

Proposal 2: As alternative to proposal 1, capture in stage 2 TS 38.305 BL CR that the LMF provides all the available data collection for all the TRPs participating in the positioning session.

- Issue#2: encoding of the Positioning Data Unavailable IE in POSITIONING DATA COLLECTION REPORT.

This IE was agreed last meeting to indicate that the LMF could not provide any positioning information when sending the DATA COLLECTION REPORT message. Currently it is encoded as “ENUMERATED(true, ...)”. The proposal is to re-encode the IE to include codepoints indicating the reason of unavailability from LMF’s side, instead of a simple “true”: e.g., to revise the codepoints into ‘no data available, user consent not granted,’, etc.

Proposal 3: Re-encode the Positioning Data Unavailable into ENUMERATED (no data available, user consent not granted, ...)”.

## Case 3a Model inference open issues

- Whether the LMF can request for AI/ML measurements?

The LMF should be able to explicitly request for inferred measurement as LMF knows the positioning QoS requirement and not leave it to gNB decide whether to perform inferred measurement or legacy measurement. This follows similar principle used for UE-based AI positioning.

Observation 2: The LMF should be able to request for AI/ML measurements, this allows more efficient use of gNB resources and signalling

The easiest way to achieve such explicit indication can be adding a new bit "Inferred measurement" in the Measurement Characteristics Request Indicator IE in 9.2.81. Another example can be to define specific codepoints for ‘inferred-UL-RTOA’ and ‘inferred gNB-Rx-Tx-TD’, in the MEASUREMENT REQUEST message. The former approach is preferred due to no ASN.1 impacts.

Proposal 4: Introduce a new bit "Inferred measurement" in the Measurement Characteristics Request Indicator IE in 9.2.81

- Indication of LoS/NLoS inferrence

It is captured as FFS if the gNB hosting the model should indicate that LoS/NLoS is inferred when reporting inferring measurements. Currently, there is no identified change to LMF behaviour with knowledge that it is inferred. It is proposed to delete it.

Proposal 5: There is no need to indicate that the LoS/NLoS form model inference is inferred. Remove and delete the FFS

# 3	Conclusions

In this paper, we have made the following observations and proposals:

Observation 1: When the NG-RAN sends the Positioning Data Collection Needed IE, it is not clear if the NG-RAN node requests for data collection in a best effort way, or if instead the NG-RAN should map the request for data collection to a specific list of measuring TRPs.

Proposal 1: RAN3 to discuss whether the Positioning Data Collection Needed IE should be re-encoded within a list of TRPs in NRPPa MEASUREMENT RESPONSE and MEASUREMENT REPORT messages.

Proposal 2: As alternative to proposal 1, capture in stage 2 TS 38.305 BL CR that the LMF provides all the available data collection for all the TRPs participating in the positioning session.

Proposal 3: Re-encode the Positioning Data Unavailable into ENUMERATED (no data available, user consent not granted, ...)”.

Observation 2: The LMF should be able to request for AI/ML measurements, this allows more efficient use of gNB resources and signalling

Proposal 4: Introduce a new bit "Inferred measurement" in the Measurement Characteristics Request Indicator IE in 9.2.81

Proposal 5: There is no need to indicate that the LoS/NLoS form model inference is inferred. Remove and delete the FFS

# 4	TP to NRPPa BL CR

<Start of changes>

### 9.1.X	Messages for Positioning Data Collection Information Transfer Procedures

#### 9.1.X.1	POSITIONING DATA COLLECTION REPORT

This message is sent to report positioning information for data collection.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.Z2 |  | YES | ignore |
| Positioning Data Unavailable | O |  | ENUMERATED(no data available, user consent not granted, …) |  | YES | ignore |

Next change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >>Sample-based UL-RTOA |  |  | 9.2.X |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >Inferred measurement | O |  | ENUMERATED(true, …) | This IE is only valid for UL-RTOA and gNB Rx-Tx Time Difference. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

Next change

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: UL SRS-RSRPP in Additional Path Fourth Bit: Multiple UL AoA in Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: SRS bandwidth aggregation used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteen bit: UL SRS-RSRPP in sample-based UL-RTOA. Fourteen bit: Inferred measurement Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

Next change

PositioningDataUnavailable ::= ENUMERATED {no-data-available, user-consent-not-granted, ...}

//End of change//

# 5	TP to TS 38.305 BL CR (one option)

<Start of changes>

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures described in Clause 8.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clause 8.

4.	The LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in step 3 that positioning data collection is needed. The message includes all available positioning data collection information from the TRPs participating in the positioning session. The message also includes correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

//End of change//