3GPP TSG-RAN WG3 Meeting #129	R3-255580

Bengaluru, India, August 25th–29th, 2025

Agenda Item:	17.3

Source:	Ericsson

Title:	Stage 2 Specifications TPs for On-demand SIB1 for UEs in idle/inactive mode

Document for:	Other

# Introduction

The revised Work Item Description (WID) for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

In this contribution, we present several TPs for stage 2 specifications regarding Objective 2 of the work item (see highlighted text above).

# Text Proposal for Stage 2 specifications

Text Proposal for BL CR for TS 38.420

*************** Start of TP ***************

### 5.2.4	Energy saving function

This function enables decreasing energy consumption by indication of cell or SSB beam activation/deactivation over the Xn interface, or by enabling the neighbour NG-RAN node(s) to broadcast the UL WUS configuration in support of on-demand SIB1 transmission.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.2.5	Energy saving procedures

### The following procedures are used by NG-RAN nodes to reduce energy consumption:

-	Cell Activation procedure: enables an NG-RAN node to request the activation of a previously deactivated cell or SSB beam hosted in another NG-RAN node.

-	UL WUS Configuration Provision procedure: enables an NG-RAN node to provide UL WUS configuration to another NG-RAN node in support of on-demand SIB1 transmission and to request the receiving, assisting NG-RAN node to start or stop broadcasting the UL WUS configuration.

-	UL WUS Configuration Provision Status procedure: enables an assisting NG-RAN node to inform the requesting NG-RAN node that the UL WUS configuration broadcasting has stopped.

*************** End of TP ***************

Text Proposal for BL CR for TS 38.470

*************** Start of TP ***************

## 5.2	F1-C functions

### 5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU. The F1 setup function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. The gNB-CU Configuration Update function may activate SSB beams of cells. The gNB-CU Configuration Update function may indicate the cells where the gNB-DU is allowed to deactivate the SSB beams. With the gNB-CU Configuration Update function, energy saving with cell activation and cell deactivation can be supported as defined in TS 38.300 [8]. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. The gNB-DU Configuration Update function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The F1 setup and gNB-DU Configuration Update functions allow to inform the S-NSSAI(s), NSAG(s), CAG ID(s) and NID(s) supported by the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide information on either RedCap access configuration, or eRedCap access configuration, or both at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide barring exemption for emergency call information at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide barring configuration information for 2Rx XR UEs at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide and request coordination of the UL WUS configuration information for on-demand SIB1 at the gNB-DU. The gNB-CU Configuration Update function allows to indicate the cells where the gNB-DU may use on-demand SIB1 transmission.

The F1 setup and gNB-CU Configuration Update functions allow to inform the NID(s) available at the gNB-CU.

The F1 resource coordination function is used to transfer information about frequency resource sharing between gNB-CU and gNB-DU. In case of split gNB architecture, the gNB-CU may consolidate the outgoing messages from multiple gNB-DUs and distribute the incoming messages to the involved gNB-DUs, to perform resource coordination.

*************** End of TP ***************

Text Proposal for TS 38.401

*************** Start of TP ***************

## 8.x	Procedures for network energy savings

### 8.x.1	Procedures for on-demand SIB1

#### 8.x.1.1	OD-SIB1 configuration coordination – start

The signalling flow for starting inter-gNB coordination of on-demand SIB1configuration provisioning is shown in Figure 8.x.1.1-1.

Figure 8.x.1.1-1: Inter-gNB coordination of on-demand SIB1configuration provisioning – start

1.	The gNB1-DU generates the OD-SIB1 configuration for one or more of its cells.

2a.	The gNB1-DU sends a GNB-DU CONFIGURATION UPDATE message to the gNB1-CU, which includes the OD-SIB1 configuration(s) for the desired cell(s).

2b.	The gNB1-CU responds to gNB1-DU with a GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message.

3a.	The gNB1-CU sends an UL WUS CONFIGURATION PROVISION REQUEST message to the gNB2-CU to start the coordination of the OD-SIB1 configuration provisioning.

3b.	In the case the gNB2-CU is able to broadcast the OD-SIB1 configuration as requested, it responds to the gNB1-CU with an UL WUS CONFIGURATION PROVISION RESPONSE message.

NOTE 1:	Step 3b may occur after step 4b.

NOTE 2:	In the case the gNB2-CU is not able to broadcast the OD-SIB1 configuration as requested, it responds to the gNB1-CU with an UL WUS CONFIGURATION PROVISION FAILURE message, and the procedure terminates.

4a.	The gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to the gNB2-DU with an updated SIBx, which includes the received OD-SIB1 configuration(s) from gNB1-CU.

4b.	The gNB2-DU responds to the gNB2-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

5.	The gNB2-DU starts broadcasting the OD-SIB1 configuration(s) in the updated SIBx.

NOTE 3:	Steps 6–7 may happen in parallel to steps 4–5, but step 6a must come after step 3b.

6a.	The gNB1-CU sends a GNB-CU CONFIGURATION UPDATE message to the gNB1-DU, including the On-Demand SIB1 Indicator IE set to “start” for the cell(s) where assistance for OD-SIB1 configuration provisioning is assured.

6b.	The gNB1-DU responds to the gNB1-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

7.	The gNB1-DU may start operating the cell(s), for which assistance for OD-SIB1 configuration provisioning is assured, in on-demand SIB1 mode.

#### 8.x.1.2	OD-SIB1 configuration coordination – requesting-node–initiated stop

The signalling flow for terminating inter-gNB coordination of on-demand SIB1configuration provisioning, initiated by the requesting node, is shown in Figure 8.x.1.2-1.

Figure 8.x.1.2-1: Inter-gNB coordination of on-demand SIB1configuration provisioning – requesting-node–initiated stop

1.	The gNB1-DU determines that it will no longer operate one or more of its cells in OD-SIB1 transmission mode. The gNB1-DU reverts the operation of the desired cell(s) to periodic SIB1 transmission mode.

2a.	The gNB1-DU sends a GNB-DU CONFIGURATION UPDATE message to the gNB1-CU, which includes the On-Demand SIB1 IE set to “stop” for the desired cell(s).

2b.	The gNB1-CU responds to gNB1-DU with a GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message.

3a.	The gNB1-CU sends an UL WUS CONFIGURATION PROVISION REQUEST message to the gNB2-CU to stop the coordination of the OD-SIB1 configuration provisioning.

3b.	The gNB2-CU responds to the gNB1-CU with an UL WUS CONFIGURATION PROVISION RESPONSE message.

4a.	The gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to the gNB2-DU with an updated SIBx, which excludes the OD-SIB1 configuration(s) for the indicated cell(s) by gNB1-CU.

4b.	The gNB2-DU responds to the gNB2-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

5.	The gNB2-DU stops broadcasting the indicated OD-SIB1 configuration(s) in SIBx.

#### 8.x.1.3	OD-SIB1 configuration coordination – assisting-node–initiated stop

The signalling flow for terminating inter-gNB coordination of on-demand SIB1configuration provisioning, initiated by the assisting node, is shown in Figure 8.x.1.3-1.

Figure 8.x.1.3-1: Inter-gNB coordination of on-demand SIB1configuration provisioning – assisting-node–initiated stop

1.	The gNB2-CU decides to stop the provisioning of the OD-SIB1 configuration(s) for certain cell(s) from gNB1-CU.

2.	The gNB2-CU sends an UL WUS CONFIGURATION PROVISION STATUS message to the gNB1-CU to indicate that it stops the provisioning of the OD-SIB1 configuration(s) for certain indicated cell(s).

3a.	The gNB2-CU sends a GNB-CU CONFIGURATION UPDATE message to the gNB2-DU with an updated SIBx, which excludes the OD-SIB1 configuration(s) for the indicated cell(s).

3b.	The gNB2-DU responds to the gNB2-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

4.	The gNB2-DU stops broadcasting the indicated OD-SIB1 configuration(s) in SIBx.

NOTE:	Steps 5–6 may happen in parallel to steps 3–4, but step 5a must come after step 2.

5a.	The gNB1-CU sends a GNB-CU CONFIGURATION UPDATE message to the gNB1-DU, including the On-Demand SIB1 Indicator IE set to “stop” for the indicated cell(s) by gNB2-CU.

5b.	The gNB1-DU responds to the gNB1-CU with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

6.	The gNB1-DU shall revert the operation of the indicated cell(s) from on-demand SIB1 transmission mode to periodic SIB1 transmission mode, if currently operating in on-demand mode. If the gNB1-DU intends to operate the indicated cell(s) in on-demand SIB1 transmission mode again, it may restart coordination for on-demand SIB1 configuration provisioning.

*************** End of TP ***************