3GPP TSG-RAN WG3 Meeting #129	R3-255588

# Bengaluru, India, 25th – 29th Aug, 2025

Title: 	AI/ML-Based CCO with Timing Aspects over Xn

Source: 	Jio Platforms, Ericsson

Agenda item:	11.3

Document Type:	Discussion and decision

# 1	Introduction

During the last RAN3 meeting, discussions highlighted an open question: whether the Time for Predicted CCO Issue should be transferred over Xn alongside already agreed-upon information (e.g., predicted coverage modification cause, future coverage states, and their timing). While these elements were standardized, no conclusion or consensus was reached on the necessity of signaling the predicted issue time, as captured in the meeting minutes:

Timing information for predicted CCO issue over Xn is not needed?.

This contribution addresses the ongoing study of AI/ML-based Coverage and Capacity Optimization (CCO) in NG-RAN architectures, building on prior agreements from RAN3#128 and preliminary discussions in RAN3#129. A key focus is the role of timing information for predicted CCO issues in multi-node scenarios, particularly over the Xn interface. In dynamic, AI/ML-driven environments, accurate scheduling of future network state changes is critical for resource efficiency and service continuity.

# 2	Discussion on AI/ML Predicted CCO issue timing aspect.

##### 2.1 Temporal Granularity in Predictive CCO Signaling

As part of the ongoing enhancements for AI/ML-based Coverage and Capacity Optimization (CCO) in NG-RAN, recent 3GPP discussions have focused on the signalling design for exchanging predictive CCO assistance information across RAN interfaces. At RAN3#128, it was agreed that mechanisms should be specified over both the F1 and Xn interfaces to support the update and cancellation of previously signaled predicted CCO issues. It was also concluded that no new information element (IE) is required to identify an earlier prediction instance, as the set of predicted cells and SSBs suffices to uniquely reference a prior CCO prediction.

A major open point remains the signaling of timing information related to predicted CCO issues over Xn. The current working assumption suggests that such timing may not be necessary “unless the usage can be identified.” However, growing architectural complexity, especially in multi-node scenarios where overlapping predictive and detected issues may coexist, has brought new relevance to the issue of timing synchronization. Indeed, determining when a predicted CCO issue is expected to occur is critical for allowing remote nodes to prepare or adjust their own coverage states in a timely manner.

Observation 1: In multi-beam, multi-cell environments, simultaneous AI/ML predictions may target different coverage modification causes (e.g., congestion, coverage hole, interference). As the temporal resolution of these predictions increases, their coordination becomes critical to prevent conflict or redundant network adjustments.

In Release 19 R3-253988 BLCR to TS 38.473, a new IE called Time for Predicted CCO Issue [1] was introduced to allow the gNB-CU to inform the gNB-DU of the forecasted time at which a predicted CCO issue is expected to occur. This time indicator ensures that the future CCO state is not applied immediately but rather only when necessary, preventing premature or unnecessary beam and coverage modifications. In parallel, the gNB-DU sends back a separate timing indicator called Time for Future Coverage Modification, denoting the planned activation time of the mitigative CCO state. This feedback loop allows the gNB-CU to assess whether predicted issues have been proactively addressed and to potentially cancel or reprioritize ongoing predictions accordingly.

When multiple predicted or detected Coverage and Capacity Optimization (CCO) issues are simultaneously valid or overlap in a partially intersecting set of resources—such as when two beams in the same or neighboring cells are predicted to experience congestion or signal degradation at different future times—the receiving gNB-CU must be equipped with a mechanism to accurately distinguish, schedule, and prioritize between the concurrent CCO events. Simply relying on cell or SSB beam identifiers is insufficient in such cases, as identifier-based signaling does not convey temporal specificity or the sequence of events with adequate granularity.

In scenarios involving overlapping predictions or conflicts between a predicted and a detected issue, the lack of clearly defined activation and validity timing for each issue introduces ambiguity in network decision-making. This is especially critical when the triggering and resolution of predicted CCO states are implicitly based on modeled assumptions, such as traffic forecasts or beam coverage degradation projections, without explicitly signaling when these changes are expected to become effective.

In particular, when predicted CCO assistance information is communicated without a well-defined time window (i.e., time of predicted issue occurrence and its expected duration), it becomes extremely challenging for the gNB-CU to coordinate resource adaptation, load balancing, or beam adjustment actions—especially if other simultaneous CCO issues are present. The result is a potential misalignment of resource optimization operations, which can lead to suboptimal or even contradictory network behaviors.

Moreover, without precise timing annotations, it is unclear whether a newly received predicted CCO issue supersedes a previously signaled one, should be merged into a multi-phase prediction profile, or ignored as redundant. This is particularly problematic in highly dynamic or AI-assisted predictive CCO frameworks, where multiple outputs from the prediction model may relate to the same resource over different anticipated time periods.

Observation 2: When multiple predicted or detected CCO issues overlap (e.g., two beams predicted for congestion at different times), the receiving gNB-CU needs a method to differentiate and prioritize between them. This cannot be achieved through cell/beam identifiers alone. Absence of timing information results in ambiguity, particularly when future coverage state activation times are derived indirectly.

Therefore, the absence of time domain parameters in the signaling of predicted and detected CCO issues—such as explicit "time of validity," "activation time," or "forecast window"—limits the ability of NG-RAN nodes to orchestrate timely and effective optimization actions. It also impairs the ability to prioritize real-time detections over scheduled predictions and introduces ambiguity when reconciling overlapping issue reports. A robust CCO coordination mechanism must include not only spatial identifiers (i.e., cell and beam indices) but also detailed time-related metadata, to ensure that multiple issues targeting the same or adjacent resources can be resolved in a deterministic, prioritized, and conflict-free manner.

Observation 3: Unlike legacy static CCO where a cell can only be assigned a single CCO state, the predictive model under AI/ML allows multiple future states per cell and SSB, with distinct timing and modification cause values. This demands explicit signaling of associated prediction time to preserve temporal alignment.

When both detected and predicted Coverage and Capacity Optimization (CCO) issues coexist within the same spatial or temporal domain, the receiving gNB-CU (or coordinating node) faces a complex decision-making scenario that cannot rely solely on issue identifiers or origin. In such cases, effective prioritization mechanisms must account for multiple dimensions, particularly the severity of each issue (e.g., expected throughput degradation, congestion risk, or coverage gap extent) as well as the timing of the anticipated impact. However, in the absence of an explicit indication of the prediction time—that is, the estimated time at which a forecasted issue is expected to materialize—the system lacks a critical input required to establish temporal ordering among competing events. This lack of timing context introduces ambiguity when two or more CCO issues are scheduled to affect overlapping or adjacent coverage areas, particularly if the window for applying corrective actions (e.g., beam adjustments, power control, or sleep mode reconfiguration) is narrow.

In such a scenario, the inability to distinguish whether a predicted issue is imminent or distant in time hinders the gNB-CU’s ability to decide whether to react immediately, defer action, or prioritize a concurrently detected real-time issue. Consequently, operational policies based solely on static identifiers or issue categories become insufficient. A robust resolution requires the integration of timestamped prediction metadata to enable dynamic prioritization logic that aligns with both the urgency and the potential network impact of each issue.

Observation 4: In the presence of coexisting detected and predicted CCO issues, priority mechanisms must consider both severity and timing. Without prediction time indication, the receiver has no basis to determine which issue should take precedence if activation windows overlap.

##### 2.2 Temporal Granularity in Predictive CCO Signaling over Xn

The introduction of a deterministic “time-to-action” parameter—explicitly indicating the expected onset time or activation window of a predicted CCO issue—would significantly enhance the orchestration capabilities of the receiving network entity, such as a gNB-CU. With this temporal anchor in place, the network could engage in forward-looking, context-aware planning to mitigate performance degradation before it materializes. For instance, time-aware scheduling of beam reconfiguration procedures could be initiated during low-traffic intervals to minimize disruption; similarly, proactive user equipment (UE) offloading strategies could be pre-calculated and executed just-in-time to avoid overload scenarios, rather than reactively after congestion is observed. Additionally, pre-emptive adjustments of antenna tilt or transmit power levels could be coordinated with surrounding nodes in a harmonized manner, ensuring that coverage optimization actions do not inadvertently induce instability in neighboring cells.

Such time-sensitive operations demand not only knowledge of what optimization to perform, but when it must be applied to yield the intended effect without overreacting or wasting energy. The absence of a well-defined “time-to-action” results in operational uncertainty, forcing the network to either delay corrective actions—thereby increasing the risk of service impact—or to apply preemptive strategies prematurely, which may lead to suboptimal resource usage or unnecessary disruption to ongoing sessions.

Moreover, without this temporal precision, coordination across distributed nodes becomes more challenging, especially in multi-gNB scenarios where concurrent optimization actions must be carefully sequenced to avoid conflicting adaptations. The orchestration logic loses a key input required for constructing a prioritized execution timeline, and this limits the potential benefits of AI/ML-driven predictive frameworks. Thus, integrating a standardized and accurate “time-to-action” field into the prediction reporting structure is not merely an enhancement—it is a foundational requirement for enabling reliable, efficient, and intelligent RAN behavior in the face of upcoming CCO challenges.

Observation 5: A deterministic "time-to-action" associated with a predicted issue would enable scheduling of beam re-configuration, UE offloading, or pre-emptive power tilt optimization — all of which require forward planning. The lack of this parameter prevents time-sensitive orchestration.

For XnAP signaling, RAN3 has already agreed to support transfer of the Time for Future Coverage Modification IE across peer gNBs, encapsulated in the NG-RAN NODE CONFIGURATION UPDATE message [2] (figure 2), as specified in R3-253986 (CR to TS 38.423). However, it is still an open question whether the Time for Predicted CCO Issue IE should also be signaled over Xn. Doing so could offer important benefits: enabling the neighbor gNB to differentiate multiple simultaneous predictions, avoid conflicting resource decisions, and properly align CCO mitigation timelines with local traffic or beam planning.

Figure 2. 3GPP TS 38.423 §8.4.2.2-1: NG-RAN node Configuration Update, successful operation

Especially in cases where overlapping predicted and detected CCO issues exist for different beams or cells, the availability of Time for Predicted CCO Issue empowers nodes to resolve prioritization and coordination challenges. Without it, ambiguity arises over which predicted event is more urgent or whether mitigation actions for one issue could inadvertently interfere with another.

Concluding, when the local node-level signaling of prediction timing is already specified, there is a strong argument for its inclusion in inter-node signaling over Xn. Such inclusion would enable end-to-end temporal consistency in the AI/ML-based CCO framework, supporting informed decision-making, better prioritization, and ultimately more stable and efficient coverage adaptation strategies across the NG-RAN.

When a predicted CCO issue is communicated over Xn, it should always be accompanied by a Time for Predicted CCO Issue field, representing the predicted occurrence time of the CCO problem. This value serves as an anchor for aligning the future CCO state in the neighbor gNB and improves interpretability of the Future Coverage Modification List IE.

- This is particularly necessary in the case where:

- Multiple predictions are transmitted from different gNB-CUs to a common neighbor.

- Recurrent predictions are periodically updated by the AI/ML model.

- Detected issues arise simultaneously or shortly before the predicted issue activation.

##### 2.3 Decision on Timing Info for Predictive CCO Signaling over Xn

By introducing a consistent Time for Predicted CCO Issue, the future activation time of the associated CCO State can be derived or validated. Moreover, the temporal relation between new and prior predictions can be used to resolve potential conflicts or redundancies.

Proposal 1: RAN3 to agree that, in the context of AI/ML-based CCO prediction exchange over Xn, the Time for Predicted CCO Issue should be explicitly included for each predicted CCO entry.

It is therefore proposed that RAN3 should reach consensus on the necessity of including and utilizing explicit timing information—such as “Time for Predicted CCO Issue”—whenever multiple predictive CCO assistance messages are associated with the same neighboring gNB or its constituent cells and beams. In scenarios where several predictions target partially overlapping spatial resources within the same neighbor node, the presence of timing metadata becomes essential to disambiguate between them. Each prediction may refer to a different forecast window, correspond to distinct causes (e.g., capacity shortage, signal degradation, or interference spike), or require mitigation actions with different urgency levels.

In such cases, the receiving gNB-CU must not only recognize that multiple predictions exist but also evaluate their relative temporal relevance. Timing information provides the foundation for determining the correct sequence of planned coverage or capacity adaptations, ensuring that mitigation actions are neither prematurely triggered nor applied too late to be effective. Without this temporal differentiation, the coordinating node may misinterpret a newer prediction as redundant or mistakenly assume that overlapping predictions refer to a single event—potentially causing ineffective or contradictory optimization behavior.

Moreover, when overlapping or conflicting CCO predictions coexist—especially across adjacent beams or cells controlled by the same neighbor gNB—the use of timing information allows the system to apply a prioritization logic based on both urgency and impact horizon. For example, a prediction indicating imminent congestion in one beam may take precedence over another forecasted degradation event that is expected to occur later in time, even if both refer to the same physical cell. Similarly, long-term, low-severity issues may be deprioritized in favor of short-term, high-impact events, provided that the timing data is available to enable such comparisons.

Therefore, the agreement by RAN3 should affirm that timing information is not merely optional but constitutes a critical component of predictive CCO signaling. It serves as an enabler for intelligent scheduling, preemptive mitigation, and effective multi-issue prioritization in increasingly dynamic and AI-assisted RAN environments. This principle holds particular importance in multi-node topologies, where local predictions may need to be reconciled against remote forecasts to ensure coordinated, non-conflicting adaptations across the network.

Proposal 2: RAN3 to agree that when multiple predicted CCO issues exist for a given neighbor gNB, the timing information shall be used to distinguish among them and to guide prioritization in case of overlapping or conflicting predictions.

# 3	Conclusion

Follows the summary of Observations and Proposals:

Observation 1: In multi-beam, multi-cell environments, simultaneous AI/ML predictions may target different coverage modification causes (e.g., congestion, coverage hole, interference). As the temporal resolution of these predictions increases, their coordination becomes critical to prevent conflict or redundant network adjustments.

Observation 2: When multiple predicted or detected CCO issues overlap (e.g., two beams predicted for congestion at different times), the receiving gNB-CU needs a method to differentiate and prioritize between them. This cannot be achieved through cell/beam identifiers alone. Absence of timing information results in ambiguity, particularly when future coverage state activation times are derived indirectly.

Observation 3: Unlike legacy static CCO where a cell can only be assigned a single CCO state, the predictive model under AI/ML allows multiple future states per cell and SSB, with distinct timing and modification cause values. This demands explicit signaling of associated prediction time to preserve temporal alignment.

Observation 4: In the presence of coexisting detected and predicted CCO issues, priority mechanisms must consider both severity and timing. Without prediction time indication, the receiver has no basis to determine which issue should take precedence if activation windows overlap.

Observation 5: A deterministic "time-to-action" associated with a predicted issue would enable scheduling of beam re-configuration, UE offloading, or pre-emptive power tilt optimization — all of which require forward planning. The lack of this parameter prevents time-sensitive orchestration.

Proposal 1: RAN3 to agree that, in the context of AI/ML-based CCO prediction exchange over Xn, the Time for Predicted CCO Issue should be explicitly included for each predicted CCO entry.

Proposal 2: RAN3 to agree that when multiple predicted CCO issues exist for a given neighbor gNB, the timing information shall be used to distinguish among them and to guide prioritization in case of overlapping or conflicting predictions.

# 4	References

[1] R3-253988

[2] 3GPP TS 38.423