3GPP TSG-RAN WG3 Meeting #129	R3-255607

Bangaluru, IND, 25 – 29 August, 2025

Agenda Item:	14.2

Source:	CATT

Title:	(TP to BLCR for TS 36.300) Left issues in IoT NTN

Document for:	Decision

- Introduction

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

In this paper, we will discuss the left issues in IoT NTN.

2. Discussion

2.1 Suspend and Resume

In case of NG Connection interruption due to hard feeder link switch over, RAN3 meeting has agreed to introduce a new IE for NG downlink transmission suspension/resumption. The corresponding description is as follows in BLCR 38.300[1]:

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

Observation 1: NR already supports an indication to suspend and resume data and signalling procedures for hard feeder link switch.

For whether need to relax the restrictions to other case, it is still further discussed in NR NTN part. From our perspective, the indicator to suspend and resume also need introduce to IoT NTN and relax the current restrictions.

The following observations explained in NR are still alive in IoT NTN case:

Observation 2: When the on-board eNB coverage leaves or re-enters the MME service area, the S1 connection could be suspended or resumed through the new indicator [2].

Observation 3: Support of S1 Suspend/Resume could make the S1 connection establishment/recovery faster, no need to frequently remove and setup the S1 interfaces between on board eNB and the MMEs on ground [2].

Based on above, it is suggested to introduce suspend and resume in BL CR 36.300, and also remove the restriction about hard feeder link switchover scenarios.

Proposal 1: Add suspend and resume related description in BL CR 36.300, and also remove the restriction about hard feeder link switchover scenarios.

2.2 OAM configuration for support TAI list

In NR NTN, there was a heated discussion about OAM based solution of provision TAI list to AMF. This discussion is also applied to IoT NTN scenario.

Currently, based on multiple interaction LSs, RAN3 and SA5 had confirmed that the OAM based solution is feasible for NTN to provide supported TAI list of the on-board eNB to AMF. It is workable to support OAM based solution without any change to SA2 specification. And RAN3 specification shall contain relative RAN3 agreements, indicating that the supported TAIs of an on-board eNB may be provided to AMF by OAM/pre-configuration,

Also, the same designation and description shall be introduced in IoT NTN.

Proposal 2: RAN3 to discuss and agree the stage 2 TP in the Annex to capture the OAM based configuration for supported TAI list.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: NR already supports an indication to suspend and resume data and signalling procedures for hard feeder link switch.

Observation 2: When the on-board eNB coverage leaves or re-enters the MME service area, the S1 connection could be suspended or resumed through the new indicator [2].

Observation 3: Support of S1 Suspend/Resume could make the S1 connection establishment/recovery faster, no need to frequently remove and setup the S1 interfaces between on board eNB and the MMEs on ground [2].

Proposal 1: Add suspend and resume related description in BL CR 36.300, and also remove the restriction about hard feeder link switchover scenarios.

Proposal 2: RAN3 to discuss and agree the stage 2 TP in the Annex to capture the OAM based configuration for supported TAI list.

4. Reference

[1]	R3-253093 (BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN

[2] 	R3-255179 TP to BL CR for TS38.300) Support of NG Suspend Resume

Annex TP for TS 36.300

3.1	Definitions

For the purposes of the present document, the following terms and definitions apply.

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, e.g. BRID or DAA.

>>Unchanged part skipped<<

FeMBMS: further enhanced multimedia broadcast multicast service.

FeMBMS/Unicast-mixed cell: cell supporting MBMS transmission and unicast transmission as SCell.

Feeder link: wireless transport link between the NTN Gateway and the NTN payload.

>>Unchanged part skipped<<

Hybrid cell: a cell broadcasting a CSG indicator set to false and a specific CSG identity. This cell is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs.

Inter-satellite link: wireless transport link between two regenerative NTN payloads.

>>Unchanged part skipped<<

NTN payload: a network node, embarked on board a satellite or high altitude platform station, providing connectivity functions, between the service link and the feeder link. Considering E-UTRAN functions, the NTN payload may be a TNL node (transparent payload), or an eNB (regeneative payload).

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

23.21.6	Signalling

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases:

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the S1 and X2 handover messages, X2 Setup and X2 eNB Configuration Update procedures is expected to be Uu Cell ID.

The mapping between Mapped Cell ID(s) and geographical area(s) is configured in the RAN and Core Network.

NOTE 1:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas (e.g. overlapping and/or with different dimensions).

The eNB is responsible for constructing the Mapped Cell ID based on the UE location information, if available. The mapping may be pre-configured (e.g., depending on operator's policy) or up to implementation.

NOTE 2:	As described in TS 23.401 [17], the User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE's location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

For regenerative payload, the supported TAIs of an on-board eNB can be configured to the MME(s) by O&M, as in TS 28.541 [49].