3GPP TSG-RAN WG3 Meeting #129	R3-255629

Bengaluru, India, 25th ~29th Aug, 2025

Title:	(TP for LTM BLCR for TS38.300): Clarification on the single Xn UE association in inter-CU LTM

Agenda Item:	13.2

Source:	Huawei, NEC, LG Electronics

Document for:	Other

- Introduction

The contribution contains a TP for LTM BLCR for TS 38.300 to clarify the single Xn UE association in inter-CU LTM.

2	Text Proposal for LTM BLCR for TS38.300

<< Start of Changes >>

9.2.3.5.2	C-Plane Handling

Cell switch command is conveyed in a MAC CE, which contains the necessary information to perform the LTM cell switch.

The overall procedure for LTM is shown in Figure 9.2.3.5.2-1 below. Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without the need to release, reconfigure or add other LTM candidate configurations after each LTM cell switch completion. The general procedure over the air interface is applicable to SCG LTM. Further details of SCG LTM can be found in TS 37.340 [21].

Figure 9.2.3.5.2-1. Signalling procedure for LTM

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure LTM.

3.	The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI resource configuration. For inter-gNB LTM, the source gNB includes the same source NG-RAN node UE XnAP ID for all HANDOVER REQUEST messages to a candidate gNB.

4.	Admission Control may be performed by the candidate cells(s)/gNB(s).

5.	The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) with HANDOVER REQUEST ACKNOWLEDGE message to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI-RS resource configuration and early sync information upon request.  For inter-gNB LTM, each candidate gNB includes the same target NG-RAN node UE XnAP ID for all HANDOVER REQUEST ACKNOWLEDGE messages it responds.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration and the LTM configuration ID mapping list.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration.

<skip unchanged part>

<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>