3GPP TSG-RAN WG3 #129                                                    R3-255642

Bengaluru, IN, 25th – 29th Aug, 2025

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 38.423 and 38.300) MRO for CHO with Candidate SCG(s) and S-CPAC

Document for:	other

Introduction

In last RAN3 meeting, the following agreements are captured:

MRO for CHO with Candidate SCG(s)

Reuse the existing “HO too early” and “HO to wrong cell” in case of too early CHO execution and CHO execution to wrong cell.

Update the MN node of wrong candidate PSCell list selection for CHO with candidate SCG(s) as candidate or target MN.

Handover Report message is not needed for case 7a.

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

MRO for S-CPAC：

The setting of the information on previousPSCellId and timeSCGFailure needs to be updated to support the failures due to the following S-CPAC. Work on LS to RAN2.

The TP in 10.18.3 Conditional PSCell addition or change failure in R3-253185 to be checked.

Whether to enhance UHI in case of S-CPAC?

- SCG UHI should be updated to the new serving MN/target SN from source MN during S-CPAC procedure, i.e. Include SCG UHI in SN Reconfiguration Complete message from MN to the target SN

Inform the SN(s) about the outcome of mobility events for SN initiated S-CPAC using the SN RELEASE REQUEST or SN RELEASE CONFIRM.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

  - MRO for CHO with candidate SCGs

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

According to legacy MRO, it is always the last serving node performing MRO analysis because MRO analysis on measurement result should base on measurement configuration. The following text in TS38.331 also descripts the reason.

SCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig.

For example, the text on cell measurement quantity based on SSB block is captured below:

1> for each cell measurement quantity to be derived based on SS/PBCH block: 2> if nrofSS-BlocksToAverage is not configured in the associated measObject in RRC_CONNECTED or in the associated entry in measIdleCarrierListNR within VarMeasIdleConfig in RRC_IDLE/RRC_INACTIVE; or 2> if absThreshSS-BlocksConsolidation is not configured in the associated measObject in RRC_CONNECTED or in the associated entry in measIdleCarrierListNR within VarMeasIdleConfig in RRC_IDLE/RRC_INACTIVE; or 2> if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation: 3> derive each cell measurement quantity based on SS/PBCH block as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [9]; 2> else: 3> derive each cell measurement quantity based on SS/PBCH block as the linear power scale average of the highest beam measurement quantity values above absThreshSS-BlocksConsolidation where the total number of averaged beams shall not exceed nrofSS-BlocksToAverage, and where each beam measurement quantity is described in TS 38.215 [9];

For Cell with different measConfig (just take absThreshSS-BlocksConsolidation and nrofSS-BlocksToAverage as an example), different cell measurement quantity may be derived from SSB 1~4.

|  | Cell A |
| --- | --- |
| measurement quantity for SSB 1 | -60 dbm |
| measurement quantity for SSB 2 | -70 dbm |
| measurement quantity for SSB 3 | -80 dbm |
| measurement quantity for SSB 4 | -90 dbm |

With different absThreshSS-BlocksConsolidation and nrofSS-BlocksToAverage, Cell measurement quantity is listed below:

| absThreshSS-BlocksConsolidation | nrofSS-BlocksToAverage | Cell measurement quantity |
| --- | --- | --- |
| not configured | not configured | -60 dbm |
| -75 dbm | 3 | -62 dbm |
| -85 dbm | 3 | -64 dbm |
| -95 dbm | 5 | -65 dbm |

Since there are many parameters in measConfig which would impact Cell measurement quantity, if Cell A and Cell B with different measConfig, comparing cell measurement result of Cell A and Cell B, measurement configuration is necessary.

Observation 1: Cell measurement result is derived based on measurement configuration. To compare cell measurement result among the reported cells, measurement configuration is necessary. So, legacy MRO sticks to the rule of last serving node (with measConfig) performing MRO analysis.

For SCG failure, there are two scenarios:

1. When SCG failure occurs, source MN receive SCG failure information message from UE.

2. When SCG failure occurs shortly after successful CHO with candidate SCGs, the target MN would receive SCG failure information message from UE.

For scenario 1, source MN has measConfig, and during handover preparation procedure, source MN send measConfig to candidate MN via Inter-node RRC messages. So, both source MN and candidate MN could perform MRO analysis.

For scenario 2, after successful CHO with candidate SCGs, target MN may configure new measConfig and it would not send such UE related configuration back to source MN. So, source MN cannot perform MRO analysis.

Proposal 1: For SCG failure after successful CHO with candidate SCG, only current MN (last serving MN) can perform MRO analysis while source MN/SN and candidate MN/SN cannot perform MRO analysis due to lack of measConfig.

For scenario 1, it may be not needed to enhance SCG Failure Indication message because candidate MN can analyze measurement result within SCG failure information message and then perform MRO analysis by itself. While for scenario 2, when SCG failure occurs shortly after successful CHO with candidate SCGs, current MN (also target MN for previous handover) performs MRO analysis, but pervious source MN could not analyze the failure report. The new SCG Failure Indication message should be sent from current (target) MN to previous source MN. For SCG failure, source MN should inform other candidate MN to perform optimization.

For example, for scenario 2, SCG failure occurs shortly after successful CHO with candidate SCGs (PCell 1, PSCell A) -> (PCell 2, PSCell B), PCell 2 would receive SCG failure information message from UE. PCell 2 (target MN) send new SCG Failure Indication message to other candidate MN/SN if other candidate MN/SN e.g., MN3/SN3 does not include suitable PSCell C into the candidate PSCell list. It is consider that candidate MN3 does not have measurement configuration hence target MN2 needs to send CPC failure type and suitable PSCell ID to candidate MN3. In case target MN2 have no Xn interface with candidate MN3, it has to send the SCG failure indication to source MN, then source MN send the SCG failure indication to candidate MN3, and may further send to SN3.

Proposal 2: For SCG failure after successful CHO with candidate SCG, the SCG failure indication message including CPC failure type and suitable PSCell ID should be sent from target MN- >candidate MN or target MN->source MN->candidate MN.

For stage 2 text, the following text in TS38.300 is captured:

- 15.5.2.6 PSCell addition/change failure For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21]. In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different M-NG-RAN node, the serving M-NG-RAN node sends SCG Failure Indication message to that M-NG-RAN node.

For CPC optimization, candidate SN is also involved, not only the M-NG-RAN node. For example, candidate MN may inform candidate SN to make optimization on SCG failure via SCG FAILURE INFORMATION REPORT message.

Moreover, the forwarding of scenario 2 i.e., target MN->source MN->candidate MN should be captured.

Stage 2 text is updated below.

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different node, the serving M-NG-RAN node sends SCG Failure Indication message to the problematic M-NG-RAN node, via source M-NG-RAN node if needed, or problematic S-NG-RAN node, via its connected M-NG-RAN node if needed.

Proposal 3: It is proposed for RAN3 to discuss the above stage-2 description.

  - MRO for Subsequent CPAC

For stage 2 description, the following text is captured:

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis.

-	For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation.

-	For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the initiating SN at CPAC preparation, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.

There is a case that initiating SN acts as both initiating node and following source node, for example:

PSCell 1 is the initiating cell in initiating SN A. After some following CPC, PSCell5 in initiating SN A is the source PSCell in following CPC, and SCG failure occurs during following S-CPC from PSCell5 of SN A to PSCell6 of SN B

For above case, SN A is not only initiating SN but also source SN. SN B is only target SN.

According to stage 2 text, the following procedure is used:

1. MN sends the SCG Failure Information Report message to SN A. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN A at CPAC preparation, SN A send SCG FAILURE TRANSFER to MN.

2. MN sends the SCG Failure Information Report message to candidate SN including candidate SN B.

However, initiating SN A not only need to identify the candidate PSCell selection but also the following CPC execution condition. The current specification only touches the candidate PSCell selection optimization, not cover the following CPC execution condition optimization.

The solution may let SN A finish all its optimization including PSCell selection and execution conditions. Then MN informs SN B to make optimization.

Proposal 4: For a case that initiating SN act as both initiating node and following source node, initiating node should make sure that both PSCell selection and following CPC execution condition are optimal i.e., other node responsible for the SCG failure, then sending the SCG failure indication to MN.

The Stage 2 text may be updated as below to show cell selection and/or execution condition in the initiating SN is finished.

-	For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the initiating SN at CPAC preparation and/or execution condition is optimal, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.

For solution 2, it does not describe the work of initiating SN (general description).

-	For a following S-CPC failure during a SN initiated S-CPAC, if  the root cause of SCG failure may have occurred in other node, the initiating SN indicates to MN, MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.

Proposal 5: It is proposed for RAN3 to discuss the above stage 2 description for two solutions.

Conclusions

MRO for CHO with candidate SCGs:

Observation 1: Cell measurement result is derived based on measurement configuration. To compare cell measurement result among the reported cells, measurement configuration is necessary. So, legacy MRO sticks to the rule of last serving node (with measConfig) performing MRO analysis.

Proposal 1: For SCG failure after successful CHO with candidate SCG, only current MN (last serving MN) can perform MRO analysis while source MN/SN and candidate MN/SN cannot perform MRO analysis due to lack of measConfig.

Proposal 2: For SCG failure after successful CHO with candidate SCG, the SCG failure indication message including CPC failure type and suitable PSCell ID should be sent from target MN- >candidate MN or target MN->source MN->candidate MN.

Proposal 3: It is proposed for RAN3 to discuss the above stage 2 description.

MRO for Subsequent CPAC:

Proposal 4: For above case, initiating node should make sure that both PSCell selection and following CPC execution condition are optimal i.e., other node responsible for the SCG failure, then sending the SCG failure indication to MN.

Proposal 5: It is proposed for RAN3 to discuss the above stage 2 description for two solutions.

Reference

[1]

TP for TS38.423 on CHO with candidate SCG(s)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.4.x	SCG Failure Indication

8.4.x.1	General

The purpose of the SCG Failure Indication procedure is to transfer SCG failure related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SCG Failure Indication, successful operation

NG-RAN node1 initiates the procedure by sending the SCG FAILURE INDICATION message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a SCG failure related problem was detected.

If the Handover Report Type IE is set to “Too Early CPC” or “CPC Execution to Wrong Cell” then NG-RAN node1 indicate NG-RAN node2, that following a successful CHO with candidate SCG, a failure occurred in SCG. The detection of Too Early CPC event and CPC Execution to Wrong Cell are made according to TS 38.300 [9].

The SCG failure indication message may include:

-	Suitable PSCell.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

8.4.x.3	Unsuccessful Operation

Not applicable.

8.4.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

- 9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the candidate target NG-RAN node |  |  |
| SCG Failure Report Container | M |  | OCTET STRING | Includes the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |
| Handover Report Type | O |  | ENUMERATED (Too Early CPC, CPC Execution to Wrong Cell…) |  | YES | ignore |
| Suitable PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP for TS 38.300 on CHO with candidate SCG(s)

- 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure has been caused by a different node, the serving M-NG-RAN node sends SCG Failure Indication message to the problematic M-NG-RAN node, via source M-NG-RAN node if needed, or, problematic S-NG-RAN node, via its connected M-NG-RAN node if needed.

TP for TS 37.340 on Subsequent CPAC

- 10.18.3	Conditional PSCell addition or change failure

Solution 1：

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis.

-	For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation.

-	For a following S-CPC failure during a SN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the initiating SN at CPAC preparation and/or execution condition is optimal, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.

Solution 2：

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the initiating SN, and the initiating SN performs root cause analysis.

-	For initial S-CPAC failure during a SN initiated S-CPAC, the forwarding mechanism is the same as that for SN initiated CPC. The source SN is also the initiating SN at S-CPAC preparation.

-	For a following S-CPC failure during a SN initiated S-CPAC, if  the root cause of SCG failure may have occurred in other node, the initiating SN indicates to MN, MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation.