3GPP TSG-RAN WG3 #129                                       R3-255659

Bangalore, IN, 25 - 29 Aug, 2025

Agenda item:	13.2

Source:	Samsung

Title:	(TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM

Document for:	Discussion & Decision

# 1 Introduction

RAN3 had some discussions on inter-CU LTM in previous meetings, and below agreements are achieved in last meeting:

RAN3#127-bis: Inter-CU LTM Semi-Persistent CSI-RS coordination - Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). - Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation. - Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress. Security Mechanism in inter-CU LTM - For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. - The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB. - The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Whether to define additional information from CU to DU to deliver NCC value. TA information transfer - Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: - Candidate Cell ID - TA value - Preamble index - RA-RNTI - FFS on the XnAP IDs. LTM Preparation for Parallel messages - Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB. UE associated context setup for subsequent LTM - For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB. Data Forwarding Info IE design - Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s). Inter-CU LTM with DC - Include the following information in SN Change Required (Source SN to MN) message: - LTM request indication (explicit way) - Suggested PScell list (implicit way in INM, FFS on explicit way) - Measurement results (implicit way in INM) - Upper limit number of PScell(s) for each candidate SN (FFS) - CSI Resource configuration request (explicit way) - Early sync information request (explicit way) - Include the following information in SN Addition Request (MN to Candidate SN) message: - LTM request indication (explicit way) - Suggested PScell list (implicit way in INM, FFS on explicit way) - Measurement results (implicit way in INM) - Upper limit number of PScell(s) for the candidate SN (explicit way) - CSI Resource configuration request (explicit way) - Early sync information request (explicit way) - It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message. - For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN. - The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN. - Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message. RAN3#128: Inter-CU LTM Semi-Persistent CSI-RS coordination - WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs. - Add description in Stage 2 TS 38.401 for describing that CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE message. - Update the online agreement to: CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message. - Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell. - FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP. - FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure. - FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message. - FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP. LTM CFRA Resource - In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation. UE Based TA Measurement - Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message. Security Mechanism in inter-CU LTM - For Inter-CU LTM, LTM CONFIGURATION UPDATE procedure is per node level basis with a list of cells, and security key is per cell. - Include the Rel-19 set IDs of source cell and each candidate cell(s) in UE Context Modification Request message. Introduce Rel-19 set IDs into LTM Security Information IE. - FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2: - Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message. - Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different. - FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB. - Remove Note in TS 38.300 BL CR “Editor’s Note: step 6 and 7 are optional.” Inter-CU LTM with DC - Update the text description of CELL SWITCH NOTIFICATION message to capture the RAN3#127bis agreement: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message, and reusing the existing IEs. - Update the TA INFORMATION TRANSFER message to provide a list of TA values of multiple candidate cells. - The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Change Required message. FFS on the SN Addition Request Acknowledge message from the source SN/candidate SN to the MN. - The SCG reference configuration is provided by an implicit way in the CG-ConfigInfo RRC container in the SN Addition Request message from the MN to the candidate SN. - A per cell indicator indicating complete candidate configuration is included in the SN Addition Request Acknowledge message. - The SN Change Required message and the SN Modification Request message from MN to source SN should design a mechanism to support multiple candidate SNs, i.e., SN can include multiple candidate SNs information in a single SN Change Required message, and MN includes multiple candidate SNs information in a single SN Modification Request message. - The SN Addition Request message should design a mechanism to support single candidate SN, i.e., MN sends SN addition request towards only one candidate SN. - The suggested PSCell list should be explicitly included in the SN Change Required message. - The maximum number of PSCells that can be prepared by each candidate SN is included in the SN Change Required message. - The LTM Configuration ID Mapping List IE may be included in the SN Change Required message and the SN Addition Request message. - The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM. - The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM. - WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM. - If the source SN has the SCG reference configuration, the source SN will provide the SCG reference configuration in the SN Change Required message and thus the MN will not request source SN to provide the SCG reference configuration. - Early data forwarding is supported in inter-CU LTM in DC. - Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation. - Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case. - MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN. - The source SN generates the common LTM CSI resource configuration for inter-CU SCG LTM and then provides the common LTM CSI resource configuration to the MN via the SN Modification Request ACK message. - For the SSB based L1 measurement, the common CSI resource configuration may be included in the SN Change Required message and the SN Addition Request message. - The CSI Resource Configuration IE is the common CSI resource configuration, which refers to the ltm-CSI-ResourceConfig-r18 IE in the RRC spec. - The L1 Configuration IE is the L1 RS configuration per candidate PSCell, which refers to the ltm-SSB-Config-r18 IE in the RRC spec for the SSB based L1 measurement, or the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 IE associated with the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 IE in the RRC spec for the CSI-RS based L1 measurement. - For the avoidance of simultaneous configurations of inter-CU MCG LTM and inter-CU SCG LTM, no RAN3 impact is foreseen. - FFS on whether LTM modification/cancel related procedures are needed. - The SP CSI-RS activation/(de)activation for the inter-CU LTM in DC is deprioritized in rel-19.

In this paper, we will further discuss essential issue for the basic procedure of inter-gNB-CU LTM based on agreed BLCR in [1], [2] and [3], and discuss the necessary enhancement on Xn/F1 interface.

# 2 Discussion

To achieve the analysis for network interface impact for the scenario of inter-gNB-CU LTM, we draw the potential message flows of the whole inter-gNB-CU LTM process as shown in the below Fig.1 to provide the basic description for each involved message flows and our view for the issues raised in stage 2/3 BLCR. After that, we finally summarize our proposals.

Fig.1 Message Flows for Inter-gNB-CU LTM

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI resource configuration.

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the LTM lower layer RRC configurations.

6.  The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) with HANDOVER REQUEST ACKNOWLEDGE message to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI-RS resource configuration and early sync information upon request.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration and the LTM configuration ID mapping list.

10.	The candidate gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information of candidate cells for subsequent LTM. The candidate gNB-CU may include the common CSI-RS resource configuration.

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message. The candidate gNB-DU may also provide the CSI-RS report configuration.

12.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration.

NOTE :	Step 9 may also be triggered after step 25, or after step 32 by implementation for subsequent LTM.

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB-DU.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

NOTE :	The source gNB may initiate CSI-RS Coordination procedure to activate or deactivate CSI-RS resource(s) of some candidate cells(s).

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

18.	Early DL and UL synchronization to some LTM candidate cell(s) may be performed. The source gNB may activate or deactivate the TCI states of the candidate LTM cells.  Depending on NW configuration, the UE may perform early UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the source gNB. With a NW triggered UL synchronization, a PDCCH order is received from the source cell to trigger CFRA to a candidate cell, the UE performs early TA acquisition by sending preamble towards the indicated candidate cell. In order to minimize the data interruption on the source cell due to CFRA towards the indicated candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB instead.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via the TA INFORMATION TRANSFER message.

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB, if configured. L1 measurement should be performed as long as RRC reconfiguration (step 13 and 14) is applicable.

23.	The source gNB-DU determines to initiate inter-gNB-CU LTM.

24.	The source gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a TA command for the target cell. In case of a security context change, the LTM cell switch command MAC CE also contains the NCC value. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID. In case of security context change, the UE generates and applies the new security keys based on the received NCC value.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 24.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID and the TCI state ID(s).

26.	The source gNB-CU sends the CELL SWITCH NOTIFICATION message to the target gNB-CU to indicate the initiation of Cell Switch command to the UE.

27.	The target gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30/31. The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell. In return, the source gNB-CU sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE: Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

32.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

33.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

NOTE :	Steps 30/31 and 32 do not have to occur one after the other. The target gNB may send the HANDOVER SUCCESS message to the source gNB after receiving the RRCReconfigurationComplete message.

34.	The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. This message may include the new security key(s) to be used with the UE.

35. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

36. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

37.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

38.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

The steps 18-38 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 13 and 14.

//////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

From the above overview of inter-gNB-CU LTM procedure, based on current discussion progress, the impacts on Xn/F1 interface are mainly reflected in the following aspects:

## Issue 1: Early TA RACH resource allocation

In RAN3#127 meeting, the following agreement was reached on this issue:

- The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration. - The format of the new introduced IE is same with the gNB-DU ID.

From the the BLCR in [2], we note that early RACH configuration of each candidate cell is allocated based on gNB-DU and corresponding gNB-DU ID is replaced with an implicit ID (Early RACH Resources Requester ID) in XnAP specification. And the dedicated different early RACH configuration is allocated by candidate cell for each candidate gNB-DU. RACH configuration could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different gNB-DU.

However, another case exists. If one LTM gNB is not CU-DU split architecture, how does source gNB request early RACH configuration to this LTM gNB for each candidate cell? We think RACH configuration allocation shoult not be based on gNB-DU, should be based on gNB in case of aggregated gNB.

Therefore, we should announce below mechanism on early RACH configuration allocation.

- Case#1: If one LTM gNB is CU-DU split architecture, early RACH configuration of candidate cell is allocated based on gNB-DU.

- Case#2: If one LTM gNB is not CU-DU split architecture, i.e., aggregated gNB, early RACH configuration of candidate cell is allocated based on gNB.

Proposal 1-1: RAN3 support the early TA RACH resource allocation is based on gNB-DU in case of split gNB and/or based on gNB in case of aggregated gNB.

Additionally, in case#1, we should note before the step that the source gNB-CU sends the pair of (gNB ID, Early RACH Resources Requester ID) to the candidate gNB-CU to request early RACH configuration is initiated from the source gNB-CU, the source gNB-CU should be aware of the gNB-DU ID of each candidate cell belonging.

Then, how does source gNB-CU acquire the gNB-DU ID information to which inter-CU candidate cell belongs? Reviewing current specification as below highlight from TS 38473, it cannot be achieved via Cell ID information due to there is no connection between gNB-DU ID and cell identifiers.

9.3.1.9 gNB-DU ID [F1 IE] The gNB-DU ID uniquely identifies the gNB-DU at least within a gNB-CU. IE/Group Name; Presence; Range; IE type and reference; Semantics description. gNB-DU ID; M; INTEGER (0 .. 236-1); The gNB-DU ID is independently configured from cell identifiers, i.e. no connection between gNB-DU ID and cell identifiers..

Therefore, RAN3 should discuss this issue on how does source gNB-CU acquire the gNB-DU ID information to which inter-CU candidate cell belongs.

Proposal 1-2: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID information to which inter-CU candidate cell belongs before initiation of LTM preparation.

Based on the mechanism of early TA RACH resource allocation proposed by Proposal 1-1, in the Case#2: If one LTM gNB is not CU-DU split architecture, i.e., aggregated gNB, early RACH configuration of candidate cell is allocated based on gNB. Let’s assume the case that source gNB is aggregated gNB, and one candidate gNB with one candidate cell is CU-DU split gNB. When source gNB initiates the HO REQ to candidate gNB with Early Sync Information Request IE for requesting early TA RACH resource from candidate cell, the IE of Early RACH Resources Requester ID seem to be reasonable as an optional IE since source gNB is an aggregated gNB.

Below IEs are from BLCR in [2] #### 9.2.1.xx3 Early Sync Information Request This IE contains a request for Early synchronization information. Editor’s note: Details on IEs need to be continued. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early Sync Configuration Request; M; ENUMERATED (true, …). Early RACH Resources List; 0..1. >Early RACH Resources Item; 1..< maxnoofEarlyRACHResourcesID>. >>Global gNB ID; M; 9.2.2.1. >>Early RACH Resources Requester ID; M; 9.2.1.xx9; Identifies the entity requesting Early RACH resources.. Range bound; Explanation. maxnoofEarlyRACHResourcesID; Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8..

Furthermore, when candidate gNB-CU initiates UE Context Setup Request to candidate gNB-DU with Early Sync Information Request IE for requesting early TA RACH resource from candidate cell, it will introduce another issue if reusing current IE tabular on LTM gNB-DUs List IE. LTM gNB-DU ID IE is a mandatory IE for intra-CU LTM but it is possible to be an optional IE for inter-CU LTM in case source gNB is an aggregated gNB.

Regarding the DU-CU TA INFORMATION TRANSFER message, the same issue is existing in current IE tabular, and the mandatory IE on the Source gNB-DU ID IE is possible to be an optional information for inter-CU LTM in case source gNB is an aggregated gNB.

Below IEs are from BLCR in [3] Early Sync Information Request; 0..1; YES; ignore. >Request for RACH Configuration; O; ENUMERATED (true, …); -. >LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).; YES; reject. >>LTM gNB-DUs Item IEs; 1..< maxnoofLTMg NBDUs>. >>>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>>LTM gNB ID; O; Global gNB ID 9.3.1.305; -. #### 9.2.1.24 DU-CU TA INFORMATION TRANSFER This message is sent by the gNB-DU to inform the gNB-CU about TA information. Direction: gNB-DU gNB-CU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Transaction ID; M; 9.3.1.23; YES; reject. DU to CU TA Information List; 0..1; YES; ignore. >DU to CU TA Information Item IEs; 1 .. <maxnoof TAList>; EACH; ignore. >>Candidate Cell ID; M; NR CGI 9.3.1.12; -. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [31].; -. >>Preamble Index; M; INTEGER (0..63); -. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [16].; -. >>Source gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8].; -. >>Source gNB ID; O; Global gNB ID 9.3.1.305; -.

Therefore, RAN3 should re-think the F1/Xn IEs design on early TA RACH resource request & allocation & TA information transfer. One simple way is that define the related gNB-DU ID IE to an optional IE in the Rel-19 specification.

Proposal 1-3: RAN3 should define F1 gNB-DU ID IE to be an optional IE in the Rel-19 specification on the procedure of early TA RACH resource request & allocation & TA information transfer.

## Issue 2: FFS the detail in TA INFORMATION TRANSFER message

This issue is related to the early TA information transfer from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU in step 20.

We already agreed to introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

- Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: - Candidate Cell ID - TA value - Preamble index - RA-RNTI - FFS on the XnAP IDs.

Furthermore, the detail IEs are agreed to be included in F1AP DU-CU/CU-DU TA INFORMATION TRANSFER, including a candidate cell list that include related TA information and RACH resource information for each candidate cell. Additionally, if source gNB is a split gNB and early TA RACH resource is allocated based on gNB-DU as mentioned in the issue#1, the implicit gNB-DU ID(Early RACH Resources Requester ID) should be indicated in the XnAP TA INFORMATION TRANSFER message to inform source gNB-CU to forward TA information to the indicated source gNB-DU.

Proposal 2-1: Propose to include an optional Early RACH Resources Requester ID information in XnAP TA INFORMATION TRANSFER message to inform source gNB-CU to forward TA information to the indicated source gNB-DU in case of split architecture for source gNB.

Regarding FFS on the XnAP IDs to be included in XnAP TA Information Transfer message, we have agreed that this procedure is a non-UE associated procedure, and related RACH resource information (Preamble index and RA-RNTI) for the indicated TA value of candidate cell is also carried to source gNB/DU to identify the UE to which this TA value is belonging. Therefore, it is following the R18 legacy mechanism. So we do not need to include the related XnAP IDs information to be used to identify UE.

Proposal 2-2: No need to include the XnAP IDs information in the XnAP TA Information Transfer message, since the RACH resource information for the indicated TA value of candidate cell is used for source gNB/DU to identify the UE to which this TA value is belonging.

## Issue 3: FFS the detail on Data forwarding Information IE in LTM Configuration Update message

For supporting data forwarding in the subsequent inter-CU LTM scenarios, RAN3 have reached the following agreement in last meeting:

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

From the agreement, we can see that Data Forwarding Informaton IE is a list including multiple data forwarding addresses from each candidate gNB(s) which is carried to other candidate gNBs via the LTM Configuration Update message for supporting early data forwarding in subsequent LTM. And after last RAN3 meeting, the merged XnAP BLCR in [2] is archived, we can see the Data Forwarding Information IE is included in the LTM Configuration Update message but the detail is FFS. Therefore, we will continue to discuss this issue on FFS the details on data forwarding information IE.

#### 9.1.1.x3 LTM CONFIGURATION UPDATE This message is sent by the NG-RAN node1 to update LTM configuration data. Direction: NG-RAN node1  NG-RAN node2. Editor’s note: Details on IEs need to be continued. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. NG-RAN node 1 UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the NG-RAN node 1; YES; reject. NG-RAN node 2 UE XnAP ID; O; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the NG-RAN node 2; YES; reject. LTM Information Request; O; 9.2.3.xx1; YES; ignore. LTM Updates to Candidate Cell Information List; 0..1; YES; ignore. ============omit the unrelated part=================. Data Forwarding Information; FFS.

Regarding FFS on the details on Data Forwarding Information IE, when reviewing the current the specification in XnAP TS 38.423, the basic principle is that one data forwarding address is allocated based on each SN in case of MR-DC, but different data forwarding address can be allocated based on each candidate PCell in case of SA. Therefore, we think that data forwarding address per-candidate cell is reasonable for inter-CU LTM.

Observation 3-1: Regarding inter-CU LTM, Data forwarding address is based on candidate cell not candidate gNB.

To indicate the data forwarding address for each candidate cell, Data Forwarding Info from target NG-RAN node IE is generally used in the some previous examples. Since this IE is per-PDU session, we can design the Data Forwarding Information IE as below:

#### 9.2.1.x14 Data Forwarding Information This IE contains different data forwarding addresses information per-PDU session for different candidate LTM cell and they are used by new source gNB for early data forwarding towards these addresses in subsequent LTM. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Data Forwarding Information List; 1; YES; ignore. >Data Forwarding Information Item; 1.. <maxnoof LTMCells>; –. >>LTM Cell ID; M; NR CGI 9.2.2.7; –. >>PDU Session Resources List; 1; –. >>>PDU Session Resources Item; 1. .<maxnoofPDU Sessions>; –. >>>>PDU Session ID; M; 9.2.3.18; –. >>>>Data Forwarding Info from target NG-RAN node; M; 9.2.1.16; –. Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8.. Range bound; Explanation. maxnoofPDUSsessions; Maximum no. of PDU sessions. Value is 256.

Proposal 3-2: RAN3 to agree to use Data Forwarding Info from target NG-RAN node IE per-PDU session per-candidate cell as the included details in the Data Forwarding Information IE.

## Issue 4: FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB for identifying the UE

In RAN3#126 meeting, below agreement is reached to support establishing UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

In plus, we need to consider how to identify the same UE when candidate gNB receives the LTM Configuration Update message from new source gNB. As below figure shows, the C-gNB2 (new source gNB2) initiates to establish UE association with C-gNB3, by sending LTM Configuration Update carrying one UE ID reference info for the candidate gNB to identify the same UE.

Through the discussion in last RAN3#127bis meeting, the below agreements are reached and old target UE XnAP ID can be included as the related UE ID reference info for the candidate gNB to identify the same UE.

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

And next, the new issue is raised FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

Regarding this issue, it is possible to have two options:

Option#1: Reuse Cell Switch Notification message when LTM is triggered.

Option#2: Reuse the LTM Configuration Update message when the procedure of sync up configurations is performed for subsequent LTM.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

From our view, even two options both can work, considering the LTM Configuration Update message is used to transfer the related information for supporting subsequent LTM, so LTM Configuration Update message seems to be better option.

Proposal 4: Reuse the LTM Configuration Update message to transfer old target UE XnAP ID(s) to new serving gNB when the procedure of sync up configurations is performed for subsequent LTM.

## Issue 5: FFS on the Rel-19 Set ID(s) assignment among CUs in Security Mechanism

Regarding the Security Mechanism in inter-CU LTM, below agreement is reached in last RAN3#128 meeting

- For Inter-CU LTM, LTM CONFIGURATION UPDATE procedure is per node level basis with a list of cells, and security key is per cell. - Include the Rel-19 set IDs of source cell and each candidate cell(s) in UE Context Modification Request message. Introduce Rel-19 set IDs into LTM Security Information IE. - FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2: - Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message. - Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different. - FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

Regarding the FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2. Through the analysis, regarding the option 1, it may involve total three times the information interaction between source gNB and candidate gNB. First step, source gNB provides the initially assigned ID range to each candidate gNB; In second step, candidate gNB final select one ID for its own cell, and feedback to source gNB; In third step, source gNB should sends all the ID information of each candidate cell to each candidate gNB for supporting subsequent LTM. In addition, it cannot know how many candidate cells are to be accepted before handover request procedure, so source gNB cannot decide how many IDs are provided to each candidate gNB. This option brings some complexity and introduces more signaling overhead.

However, regarding the option 2, it only needs two steps information interaction between source gNB and candidate gNB. First step, each candidate gNB provides the initially assigned ID for its own candidate cell to source gNB; In second step, source gNB finally uniforms ID configuration for all the candidate cell, and then provides these to each candidate gNB for supporting subsequent LTM.

Therefore, option 2 is an easy configuration way and involves less signaling overhead.

Proposal 5-1: RAN3 to adopt the Option 2 on Rel-19 Set ID(s) assignment among CUs, which involves less signaling overhead.

In plus, regarding the FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB, considering the LTM Configuration Update message is used to transfer the related information for supporting subsequent LTM, so we suggest to use LTM Configuration Update message to transfer Rel-19 set ID per candidate cell to the candidate gNB.

Proposal 5-2: Reuse LTM Configuration Update message to transfer Rel-19 set ID per candidate cell to the candidate gNB.

Regarding the Inter-CU LTM with DC, below agreement is reached in last RAN3#128 meeting:

- The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM. - The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM. - WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

We can see the similar issue need to be discussed on Rel-19 set ID assignment per candidate PSCell among SNs. We support to turn the WA into agreement, different candidate PSCells in the same SN can have different Rel-19 set IDs, because there seem be no obstacle to must have a restriction. We also support to reuse the similar option 2 for inter-CU LTM as a solution for inter-SN SCG LTM: Candidate SN provides Rel-19 set ID per candidate PSCell in SN ADDITION REQUEST ACKNOWLEDGE message to MN, then MN / source SN finally uniforms ID configuration for all the candidate PSCell to ensure that the Rel-19 set IDs under different candidate SN are different, and then to use SN MODIFICATION REQUEST message to transfer Rel-19 set ID per candidate PSCell to the candidate SN.

Proposal 5-3: Support to turn the WA into agreement, Different candidate PSCells in the same SN can have different Rel-19 set IDs.

## Proposal 5-4:Regarding the issue on Rel-19 set ID assignment per candidate PSCell among SNs, reuse the same solution as for inter-CU LTM: Candidate SN provides Rel-19 set ID per candidate PSCell in SN ADDITION REQUEST ACKNOWLEDGE message to MN, then MN / source SN finally uniforms ID configuration for all the candidate PSCell to ensure that the Rel-19 set IDs under different candidate SN are different, and then to use SN MODIFICATION REQUEST message to transfer Rel-19 set ID per candidate PSCell to the candidate SN for the subsequent SCG LTM usage.

## Issue 6: FFS on other ID configuration of LTM candidate Cells (e.g., UE Based TA Measurement ID)

As we know, the co-existence between inter-CU LTM and intra-CU LTM is possible, because source gNB can configure intra-CU (including inter/intra-DU) LTM candidate cell, furthermore, source gNB can configure more than one LTM candidate cells within one candidate gNB via respective handover request procedure per-cell. Take an example, cell1 and cell2 in the same candidate gNB1, regarding the configuration of UE Based TA Measurement ID, it is reasonable for candidate gNB1 to decide the whether corresponding UE Based TA Measurement ID for cell1 and cell2 is different or same. Source gNB CAN only decide whether UE Based TA Measurement ID is same or different between source cell0 and other candidate cells (including cell1 and cell2), but it cannot decide whether UE Based TA Measurement ID is same or different between cell1 and cell2 with one candidate gNB1. Otherwise, it violates the principles of R18 principle of intra-CU LTM, the gNB-CU configures its own cells on the UE Based TA Measurement ID which is same or different between any two cells.

Therefore, regarding below agreement from last meeting, we suggest to revisit and continue to revise.

UE Based TA Measurement − Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message.

Take more consideration, UE Based TA Measurement ID of source cell and each candidate cell(s) can also reuse the solution on Rel-19 set IDs assignment among CUs, so we suggest to RAN3 to agree below proposal:

Proposal 6: UE Based TA Measurement ID assignment of source cell and each candidate cell(s), REUSE the solution on Rel-19 set IDs assignment among CUs.

# 3 Conclusion

This contribution firstly analyses the whole message flows of inter-CU LTM and propose some enhancements for Xn interface on below aspects. Therefore, we invite RAN3 to agree below proposals for supporting the basic inter-CU LTM procedure.

## Issue 1: Early TA RACH resource allocation

Proposal 1-1: RAN3 support the early TA RACH resource allocation is based on gNB-DU in case of split gNB and/or based on gNB in case of aggregated gNB.

Proposal 1-2: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID information to which inter-CU candidate cell belongs before initiation of LTM preparation.

Proposal 1-3: RAN3 should define F1 gNB-DU ID IE to be an optional IE in the Rel-19 specification on the procedure of early TA RACH resource request & allocation & TA information transfer.

## Issue 2: FFS the detail in TA INFORMATION TRANSFER message

Proposal 2-1: Propose to include an optional Early RACH Resources Requester ID information in XnAP TA INFORMATION TRANSFER message to inform source gNB-CU to forward TA information to the indicated source gNB-DU in case of split architecture for source gNB.

Proposal 2-2: No need to include the XnAP IDs information in the XnAP TA Information Transfer message, since the RACH resource information for the indicated TA value of candidate cell is used for source gNB/DU to identify the UE to which this TA value is belonging.

## Issue 3: FFS the detail on Data forwarding Information IE in LTM Configuration Update message

Observation 3-1: Regarding inter-CU LTM, Data forwarding address is based on candidate cell not candidate gNB.

Proposal 3-2: RAN3 to agree to use Data Forwarding Info from target NG-RAN node IE per-PDU session per-candidate cell as the included details in the Data Forwarding Information IE.

## Issue 4: FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB for identifying the UE

Proposal 4: Reuse the LTM Configuration Update message to transfer old target UE XnAP ID(s) to new serving gNB when the procedure of sync up configurations is performed for subsequent LTM.

## Issue 5: FFS on the Rel-19 Set ID(s) assignment among CUs in Security Mechanism

Proposal 5-1: RAN3 to adopt the Option 2 on Rel-19 Set ID(s) assignment among CUs, which involves less signaling overhead.

Proposal 5-2: Reuse LTM Configuration Update message to transfer Rel-19 set ID per candidate cell to the candidate gNB.

Proposal 5-3: Support to turn the WA into agreement, Different candidate PSCells in the same SN can have different Rel-19 set IDs.

Proposal 5-4: Regarding the issue on Rel-19 set ID assignment per candidate PSCell among SNs, reuse the same solution as for inter-CU LTM: Candidate SN provides Rel-19 set ID per candidate PSCell in SN ADDITION REQUEST ACKNOWLEDGE message to MN, then MN / source SN finally uniforms ID configuration for all the candidate PSCell to ensure that the Rel-19 set IDs under different candidate SN are different, and then to use SN MODIFICATION REQUEST message to transfer Rel-19 set ID per candidate PSCell to the candidate SN for the subsequent SCG LTM usage.

## Issue 6: FFS on other ID configuration of LTM candidate Cells (e.g., UE Based TA Measurement ID)

Proposal 6: UE Based TA Measurement ID assignment of source cell and each candidate cell(s), REUSE the solution on Rel-19 set IDs assignment among CUs.

# 4 TP to TS 38.423

---------------------------Start of the change--------------------------------------------

#### 9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM Handover Information Response | O |  |  |  | YES | ignore |
| >SSB Information | M |  | 9.2.3.x10 |  | – |  |
| >LTM Information Response | O |  | 9.2.3.xx2 |  | – |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. | – |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [10]. This IE applies for SUL carrier. | – |  |
| >No Security Change Configuration | O |  | OCTET STRING | Includes the ltm-NoSecurityChangeID contained in the LTM-Candidate IE, as defined in TS 38.331 [10]. | - |  |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10]. | - |  |
| Early Sync Information Response | O |  | 9.2.3.xx4 |  | YES | ignore |

#### 9.1.x.x2	TA INFORMATION TRANSFER

This message is sent by the NG-RAN node1 to inform the NG-RAN node2 about the Timing Advance value and related information.

Editor’s note: Details on IEs need to be continued.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TA Information List |  | 1 |  |  | YES | reject |
| >TA Information Item IEs |  | <1… maxnoofTAList> |  |  |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [40]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [35]. | - |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.1.xx9 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

--------------------------- Next of the change --------------------------------------------

#### 9.1.x.x3	LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| LTM Information Request | O |  | 9.2.3.xx1 |  | YES | ignore |
| LTM Updates to Candidate Cell Information List | 0..1 |  |  |  | YES | ignore |
| >LTM Updates to Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.2.2.7 |  | - |  |
| >>Request for CSI-RS Resource Configuration (FFS) | O |  | ENUMERATED (true, …) | Indicates the request to the candidate cell to provide CSI-RS resource configuration. | - |  |
| >>Early Sync Information Request | O |  | 9.2.3.xx3 | Indicates the request to the candidate cell for early synchronization information. | - |  |
| >>Early Sync Information | O |  | 9.2.3.xx6 | Indicates the consolidated early synchronization information to be used for subsequent LTM. | - |  |
| >>Early Sync Information for SUL | O |  | 9.2.3.xx6 | Indicates the consolidated early synchronization information for SUL to be used for subsequent LTM. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell identified by the Candidate Cell ID IE. | - |  |
| >>AS Security Information | O |  | 9.2.3.50 |  | - |  |
| >>No Security Change Configuration | O |  | OCTET STRING | Includes the ltm-NoSecurityChangeID contained in the LTM-Candidate IE, as defined in TS 38.331 [10], for the LTM candidate cell identified by the Candidate Cell ID IE. | - |  |
| UE Security Information for LTM | O |  | 9.2.3.x12 |  | YES | ignore |
| Data Forwarding Information | O |  | 9.2.3.x14 |  | YES | ignore |
| UE Context Reference at the Candidate NG-RAN node List |  | 0..1 |  |  | YES | ignore |
| >UE Context Reference at the Candidate NG-RAN node Item |  | 1 .. <maxnoofLTMNodes> |  |  | – |  |
| >>Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >>NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMNodes | Maximum no. of Nodes configured for LTM allowed towards one UE, the maximum value is 8. |

--------------------------- Next of the change --------------------------------------------

#### 9.2.3.xx3	Early Sync Information Request

This IE contains a request for the early synchronization information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Early Sync Configuration Request | M |  | ENUMERATED (true, …) |  |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.3.xx9 | Identifies the entity requesting Early RACH resources. |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

#### 9.2.3.xx4	Early Sync Information Response

This IE contains the early synchronization related information to be provided in response to information provided in the Early Sync Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. |
| Early UL Sync Configuration | O |  | 9.2.3.xx6 | Early UL sync configurations for the UE. |
| Early UL Sync Configuration for SUL | O |  | 9.2.3.xx6 | Early UL sync configurations for the UE for SUL carrier. |

#### 9.2.3.xx6	Early UL Sync Configuration

This IE indicates the Early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [10]. |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item IEs |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >>Early RACH Resources Requester ID | O |  | 9.2.3.xx9 | Identifies the entity requesting Early RACH resources. |
| >>Preamble Index List | O |  | 9.2.3.xx7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

--------------------------- Next of the change --------------------------------------------

#### 9.2.1.x14	Data Forwarding Information

This IE contains different data forwarding addresses information per-PDU session for different candidate LTM cell and they are used by new source gNB for early data forwarding towards these addresses in subsequent LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Data Forwarding Information List |  | 1 |  |  | YES | ignore |
| >Data Forwarding Information Item |  | 1.. <maxnoof LTMCells> |  |  | – |  |
| >>LTM Cell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>PDU Session Resources List |  | 1 |  |  | – |  |
| >>>PDU Session Resources Item |  | 1. .<maxnoofPDU Sessions> |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>>Data Forwarding Info from target NG-RAN node | M |  | 9.2.1.16 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSsessions | Maximum no. of PDU sessions. Value is 256 |

--------------------------- End of the change --------------------------------------------

# 5 TP to TS 38.473

---------------------------Start of the change--------------------------------------------

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >Request for CSI-RS Resource Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >Request for L1 Execution Condition | O |  | 9.3.1.x |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >LTM Nodes List |  | 0..1 |  | This IE contains the IDs of the source Node and candidate Node(s). | YES | reject |
| >>LTM Nodes Item IE |  | 1..< maxnoofLTMNodes> |  |  | - |  |
| >>>LTM gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | M |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMNodes | Maximum no. of Nodes allowed to be configured with LTM towards one UE, the maximum value is 8. |

--------------------------- Next of the change --------------------------------------------

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >Request for CSI-RS Resource Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >Request for L1 Execution Condition | O |  | 9.3.1.x |  |  |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >LTM Nodes List |  | 0..1 |  | This IE contains the IDs of the source Node and candidate Node(s). | YES | reject |
| >>LTM Nodes Item IE |  | 1..< maxnoofLTMNodes> |  |  | - |  |
| >>>LTM gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | M |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |
| LTM Security Information | O |  | 9.3.1.XX |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMNodes | Maximum no. of Nodes allowed to be configured with LTM towards one UE, the maximum value is 8. |

--------------------------- Next of the change --------------------------------------------

#### 9.2.1.24	DU-CU TA INFORMATION TRANSFER

This message is sent by the gNB-DU to inform the gNB-CU about TA information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DU to CU TA Information List |  | 0..1 |  |  | YES | ignore |
| >DU to CU TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Source gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | - |  |
| >>Source gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |

--------------------------- Next of the change --------------------------------------------

#### 9.3.1.328	Early UL Sync Configuration

This IE indicates the early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8]. |
| LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |
| >LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |
| >>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |
| >>Preamble Index List | O |  | 9.3.1.329 |  |
| LTM Nodes List |  | 0..1 |  | This IE contains the IDs of the source node and candidate node(s). |
| >LTM Nodes Item |  | 1..< maxnoofLTMNodes> |  |  |
| >>LTM gNB-DU ID | O |  | gNB-DU ID 9.3.1.9 |  |
| >>LTM gNB ID | M |  | Global gNB ID 9.3.1.305 |  |
| >>Preamble Index List | O |  | 9.3.1.329 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMNodes | Maximum no. of Nodes allowed to be configured with LTM towards one UE, the maximum value is 8. |

--------------------------- End of the change --------------------------------------------

# 6 Reference

[1] R3-254007 (BL CR to 38.300) Support for Inter-CU LTM

[2] R3-254009	(BL CR to 38.423) Xn support for inter-CU LTM

[3] R3-254011	(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM