3GPP TSG-RAN WG3 Meeting #129	R3-255667

# Bangaluru, India, 25 - 29 August, 2025

Agenda item:	19.2

Source: 	Samsung

Title: 	Further discussion on information exchange for CLI handling

Document for:	Discussion and decision

Introduction

This paper provides some further discussion on the information exchange for CLI handling, based on the latest progress of RAN plenary and RAN WGs.

Discussion

In the current stage-3 BLCRs, the definition of SBFD configuration is kept as a sub-IE for further study, pending RAN2 progress.

| >>>SBFD Configuration | O |  | FFS (pending on RAN2 progress) | FFS | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

It is noticed that, during the post-meeting email disscussion of RAN2, a running CR for SBFD [1] has been created, to capture the agreements achieved in RAN2. When it comes to the SBFD configuration, according to the speific changes in the running CR, the time configuration parameters are defined separately within the TDD-UL-DL-ConfigCommon IE, as part of TDD-UL-DL-Pattern field, as shown below:

As for the frequency information, the configuration for SBFD subbands is specififed below, according to the running CR in RAN2:

According to the IE design in RAN2 runnig CR, what we could conclude from the progress of RAN2 is that, it is impossible to define a single container in network interface to cover all the SBFD configuration related parameters defined by RAN2, because these parameters are defined separately upon the existing TDD configuration. Thus, we would propose to define a new separated IE over XnAP and F1AP to cover the time and frequency configuration of SBFD.

Proposal 1: Define a new separated IE in XnAP and F1AP to cover the time and frequency configuration of SBFD.

With respect to the detailed sub-IEs of the time configuration and frequency configuration, there is no single IE in 38.331for time configuration to be referred to, while there is indeed a single IE for frequency information, i.e., the sbfd-Sunbband-Allocation-r19 IE. To keep a harmonized manner in RAN3 specs, we think it is better to define the sub-IEs of time and frequency configurations explicitly over network interfaces. Based on the above discussion, a stage 3 TP to the BLCR of 38.473 is provided in [2]. Similar changes could be taken for XnAP.

Proposal 2: Define the sub-IEs of time and frequency configuration explicitly over Xn/F1.

After the discussion of last RAN plenary meeting, i.e., RAN#108, the WID of the SBFD WI is updated, with a new bullet added for the information exchange between gNBs:

- Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - One or more SRS resource configurations

Although it has been approved in RAN plenary to add this new bullet, we think it is better for RAN3 to first evaluate the necessity to exchange the SRS resource configurations among gNBs, instead of simply digging into the details of adding the information into network interfaces.

Our basic understanding on the addition of this bullet is that, in order to configuration L1 CLI measurement to the UE, to measure the UE-to-UE CLI this UE may endure due to the UL transmission of another UE in the neighbour cell (which activates SBFD), the SRS configuration of the neighbour cell shall be informed to the receiving node. However, when considering the motivation to exchange the SRS resource configuration, a fact that cannot be ignored is that, in Rel-19 SBFD WI, RAN1 and RAN4 defined UE-to-UE CLR measurement clearly targeted for co-channel case, not adjacent-channel case, while the intention to exchange SRS resource configuration above seems to be measuring adjacent-channel UE-to-UE CLI. With this consideration in mind, we would prefer that RAN3 shall first check whether it is necessary to exchange SRS resource configuration for adjacent-channel UE-to-UE CLI measurements, before any normative work on specifications is performed.

Proposal 3: RAN3 shall first check whether it is necessary to exchange SRS resource configuration for adjacent-channel UE-to-UE CLI measurements, before any normative work on specifications is performed.

If companies all confirm the necessity to exchange the SRS resource configuration for adjacent-channel UE-to-UE CLI, the RAN3 impact can be further considered them. Note that one of the new defined L1 CLI measurement, i.e., SRS-RSRP measurement, is configured based on the SRS resource of the aggressor UE, which further justifies that the SRS resource of the aggressor UE shall be passed from the neighbour cell to the receiving cell, the served UE of which could be interfered. Specifically, we believe a list of SRS-resource IEs can be transmitted via XnAP and F1AP, for the purpose of configuring SRS-RSRP measurement resources for the victim UEs.

Proposal 4: A list of SRS-resource IEs of the aggressor UEs can be transmitted via XnAP and F1AP, for the purpose of configuring SRS-RSRP measurement resources for the victim UEs, only if the adjacent-channel UE-to-UE CLI measurement is confirmed.

Conclusion

Proposal 1: Define a new separated IE in XnAP and F1AP to cover the time and frequency configuration of SBFD.

Proposal 2: Define the sub-IEs of time and frequency configuration explicitly over Xn/F1.

Proposal 3: RAN3 shall first check whether it is necessary to exchange SRS resource configuration for adjacent-channel UE-to-UE CLI measurements, before any normative work on specifications is performed.

Proposal 4: A list of SRS-resource IEs of the aggressor UEs can be transmitted via XnAP and F1AP, for the purpose of configuring SRS-RSRP measurement resources for the victim UEs, only if the adjacent-channel UE-to-UE CLI measurement is confirmed.

References

[1] R2-25xxxxx RRC running CR for Evolution of NR duplex operation (SBFD)_v08_mark_Rapp

[2] R3-255668 Text proposals to BLCRs of TS38.473 on CLI handling