3GPP TSG-RAN WG3 Meeting #129	R3- 255671

Bengaluru, India, August 25th-29th, 2025

Agenda Item:	14.3

Serving:	CSCN

Title:	Discussion on impacts on Xn interface with SMTC enhancment

Document for:	Discussion

1	Introduction

In RAN2#130, “a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbour cells that need to be measured by the UE” was agreed in the last RAN2 meeting [1].

From RAN3 perspective, there are still some remaining open issues regarding the impacts on the Xn interface related to the SMTC selection procedure based on location-based SMTC selection, which require further discussion.

Issue 1: How can the serving cell acquire the reference location of the neighbour cells?

Issue 2: How can the serving cell establish associations between SMTC configurations and the reference locations?

In this contribution, we will further discuss the corresponding issues.

2	Discussion

Figure 1: SMTC deployment

During the last meeting, RAN2 agreed to introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbour cells that need to be measured by the UE. As Figure 1 shows, UE A knows the reference location of all neighbour cells and its own position, so UE A can determine it is closest to NCell1. Similarly, UE B can determine it is close to NCell1, NCell2 and NCell3. By correlating the SMTC configurations with the neighbour cell reference locations, UE can select the necessary SMTC configurations for measurement. For example, UE A can select SMTC1; UE B can select SMTC1, SMTC2 and SMTC3.

However, the serving cell has no awareness of the overall network deployment, so it cannot establish associations between SMTC configurations and neighbour cell locations. In order to establish such associations, the following two key issues need to be addressed.

Issue 1: How can the serving cell acquire the reference location of the neighbour cells?

In order to acquire the reference location of the neighbour cells, there are two possible solutions: one is the Xn interface solution, and the other is the OAM solution.

The Xn interface-based solution is that the serving cell acquires the reference location of neighbour cells through Xn interface. Specifically, the serving gNB transmits SMTC request messages to all neighbouring gNBs, and then each neighbouring gNB responds to the serving gNB with an SMTC Request Acknowledge message once or periodically. This message contains the cell information, including CGI/PCI, reference location, and the SMTC of each neighboring cell.

The OAM-based solution is that the serving cell acquires the reference location of neighbour cells from OAM. As the cell coverage is predefined by the network operator, OAM knows the cell coverage and the adjacent cells at any time. With OAM periodically providing the cell information including CGI/PCI, reference location and the SMTC of each neighbour cell, the serving cell can acquire the SMTC configuration and the reference location of each neighbour cell.

Proposal 1: Xn interface-based solution or OAM-based solution can be introduced to solve the issue of how the serving cell can acquire the reference location of the neighbour cells in SMTC enhancement for downlink coverage enhancement.

Issue 2: How can the serving cell establish associations between SMTC configurations and the reference locations?

For the Xn interface solution, the serving cell needs to obtain more information to establish associations between SMTC configurations and reference locations. One necessary element is the neighbour cell coverage/beam coverage. The serving cell establishes associations between SMTC configurations and reference locations based on the information provided by neighbour cells including CGI/PCI, reference location, cell coverage/beam coverage and the SMTC of each neighbour cell.

Proposal 2: Information such as CGI/PCI, reference location, cell coverage/beam coverage and the SMTC configuration needs to be transferred over Xn interface in the Xn interface-based solution.

For the OAM-based solution, OAM have the overall information, and it can decide the associations between SMTC configurations and reference locaitons.  So, it can send the association information to the serving gNB directly.

Proposal 3: OAM can directly provide associations between SMTC configurations and reference locations to the serving cell in the OAM-based solution.

3	Conclusion

In this contribution, we provide considerations on how the serving cell can acquire the SMTC configuration pattern of neighbouring cells in RAN3, following observations and proposals are made:

Proposal 1: Xn interface-based solution or OAM-based solution can be introduced to solve the issue of how the serving cell can acquire the reference location of the neighbour cells in SMTC enhancement for downlink coverage enhancement.

Proposal 2: Information such as CGI/PCI, reference location, cell coverage/beam coverage and the SMTC configuration needs to be transferred over Xn interface in the Xn interface-based solution.

Proposal 3: OAM can directly provide associations between SMTC configurations and reference locations to the serving cell in the OAM-based solution.

4	Reference

[1] Report of 3GPP TSG RAN WG2 meeting #130