3GPP TSG-RAN3 Meeting #129	R3-255692

Bengaluru, IN, 25-29 Aug 2025

# Title:	Discussion on information exchange for SBFD

# Source:	ZTE Corporation

# Agenda Item:	19.2

# Document for:	Discussion, Decision

Introduction

WID for SBFD is agreed in [1], following aspects are related to RAN3.

- Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - One or more SRS resource configurations - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

In this paper, we discus RAN3 related SBFD topics.

Discussion

SBFD Location Configuration

From our view, depending on OAM configuration and channel condition, one gNB may turn on/off the SBFD feature and corresponding SBFD resource configuration will be released/added. SBFD frequency/time related configuration may contains many parameters, “release/add” signaling is not efficient to exchange the enable or disable information. So, we suggest to introduce an on/off indication to indicate the state of SBFD resource configuration.

Based on RAN2 progress, both time domain and frequency domain related SBFD information has been defined in the running CR for TS 38.331. Currently, the granularity of the time domain SBFD info is per cell level. The granularity of the frequency domain SBFD info is per cell and per SCS level. Hence, to fulfill the RAN2 requirement, RAN3 may define separate IEs for time domain and frequency domain SBFD information transmission in XnAP and F1AP.

Proposal 1: RAN3 defines separate IEs for time domain and frequency domain SBFD information transmission for XnAP and F1AP.

Time domain SBFD location configuration is transmitted via tdd-UL-DL-ConfigurationCommon IE in RRC which has already been included in the existing IE(e.g. TDD UL-DL Configuration Common NR IE) in XnAP. No further enhancement is needed. Besides, this RAN3 IE has also been used for gNB-DU to transmit the same information to gNB-CU in the Served Cell Information IE. For gNB-CU to gNB-DU direction, there is only Intended TDD DL-UL configuration IE exists in Neighbour Cell Information List Item IE, F1AP GNB-CU CONFIGURATION UPDATE message. So, we prefer to let the new introduced SBFD Configuration IE to capture the time domain SBFD information and update the semantic description for further explanation which is shown below:

Tabular1. SBFD Configuration IE in BLCR for TS 38.473

| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| >>SBFD Configuration | O |  | OCTET STRING | Include the tdd-UL-DL-ConfigurtaionCommon IE as defined in TS 38.331[8]. | - |  |

Proposal 2: Update existing SBFD Configuration IE in BLCR for TS 38.473 to capture the tdd-UL-DL-ConfigurationCommon IE defined in RRC.

For frequency domain configuration, based on the existing BLCR design, we prefer to introduce a new IE(e.g. SBFD Frequency Configuration IE) which is parallel to the SBFD Configuration in existing XnAP and F1AP BLCR to capture the frequency domain related configuration. One example tabular is shown below:

Tabular2. SBFD Frequency Configuration IE in BLCR for TS 38.473

| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| >>SBFD Configuration | O |  | OCTET STRING | Include the tdd-UL-DL-ConfigurtaionCommon IE as defined in TS 38.331[8]. | - |  |
| >>SBFD Frequency Configuration | O |  | OCTET STRING | Includes the scs-SpecificCarrierList Contained in the SIB1 message as defined in TS 38.331[8] | - |  |

Proposal 3: Introduce a new IE(e.g. SBFD Frequency Configuration IE) which is parallel to the SBFD Configuration in existing XnAP and F1AP BLCR to capture the FDD related configuration.

SBFD RACH Configuration Exchange

Currently, F1 and NR interfaces exchange PRACH configuration of cells. Based on RAN1 and RAN2 discussions, for SBFD cells, dedicated SBFD RACH configurations will be introduced. This means that an SBFD cell may simultaneously configure two different types of RACH: legacy RACH and SBFD RACH.

However, in the current specification, only one type of RACH configuration is exchanged, making it impossible for the receiving node to determine whether the received configuration is for legacy RACH or SBFD RACH. Furthermore, without exchanging SBFD RACH configuration, the receiving gNB may transmit/receive data on resources allocated for SBFD RACH by the transmitting gNB, potentially causing interference to the transmitting gNB.We have two following alternatives:

-Alt 1: Confirm that the RACH transmitted over existing Xn interface is only for legacy RACH, and specify that SBFD cells should not populate SBFD RACH information in related IE

- Alt 2: Enhance Xn and F1 interfaces to support exchange of SBFD RACH configuration

While Alt 1 requires no signaling enhancement, it may lead to resource conflicts and interference issues. Alt 2 requires signaling enhancement but provides a complete solution with better resource coordination. Hence, it is proposed for RAN3 to enhance XnAP and F1AP to support exchanging SBFD specific RACH configuration. More specifically, introducing a new IE for this purpose in Served Cell Information NR IE

Proposal 4: Enhance Xn and F1 interfaces to support exchange of SBFD RACH configuration.

Sets of CSI-RS resource

In previous meeting, companies discussed the RAN1 agreement and make no consensus on the following topic:

Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is the relative index of the CSI-RS resources within a set of resources, the details to be checked.

Based on the internal discussion with our RAN1 and RAN2 friends after meeting, it is clear that RAN1 focus their view on transmission the NZP CSI-RS resources, whether one set or multiple sets of such info is not important. Meanwhile, RAN2 has already defined multiple sets of NZP CSI-RS resources in RRC signalling(e.g. CSI-ResourceConfig). From this point of view, we prefer to follow RAN2’s design and also introduce multiple sets of NZP CSI-RS resource sets.

Proposal 5: Multiple sets of NZP CSI-RS resource sets can be transmitted via XnAP and F1AP.

New Added Content in WID

The WID has been updated after RP#108, with the RAN3 related modification shown below:

==== content in RP-251874 ====

    - Information exchange among gNBs, including [RAN3]

      - Semi-static cell-specific SBFD time and frequency location configuration

      - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

      - Strongest DL beam information

      - CLI-mitigation request

      - One or more SRS resource configurations

==== content in RP-251874 ====

Based on the modified WID, RAN3 may support transmitting SRS resource configuration information for SBFD between gNBs. More specifically, the SRS resource configurations may encoded as OCTET STRING and be transmitted via Served Cell Information NR IE, in Xn Setup procedure and NG-RAN node Configuration Update procedure. Example tabular is shown below:

Tabular 3. Tabular design for transmitting SRS resource configuration

      - 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| SKIP UNRATELED PART |  |  |  |  |  |  |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| SKIP UNRATELED PART |  |  |  |  |  |  |
| >>>SBFD Configuration | O |  | FFS (pending on RAN2 progress) |  | YES | ignore |
| >>>SRS Resource Configuration List |  | 0..<maxnoofSRSResourceConfiguration> |  |  | YES | ignore |
| >>>>SRS Resource Configuration | M |  | OCTET STRING | Corresponds to information provided in the SRS-Resource contained in SRS-Config as defined in 38.331[10]. | - |  |

Proposal 6: Introduce the SRS Configuration Information List IE in the Served Cell Information NR IE and each SRS Configuration Information IE is encoded as OCTET STRING in XnAP.

Proposal 7: The maximum number of SRS Resource Configuration is 64.

Conclusion

Proposal 1: RAN3 defines separate IEs for time domain and frequency domain SBFD information transmission for XnAP and F1AP.

Proposal 2: Update existing SBFD Configuration IE in BLCR for TS 38.473 to capture the tdd-UL-DL-ConfigurationCommon IE defined in RRC.

Proposal 3: Introduce a new IE(e.g. SBFD Frequency Configuration IE) which is parallel to the SBFD Configuration in existing XnAP and F1AP BLCR to capture the FDD related configuration.

Proposal 4: Enhance Xn and F1 interfaces to support exchange of SBFD RACH configuration.

Proposal 5: Multiple sets of NZP CSI-RS resource sets can be transmitted via XnAP and F1AP.

Proposal 6: Introduce the SRS Configuration Information List IE in the Served Cell Information NR IE and each SRS Configuration Information IE is encoded as OCTET STRING in XnAP.

Proposal 7: The maximum number of SRS Resource Configuration is 64.

Reference

- RP-251874 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)