3GPP TSG-RAN WG3 Meeting #129														R3-255705

Bengaluru, IN, Aug 25 - 29, 2025

Agenda item:			11.3

Source:					CMCC

Title:						(TP to BLCR for TS 38.401, TS 38.423, TS 38.473 and TS 38.300) AI/ML based CCO

Document Type:		Other

Introduction

During RAN3#128, the following agreements have been captured for AI/ML enabled Coverage and Capacity Optimization.

Adopt Opt2 (gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU)

Timing information for predicted CCO issue over Xn is not needed?

Current mechanism enables the update of predicted CCO issue and/or future CCO state. A new detected CCO issue/a new predicted CCO issue for the same affected cells and beams after a predicted CCO issue will cancel the prediction.

Check the scenario whether has the possibility that there will be isolated multiple CCO issues detected for different cells or beams? Specific Cancel towards each CCO issue is needed?

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

In this paper, we provide our views on the FFS of AI/ML based Coverage and Capacity Optimization.

Discussion

Remaining open issues

Timing information for predicted CCO issue over Xn is not needed?

In RAN3#128, RAN3 has left an FFS on whether to exchange time information of predicted CCO issue over Xn. From our understanding, whether the time information of the coverage modification cause needs to be transferred over Xn depends on whether this time information is needed in the target node. For the target node, what it needs to do is to modify its own coverage at the time the coverage of source node changes. So, the target node needs to know when to make what kind of adjustment. Since the target node has received the future CCO state and the corresponding time information, it can be used by the target node to determine when to make the corresponding modification. Therefore, the time information of the predicted CCO issue seems to be less essential.

Proposal 1: No need to send time information related to coverage modification cause over Xn.

Current mechanism enables the update of predicted CCO issue and/or future CCO state. A new detected CCO issue/a new predicted CCO issue for the same affected cells and beams after a predicted CCO issue will cancel the prediction.

Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

Then, for the mechanism to update/cancel the predicted CCO issue and/or future CCO state, last meeting discussed whether to introduce a new detected CCO issue/a new predicted CCO issue to cancel the prediction. From our understanding, the current mechanism is supported to update a new predicted CCO issue and/or future CCO state, with reusing the NG-RAN node Configuration Update message to transmit the new one and overwrite the old one. But it’s no way to cancel the prediction. As non-AI/ML enabled CCO related information can also be exchanged via NG-RAN node Configuration Update message with "Future Coverage Modification Item" IE absence, if the absence of "Future Coverage Modification Item" IE represents the cancel of the prediction, the legacy procedure may cancel the prediction, which is not expected. So, we proposed to introduce a new Predicted Coverage Modification Cause or a separate IE to indicate the prediction is canceled. As RAN3 already discussed to add new code-point with details FFS, add a new cause value is reasonable. When the previous prediction needs to be canceled, the “Future Coverage Modification Item” IE can be the same as the prediction needs to be canceled, and the cause value is set to “cancel”. The signalling is reused, and only one new cause value needs to be introduced to cancel the prediction.

Observation 1: The current mechanism is supported to update a new predicted CCO issue and/or future CCO state, but no way to cancel the prediction.

Observation 2: The absence of "Future Coverage Modification Item" IE cannot represent the cancel of prediction, as legacy information can also be exchanged via NG-RAN node Configuration Update message without this IE. The legacy procedure may cancel the prediction, which is not expected.

Proposal 2: Introduce a new cause value in Predicted Coverage Modification Cause IE and Predicted CCO Assistance Information IE to cancel the prediction.

Check the scenario whether has the possibility that there will be isolated multiple CCO issues detected for different cells or beams? Specific Cancel towards each CCO issue is needed?

For multiple isolated CCO issues for different cells or beams, we think the scenario exists. But the cancel procedure for each CCO issue can be applied by introduce a new cause value as in P2. For one CCO issue needs to be canceled, the “Future Coverage Modification Item” IE can be the same as that of the predicted/detected CCO issue needs to be canceled, and the “Predicted Coverage Modification Cause” is set to “cancel”. Then, each of the isolated multiple CCO issues can be canceled separately.

Proposal 3: If P2 is agreed, cancellation of each isolated CCO issue for different cells or beams is supported.

Others

In RAN3#127bis, RAN3 has agreed that the legacy UE performance measurements can be reused for CCO.

But in order to evaluate the predictions or future information exchanged via Xn, the collected UE performance is proposed to be the coverage/cell edge UE(s)’ performance, i.e. the UE(s) serving gNB is changed after CCO, which is valuable to be checked as these UE(s) may mainly affected by CCO actions.

Proposal 4: It’s proposed to collect the performance of coverage/cell edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

Then, we also think the cell/node level gNB performance needs to be feedback from the target nodes. When the coverage configuration of local node changes, the coverage of neighbor nodes also need to be changed. Whether the coverage configuration modification caused by the local node results in severe performance degradation of neighbor node needs to be considered. For example, after the coverage configuration modification, the number of RRC connections of neighbor node increases, which may cause the neighbor node overload, or the PRB usage decrease, which waste the neighbor node resources. The neighbor nodes need to inform the local node these information and the local node can update the AI/ML model with these information.

Proposal 5: It’s proposed to collect the cell/node level performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage.

Conclusion

In this contribution, we propose the following proposals:

Proposal 1: No need to send time information related to coverage modification cause over Xn.

Observation 1: The current mechanism is supported to update a new predicted CCO issue and/or future CCO state, but no way to cancel the prediction.

Observation 2: The absence of "Future Coverage Modification Item" IE cannot represent the cancel of prediction, as legacy information can also be exchanged via NG-RAN node Configuration Update message without this IE. The legacy procedure may cancel the prediction, which is not expected.

Proposal 2: Introduce a new cause value in Predicted Coverage Modification Cause IE and Predicted CCO Assistance Information IE to cancel the prediction.

Proposal 3: If P2 is agreed, cancellation of each isolated CCO issue for different cells or beams is supported.

Proposal 4: It’s proposed to collect the performance of coverage/cell edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

Proposal 5: It’s proposed to collect the cell/node level performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.401

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

# 7.11	Support of AI/ML for NG-RAN

### 7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

For a split gNB, CCO issue is predicted at the gNB-CU. The gNB-CU sends to the gNB-DU the predicted CCO issue with the predicted affected cells and beams and the time for predicted CCO issue. The gNB-DU generates the future coverage state(s) and the time when the future coverage state(s) will be applied based on the predicted CCO issue and other local information. The gNB-DU may also take into account information received for other cells when generates the future CCO configuration. The gNB-DU informs the gNB-CU of the future coverage state and the time when the future coverage state will be applied. If the Neighbour Future Coverage Modifications for cells and beams is available at gNB-CU, gNB-CU sends it to gNB-DU, the gNB-DU shall, if supported, take it into account for Coverage and Capacity Optimization.

### 7.11.2	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

-	the recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

- Annex: TP for 38.423

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Update of Cell Coverage:

If the Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

-	If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell.

-	If the Cell Deployment Status Indicator IE is present and the Cell Replacing Info IE contains non-empty cell list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets.

-	If the Cell Deployment Status Indicator IE is not present, the NG-RAN node2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE.

If the SSB Coverage Modification List IE is present in the Coverage Modification List IE, the NG-RAN node2 may use the information in the SSB Coverage State IE to identify the SSB beam deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue detected at NG-RAN node1 and for configuring coverage state of its served cell(s).

If the Coverage Modification Cause IE set to "network energy saving" is present and the SSB Coverage State IE is zero for a set of SSB beams in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to decide the SSB beam activation for the concerned beams when necessary.

Update of Future Cell Coverage:

If the Future Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information to identify the future cell deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9].

If the Future SSB Modification List IE is present in the Future Coverage Modification List IE, the NG-RAN node2 may use the information to identify the future SSB beam deployment configuration to be enabled by the NG-RAN node1 at a time indicated by Time for Future Coverage State IE and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9].

If the Predicted Coverage Modification Cause IE set to "coverage" or "cell edge capacity" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use the information for deducing the CCO issue predicted in NG-RAN node1 and for configuring coverage state of its served cell(s).

If the Predicted Coverage Modification Cause IE set to "cancel" is present in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may cancel the configured coverage state of its served cell(s) for the CCO issue predicted in NG-RAN node1.

Interactions with other procedures:

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

If the NG-RAN node1 receives a NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message containing a Local NG-RAN Node Identifier within the Neighbour NG-RAN Node List IE identical to the Local NG-RAN Node Identifier included in the corresponding NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node1 may initiate the NG-RAN node Configuration Update procedure including in the NG-RAN NODE CONFIGURATION UPDATE message a new Local NG-RAN Node Identifier, different from the Local NG-RAN Node Identifier of each of its neighbour NG-RAN Nodes.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |
| Future Coverage Modification List |  | 0..1 |  | List of cells whose coverage will be modified. | YES | ignore |
| >Future Coverage Modification Item |  | 1..<maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 | NG-RAN Cell Global Identifier of the cell whose coverage will be modified. | – |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicates that the cell will be active and also indicates the future coverage configuration of the concerned cell. | – |  |
| >>Future SSB Coverage Modification List |  | 0..1 |  | List of SSB beams whose coverage will be modified. | – |  |
| >>>Future SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB index | M |  | INTEGER (0..63) | Identifier of the SSB beam whose coverage will be modified. | – |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also Indicates the future coverage configuration of the concerned SSB beams. | – |  |
| >>Predicted Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, cancel, ...) | Indicates the reason for the predicted coverage modification in NG-RAN node1. | – |  |
| >>Time for Future Coverage State | O |  | INTEGER (1..60, …) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information, in seconds. | – |  |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

- Annex: TP for 38.473

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a future cell and/or beam configuration.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust the coverage of its cells. 
Editor´s note: to be further discussed whether any more details need to be added to the above paragraph.

If the Neighbour Future Coverage Modification Notification IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, take it into account for Coverage and Capacity Optimization.

If the Predicted CCO issue IE set to "cancel" is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, cancel the determined future cell and/or beam configuration of its served cell(s) contained in the Predicted Affected Cells and Beams IE for the CCO issue predicted in gNB-CU.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, cancel, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

- Annex: TP for 38.300

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization, Network Slicing, Coverage and Capacity Optimization.

### 16.20.2	Mechanisms and Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information;

-	Predicted resource status information per slice;

-	Predicted slice available capacity;

-	UE performance feedback;

-	Measured UE trajectory;

-	Energy Cost (EC).

The collection and reporting are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

The Future Coverage Modification List, including the future coverage state, predicted coverage modification cause and time for future coverage state, is transferred to the target NG-RAN node via the NG-RAN NODE CONFIGURATION UPDATE procedure to provide information for the target NG-RAN node to modify its coverage correspondingly.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>