3GPP TSG RAN3 meeting #129   	         	                        R3-255716

18th – 22nd August 2025  Bengaluru, IN

Agenda Item:	17.3. Support on-demand SIB1 for UEs

Source:	ISSDU, NTU

Title:	On-Demand SIB1 Availability for Energy Saving in the Handover Scenario

Document for:	Discussion

Introduction

As part of the overall direction to enhance sustainability and energy efficiency in 5G-Advanced, the Release 19 Network Energy Saving (NES) work item was approved under [1]. One of its key objectives is to support on-demand SIB1 transmission, aiming to reduce unnecessary signaling in cells with low UE activity[2].

To improve idle/inactive mode energy performance, RAN1 has studied several on-demand SIB1 configurations based on WUS (Wake-Up Signal) and SIB1 coordination between cells. Three major options were outlined:

Case 1: Option 1+A+X, UL WUS Config in SIB1 and SIB1 both from the NES Cell

Case 2: Option 1+B+X, UL WUS Config in SIB1 from the Cell A, SIB1 from the NES Cell

Case 3: Option 2+B+Y, UL WUS Config in SIB1 and SIB1 both from the Cell A

These combinations vary in terms of UE awareness of UL WUS configuration and gNB coordination. Among them, RAN1 agreed at RAN#105 to prioritize Case 2 for further study.

While current RAN1 discussions primarily focus on idle/inactive mode UEs, potential implications for mobility and handover scenarios may be of interest for further evaluation in RAN3.

This document explores the applicability of on-demand SIB1 availability during HO, aiming to reduce energy consumption while maintaining HO performance. The analysis assumes reuse of SI scheduling and NES mechanisms, and also considers whether additional RRC signaling enhancements (e.g., IEs) may be beneficial.

Discussion

In current HO procedures, the target gNB typically maintains a fixed SIB1 periodicity, ensuring that SIB1 is available at all times. While robust, this approach may result in unnecessary energy consumption in low UE density cells.

Two operational approaches are considered:

1. Conventional HO (Fixed SIB1 Periodicity):

The target cell continuously broadcasts SIB1 regardless of UE activity. Upon HO execution, the UE synchronizes via SSB and receives SIB1 as part of the standard procedure. This ensures high availability but consumes energy even when the target cell has no active UEs.

2. HO with On-Demand SIB1 Availability:

The target cell ensures an SIB1 availability window during the HO process. This can be achieved via implementation-specific coordination during HO preparation, using existing SI scheduling and NES mechanisms (e.g., cell DTX/DRX, SSB periodicity adaptation). Optionally, enhancements to the HO preparation or configuration steps (e.g., signaling IE updates) may be considered to support this operation more reliably.

Figures 1 illustrate the conceptual difference and a possible message sequence, respectively.

Figure 1: Conceptual comparison between conventional HO and HO with On-Demand SIB1 availability.

The following four steps describe a proposed HO message sequence integrating On-Demand SIB1 availability to ensure reliable reception during HO while leveraging NES for energy efficiency, covering phases from preparation to post-execution optimization

Step 1: During HO Preparation, the serving gNB coordinates with the target gNB to configure an SIB1 availability window using existing SI scheduling and NES settings.

Step 2: Before sending the RRC Reconfiguration to the UE, determine the maximum acceptable time from HO command execution to SIB1 reception, ensuring it meets both performance and energy-saving targets.

Step 3: During HO Preparation, align NES-related parameters (e.g., adaptive SSB periodicity, cell DTX/DRX) with the planned SIB1 availability window to avoid timing conflicts.

Step 4: Based on the HO execution and Path Switch Request results, collect performance data to refine deployment guidelines or propose specification enhancements for On-Demand SIB1 availability in HO scenarios.

Proposal 1: Include mobility scenarios such as handover (HO) in the RAN3 discussion for On-Demand SIB1 availability, referencing the four-step HO message sequence.

Simulation Analysis

Figure 2 illustrates the trade-off between energy saving and HO success rate when applying different SIB1 availability window sizes (W = 20, 40, 80 ms) under the On-Demand SIB1 scheme. The results show that a larger availability window contributes to higher energy saving, yet leads to a lower HO success rate due to increased chances of missing the optimal SIB1 reception timing. In contrast, smaller windows (e.g., 20 ms) yield less energy saving but maintain better HO performance. A baseline case using periodic SIB1 broadcast with 80 ms periodicity is also included for reference.

Figure 2: Handover Success Rate vs. SIB1 Availability Window

Proposal 2: For Case 2: Option 1+B+X (WUS from Cell A, SIB1 from NES cell), evaluate feasibility, delay tolerance, NES interaction, and potential specification enhancements in HO scenarios.

Conclusion

In this contribution, we have the following proposals for On-Demand SIB1 Availability for Energy Saving in the Handover Scenario:

Proposal 1: Include mobility scenarios such as handover (HO) in the RAN3 discussion for On-Demand SIB1 availability, referencing the four-step HO message sequence.

Proposal 2: For Case 2: Option 1+B+X (WUS from Cell A, SIB1 from NES cell), evaluate feasibility, delay tolerance, NES interaction, and potential specification enhancements in HO scenarios.

References

[1] RP-234065: New WID: Enhancements of network energy savings for NR

[2] TR 38.864: Study on network energy savings for NR