3GPP TSG-RAN WG3 Meeting #129	R3-255736

Bengaluru, India, 25 – 29 August 2025

Agenda Item:       10.2

Source:     ZTE Corporation

Title: 	(TP for SON BLCR for TS38.423) MRO for others

Document for:	Discussion and Approval

1	Introduction

This contribution provides  a discussion on left issues on MRO for others.

2	MRO for CHO with SCG(s)

## 2.1 SCGFailureInformation

After RAN3#128 meeting, the following issue is left:

In addition to SCGFailureInformation, the following information needed in the SCG Failure Indication message

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

- Only in user Case 1b 2b 3b of tool late CPC execution,  the source MN received SCGFailureInformation from the UE and forwards it to the candidate MN. Since the SCGFailureInformation includes failedPSCellId 和 MeasResultFreqList and candidate MN has the knowledge of CPAC configuration of the UE, the candidate MN can deduce the CPC failure type and suitable PSCell ID, there is no need to provide duplicated information from source MN.

Proposal 1：No further information in addition to SCGFailureInformation in the SCG Failure Indication message.e.g CPC failure type Suitable PSCell ID.

## 2.2 Stage 3 updates

- Semantic description update for XnAP HANDOVER REPORT message:

- It was agreed to reuse HANDOVER REPORT to in case of too late CHO execution, reusing this message from the last serving MN to the relevant candidate MN.

- The message was used for the case of HO too early, HO to wrong cell, Inter-system ping-pong which means the NG-RAN node 1 is the target Node in previous handover.

- However the direction of the message is opposite in case of Too Late CHO with candidate SCG. In this case, NG-RAN node 2 is the target Node in previous handover.

Take Handover cause IE as an example, the source Node is NG-RAN node 1 instead of NG-RAN node 2.

| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it indicates the handover cause employed for handover from NG-RAN node 1. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 2: To update semantic description of HANDOVER REPORT message if Handover Report Type is set to “Too Late CHO with candidate SCG”.

3	MRO for SCPAC

After the RAN3#128 meeting, there is an open issue on S-CPAC ping-pong , as below:

MRO for S-CPAC Whether to enhance UHI in case of S-CPAC? - SCG UHI should be updated to the new serving MN/target SN from source MN during S-CPAC procedure, i.e. Include SCG UHI in SN Reconfiguration Complete message from MN to the target SN Inform the SN(s) about the outcome of mobility events for SN initiated S-CPAC using the SN RELEASE REQUEST or SN RELEASE CONFIRM.

The issue was raised of whether to support forwarding the SCG UHI to the target node during the S-CPAC procedure to avoid ping-pong immediately. However, we believe this type of real-time optimization introduces significant complexity of SCPAC reconfiguration at network side and  is not aligned with the principles of MRO, which focuses on long-term optimization. Additionally, it would require modifications to the UE's SCPAC configuration whenever a ping-pong is detected. From our perspective,  the real-time reconfiguration of SCPAC to avoid ping-pong is not necessary.

Proposal 3:  There's no need for a real-time reconfiguration scheme during SCPAC execution to avoid ping-pong.

- 4 Conclusion:

Proposal 1：No further information in addition to SCGFailureInformation in the SCG Failure Indication message.e.g CPC failure type Suitable PSCell ID.

Proposal 2: To update semantic description of HANDOVER REPORT message if Handover Report Type is set to “Too Late CHO with candidate SCG”.

Proposal 3:  There's no need for a real-time reconfiguration scheme during SCPAC execution to avoid ping-pong.

Annex : TP for BL CLR for TS38.423 for CHO with SCG(s)

<<<<<<<<<<<<<<<<<<<< Start Change >>>>>>>>>>>>>>>>>>>>

9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …, Too Late CHO with candidate SCG) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it indicates the handover cause employed for handover from NG-RAN node 1. | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2). If the Handover Report Type is set to “Too Late CHO with candidate SCG”,it indicates NG-RAN CGI of source cell for handover procedure (in NG-RAN node 1) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the suitable cell in the NG-RAN node 2. | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the candidate target NG-RAN node | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- SCG FAILURE INDICATION

--

-- **************************************************************

SCGFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SCGFailureIndication-IEs}},

...

}

SCGFailureIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TargetNG-RANnodeUEXnAPID	CRITICALITY ignore      	TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory}|

{ ID id-SCGFailureReportContainer   CRITICALITY ignore		TYPE SCGFailureReportContainer		PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< End Change >>>>>>>>>>>>>>>>>>>>