3GPP TSG-RAN WG3 Meeting #R3-129		           R3-255743

Bangalore, India, 25-29 Aug, 2025

Agenda Item:	17.3

Source: 	Rakuten

Title:  	Signalling to support OD-SIB1

Document for:	Discussion and decision

# Introduction

We have the following agreements from the previous RAN3 meetings:

RAN3#127-bis:

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

RAN3#128:

One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M)  +  one NES Cell ID (presence M ) +  one Cell-A ID (presence O )

Cell A gNB-CU encoding the SIBxx.

The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision.

# Signalling at NES Cell and Cell A

Based on the above agreements, there are some open points still to be finalized. These will be covered in the subsequent sections.

## NES Mode Allowed/NES Support Allowed

It has been agreed that a NES Cell gNB-DU is the entity which initiates the transition from default periodic SIB1 transmission to OD-SIB1 transmission. For this, it sends a provision request to the NES Cell gNB-CU which then determines which Cell(s) A gNB-CU to forward the request to. However, there might be cases where in a gNB, the CU may determine that certain cells under a DU may not transition to OD-SIB1 mode (NES mode) at all. In which case, we propose that the CU informs the DU of the list of cells, which are allowed to transition to OD-SIB1 mode. It may introduce an IE (say Cells Allowed to be in OD-SIB1 Mode List) in the GNB-CU CONFIGURATION UPDATE message. The receiving gNB-DU can use this information to decide which cells can transition to the NES mode when appropriate.

We propose that the CU informs the DU the list of cells, which are allowed to transition to OD-SIB1 mode. It may introduce an IE (Cells Allowed to be in OD-SIB1 Mode List) in the GNB-CU CONFIGURATION UPDATE message.

The same concept can be extended to the support from a Cell A gNB in transmission of the UL WUS Configuration of a NES Cell. If a Cell A gNB makes a decision to NOT support any transmission of UL WUS Configuration, it may inform the neighboring gNBs. On receiving this message, the target gNBs may deduce not to send a provision request to the source gNB. One way is for the source gNB to introduce an IE (say NES Support Unavailable) in the NG-RAN NODE CONFIGURATION UPDATE message. This message can be initiated by Cell A gNB when it first receives a provision request message. Or it may choose to send it independent of receiving the provision request message.

We propose that if a Cell A gNB makes a decision to NOT support transmission of any UL WUS Configuration, it may inform the neighbouring gNBs. On receiving this message, the target gNBs may deduce not to send a provision request to the source gNB.

## Provision Request/Response Message

The UL WUS CONFIGURATION PROVISION REQUEST is sent from NES Cell gNB-CU to Cell A gNB-CU over the Xn interface. We have agreed in the previous meeting that the provision request message contains one NES Cell ID and it may optionally include one Cell A ID. Therefore, it is understood that if the Cell A ID is not included in the provision request message, then it is up to the implementation at Cell A to decide which cell(s) under Cell A gNB-DU should transmit the UL WUS Configuration.

However, if the Cell A ID is included in the provision request message, then we propose that the request message be extended to include multiple Cell A IDs. This is to optimize the signaling from NES Cell gNB to Cell A gNB.

Therefore, the message format will be of the form:

UL WUS CONFIGURATION PROVISION REQUEST (NES Cell ID, UL WUS Configuration, (Optional) list of Cell A cells, START)

We propose that if the optional Cell A ID is included in the provision request message, then the provision request message be extended to optionally include multiple Cell A IDs.

Similarly, if the optional Cell A ID is included in the provision request message, then we propose that the provision response message from Cell A gNB-CU to NES Cell gNB-CU also include the list of Cell A cells which has accepted the provision request successfully.

We propose that if the optional Cell A ID is included in the provision request message, then the provision response message from Cell A gNB-CU to NES Cell gNB-CU also include the list of Cell A cells which has accepted the provision request successfully.

To “start” the transition from default SIB1 mode to OD-SIB1, even if the NES Cell gNB-CU has included an optional list of Cell A IDs, we believe that to “stop” the transmission of UL WUS Configuration in the Cell A, the NES Cell gNB-CU does not need to include any Cell A ID when it sends a provision message to the Cell A gNB-CU. It is understood that on receiving the stop request, all the Cell A cells which transmit the UL WUS Configuration of the NES Cell in its SIBx may stop doing so.

## NES Update

It is possible that the UL WUS Configuration of the NES Cell gNB changes. And if this happens while the NES Cell gNB-DU is in the NES mode, then the NES Cell gNB-CU and all the Cell A gNBs supporting the transmission of UL WUS Configuration need to be informed. Towards this we propose extending the earlier agreed messages with an “Update Provision” option.

The first IE to be updated is the Served Cell Information IE in the gNB-DU Configuration Update procedures sent from the NES Cell gNB-DU to the NES Cell gNB-CU over the F1 interface.

The second method to be updated is the UL WUS CONFIGURATION PROVISION REQUEST sent from the NES Cell gNB-CU to the Cell A gNB-CU over the Xn interface. To update the UL WUS Configuration of a NES Cell which is successfully supported by Cell A, the NES Cell gNB-CU does not need to include any Cell A ID when it sends an update message to the Cell A gNB-CU. It is understood that all the Cell A cells which transmit the UL WUS Configuration in SIBx of a given NES Cell will be informed on the update by the Cell a gNB-CU.

We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-DU informs the NES Cell gNB-CU using an “Update Provision” in the Served Cell Information IE over the F1 interface.

We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-CU informs the Cell A gNB-CU using an “Update” in the UL WUS CONFIGURATION PROVISION REQUEST over the Xn Interface.

Once the updated UL WUS Configuration has been accepted at the Cell A gNB-CU, it needs to inform the Cell A gNB-DU. We propose that to update the UL WUS Configuration of the NES Cell gNB, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface. The content of SIBx encoded by Cell A gNB-CU reflects the update of NES Cell gNB’s UL WUS Configuration.

We propose that to update the UL WUS Configuration of the NES Cell gNB, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface. The content of SIBx encoded by Cell A gNB-CU reflects the update of NES Cell gNB’s UL WUS Configuration.

It can be left to the internal implementation at the NES Cell gNB-DU to transition to the default periodic SIB1 transmission while the updated UL WUS Configuration change has been reflected in the SIBx at the Cell A gNB.

## NES Support Withdrawn

When Cell A wants to stop transmitting the UL WUS Configuration of the NES Cell, it may use the agreed Class 2 procedure to inform the NES Cell of the same. We have the following agreement from a previous meeting:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

The decision of whether the Cell A gNB should support the NES Cell gNB’s request of transmitting the NES Cell gNB’s UL WUS Configuration in its SIBx lies with the Cell A gNB-CU. To continue along the same line, we propose that the decision to stop the support of NES Cell gNB’s UL WUS Configuration transmission also lies with Cell A gNB-CU.

We propose that the decision to stop the support of NES Cell gNB’s UL WUS Configuration transmission also lies with Cell A gNB-CU.

To inform the Cell A gNB-DU of the same, the Cell A gNB-CU may follow the same process as in NES Deactivation process, where the encoded SIBx transmitted as part of gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface will have null content for NES Cell gNB’s UL WUS Configuration, or absence of NES Cell gNB’s UL WUS Configuration. From this, the Cell A gNB-DU may infer that it need not broadcast SIBx for the NES Cell gNB. However, If Cell A gNB is supporting other NES Cell gNBs’ NES mode, then Cell A gNB-DU may continue to broadcast SIBx.

The Cell A gNB-CU may then use the new Class 2 procedure to inform the NES Cell gNB-CU of its decision to stop the support of UL WUS Configuration transmission in its SIBx. Once the NES Cell gNB-CU gets this information, it may decide to do nothing if there are other Cell As which continue to transmit the UL WUS Configuration of the NES Cell gNB-DU. However, if there are no other Cell As which support the NES mode, then the NES Cell gNB-CU uses another Class 2 procedure to inform the NES Cell gNB-DU of the Cell A’s decision to stop the transmission of UL WUS Configuration. On reception of this message, the NES Cell gNB-DU must stop the NES mode, and transition from OD-SIB1 transmission to default periodic SIB1 transmission.

We propose the use of a new class 2 procedure for NES Cell gNB-CU to inform NES Cell gNB-DU to transition from OD-SIB1 mode to the default periodic SIB1 transmission.

# Conclusion

We have the following observations and proposals:

- Proposal 1	We propose that the CU informs the DU the list of cells, which are allowed to transition to OD-SIB1 mode. It may introduce an IE (Cells Allowed to be in OD-SIB1 Mode List) in the GNB-CU CONFIGURATION UPDATE message.

- Proposal 2	We propose that if a Cell A gNB makes a decision to NOT support any transmission of UL WUS Configuration, it may inform the neighbouring gNBs. On receiving this message, the target gNBs may deduce not to send a provision request to the source gNB.

- Proposal 3	We propose that if the optional Cell A ID is included in the provision request message, then the provision request message be extended to optionally include multiple Cell A IDs.

- Proposal 4	We propose that if the optional Cell A ID is included in the provision request message, then the provision response message from Cell A gNB-CU to NES Cell gNB-CU also include the list of Cell A cells which has accepted the provision request successfully.

- Proposal 5	We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-DU informs the NES Cell gNB-CU using an “Update Provision” in the Served Cell Information IE over the F1 interface.

- Proposal 6	We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-CU informs the Cell A gNB-CU using an “Update” in the UL WUS CONFIGURATION PROVISION REQUEST over the Xn Interface.

- Proposal 7	We propose that to update the UL WUS Configuration of the NES Cell gNB, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface. The content of SIBx encoded by Cell A gNB-CU reflects the update of NES Cell gNB’s UL WUS Configuration.

- Proposal 8	We propose that the decision to stop the support of NES Cell gNB’s UL WUS Configuration transmission also lies with Cell A gNB-CU.

We propose the use of a new class 2 procedure for NES Cell gNB-CU to inform NES Cell gNB-DU to transition from OD-SIB1 mode to the default periodic SIB1 transmission.

# Reference

Chair Notes RAN#128

Chair Notes RAN#127-bis

R3-252386