3GPP TSG-RAN WG3 Meeting #R3-129		           		   R3-255745

Bangalore, India, 25-29 Aug, 2025

Agenda item:	 11.3

Source:		Rakuten

Title:		Timing Information for AIML based CCO

Document for:		Discussion and Decision

# 1	Introduction

# Following are the agreements and WA from the previous meeting for the topic of prediction timing:

RAN3#126: WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.

RAN3#127: Exchange timer infor for predicted CCO issue over XnAP is not needed unless the usage can be identified?

RAN3#128: Timing information for predicted CCO issue is NOT needed to exchange over Xn?

# 2	Discussion

# In the legacy CCO mechanism:

Step 1: The gNB-CU detects the CCO issue (i.e., either coverage issue or cell edge capacity issue) and affected cells/SSBs;

Step 2: The gNB-CU sends the CCO Assistance Information IE (within the GNB-CU CONFIGURATION UPDATE message over F1) containing the CCO issue type and affected cells/SSBs to the gNB-DU;

Step3: The gNB-DU uses the received CCO Assistance Information IE to determine the new coverage state (CCO state) for the affected gNB-DU’s cells/SSBs;

Step 4: The gNB-DU sends the Coverage Modification Notification IE (within the GNB-DU CONFIGURATION UPDATE message over F1) containing the new CCO state for the affected gNB-DU’s cells/SSBs to the gNB-CU;

Step 5: The gNB-CU sends the Coverage Modification List IE (within the NG-RAN NODE CONFIGURATION UPDATE message over Xn) containing the detected CCO issue type and the new CCO state for the affected gNB-DU’s cells/SSBs to the neighbour gNB to allow for the neighbour gNB to determine a new CCO state for its own cells/SSBs that best matches the new CCO state of the gNB-CU’s cells/SSBs resulting from the detected CCO issue.

For the AIML-based CCO mechanism:

-	Step 0: AIML training entity generates predicted CCO issue

-	Step 1: gNB-CU predicts the CCO issue.

-	Step 2: gNB-CU sends the predicted CCO issue to gNB-DU.

-	Step 3: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

-	Step 4: gNB-DU sends the future coverage status to gNB-CU.

-	Step 5: gNB-CU sends the detected CCO issue and the new CCO state for the affected gNB-DU’s cells/SSBs to neighbour gNBs (say gNB2) along with the timing information of the future CCO state.

- 	Step 7: gNB2-CU sends this information to gNB2-DU which decides its own coverage/capacity configuration (future CCO state) in response to gNB1-DU’s future CCO state

Based on the above steps, it is clear that in the legacy mechanism, there is no need for exchanging any timing information between the source gNB and the neighbouring gNB for co-ordinating the actions to mitigate the detected CCO issue. This is because, once the issue is detected, all the following steps are executed immediately in the order mentioned above.

However, this is not the case in the case of AIML based CCO. Since there is a lag (up to 60s) between when the issue is predicted, and the time at which the action is taken (CCO state is changed) at both the source and the neighbouring gNBs, it is possible that different events may occur which might end up changing the planned future CCO state.

Observation 1	Since there is a lag between when the issue is predicted, and the time at which the action is taken (CCO state is changed) at both the source and the neighbouring gNBs, it is possible that different events may occur which might end up changing the planned future CCO state.

For the sake of discussion, we define participating entities comprising of the (source) gNB where the CCO issue has been detected/predicted, and the target gNB(s) which are neighbouring to the source gNB which end up taking a reactionary measure by changing their CCO state in response to the CCO state change of the source gNB.

Given the non-deterministic temporal nature of the AIML based CCO, we believe that the participating nodes (source gNB and the target gNB) must be armed with all the timing information. It has been agreed in RAN3#126 that a time is associated with the predicted CCO issue and a time is associated with the future CCO state at the source gNB (gNB1). However, so far, it has been agreed that only the time of the future CCO state is shared from the source gNB (CU1) to the neighbouring gNB (CU2). It has been left for further discussion whether the timing information of the predicted CCO issue is needed at the neighbouring gNB (CU2).

In the following paragraphs, we will present some use cases which will motivate the need for sharing the timing information of the predicted CCO issue over the Xn interface from CU1 and CU2.

Relevant duration of applying future CCO state at DU2

The timing of the predicted CCO issue at gNB1 is essential information for gNB2 to decide what is the relevant time duration during which it may apply its own CCO state to minimize the impact of the predicted CCO issue. For example, if the logic is to apply the CCO state before the CCO issue has occurred, gNB2 has no information on the time window available to apply the CCO state.  It only knows when gNB1-DU has planned to change its state but has no information on the upper time limit of applying its future CCO state, before the issue actually occurs in gNB1.

Conflict due to multiple predicted/detected CCO issues

The afore mentioned issue becomes even more prominent in case of multiple concurrent CCO predictions. For multiple co-existing CCO issues, the time information of the predicted CCO issue at gNB2 becomes imperative for optimal handling of the situation. For example, if a gNB receives multiple NG-RAN NODE CONFIGURATION UPDATE messages from multiple neighbouring gNBs, then it needs the additional time information about when each of the neighbouring cells is expecting to face the CCO issue to figure out in which order it should change its future CCO state to effectively prevent those CCO issues from occurring.

Monitoring the network condition to ensure the validity of the CCO issue prediction

When CU1 sends the CCO issue and the future CCO state along with its timing information to the CU2, it is important for the DU2 to know what the predicted time of the CCO issue by CU1 is. DU1 might apply the future CCO state before/after/same time as the CCO issue. However, if this information is not sent to the gNB2, then DU2 has only partial information about what is happening at gNB1 to which it is supposed to react to. If the timing information of the predicted CCO issue is also made available to gNB2, then it may decide its own strategy of validating the prediction and applying any necessary correction in terms of the CCO state. This could potentially include not going ahead with the planned CCO state change if the monitoring of network condition indicates that the CCO issue at gNB1 might be an inaccurate prediction.

Given these use cases, we propose that the timing information of the predicted CCO issue should be shared by CU1 to CU2 over the Xn interface. Furthermore, at the target NG-RAN node, we propose the CU2 forwards the CCO issue and the predicted time information it has received from the source NG=RAN node CU1 to the DU2, so that DU2 can decide its future CCO state. This is in addition to the future CCO state and the time of the future CCO state of DU1 that is sent by CU1 to CU2.

Proposal 1	We propose that the timing information of the predicted CCO issue should be shared by CU1 to CU2 over the Xn interface

Proposal 2	We propose that at the target NG-RAN node. the CU2 forwards the CCO issue and the predicted time information it has received from the source NG-RAN node CU1 to the DU2, so that DU2 can decide its future CCO state. This is in addition to the future CCO state and the time of the future CCO state of DU1 that is sent by CU1 to CU2.

# 3	Conclusion

Observation 1	Since there is a lag between when the issue is predicted, and the time at which the action is taken (CCO state is changed) at both the source and the neighbouring gNBs, it is possible that different events may occur which might end up changing the planned future CCO state.

Proposal 1	We propose that the timing information of the predicted CCO issue should be shared by CU1 to CU2 over the Xn interface

Proposal 2	We propose that at the target NG-RAN node. the CU2 forwards the CCO issue and the predicted time information it has received from the source NG-RAN node CU1 to the DU2, so that DU2 can decide its future CCO state.

# Reference

[1] TR 38.743

[2] TS 38.473