3GPP TSG-RAN WG3 Meeting #129	             R3-255821

Bangalore, India, 25th – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1314 | rev | 1 | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE radio capability for paging information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, CATT, ZTE, Qualcomm, Ericsson, Nokia, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2025-08-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, the UE Radio Capability for Paging IE is included in the INITIAL CONTEXT SETUP REQUEST message, but there are no procedural texts with regards to the NG-RAN behaviours. The main reason to introduce this IE in the INITIAL CONTEXT SETUP REQUEST message is specified in 5.4.4.3 Paging assistance information of TS 23.501, and copied as follows. - As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging Information (which is derived by the NG-RAN node) contains information on all NG-RAN RATs that the UE supports in that PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging Information in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN. It can be observed that when the NG-RAN receives the UE Radio Capability for Paging IE in the INITIAL CONTEXT SETUP REQUEST message, it shall, if supported, ensure that the received UE Radio Capability for Paging contains information on all NG-RAN RATs. In RAN3#127bis meeting, RAN3 sent an LS asking SA2 to provide the feedback on the NG-RAN node behavior in the case that the NG-RAN node receives the UE Radio Capability for Paging IE in the INITIAL UE CONTEXT SETUP REQUEST message, and if the UE Radio Capability for Paging Information does not contain information on all NG-RAN RATs that the UE supports. According to the SA2 response, it clearly stated that NG-RAN node is responsible to upload all “UE Radio Capability for Paging” as a single Information Element, instead of just providing the missing NG-RAN RATs when the NG-RAN checks the UE Radio Capability for Paging does not contain paging information on all NG-RAN RATs that the UE supports. In the RAN3#129 meeting, companies also found that the procedural text of NG-RAN behaviour is missing when the UE Radio Capability for Paging IE is included in Core Network Assistance Information for RRC INACTIVE IE in the INITIAL CONTEXT SETUP REQUEST message. The main reason to introduce this IE is used for NG-RAN to check and update it with all the RAT(s) the UE supports in the serving PLMN, use it for subsequent RAN paging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedural text for the behaviour of the NG-RAN node handling the UE Radio Capability for Paging IE when it receives the INITIAL CONTEXT SETUP REQUEST message. Add procedural text for the behaviour of the NG-RAN node handling the UE Radio Capability for Paging IE included in the Core Network Assistance Information for RRC INACTIVE IE when it receives the INITIAL CONTEXT SETUP REQUEST message. Add procedural text for the Interactions with UE Radio Capability Info Indication procedure if the UE Radio Capability for Paging IE does not include the UE paging capability of all the NG-RAN RATs that the UE supports in the serving PLMN. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change affects only the Initial Context Setup procedure. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NG-RAN behaviour remains unclear when the UE Radio Capability for Paging IE is provided in the INITIAL CONTEXT SETUP REQUEST message directly or the UE Radio Capability for Paging IE is included in the Core Network Assistance Information for RRC INACTIVE IE in the INITIAL CONTEXT SETUP REQUEST message. The missing procedural text handling the UE Radio Capability for Paging Information IE causes misalignment with SA2 stage2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-0: Submit in RAN3#129 meeting. Rev-1: Add procedure texts for the UE Radio Capability for Paging IE included in the Core Network Assistance Information for RRC INACTIVE IE. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

In case of the establishment of a PDU session the 5GC shall be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the AMF. If no UE-associated logical NG-connection exists, the UE-associated logical NG-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Index to RAT/Frequency Selection Priority IE, if available in the AMF.

If the NAS-PDU IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall pass it transparently towards the UE.

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the received UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9];

-	store the received Mobility Restriction List in the UE context;

-	store the received UE Radio Capability in the UE context;

-	store the received Index to RAT/Frequency Selection Priority in the UE context and use it as defined in TS 23.501 [9];

/******* Unchanged part skipped ******/

If the Core Network Assistance Information for RRC INACTIVE IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the UE Radio Capability for Paging IE is included in the Core Network Assistance Information for RRC INACTIVE IE in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, check and update it with all the RAT(s) the UE supports in the serving PLMN, use it for subsequent RAN paging.

/******* Unchanged part skipped ******/

If the UE Slice Maximum Bit Rate List IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store the received UE Slice Maximum Bit Rate List in the UE context, and use it for each S-NSSAI for the concerned UE as specified in TS 23.501 [9].

If the UE Radio Capability for Paging IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, use it as specified in TS 23.501 [9].

Interactions with Initial UE Message procedure:

The NG-RAN node shall use the AMF UE NGAP ID IE and RAN UE NGAP ID IE received in the INITIAL CONTEXT SETUP REQUEST message as identification of the logical connection even if the RAN UE NGAP ID IE had been allocated in an INITIAL UE MESSAGE message sent over a different NG interface instance.

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the INITIAL CONTEXT SETUP REQUEST message and set to "subsequent state transition report", the NG-RAN node shall, if supported, send the RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state.

Interactions with UE Radio Capability Info Indication procedure:

If the UE Radio Capability for Paging IE is included in the INITIAL CONTEXT SETUP REQUEST message, which does not include UE radio capability for paging of all the NG-RAN RAT(s) that the UE supports, the NG-RAN node shall, if supported, send the UE RADIO CAPABILITY INFO INDICATION message to the AMF containing the UE radio capability for paging of all the NG-RAN RAT(s) the UE supports in the serving PLMN.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>