3GPP TSG-RAN WG3 Meeting #129	R3-255890

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	14.3

Source: 	LG Electronics Inc., Ericsson, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT

Title: 	(TP for NR_NTN_Ph3 TS 38.300 BL CR) Stage-2 capturing logical connection between NG Removal/Setup and ephemeris info

Document for:	Other

# Introduction

In this contribution, we propose a TP to complete the reflection of the agreed NG setup/removal into stage-2.

# Discussion

For NG connection management of the regenerative payload, RAN3-125 agreed as follows:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

For this, a moving satellite on-board gNB needs to know “when” to initiate NG Removal or Setup (i.e. when moving in and out of AMF service areas), and we had proposed to capture this as an OAM requirement [1][2] (i.e. pre-configuration of AMF services areas into regenerative payloads).

However, recent discussions suggest that OAM configuration is not needed. Instead, it was observed that a moving satellite gNB could be configured to initiate NG Removal with the current AMF or NG Setup with a new AMF based on its pre-planned and pre-configured ephemeris data. Given that satellite trajectories and AMF coverage areas are relatively stable and predictable, it is feasible to configure “when and with which AMF” the satellite gNB should establish or remove NG connection (along with their transport network info to begin with), or, depending on the operator’s policy/configuration, simply rely on the implementation-specific behavior. This can remove the need to configure AMFs’ service areas and avoid requiring a satellite gNB to process such information to dynamically make decisions in real-time.

While this behavior is reasonable (with no stage-3 impact), it has not been clearly captured into stage-2 yet. Therefore, we propose to clarify in stage-2 that a moving satellite gNB may initiate NG Removal toward AMF based on e.g. its ephemeris information, and may then initiate NG Setup toward the same or a different AMF if required (without relying on pre-configuration of AMF service areas).

Proposal 1: Clarify in stage-2 that a satellite on-board gNB may initiate NG Removal toward AMF based on e.g. its ephemeris information, and may then initiate NG Setup toward the same or a different AMF (without requiring pre-configuration of AMF service areas).

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: Clarify in stage-2 that a satellite on-board gNB may initiate NG Removal toward AMF based on its e.g. ephemeris information, and may then initiate NG Setup toward the same or a different AMF (without requiring pre-configuration of AMF service areas).

The TP for Proposal 1 can be found in Section 5.

# Reference

[1] R3-253662, “OAM requirement for NG management”, LG Electronics Inc., Nokia, Nokia Shanghai Bell

[2] R3-253663, “(TP for NR_NTN_Ph3 TS 38.300 BL CR) OAM for NG management”, LG Electronics Inc., Nokia, Nokia Shanghai Bell

[3] R3-255067, “(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN”, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, ETRI, Jio Platforms

# TP for NR_NTN_Ph3 TS 38.300 BL CR [3]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 16.14.x	NG Removal

For the case of regenerative payload, when disconnecting from an AMF is required due to e.g. the satellite movement in a LEO constellation, the gNB on board the satellite may request the removal of the NG interface by initiating the NG Removal procedure toward the AMF based on e.g. ephemeris information.

NOTE:	After a successful NG removal, the gNB may initiate a new NG Setup procedure, if required, based on e.g. ephemeris information.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# Annex (RAN3 agreement for Regenerative Payload)

## RAN3-128 (Malta, May 2025)

NG Connection interruption due to hard feeder link switch over WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure. Check the use cases including Hard Feederlink Switch, other use cases to be further justified.

## RAN3-127bis (Wuhan, April 2025)

NG management: No new NGAP suspend/resume procedures in R19. TNL address updates due to feeder link switch In case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. NG Connection interruption due to hard feeder link switch over Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any). Inactive UE support Support RRC_INACTIVE UE in NTN by implementation in R19. Location based CHO: No enhancements on location based CHO for NTN in R19.

## RAN3-127 (Athens, February 2025)

NG management: Clarify the concerns mentioned in the incoming LS without any change on RAN3 previous agreements.

## RAN3-126 (Orlando, November 2024)

NG management: Whether to support NG suspend/Resume? Inactive UE support Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

## RAN3-125bis (Hefei, October 2024)

NG management: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. Feeder link switch issue The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. Intra-cell Inter-AMF HO Legacy NG HO procedure can be reused for inter AMF HO for NTN regenerative payload.

## RAN3-125 (Maastricht, August 2024)

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

## RAN3-124 (Fukuoka, May 2024)

## RAN3-123bis (Changsha, April 2024)

There is no consensus to discuss new NTN architecture now; wait for an LS from SA2 on this particular issue. Technical discussion based on current architecture can be discussed in next meeting.