3GPP TSG-RAN WG3 #129                                                    R3-255917

Bengaluru, IN, 25th – 29th Aug, 2025

Agenda Item:	10.2

Source:	CATT, Samsung, Nokia

Title:	(TP for 38.423) Add CPC failure type in SCG failure indication

Document for:	other

Introduction

This TP capture the following agreement in RAN3#129.

Include CPC failure type as an optional IE in SCG Failure Indication message with codepoint ‘To Wrong PSCell’ and ‘Too late CPC’

TP for 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

- 8.4.x	SCG Failure Indication

- 8.4.x.1	General

The purpose of the SCG Failure Indication procedure is to transfer SCG failure related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

- 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SCG Failure Indication, successful operation

NG-RAN node1 initiates the procedure by sending the SCG FAILURE INDICATION message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a SCG failure related problem was detected.

If the Failure Type IE is contained in the SCG FAILURE INDICATION and is set to “Too Late CPC Execution” or “CPC/CPA Execution to wrong PSCell”, the NG-RAN node2 shall, if supported, assume that a failure occurred in a SCG. The detection of Too Late CPC event and CPC/CPA Execution to Wrong PSCell are made according to TS 38.300 [9].

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

- 8.4.x.3	Unsuccessful Operation

Not applicable.

- 8.4.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

- 9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the candidate target NG-RAN node |  |  |
| SCG Failure Report Container | M |  | OCTET STRING | Includes the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |
| Failure Type | O |  | ENUMERATED (Too Late CPC Execution, CPC/CPA Execution to wrong PSCell…) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<End of the changes >>>>>>>>>>>>>>>>>>>>>>>

- 9.3.4	PDU Definitions

SRSPositioningConfigOrActivationRequest,

NRPPaPositioningInformation,

TimeSCG-Failure,

FailureType

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-TimeSCG-Failure,

id-TargetNG-RANnodeUEXnAPID,

idPCFailureType,

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- SCG FAILURE INDICATION

--

-- **************************************************************

SCGFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SCGFailureIndication-IEs}},

...

}

SCGFailureIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TargetNG-RANnodeUEXnAPID	CRITICALITY			TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory}|

{ ID id-SCGFailureReportContainer CRITICALITY ignore	TYPE SCGFailureReportContainer		PRESENCE mandatory}|

{ ID id-FailureType 			CRITICALITY ignore		TYPE 		FailureType			PRESENCE optional},

...

}

- 9.3.5	Information Element definitions

>>>skip unchanged part<<<

#### -- F

FailureType ::= ENUMERATED {

toolateCPCExecution,

cPCorCPA-ExecutiontowrongPSCell,

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

- 9.3.7	Constant definitions

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-TimeSCG-Failure																					ProtocolIE-ID ::= aaa

id-TargetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= bbb

id-FailureType                                                                                                   ProtocolIE-ID ::= ccc

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>