3GPP TSG-RAN WG3 Meeting #129	R3-255985

Bengaluru, India, 25th ~29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1532 | rev | 9 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson (Rapporteur), ZTE, NEC, Nokia, Huawei, Ofinno, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To enable AI/ML support for the use cases in scope of Rel19 work |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of function and procedure enhancements to support AI/ML supported use cases in scope for Rel19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The objectives of the AI/ML for NG-RAN work item cannot be fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2, 8.2.5.2, 8.2.5.4, 8.2.10.2, 8.3.1.2, 8.3.4.2, 9.2.1.7, 9.2.1.10, 9.2.1.20, 9.2.1.23, 9.2.2.1, 9.2.2.7, 9.3.1.212, 9.3.1.A (new), 9.3.1.B (new), 9.3.1.C (new), 9.3.1.xx (new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423 CR1411 TS37.483 CR0170 TS38.425 CR0157 TS38.401 CR0478 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmission of Rev 0 Rev2: Correction of Cover Page Rev3: addition of agreements from RAN3-127. Addition of rapporteur corrections to ASN.1 Rev4: Resubmission Rev5: addition of agreements from RAN3-127bis. Rev6: Resubmission Rev7: addition of changes from R3-253948 and R3-253953 Rev8: Resubmission Rev9: Addition of changes from R3-255698, R3-255829, R3-255930 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization and network energy saving. If the Coverage Modification Cause IE is set to the "network energy saving", gNB-CU may consider those deactivated SSB beams are due to network energy saving.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the eRedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store and use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

If the RRC Terminating IAB-Donor Related Info IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider that the BAP address indicated by the Mobile IAB-MT BAP Address IE is assigned by the gNB-CU of the RRC-terminating IAB-donor indicated  by the RRC Terminating IAB-Donor gNB-ID IE, and it shall use this  BAP address and gNB ID for the subsequent IAB Transport Migration Management procedure towards the RRC-terminating IAB-donor of the mobile IAB-node as needed, as specified in TS 38.423 [28].

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

If the Future Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message and if the Future Coverage Modification Cause IE is set to “cancel”, the gNB-CU shall, if supported, consider it as a notification that the gNB-DU has cancelled the future coverage modifications indicated for the cells and beams listed in the Future Coverage Modification Notification IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

<<<<SKIP UNRELATED PART>>>>

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.If the CCO issue detection IE set to "network energy saving" is included in the CCO Assistance Information IE, the gNB-DU may consider the indicated SSB beams by the Affected Cells and Beam IE are deactivated due to network energy saving.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

If the Mobile IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows mobile IAB-node access.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a future cell and/or beam configuration.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Predicted Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust the coverage of its cells and/or beam configuration.

If the Neighbour Future Coverage Modification Notification IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, take it into account for Coverage and Capacity Optimization.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and if the Predicted CCO Issue IE is set to “cancel”, the gNB-DU shall discard the Predicted CCO Assistance Information IE previously received together with the same list of cells and beams included in the Predicted Affected Cells and Beams IE, and it should cancel the future coverage states associated to the Predicted CCO Issue IE for the same list of cells and beams that have not been applied.

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, the Predicted CCO Issue IE is set to “cancel” and the Neighbour Future Coverage Modification Notification IE is present, the gNB-DU shall consider it as a notification of cancellation of the future coverage modifications included in the Neighbour Future Coverage Modification Notification IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 8.2.5.4	Abnormal Conditions

If the Predicted CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and if:

- the Predicted CCO Issue IE is set to “cancel”, and

- the Predicted Affected Cells and Beams IE is not present,

then the gNB-DU shall discard the received Predicted CCO Assistance Information IE.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.10	Resource Status Reporting Initiation

#### 8.2.10.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the RESOURCE STATUS REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the RESOURCE STATUS REQUEST message, the Cell To Report List IE shall be included.

If gNB-DU is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-DU shall perform measurements on. For each cell, gNB-DU shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource IE for such cell shall only include the SSB Area Radio Resource Status List IE; If the cell for which Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE; If the cell for which Radio Resource Status IE is requested to be reported supports MIMO the Radio Resource Status IE for such cell may include the MIMO PRB usage Information IE;

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List IE for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value IE. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE providing the SSB area capacity with respect to the Cell Capacity Class Value. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

-	the Hardware Load Indicator IE, if the fourth bit, " HW LoadInd Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Number of Active UEs IE, if the fifth bit, "Number of Active UEs Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the NR-U Channel List IE, if the sixth bit, " NR-U Channel List Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1";

-	the Energy Cost IE, if the seventh bit, "Energy Cost" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE in the RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.2.10.3	Unsuccessful Operation

Figure 8.2.10.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-DU shall send the RESOURCE STATUS FAILURE message with an appropriate cause value.

#### 8.2.10.4	Abnormal Conditions

If the initiating gNB-CU does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the gNB-CU may reinitiate the Resource Status Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

If the gNB-DU receives a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the DL LBT Failure Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the gNB- CU requests collection of DL LBT failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6], and act as specified in TS 38.401 [4].

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for the UE’s Ranging and Sidelink Positioning service.

For each DRB that has been successfully established and for which the Performance Delay Monitoring IE was included in the DRB to Be Setup List IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and use it to perform delay measurements on the successfully established DRBs.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message. If the Conditional Inter-DU Mobility Information IE or the LTM Indicator IE was included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

#### 8.3.1.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value. If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT SETUP REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

In case of "CHO-replace" when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT SETUP REQUEST message was not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore this candidate cell.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message and neither the LTM Information Modify IE nor the Conditional Intra-DU Mobility Information IE is present, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly. If the servingCellMO List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, configure servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT MODIFICATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the DL LBT Failure Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the gNB-CU requests collection of DL LBT failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6], , and act as specified in TS 38.401 [4].

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

For each DRB that has been successfully established or modified and for which the Performance Delay Monitoring IE was included in the DRB to Be Setup List IE or in the DRB to Be Modified List IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and use it to perform or update delay measurements on the successfully established or modified DRBs.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the UE context can be successfully performed, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. If the Conditional Intra-DU Mobility Information IE was included in the UE CONTEXT MODIFICATION REQUEST message and set to "CHO-initiation", the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT MODIFICATION REQUEST message, it shall reply with the UE CONTEXT MODIFICATION FAILURE message.

If the Conditional Intra-DU Mobility Information IE was included and set to "CHO-initiation" or "CHO-replace", but the SpCell ID IE was not included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

If the LTM Information Modify IE was included, but the SpCell ID IE and the CSI Resource Configuration IE were not included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

If the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.3.4.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT MODIFICATION REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated candidate cells.

If more than one of the following IEs, i.e., the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a logical error.

If one or more LTM cells in the LTM Cells To Be Released List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated LTM cells.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.1	Interface Management messages

<<<<SKIP UNRELATED PART>>>>

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |
| RRC Terminating IAB-Donor Related Info | O |  | 9.3.1.306 | Indicates the information related to a mobile IAB-node’s RRC-terminating IAB-donor. | YES | reject |
| Mobile IAB-MT User Location Information | O |  | 9.3.1.307 |  | YES | ignore |
| Future Coverage Modification Notification | O |  | 9.3.1.B |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| <<<<SKIP UNRELATED PART>>>> |  |  |  |  |  |  |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Predicted CCO Assistance Information | O |  | 9.3.1.A | Indicates predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | ignore |
| Neighbour Future Coverage Modification Notification | O |  | 9.3.1.C | Indicates Future Coverage Modifications for cells and beams not served by the gNB-DU. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs Periodic, Sixth Bit = NR-U Channel List Periodic, Seventh Bit = Energy Cost. Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) |  | - |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  | 1..< maxnoofBPLMNsNR> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to “add”. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

#### 9.2.1.21	RESOURCE STATUS RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested measurement is successfully initiated.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.22	RESOURCE STATUS FAILURE

This message is sent by gNB-DU to gNB-CU to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel Occupancy Time Percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |
| >>>>Channel Occupancy Time Percentage UL | O |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for UL traffic served by the corresponding NR-U Channel of the serving cell for UEs that transmit to the serving cell. Value 100 indicates that the channel resources have been utilized for UL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | YES | ignore |
| >>>>Radio Resource Status NR-U | O |  | 9.3.1.295 | Indicates the radio resource status per NR-U channel. | YES | ignore |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,...) | The node level Energy Consumption index measured at the gNB-DU. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31, ...) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SDT RLC Bearer Configuration | O |  | OCTET STRING | Includes the RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>Performance Delay Monitoring | O |  | 9.3.1.xx | Only the “UL and DL” codepoint value is used for this IE. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT RLC Bearer Configuration | O |  | OCTET STRING | RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN Identity 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS Information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. | - |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. | - |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| Configured BAP Address | O |  | BAP Address 9.3.1.111 | The BAP address configured for the corresponding child IAB-node. | YES | reject |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | - |  |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier |  |  | 9.3.1.121 |  | - |  |
| >>RLC mode | M |  | 9.3.1.27 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - |  |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | gNB-DU UE F1AP ID 9.3.1.5 | Allocated at the target gNB-DU | - |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| >S-CPAC Request | O |  | ENUMERATED (initiation, …) | Indicates that SN change is for S-CPAC preparation. | YES | reject |
| >S-CPAC Lower Layer Reference Config Request | O |  | ENUMERATED (true, …) |  | YES | reject |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.151 |  | YES | ignore |
| Serving NID | O |  | NID 9.3.1.155 |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 |  | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| Old CG-SDT Session Info | O |  | CG-SDT Session Info 9.3.1.261 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  | This IE is not used in this version of the specification. | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>>U2U RLC Channel QoS |  |  |  |  | YES | reject |
| >>>>U2U RLC Channel QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>Peer UE ID | O |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the sl-DestinationIdentityL2-U2U contained in the SL-TxResourceReqL2-U2U IE, defined in TS 38.331 [8]. This IE is included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2U Remote UE or L2 U2U Relay UE. | YES | reject |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. This IE is ignored if the DRB to Be Setup List IE is not present. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| UE Multicast MRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| >>Source MRB ID | O |  | MRB ID 9.3.1.224 | In case of inter-DU handover, indicates the MRB ID provided to the UE in the source cell. | YES | ignore |
| ServingCellMO List |  | 0..1 |  | For NCD-SSBs | YES | ignore |
| >ServingCellMO Item IEs |  | 1 .. <maxnoofServingCellMOs> |  |  | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>SSB frequency | M |  | INTEGER (0..3279165) | ARFCN | - |  |
| Network Controlled Repeater Authorized | O |  | 9.3.1.288 |  | YES | ignore |
| SDT Volume Threshold | O |  | INTEGER(1.. 192000,...) | Unit: byte. | YES | ignore |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31, ...) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT SRB. | YES | reject |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>Performance Delay Monitoring | O |  | 9.3.1.xx |  | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based or multi-path relay based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | YES | reject |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>>>ECN Marking or Congestion Information Reporting Request | O |  | 9.3.1.321 |  | YES | ignore |
| >>>>PSI based SDU Discard UL | O |  | ENUMERATED (start, stop, …) | Indicates whether UL PSI based SDU discard is (re)configured or released for the DRB. The codepoint “start” means that UL PSI based discarding is (re)configured, while the codepoint “stop” means that UL PSI based discarding is released. Up to 8 DRBs can be set as “start”. | YES | ignore |
| >>>>Performance Delay Monitoring | O |  | 9.3.1.xx | Only the “stop” codepoint value is used for this IE. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 |  | YES | ignore |
| >>UL Configuration | O |  | 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based or multi-path relay based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| >>SDT Indicator Modify | O |  | ENUMTERATED (true, false, …) | Indicates SDT DRB or not. | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | YES | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. | - |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. | - |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | - |  |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | - |  |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| >S-CPAC Request | O |  | ENUMERATED (initiation, …) | Indicates that SN change is for S-CPAC preparation. | YES | reject |
| >S-CPAC Lower Layer Reference Config Request | O |  | ENUMERATED (true, …) |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| Uplink TxDirectCurrentTwoCarrierList Information | O |  | 9.3.1.283 |  | YES | ignore |
| IAB Conditional RRC Message Delivery Indication | O |  | ENUMERATED (true, …) | Indicates whether the RRC message within should be withheld. This IE is only applicable if the UE is an IAB-MT, and the gNB-DU is an IAB-DU. | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 | This IE is only applicable if the UE is an IAB-MT. | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Updated Remote UE Local ID | O |  | Remote UE Local ID 9.3.1.267 | This IE indicates the updated Remote UE Local ID for the U2N Remote UE associated with the F1AP-IDs | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | O |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to Be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>>U2U RLC Channel QoS |  |  |  |  | YES | reject |
| >>>>U2U RLC Channel QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>Peer UE ID | O |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the sl-DestinationIdentityL2-U2U contained in the SL-TxResourceReqL2-U2U IE, defined in TS 38.331 [8]. This IE is included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2U Remote UE or L2 U2U Relay UE. | YES | reject |
| PC5 RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  |  |  |
| >>CHOICE PC5 RLC Channel QoS Information | O |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicate the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>>U2U RLC Channel QoS |  |  |  |  | YES | reject |
| >>>>U2U RLC Channel QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| Multicast MBS Session Remove List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has left. | YES | reject |
| UE Multicast MRB to Be Setup at Modify List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup at Modify Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| UE Multicast MRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| SL DRX Cycle List |  | 0..1 |  |  | YES | ignore |
| >SL DRX Cycle Item IEs |  | 1 .. <maxnoofSLdestinations > |  |  | EACH | ignore |
| >>RX UE ID | M |  | BIT STRING (SIZE(24)) | Indicates the destination L2 ID of RX UE associated to this UE. | - |  |
| >>CHOICE SL DRX Information | M |  |  |  | - |  |
| >>>SL DRX Cycle |  |  |  |  |  |  |
| >>>>SL DRX Cycle Length | M |  | ENUMERATED (ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...) | Indicates the desired SL DRX cycle for RX UE associated to this UE. | - |  |
| >>>No SL DRX |  |  |  |  | - |  |
| >>>>SL DRX configuration indicator | M |  | ENUMERATED(release,...) |  | - |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.274 |  | YES | ignore |
| SDT Bearer Configuration Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| DAPS HO status | O |  | ENUMERATED(initiation, …) | This IE is used if DAPS HO is initiated. | YES | ignore |
| ServingCellMO List |  | 0..1 |  | For NCD-SSBs | YES | ignore |
| >ServingCellMO Item IEs |  | 1 .. <maxnoofServingCellMOs> |  |  | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>SSB frequency | M |  | INTEGER (0..3279165) | ARFCN | - |  |
| Uplink TxDirectCurrentMoreCarrierList Information | O |  | 9.3.1.284 |  | YES | ignore |
| CPAC MCG Information |  | 0..1 |  | This IE is used at the MN for MCG configuration as specified in TS 37.340 [7] for CPAC. | YES | ignore |
| >CPAC Trigger | M |  | ENUMERATED (CPAC-preparation, CPAC-executed, …) |  | - |  |
| >PSCell ID | M |  | NR CGI 9.3.1.12 | The PSCell corresponding to the included CG-Config IE at CPAC-preparation or the selected PSCell by the UE at CPAC-executed. | - |  |
| Network Controlled Repeater Authorized | O |  | 9.3.1.288 |  | YES | ignore |
| SDT Volume Threshold | O |  | INTEGER(1.. 192000,...) | Unit: byte. | YES | ignore |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE shall be present if the CHO Trigger IE is present and set to "CHO-cancel". |
| ifEarlyUL | This IE shall be present if the Early UL Sync Configuration IE or the Early UL Sync Configuration for SUL IE is present. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.3.1	Radio Network Layer Related IEs

<<<<SKIP UNRELATED PART>>>>

#### 9.3.1.212	Affected Cells and Beams

This IE includes a list of cells and/or SS/PBCH block indexes affected by the detected CCO issue or predicted to be affected by the predicted CCO issue.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Affected Cell List |  | 1 .. < maxAffectedCells> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >Affected SSB List |  | 0..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxAffectedCells | Maximum numbers of cells affected by a CCO issue. Value is 32. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO Issue | O |  | ENUMERATED (coverage, cell edge capacity, cancel, ...) | Indicates the type of predicted CCO issue or that the previously sent predicted CCO issue is cancelled. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O | INTEGER (1..60, ...) |  | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. The IE is ignored if the Future Coverage Modification Cause is set to “cancel” |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, cancel, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.3.1.C	Neighbour Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by one or more neighbour node(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Future Coverage Modification Notification List |  | 1 |  |  |
| >Neighbour Future Coverage Modification Notification Item |  | 1..< maxNeighbourCellReport > |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Neighbour Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Neighbour Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Neighbour Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Neighbour Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Neighbour Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |

| Range bound | Explanation |
| --- | --- |
| maxNeighbourCellReport | Maximum no. neighbour cells for which the future neighbour state can be reported. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.3.1.xx	Performance Delay Monitoring

This IE defines the parameters for performance delay measurements, and whether to stop an ongoing measurement.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Performance Delay Monitoring Request | M |  | ENUMERATED (UL and DL, stop,…) | Indicates to measure UL and DL delay for the DRB, or to stop the ongoing measurement. | YES | ignore |
| Performance Delay Monitoring Reporting Periodicity | O |  | ENUMERATED(ms500, ms1000, ms2000, ms5000, ms10000, …) | Periodicity of reporting of UL and DL delay for the DRB. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Allowed-to-be-Deactivated-List-Item,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

ExecuteDuplication,

FullConfiguration,

GNB-CU-MBS-F1AP-ID,

GNB-CU-UE-F1AP-ID,

GNB-DU-MBS-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

LowerLayerPresenceStatusChange,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

MulticastMRBs-Setup-Item,

MulticastMRBs-FailedToBeSetup-Item,

MulticastMRBs-ToBeSetupMod-Item,

MulticastMRBs-ToBeModified-Item,

MulticastMRBs-ToBeReleased-Item,

MulticastMRBs-SetupMod-Item,

MulticastMRBs-FailedToBeSetupMod-Item,

MulticastMRBs-Modified-Item,

MulticastMRBs-FailedToBeModified-Item,

BroadcastAreaScope,

NetworkControlledRepeaterAuthorized,

NodeAssociatedInfoResult,

NRCGI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RANSharingAssistanceInformation,

RAT-FrequencyPriorityInformation,

RequestedSRSTransmissionCharacteristics,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCContainer-RRCSetupComplete,

RRCReconfigurationCompleteIndicator,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

SDT-Volume-Threshold,

ServCellIndex,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SNSSAI,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

SupportedUETypeList,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

UEIdentity-List-For-Paging-Item,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

NotificationInformation,

TraceActivation,

TraceID,

Neighbour-Cell-Information-Item,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Address-Info,

BHChannels-ToBeSetup-Item,

BHChannels-Setup-Item,

BHChannels-FailedToBeSetup-Item,

BHChannels-ToBeModified-Item,

BHChannels-ToBeReleased-Item,

BHChannels-ToBeSetupMod-Item,

BHChannels-FailedToBeModified-Item,

BHChannels-FailedToBeSetupMod-Item,

BHChannels-Modified-Item,

BHChannels-SetupMod-Item,

BHChannels-Required-ToBeReleased-Item,

BAPAddress,

BH-Routing-Information-Added-List-Item,

BH-Routing-Information-Removed-List-Item,

Child-Nodes-List,

Activated-Cells-to-be-Updated-List,

UL-BH-Non-UP-Traffic-Mapping,

IABIPv6RequestType,

IAB-TNL-Addresses-To-Remove-Item,

IABTNLAddress,

IAB-Allocated-TNL-Address-Item,

IABv4AddressesRequested,

TrafficMappingInfo,

UL-UP-TNL-Information-to-Update-List-Item,

UL-UP-TNL-Address-to-Update-List-Item,

DL-UP-TNL-Address-to-Update-List-Item,

NRV2XServicesAuthorized,

LTEV2XServicesAuthorized,

NRUESidelinkAggregateMaximumBitrate,

LTEUESidelinkAggregateMaximumBitrate,

SLDRBs-SetupMod-Item,

SLDRBs-ModifiedConf-Item,

SLDRBs-FailedToBeModified-Item,

SLDRBs-FailedToBeSetup-Item,

SLDRBs-FailedToBeSetupMod-Item,

SLDRBs-Modified-Item,

SLDRBs-Required-ToBeModified-Item,

SLDRBs-Required-ToBeReleased-Item,

SLDRBs-Setup-Item,

SLDRBs-ToBeModified-Item,

SLDRBs-ToBeReleased-Item,

SLDRBs-ToBeSetup-Item,

SLDRBs-ToBeSetupMod-Item,

GNBCUMeasurementID,

GNBDUMeasurementID,

RegistrationRequest,

ReportCharacteristics,

CellToReportList,

HardwareLoadIndicator,

CellMeasurementResultList,

ReportingPeriodicity,

TNLCapacityIndicator,

RAReportList,

RLFReportInformationList,

ReportingRequestType,

TimeReferenceInformation,

ConditionalInterDUMobilityInformation,

ConditionalIntraDUMobilityInformation,

TargetCellList,

MDTPLMNList,

PrivacyIndicator,

TransportLayerAddress,

URI-address,

NID,

PosAssistance-Information,

PosBroadcast,

PositioningBroadcastCells,

RoutingID,

PosAssistanceInformationFailureList,

PosMeasurementQuantities,

PosMeasurementResultList,

PosReportCharacteristics,

TRPInformationTypeItem,

TRPInformationItem,

LMF-MeasurementID,

RAN-MeasurementID,

SDT-Termination-Request,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

SRSConfiguration,

TRPList,

E-CID-MeasurementQuantities,

MeasurementPeriodicity,

E-CID-MeasurementResult,

Cell-Portion-ID,

LMF-UE-MeasurementID,

RAN-UE-MeasurementID,

RelativeTime1900,

SystemFrameNumber,

SlotNumber,

AbortTransmission,

TRP-MeasurementRequestList,

MeasurementBeamInfoRequest,

E-CID-ReportCharacteristics,

Extended-GNB-CU-Name,

Extended-GNB-DU-Name,

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

SuccessfulHOReportInformationList,

Coverage-Modification-Notification,

CCO-Assistance-Information,

CellsForSON-List,

IABCongestionIndication,

IABConditionalRRCMessageDeliveryIndication,

F1CTransferPathNRDC,

BufferSizeThresh,

IAB-TNL-Addresses-Exception,

BAP-Header-Rewriting-Added-List-Item,

Re-routingEnableIndicator,

Neighbour-Node-Cells-List,

Serving-Cells-List,

RBSetConfiguration,

PDCMeasurementPeriodicity,

PDCMeasurementQuantities,

PDCMeasurementResult,

PDCReportType,

RAN-UE-PDC-MeasID,

SCGActivationRequest,

SCGActivationStatus,

TRP-MeasurementUpdateList,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

TRP-PRS-Info-List,

PRS-Measurement-Info-List,

PRSConfigRequestType,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

UEReportingInformation,

PosContextRevIndication,

NRRedCapUEIndication,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

QoEInformation,

CG-SDTQueryIndication,

CG-SDTKeptIndicator,

CG-SDTSessionInfo,

SDTInformation,

FiveG-ProSeAuthorized,

UuRLCChannelToBeSetupList,

UuRLCChannelToBeModifiedList,

UuRLCChannelToBeReleasedList,

UuRLCChannelSetupList,

UuRLCChannelFailedToBeSetupList,

UuRLCChannelModifiedList,

UuRLCChannelFailedToBeModifiedList,

UuRLCChannelRequiredToBeModifiedList,

UuRLCChannelRequiredToBeReleasedList,

PC5RLCChannelToBeSetupList,

PC5RLCChannelToBeModifiedList,

PC5RLCChannelToBeReleasedList,

PC5RLCChannelSetupList,

PC5RLCChannelFailedToBeSetupList,

PC5RLCChannelFailedToBeModifiedList,

PC5RLCChannelRequiredToBeModifiedList,

PC5RLCChannelRequiredToBeReleasedList,

PC5RLCChannelModifiedList,

RemoteUELocalID,

PathSwitchConfiguration,

SidelinkRelayConfiguration,

PagingCause,

PEIPSAssistanceInfo,

UEPagingCapability,

GNBDUUESliceMaximumBitRateList,

MDTPollutedMeasurementIndicator,

UE-MulticastMRBs-ConfirmedToBeModified-Item,

UE-MulticastMRBs-RequiredToBeModified-Item,

UE-MulticastMRBs-RequiredToBeReleased-Item,

UE-MulticastMRBs-Setup-Item,

UE-MulticastMRBs-Setupnew-Item,

UE-MulticastMRBs-ToBeReleased-Item,

UE-MulticastMRBs-ToBeSetup-Item,

UE-MulticastMRBs-ToBeSetup-atModify-Item,

PosMeasurementAmount,

BAP-Header-Rewriting-Removed-List-Item,

SLDRXCycleList,

MDTPLMNModificationList,

ActivationRequestType,

PosMeasGapPreConfigList,

PosMeasurementPeriodicityNR-AoA,

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

ServingCellMO-List-Item,

ServingCellMO-encoded-in-CGC-List,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

DedicatedSIDeliveryIndication,

Configured-BWP-List,

MT-SDT-Information,

LTMInformation-Setup,

LTMConfigurationIDMappingList,

LTMInformation-Modify,

LTMCells-ToBeReleased-List,

LTMCFRAResourceConfig-List,

LTMConfiguration,

EarlySyncInformation-Request,

EarlySyncInformation,

EarlySyncCandidateCellInformation-List,

EarlySyncServingCellInformation,

LTMCellSwitchInformation,

DUtoCUTAInformation-List,

CUtoDUTAInformation-List,

DeactivationIndication,

RAReportIndicationList,

SuccessfulPSCellChangeReportInformationList,

PathAdditionInformation,

RANTSSRequestType,

RANTimingSynchronisationStatusInfo,

GlobalGNB-ID,

Activated-Cells-Mapping-List-Item,

RRC-Terminating-IAB-Donor-Related-Info,

NCGI-to-be-Updated-List-Item,

Mobile-IAB-MTUserLocationInformation,

TAI,

IndicationMCInactiveReception,

MulticastCU2DURRCInfo,

MulticastDU2CURRCInfo,

MBSMulticastSessionReceptionState,

MulticastCU2DUCommonRRCInfo,

NRA2XServicesAuthorized,

LTEA2XServicesAuthorized,

NReRedCapUEIndication,

NRPaginglongeDRXInformationforRRCINACTIVE,

Cells-With-SSBs-Activated-List,

Recommended-SSBs-for-Paging-List,

S-CPAC-Configuration,

DLLBTFailureInformationRequest,

DLLBTFailureInformationList,

SLPositioning-Ranging-Service-Info,

TimeWindowInformation-SRS-List,

TimeWindowInformation-Measurement-List,

SRSPosRRCInactiveValidityAreaConfig,

SRSReservationType,

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

Future-Coverage-Modification-Notification,

Predicted-CCO-Assistance-Information,

Neighbour-Future-Coverage-Modification-Notification

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

id-BroadcastMRBs-SetupMod-Item,

id-BroadcastMRBs-ToBeModified-List,

id-BroadcastMRBs-ToBeModified-Item,

id-BroadcastMRBs-ToBeReleased-List,

id-BroadcastMRBs-ToBeReleased-Item,

id-BroadcastMRBs-ToBeSetup-List,

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-Cells-Allowed-to-be-Deactivated-List,

id-Cells-Allowed-to-be-Deactivated-List-Item,

id-Cells-With-SSBs-Activated-List,

id-Recommended-SSBs-for-Paging-List,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-MBS-F1AP-ID,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-MBS-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-Extended-GNB-CU-Name,

id-Extended-GNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

id-MulticastMRBs-FailedToBeSetupMod-Item,

id-MulticastMRBs-Modified-List,

id-MulticastMRBs-Modified-Item,

id-MulticastMRBs-Setup-List,

id-MulticastMRBs-Setup-Item,

id-MulticastMRBs-SetupMod-List,

id-MulticastMRBs-SetupMod-Item,

id-MulticastMRBs-ToBeModified-List,

id-MulticastMRBs-ToBeModified-Item,

id-MulticastMRBs-ToBeReleased-List,

id-MulticastMRBs-ToBeReleased-Item,

id-MulticastMRBs-ToBeSetup-List,

id-MulticastMRBs-ToBeSetup-Item,

id-MulticastMRBs-ToBeSetupMod-List,

id-MulticastMRBs-ToBeSetupMod-Item,

id-MulticastF1UContext-ToBeSetup-List,

id-MulticastF1UContext-ToBeSetup-Item,

id-MulticastF1UContext-Setup-List,

id-MulticastF1UContext-Setup-Item,

id-MulticastF1UContext-FailedToBeSetup-List,

id-MulticastF1UContext-FailedToBeSetup-Item,

id-BroadcastAreaScope,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-NodeAssociatedInfoResult,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-RequestedSRSTransmissionCharacteristics,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCContainer-RRCSetupComplete,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SDT-Termination-Request,

id-SDT-Volume-Threshold,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SNSSAI,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-SupportedUETypeList,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-UEIdentity-List-For-Paging-List,

id-UEIdentity-List-For-Paging-Item,

id-UE-MulticastMRBs-ConfirmedToBeModified-List,

id-UE-MulticastMRBs-ConfirmedToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeModified-List,

id-UE-MulticastMRBs-RequiredToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeReleased-List,

id-UE-MulticastMRBs-RequiredToBeReleased-Item,

id-UE-MulticastMRBs-Setup-List,

id-UE-MulticastMRBs-Setup-Item,

id-UE-MulticastMRBs-Setupnew-List,

id-UE-MulticastMRBs-Setupnew-Item,

id-UE-MulticastMRBs-ToBeReleased-List,

id-UE-MulticastMRBs-ToBeReleased-Item,

id-UE-MulticastMRBs-ToBeSetup-atModify-List,

id-UE-MulticastMRBs-ToBeSetup-atModify-Item,

id-UE-MulticastMRBs-ToBeSetup-List,

id-UE-MulticastMRBs-ToBeSetup-Item,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-NotificationInformation,

id-TraceActivation,

id-TraceID,

id-Neighbour-Cell-Information-List,

id-Neighbour-Cell-Information-Item,

id-AdditionalRRMPriorityIndex,

id-DUCURadioInformationType,

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Address-Info,

id-BHChannels-ToBeSetup-List,

id-BHChannels-ToBeSetup-Item,

id-BHChannels-Setup-List,

id-BHChannels-Setup-Item,

id-BHChannels-ToBeModified-Item,

id-BHChannels-ToBeModified-List,

id-BHChannels-ToBeReleased-Item,

id-BHChannels-ToBeReleased-List,

id-BHChannels-ToBeSetupMod-Item,

id-BHChannels-ToBeSetupMod-List,

id-BHChannels-FailedToBeSetup-Item,

id-BHChannels-FailedToBeSetup-List,

id-BHChannels-FailedToBeModified-Item,

id-BHChannels-FailedToBeModified-List,

id-BHChannels-FailedToBeSetupMod-Item,

id-BHChannels-FailedToBeSetupMod-List,

id-BHChannels-Modified-Item,

id-BHChannels-Modified-List,

id-BHChannels-SetupMod-Item,

id-BHChannels-SetupMod-List,

id-BHChannels-Required-ToBeReleased-Item,

id-BHChannels-Required-ToBeReleased-List,

id-BAPAddress,

id-ConfiguredBAPAddress,

id-BH-Routing-Information-Added-List,

id-BH-Routing-Information-Added-List-Item,

id-BH-Routing-Information-Removed-List,

id-BH-Routing-Information-Removed-List-Item,

id-UL-BH-Non-UP-Traffic-Mapping,

id-Child-Nodes-List,

id-Activated-Cells-to-be-Updated-List,

id-IABIPv6RequestType,

id-IAB-TNL-Addresses-To-Remove-List,

id-IAB-TNL-Addresses-To-Remove-Item,

id-IAB-Allocated-TNL-Address-List,

id-IAB-Allocated-TNL-Address-Item,

id-IABv4AddressesRequested,

id-TrafficMappingInformation,

id-UL-UP-TNL-Information-to-Update-List,

id-UL-UP-TNL-Information-to-Update-List-Item,

id-UL-UP-TNL-Address-to-Update-List,

id-UL-UP-TNL-Address-to-Update-List-Item,

id-DL-UP-TNL-Address-to-Update-List,

id-DL-UP-TNL-Address-to-Update-List-Item,

id-NRV2XServicesAuthorized,

id-LTEV2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrate,

id-LTEUESidelinkAggregateMaximumBitrate,

id-PC5LinkAMBR,

id-SLDRBs-FailedToBeModified-Item,

id-SLDRBs-FailedToBeModified-List,

id-SLDRBs-FailedToBeSetup-Item,

id-SLDRBs-FailedToBeSetup-List,

id-SLDRBs-Modified-Item,

id-SLDRBs-Modified-List,

id-SLDRBs-Required-ToBeModified-Item,

id-SLDRBs-Required-ToBeModified-List,

id-SLDRBs-Required-ToBeReleased-Item,

id-SLDRBs-Required-ToBeReleased-List,

id-SLDRBs-Setup-Item,

id-SLDRBs-Setup-List,

id-SLDRBs-ToBeModified-Item,

id-SLDRBs-ToBeModified-List,

id-SLDRBs-ToBeReleased-Item,

id-SLDRBs-ToBeReleased-List,

id-SLDRBs-ToBeSetup-Item,

id-SLDRBs-ToBeSetup-List,

id-SLDRBs-ToBeSetupMod-Item,

id-SLDRBs-ToBeSetupMod-List,

id-SLDRBs-SetupMod-List,

id-SLDRBs-FailedToBeSetupMod-List,

id-SLDRBs-SetupMod-Item,

id-SLDRBs-FailedToBeSetupMod-Item,

id-SLDRBs-ModifiedConf-List,

id-SLDRBs-ModifiedConf-Item,

id-gNBCUMeasurementID,

id-gNBDUMeasurementID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReportList,

id-CellMeasurementResultList,

id-HardwareLoadIndicator,

id-ReportingPeriodicity,

id-TNLCapacityIndicator,

id-RAReportList,

id-RLFReportInformationList,

id-ReportingRequestType,

id-TimeReferenceInformation,

id-ConditionalInterDUMobilityInformation,

id-ConditionalIntraDUMobilityInformation,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-TraceCollectionEntityIPAddress,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityURI,

id-ServingNID,

id-PosAssistance-Information,

id-PosBroadcast,

id-PositioningBroadcastCells,

id-RoutingID,

id-PosAssistanceInformationFailureList,

id-PosMeasurementQuantities,

id-PosMeasurementResultList,

id-PosMeasurementPeriodicity,

id-PosReportCharacteristics,

id-TRPInformationTypeListTRPReq,

id-TRPInformationTypeItem,

id-TRPInformationListTRPResp,

id-TRPInformationItem,

id-LMF-MeasurementID,

id-RAN-MeasurementID,

id-SRSType,

id-ActivationTime,

id-AbortTransmission,

id-SRSConfiguration,

id-TRPList,

id-E-CID-MeasurementQuantities,

id-E-CID-MeasurementPeriodicity,

id-E-CID-MeasurementResult,

id-Cell-Portion-ID,

id-LMF-UE-MeasurementID,

id-RAN-UE-MeasurementID,

id-SFNInitialisationTime,

id-SystemFrameNumber,

id-SlotNumber,

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-PosMeasurementPeriodicityExtended,

id-SuccessfulHOReportInformationList,

id-Coverage-Modification-Notification,

id-CCO-Assistance-Information,

id-CellsForSON-List,

id-IABCongestionIndication,

id-IABConditionalRRCMessageDeliveryIndication,

id-F1CTransferPathNRDC,

id-BufferSizeThresh,

id-IAB-TNL-Addresses-Exception,

id-BAP-Header-Rewriting-Added-List,

id-BAP-Header-Rewriting-Added-List-Item,

id-Re-routingEnableIndicator,

id-Neighbour-Node-Cells-List,

id-Serving-Cells-List,

id-MDTPollutedMeasurementIndicator,

id-PDCMeasurementPeriodicity,

id-PDCMeasurementQuantities,

id-PDCMeasurementResult,

id-PDCReportType,

id-RAN-UE-PDC-MeasID,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-TRP-MeasurementUpdateList,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-TRP-PRS-Info-List,

id-PRS-Measurement-Info-List,

id-PRSConfigRequestType,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-UEReportingInformation,

id-PosContextRevIndication,

id-NRRedCapUEIndication,

id-RANUEPagingDRX,

id-CNUEPagingDRX,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-QoEInformation,

id-CG-SDTQueryIndication,

id-CG-SDTKeptIndicator,

id-CG-SDTSessionInfoOld,

id-SDTInformation,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5LinkAMBR,

id-FiveG-ProSeUEPC5AggregateMaximumBitrate,

id-UuRLCChannelToBeSetupList,

id-UuRLCChannelToBeModifiedList,

id-UuRLCChannelToBeReleasedList,

id-UuRLCChannelSetupList,

id-UuRLCChannelFailedToBeSetupList,

id-UuRLCChannelModifiedList,

id-UuRLCChannelFailedToBeModifiedList,

id-UuRLCChannelRequiredToBeModifiedList,

id-UuRLCChannelRequiredToBeReleasedList,

id-PC5RLCChannelToBeSetupList,

id-PC5RLCChannelToBeModifiedList,

id-PC5RLCChannelToBeReleasedList,

id-PC5RLCChannelSetupList,

id-PC5RLCChannelFailedToBeSetupList,

id-PC5RLCChannelModifiedList,

id-PC5RLCChannelFailedToBeModifiedList,

id-PC5RLCChannelRequiredToBeModifiedList,

id-PC5RLCChannelRequiredToBeReleasedList,

id-SidelinkRelayConfiguration,

id-UpdatedRemoteUELocalID,

id-PathSwitchConfiguration,

id-PagingCause,

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-PosMeasurementAmount,

id-BAP-Header-Rewriting-Removed-List,

id-BAP-Header-Rewriting-Removed-List-Item,

id-SLDRXCycleList,

id-ManagementBasedMDTPLMNModificationList,

id-ActivationRequestType,

id-PosMeasGapPreConfigList,

id-PosMeasurementPeriodicityNR-AoA,

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-ServingCellMO-List,

id-ServingCellMO-List-Item,

id-ServingCellMO-encoded-in-CGC-List,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-DedicatedSIDeliveryIndication,

id-Configured-BWP-List,

id-NetworkControlledRepeaterAuthorized,

id-MT-SDT-Information,

id-LTMInformation-Setup,

id-LTMConfigurationIDMappingList,

id-LTMInformation-Modify,

id-LTMCells-ToBeReleased-List,

id-LTMConfiguration,

id-LTMCFRAResourceConfig-List,

id-EarlySyncInformation-Request,

id-EarlySyncInformation,

id-EarlySyncCandidateCellInformation-List,

id-EarlySyncServingCellInformation,

id-LTMCellSwitchInformation,

id-DUtoCUTAInformation-List,

id-CUtoDUTAInformation-List,

id-DeactivationIndication,

id-RAReportIndicationList,

id-SuccessfulPSCellChangeReportInformationList,

id-PathAdditionInformation,

id-RANTSSRequestType,

id-RANTimingSynchronisationStatusInfo,

id-Target-gNB-ID,

id-Target-gNB-IP-address,

id-Target-SeGW-IP-address,

id-Activated-Cells-Mapping-List,

id-Activated-Cells-Mapping-List-Item,

id-F1SetupOutcome,

id-RRC-Terminating-IAB-Donor-Related-Info,

id-RRC-Terminating-IAB-Donor-gNB-ID,

id-NCGI-to-be-Updated-List,

id-NCGI-to-be-Updated-List-Item,

id-Mobile-IAB-MTUserLocationInformation,

id-IndicationMCInactiveReception,

id-MulticastCU2DURRCInfo,

id-MulticastDU2CURRCInfo,

id-MBSMulticastSessionReceptionState,

id-MulticastCU2DUCommonRRCInfo,

id-NRA2XServicesAuthorized,

id-LTEA2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrateForA2X,

id-LTEUESidelinkAggregateMaximumBitrateForA2X,

id-NReRedCapUEIndication,

id-NRPaginglongeDRXInformationforRRCINACTIVE,

id-Target-F1-Terminating-Donor-gNB-ID,

id-Broadcast-MRBs-Transport-Request-List,

id-Broadcast-MRBs-Transport-Request-Item,

id-S-CPAC-Configuration,

id-DLLBTFailureInformationRequest,

id-DLLBTFailureInformationList,

id-SLPositioning-Ranging-Service-Info,

id-TimeWindowInformation-SRS-List,

id-TimeWindowInformation-Measurement-List,

id-SRSPosRRCInactiveValidityAreaConfig,

id-SRSReservationType,

id-RequestedSRSPreconfigurationCharacteristics-List,

id-SRSPreconfiguration-List,

id-SRSInformation,

id-TAInformation-List,

id-NonIntegerDRXCycle,

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-Future-Coverage-Modification-Notification,

id-Predicted-CCO-Assistance-Information,

id-NeighbourFutureCoverageModNotification,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- GNB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE

--

-- **************************************************************

GNBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateIEs} },

...

}

GNBDUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Served-Cells-To-Add-List				CRITICALITY reject	TYPE Served-Cells-To-Add-List								PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List				CRITICALITY reject	TYPE Served-Cells-To-Modify-List							PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List				CRITICALITY reject	TYPE Served-Cells-To-Delete-List							PRESENCE optional	}|

{ ID id-Cells-Status-List						CRITICALITY reject	TYPE Cells-Status-List											PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List					PRESENCE optional	}|

{ ID id-gNB-DU-ID								CRITICALITY reject	TYPE GNB-DU-ID													PRESENCE optional	}|

{ ID id-GNB-DU-TNL-Association-To-Remove-List	CRITICALITY reject	TYPE GNB-DU-TNL-Association-To-Remove-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE Transport-Layer-Address-Info							PRESENCE optional	}|

{ ID id-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Coverage-Modification-Notification					PRESENCE optional	}|

{ ID id-gNB-DU-Name								CRITICALITY ignore	TYPE GNB-DU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name					CRITICALITY ignore	TYPE Extended-GNB-DU-Name										PRESENCE optional	}|

{ ID id-RRC-Terminating-IAB-Donor-Related-Info	CRITICALITY reject	TYPE RRC-Terminating-IAB-Donor-Related-Info	         PRESENCE optional }|

{ ID id-Mobile-IAB-MTUserLocationInformation    CRITICALITY ignore	TYPE Mobile-IAB-MTUserLocationInformation 		      PRESENCE optional	}|

{ ID id-Future-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Future-Coverage-Modification-Notification					PRESENCE optional	}},

...

}

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- GNB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	}|

{ ID id-Predicted-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	Predicted-CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-NeighbourFutureCoverageModNotification	CRITICALITY ignore	TYPE	Neighbour-Future-Coverage-Modification-Notification		PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

--**************************************************************

#### -- Resource Status Reporting Initiation ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Request

--

-- **************************************************************

ResourceStatusRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResourceStatusRequestIEs} },

...

}

ResourceStatusRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE conditional	}|

-- The above IE shall be present if the Registration Request IE is set to the value "stop" or "add".

{ ID id-RegistrationRequest		CRITICALITY ignore	TYPE RegistrationRequest	PRESENCE mandatory	}|

{ ID id-ReportCharacteristics	CRITICALITY ignore	TYPE ReportCharacteristics	PRESENCE conditional	}|

-- The above IE shall be present if the Registration Request IE is set to the value "start".

{ ID id-CellToReportList		CRITICALITY ignore	TYPE CellToReportList		PRESENCE optional	}|

{ ID id-ReportingPeriodicity	CRITICALITY ignore	TYPE ReportingPeriodicity	PRESENCE  optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Response

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusResponseIEs} },

...

}

ResourceStatusResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Failure

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusFailureIEs} },

...

}

ResourceStatusFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Resource Status Reporting ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Update

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ResourceStatusUpdateIEs}},

...

}

ResourceStatusUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID			CRITICALITY reject	TYPE GNBCUMeasurementID				PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID			CRITICALITY ignore	TYPE GNBDUMeasurementID			PRESENCE mandatory	}|

{ ID id-HardwareLoadIndicator			CRITICALITY ignore	TYPE HardwareLoadIndicator			PRESENCE optional	}|

{ ID id-TNLCapacityIndicator			CRITICALITY ignore	TYPE TNLCapacityIndicator		PRESENCE optional	}|

{ ID id-CellMeasurementResultList		CRITICALITY ignore	TYPE CellMeasurementResultList	PRESENCE optional	}|

{ ID id-NodeAssociatedInfoResult		CRITICALITY ignore	TYPE NodeAssociatedInfoResult	PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Resource Status Reporting Initiation ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Request

--

-- **************************************************************

ResourceStatusRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResourceStatusRequestIEs} },

...

}

ResourceStatusRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE conditional	}|

-- The above IE shall be present if the Registration Request IE is set to the value "stop" or "add".

{ ID id-RegistrationRequest		CRITICALITY ignore	TYPE RegistrationRequest	PRESENCE mandatory	}|

{ ID id-ReportCharacteristics	CRITICALITY ignore	TYPE ReportCharacteristics	PRESENCE conditional	}|

-- The above IE shall be present if the Registration Request IE is set to the value "start".

{ ID id-CellToReportList		CRITICALITY ignore	TYPE CellToReportList		PRESENCE optional	}|

{ ID id-ReportingPeriodicity	CRITICALITY ignore	TYPE ReportingPeriodicity	PRESENCE  optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Response

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusResponseIEs} },

...

}

ResourceStatusResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Failure

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusFailureIEs} },

...

}

ResourceStatusFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Resource Status Reporting ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Update

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ResourceStatusUpdateIEs}},

...

}

ResourceStatusUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID			CRITICALITY reject	TYPE GNBCUMeasurementID				PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID			CRITICALITY ignore	TYPE GNBDUMeasurementID			PRESENCE mandatory	}|

{ ID id-HardwareLoadIndicator			CRITICALITY ignore	TYPE HardwareLoadIndicator			PRESENCE optional	}|

{ ID id-TNLCapacityIndicator			CRITICALITY ignore	TYPE TNLCapacityIndicator		PRESENCE optional	}|

{ ID id-CellMeasurementResultList		CRITICALITY ignore	TYPE CellMeasurementResultList	PRESENCE optional	}|

{ ID id-NodeAssociatedInfoResult		CRITICALITY ignore	TYPE NodeAssociatedInfoResult	PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Coverage-Modification-Cause,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-areaScope,

id-IntendedTDD-DL-ULConfig,

id-QosMonitoringRequest,

id-BHInfo,

id-IAB-Info-IAB-DU,

id-IAB-Info-IAB-donor-CU,

id-IAB-Barred,

id-SIB12-message,

id-SIB13-message,

id-SIB14-message,

id-UEAssistanceInformationEUTRA,

id-SL-PHY-MAC-RLC-Config,

id-SL-ConfigDedicatedEUTRA-Info,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRPRACHConfig,

id-TDD-UL-DLConfigCommonNR,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-TSCTrafficCharacteristics,

id-AdditionalPDCPDuplicationTNL-List,

id-RLCDuplicationInformation,

id-AdditionalDuplicationIndication,

id-mdtConfiguration,

id-TraceCollectionEntityURI,

id-NID,

id-NPNSupportInfo,

id-NPNBroadcastInformation,

id-AvailableSNPN-ID-List,

id-SIB10-message,

id-RequestedP-MaxFR2,

id-DLCarrierList,

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-Supported-MBS-FSA-ID-List,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-MBS-Broadcast-NeighbourCellList,

id-PDCPTerminatingNodeDLTNLAddrInfo,

id-ENBDLTNLAddress,

id-PRS-Resource-ID,

id-LocationMeasurementInformation,

id-SliceRadioResourceStatus,

id-CompositeAvailableCapacity-SUL,

id-NR-U,

id-NR-U-Channel-List,

id-MIMOPRBusageInformation,

id-IngressNonF1terminatingTopologyIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-rBSetConfiguration,

id-frequency-Domain-HSNA-Configuration-List,

id-child-IAB-Nodes-NA-Resource-List,

id-Parent-IAB-Nodes-NA-Resource-Configuration-List,

id-uL-FreqInfo,

id-uL-Transmission-Bandwidth,

id-dL-FreqInfo,

id-dL-Transmission-Bandwidth,

id-uL-NR-Carrier-List,

id-dL-NR-Carrier-List,

id-nRFreqInfo,

id-transmission-Bandwidth,

id-nR-Carrier-List,

id-permutation,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-SurvivalTime,

id-PDCMeasurementQuantities-Item,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoAInformation,

id-ARPLocationInfo,

id-ARP-ID,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRPRx-TEGInformation,

id-TRPBeamAntennaInformation,

id-Redcap-Bcast-Information,

id-NR-TADV,

id-SDT-MAC-PHY-CG-Config,

id-CG-SDTindicatorSetup,

id-CG-SDTindicatorMod,

id-SDTRLCBearerConfiguration,

id-SRBMappingInfo,

id-DRBMappingInfo,

id-LastUsedCellIndication,

id-Recommended-SSBs-List,

id-SSBs-withinTheCell-tobe-Activated-List,

id-SIB17-message,

id-MUSIM-GapConfig,

id-SIB20-message,

id-pathPower,

id-DU-RX-MT-RX-Extend,

id-DU-TX-MT-TX-Extend,

id-DU-RX-MT-TX-Extend,

id-DU-TX-MT-RX-Extend,

id-TAINSAGSupportList,

id-SL-RLC-ChannelToAddModList,

id-SIB15-message,

id-InterFrequencyConfig-NoGap,

id-MBSInterestIndication,

id-L571Info,

id-L1151Info,

id-SCS-480,

id-SCS-960,

id-SRSPortIndex,

id-PEISubgroupingSupportIndication,

id-NeedForGapsInfoNR,

id-NeedForGapNCSGInfoNR,

id-NeedForGapNCSGInfoEUTRA,

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-Preconfigured-measurement-GAP-Request,

id-BWP-Id,

id-ExtendedResourceSymbolOffset,

id-MusimCapabilityRestrictionIndication,

id-duplicationIndication,

id-dRB-List,

id-ChannelOccupancyTimePercentageUL,

id-RadioResourceStatusNR-U,

id-FiveG-ProSeLayer2Multipath,

id-FiveG-ProSeLayer2UEtoUERelay,

id-FiveG-ProSeLayer2UEtoUERemote,

id-TSCTrafficCharacteristicsFeedback,

id-RANfeedbacktype,

id-Mobile-TRP-LocationInformation,

id-Mobile-IAB-MT-UE-ID,

id-MobileAccessPointLocation,

id-SIB24-message,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-ERedcap-Bcast-Information,

id-NeedForInterruptionInfoNR,

id-SCPAC-Request,

id-MobileIAB-Barred,

id-F1UTunnelNotEstablished,

id-S-CPACLowerLayerReferenceConfigRequest,

id-MusimCandidateBandList,

id-PSIbasedSDUdiscardUL,

id-SIB22-message,

id-U2URLCChannelQoS,

id-SL-PHY-MAC-RLC-ConfigExt,

id-UL-RSCP,

id-BW-Aggregation-Request-Indication,

id-ReportingGranularitykminus1,

id-ReportingGranularitykminus1additionalpath,

id-ReportingGranularitykminus2,

id-ReportingGranularitykminus2additionalpath,

id-ReportingGranularitykminus3,

id-ReportingGranularitykminus3additionalpath,

id-ReportingGranularitykminus4,

id-ReportingGranularitykminus4additionalpath,

id-ReportingGranularitykminus5,

id-ReportingGranularitykminus5additionalpath,

id-ReportingGranularitykminus6,

id-ReportingGranularitykminus6additionalpath,

id-TimingReportingGranularityFactorExtended,

id-PosValidityAreaCellList,

id-SymbolIndex,

id-AggregatedPosSRSResourceIDList,

id-PhaseQuality,

id-PRSBandwidthAggregationRequestIndication,

id-AggregatedPRSResourceSetList,

id-MeasuredFrequencyHops,

id-TxHoppingConfiguration,

id-AggregatedPosSRSResourceSetList,

id-ValidityAreaSpecificSRSInformation,

id-PeerUE-ID,

id-MeasBasedOnAggregatedResources,

id-SIB23-message,

id-PointA,

id-SCS-SpecificCarrier,

id-NR-PCI,

id-E-CID-MeasuredResultsAssociatedInfoList,

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-PerformanceDelayMonitoring,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxCellingNBDU,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofslots,

maxnoofNonUPTrafficMappings,

maxnoofServingCells,

maxnoofServedCellsIAB,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofDUFSlots,

maxnoofHSNASlots,

maxnoofEgressLinks,

maxnoofMappingEntries,

maxnoofDSInfo,

maxnoofQoSParaSets,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofPhysicalResourceBlocks-1,

maxnoofPRACHconfigs,

maxnoofRAReports,

maxnoofRLFReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationState,

maxnoofCHOcells,

maxnoofMDTPLMNs,

maxnoofCAGsupported,

maxnoofNIDsupported,

maxnoofExtSliceItems,

maxnoofPosMeas,

maxnoofTRPInfoTypes,

maxnoofSRSTriggerStates,

maxnoofSpatialRelations,

maxnoBcastCell,

maxnoofTRPs,

maxnooflcs-gcs-translation,

maxnoofPath,

maxnoofMeasE-CID,

maxnoofSSBs,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofSuccessfulHOReports,

maxnoofNR-UChannelIDs,

maxServedCellforSON,

maxNeighbourCellforSON,

maxAffectedCells,

maxnoofMBSQoSFlows,

maxnoofMBSFSAs,

maxnoofMBSAreaSessionIDs,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofCellsforMBS,

maxnoofIABCongInd,

maxnoofBHRLCChannels,

maxnoofTLAsIAB,

maxnoofRBsetsPerCell,

maxnoofRBsetsPerCell-1,

maxnoofNeighbourNodeCellsIAB,

maxnoofMeasPDC,

maxnoARPs,

maxnoofULAoAs,

maxNoPathExtended,

maxnoTRPTEGs,

maxFreqLayers,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

maxnoofPRSTRPs,

maxnoofQoEInformation,

maxnoofUuRLCChannels,

maxnoofPC5RLCChannels,

maxnoofSMBRValues,

maxnoofMBSSessionsofUE,

maxnoofSLdestinations,

maxnoofNSAGs,

maxnoofSDTBearers,

maxnoofPosSITypes,

maxnoofMRBs,

maxNrofBWPs,

maxnoofUETypes,

maxnoofLTMCells,

maxnoofLTMgNB-DUs,

maxnoofTAList,

maxnoofDRBs,

maxnoofUEsInQMCTransferControlMessage,

maxnoofUEsforRAReportIndications,

maxnoofSuccessfulPSCellChangeReports,

maxnoofPeriodicities,

maxnoofThresholdMBS-1,

maxMBSSessionsinSessionInfoList,

maxnoofLBTFailureInformation,

maxnoofRSPPQoSFlows,

maxnoVACell,

maxnoAggregatedSRS-Resources,

maxnoAggregatedPosSRSResourceSets,

maxnoAggregatedPosPRSResourceSets,

maxnoofTimeWindowSRS,

maxnoofTimeWindowMea,

maxnoPreconfiguredSRS,

maxnoHopsMinusOne,

maxnoAggCombinations,

maxnoAggregatedPosSRSCombinations,

maxNeighbourCellReport

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardUL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardUL		PRESENCE optional	}|

{	ID id-PerformanceDelayMonitoring	CRITICALITY ignore	EXTENSION PerformanceDelayMonitoring		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### -- E

EarlyULSyncConfig ::= SEQUENCE {

rACH 								RACHConfiguration,

lTMgNB-DU-IDs-PreambleIndexList		LTMgNB-DU-IDs-PreambleIndexList							OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { EarlyULSyncConfig-ExtIEs} } OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

ECNmarkingRequest ::= ENUMERATED { ul, dl, both, stop, ... }

CongestionInformationRequest ::= ENUMERATED { ul, dl, both, stop, ... }

ECNMarkingorCongestionInformationReportingStatus ::= ENUMERATED { active, not-active, ...}

EnergyCost ::= INTEGER (0..10000, ...)

#### -- F

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

Future-Coverage-Modification-Notification ::= SEQUENCE {

future-coverage-Modification-List			Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Future-Coverage-Modification-Item

Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

futurecellCoverageState				FutureCellCoverageState,

futureSSBCoverageModificationList	FutureSSBCoverageModification-List	OPTIONAL,

timeforFutureCoverageModification	TimeforFutureCoverageModification	OPTIONAL,

futureCoverageModificationCause		Predicted-CCO-issue-detection		OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FutureCellCoverageState ::= INTEGER (0..63, ...)

FutureSSBCoverageModification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF FutureSSBCoverageModification-Item

FutureSSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

futureSSBCoverageState			FutureSSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { FutureSSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

FutureSSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FutureSSBCoverageState ::= INTEGER (0..15, ...)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- N

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

Neighbour-Future-Coverage-Modification-Notification ::= SEQUENCE {

neighbour-future-coverage-Modification-List			Neighbour-Future-Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Neighbour-Future-Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Neighbour-Future-Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Neighbour-Future-Coverage-Modification-List ::= SEQUENCE (SIZE (1.. maxNeighbourCellReport)) OF Neighbour-Future-Coverage-Modification-Item

Neighbour-Future-Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

neighbourfuturecellCoverageState				NeighbourFutureCellCoverageState,

neighbourfutureSSBCoverageModificationList	NeighbourFutureSSBCoverageModification-List	OPTIONAL,

timeforneighbourFutureCoverageModificaiton	TimeforNeighbourFutureCoverageModification	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Neighbour-Future-Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Neighbour-Future-Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourFutureCellCoverageState ::= INTEGER (0..63, ...)

NeighbourFutureSSBCoverageModification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF NeighbourFutureSSBCoverageModification-Item

NeighbourFutureSSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

neighbourfutureSSBCoverageState			NeighbourFutureSSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { NeighbourFutureSSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourFutureSSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourFutureSSBCoverageState ::= INTEGER (0..15, ...)

NodeAssociatedInfoResult ::= SEQUENCE {

energyCost				EnergyCost			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { NodeAssociatedInfoResult-ExtIEs} } OPTIONAL,

...

}

NodeAssociatedInfoResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- P

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

PEISubgroupingSupportIndication ::= ENUMERATED {true, ...}

ReportingPeriodicityValue ::= INTEGER (0..512, ...)

PerformanceDelayMonitoring ::= SEQUENCE {

performanceDelayMonitoringRequest			PerformanceDelayMonitoringRequest,

performanceDelayMonitoringPeriodicity		PerformanceDelayMonitoringPeriodicity	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { PerformanceDelayMonitoring-ExtIEs} } OPTIONAL,

...

}

PerformanceDelayMonitoring-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PerformanceDelayMonitoringPeriodicity ::= ENUMERATED	{

ms500,

ms1000,

ms2000,

ms5000,

ms10000,

...

}

PerformanceDelayMonitoringRequest ::=ENUMERATED	{ul-and-dl, stop, ...}

Periodicity ::= INTEGER (0..640000, ...)

PeriodicitySRS ::= ENUMERATED { ms0p125, ms0p25, ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms4, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ...}

Predicted-CCO-Assistance-Information ::= SEQUENCE {

predicted-CCO-issue-detection	Predicted-CCO-issue-detection	OPTIONAL,

predictedAffectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

timeforPredictedCCOIssue		TimeforPredictedCCOIssue	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Predicted-CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

Predicted-CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Predicted-CCO-issue-detection	::=	ENUMERATED {coverage, cell-edge-capacity, cancel,...}

<<<<<<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>>>>>>>>

#### -- T

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

TimeforFutureCoverageModification ::= INTEGER (1..60, ...)

TimeforPredictedCCOIssue ::= INTEGER (1..60, ...)

TimeforNeighbourFutureCoverageModification ::= INTEGER (1..60, ...)

### 9.4.7	Constant Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofSSBarea							INTEGER ::=64

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRAReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofSuccessfulHOReports				INTEGER ::= 64

maxnoofNR-UChannelIDs					INTEGER ::= 16

maxServedCellforSON						INTEGER ::= 256

maxNeighbourCellforSON					INTEGER ::= 32

maxAffectedCells						INTEGER ::= 32

maxnoofMRBs								INTEGER ::= 32

maxnoofMBSQoSFlows						INTEGER ::= 64

maxnoofMBSFSAs 							INTEGER ::= 256

maxnoofUEIDforPaging 					INTEGER ::= 4096

maxnoofCellsforMBS						INTEGER ::= 512

maxnoofTAIforMBS						INTEGER ::= 512

maxnoofMBSAreaSessionIDs				INTEGER ::= 256

maxnoofMBSServiceAreaInformation		INTEGER ::= 256

maxnoofIABCongInd						INTEGER ::= 1024

maxnoofNeighbourNodeCellsIAB			INTEGER ::= 1024

maxnoofRBsetsPerCell					INTEGER ::= 8

maxnoofRBsetsPerCell-1					INTEGER ::= 7

maxnoofMeasPDC							INTEGER ::= 16

maxnoARPs								INTEGER ::=	16

maxnoofULAoAs							INTEGER ::= 8

maxNoPathExtended						INTEGER ::= 8

maxnoTRPTEGs							INTEGER ::= 8

maxFreqLayers							INTEGER ::= 4

maxNumResourcesPerAngle					INTEGER ::= 24

maxnoAzimuthAngles						INTEGER ::= 3600

maxnoElevationAngles					INTEGER ::= 1801

maxnoofPRSTRPs							INTEGER ::= 256

maxnoofQoEInformation					INTEGER ::= 16

maxnoofUuRLCChannels					INTEGER ::= 32

maxnoofPC5RLCChannels					INTEGER ::= 512

maxnoofSMBRValues						INTEGER ::= 8

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

maxnoofUETypes							INTEGER ::= 8

maxnoofLTMCells							INTEGER	::= 8

maxnoofTAList							INTEGER	::= 8

maxnoofLTMgNB-DUs						INTEGER	::= 8

maxnoofUEsInQMCTransferControlMessage	INTEGER ::= 512

maxnoofUEsforRAReportIndications		INTEGER ::= 64

maxnoofSuccessfulPSCellChangeReports	INTEGER ::= 64

maxnoofPeriodicities					INTEGER ::= 8

maxnoofThresholdMBS-1					INTEGER ::= 7

maxMBSSessionsinSessionInfoList			INTEGER ::= 1024

maxnoofLBTFailureInformation			INTEGER ::= 64

maxnoofRSPPQoSFlows						INTEGER ::= 2048

maxnoVACell								INTEGER ::= 32

maxnoAggregatedSRS-Resources			INTEGER ::= 3

maxnoAggregatedPosSRSResourceSets		INTEGER ::= 3

maxnoAggregatedPosPRSResourceSets		INTEGER ::= 3

maxnoofTimeWindowSRS					INTEGER ::= 16

maxnoofTimeWindowMea					INTEGER ::= 16

maxnoPreconfiguredSRS					INTEGER ::= 16

maxnoHopsMinusOne						INTEGER ::= 5

maxnoAggCombinations					INTEGER ::= 2

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxNeighbourCellReport					INTEGER ::= 512

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-ProtocolIE-ID-46-not-to-be-used					ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container	ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-ProtocolIE-ID-138-not-to-be-used					ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-MeasGapSharingConfig								ProtocolIE-ID ::= 238

id-systemInformationAreaID							ProtocolIE-ID ::= 239

id-areaScope										ProtocolIE-ID ::= 240

id-RRCContainer-RRCSetupComplete					ProtocolIE-ID ::= 241

id-TraceActivation									ProtocolIE-ID ::= 242

id-TraceID											ProtocolIE-ID ::= 243

id-Neighbour-Cell-Information-List					ProtocolIE-ID ::= 244

id-ProtocolIE-ID-246-not-to-be-used					ProtocolIE-ID ::= 246

id-ProtocolIE-ID-247-not-to-be-used					ProtocolIE-ID ::= 247

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 248

id-DUCURadioInformationType							ProtocolIE-ID ::= 249

id-CUDURadioInformationType 						ProtocolIE-ID ::= 250

id-AggressorgNBSetID								ProtocolIE-ID ::= 251

id-VictimgNBSetID									ProtocolIE-ID ::= 252

id-LowerLayerPresenceStatusChange					ProtocolIE-ID ::= 253

id-Transport-Layer-Address-Info						ProtocolIE-ID ::= 254

id-Neighbour-Cell-Information-Item					ProtocolIE-ID ::= 255

id-IntendedTDD-DL-ULConfig							ProtocolIE-ID ::= 256

id-QosMonitoringRequest								ProtocolIE-ID ::= 257

id-BHChannels-ToBeSetup-List						ProtocolIE-ID ::= 258

id-BHChannels-ToBeSetup-Item						ProtocolIE-ID ::= 259

id-BHChannels-Setup-List							ProtocolIE-ID ::= 260

id-BHChannels-Setup-Item							ProtocolIE-ID ::= 261

id-BHChannels-ToBeModified-Item						ProtocolIE-ID ::= 262

id-BHChannels-ToBeModified-List						ProtocolIE-ID ::= 263

id-BHChannels-ToBeReleased-Item						ProtocolIE-ID ::= 264

id-BHChannels-ToBeReleased-List						ProtocolIE-ID ::= 265

id-BHChannels-ToBeSetupMod-Item						ProtocolIE-ID ::= 266

id-BHChannels-ToBeSetupMod-List						ProtocolIE-ID ::= 267

id-BHChannels-FailedToBeModified-Item				ProtocolIE-ID ::= 268

id-BHChannels-FailedToBeModified-List				ProtocolIE-ID ::= 269

id-BHChannels-FailedToBeSetupMod-Item				ProtocolIE-ID ::= 270

id-BHChannels-FailedToBeSetupMod-List				ProtocolIE-ID ::= 271

id-BHChannels-Modified-Item							ProtocolIE-ID ::= 272

id-BHChannels-Modified-List							ProtocolIE-ID ::= 273

id-BHChannels-SetupMod-Item							ProtocolIE-ID ::= 274

id-BHChannels-SetupMod-List							ProtocolIE-ID ::= 275

id-BHChannels-Required-ToBeReleased-Item			ProtocolIE-ID ::= 276

id-BHChannels-Required-ToBeReleased-List			ProtocolIE-ID ::= 277

id-BHChannels-FailedToBeSetup-Item					ProtocolIE-ID ::= 278

id-BHChannels-FailedToBeSetup-List					ProtocolIE-ID ::= 279

id-BHInfo											ProtocolIE-ID ::= 280

id-BAPAddress										ProtocolIE-ID ::= 281

id-ConfiguredBAPAddress								ProtocolIE-ID ::= 282

id-BH-Routing-Information-Added-List				ProtocolIE-ID ::= 283

id-BH-Routing-Information-Added-List-Item			ProtocolIE-ID ::= 284

id-BH-Routing-Information-Removed-List				ProtocolIE-ID ::= 285

id-BH-Routing-Information-Removed-List-Item			ProtocolIE-ID ::= 286

id-UL-BH-Non-UP-Traffic-Mapping						ProtocolIE-ID ::= 287

id-Activated-Cells-to-be-Updated-List				ProtocolIE-ID ::= 288

id-Child-Nodes-List									ProtocolIE-ID ::= 289

id-IAB-Info-IAB-DU									ProtocolIE-ID ::= 290

id-IAB-Info-IAB-donor-CU							ProtocolIE-ID ::= 291

id-IAB-TNL-Addresses-To-Remove-List					ProtocolIE-ID ::= 292

id-IAB-TNL-Addresses-To-Remove-Item					ProtocolIE-ID ::= 293

id-IAB-Allocated-TNL-Address-List					ProtocolIE-ID ::= 294

id-IAB-Allocated-TNL-Address-Item					ProtocolIE-ID ::= 295

id-IABIPv6RequestType								ProtocolIE-ID ::= 296

id-IABv4AddressesRequested							ProtocolIE-ID ::= 297

id-IAB-Barred										ProtocolIE-ID ::= 298

id-TrafficMappingInformation						ProtocolIE-ID ::= 299

id-UL-UP-TNL-Information-to-Update-List				ProtocolIE-ID ::= 300

id-UL-UP-TNL-Information-to-Update-List-Item		ProtocolIE-ID ::= 301

id-UL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 302

id-UL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 303

id-DL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 304

id-DL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 305

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= 306

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= 307

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 308

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 309

id-SIB12-message									ProtocolIE-ID ::= 310

id-SIB13-message									ProtocolIE-ID ::= 311

id-SIB14-message									ProtocolIE-ID ::= 312

id-SLDRBs-FailedToBeModified-Item					ProtocolIE-ID ::= 313

id-SLDRBs-FailedToBeModified-List					ProtocolIE-ID ::= 314

id-SLDRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 315

id-SLDRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 316

id-SLDRBs-Modified-Item								ProtocolIE-ID ::= 317

id-SLDRBs-Modified-List								ProtocolIE-ID ::= 318

id-SLDRBs-Required-ToBeModified-Item				ProtocolIE-ID ::= 319

id-SLDRBs-Required-ToBeModified-List				ProtocolIE-ID ::= 320

id-SLDRBs-Required-ToBeReleased-Item				ProtocolIE-ID ::= 321

id-SLDRBs-Required-ToBeReleased-List				ProtocolIE-ID ::= 322

id-SLDRBs-Setup-Item								ProtocolIE-ID ::= 323

id-SLDRBs-Setup-List								ProtocolIE-ID ::= 324

id-SLDRBs-ToBeModified-Item							ProtocolIE-ID ::= 325

id-SLDRBs-ToBeModified-List							ProtocolIE-ID ::= 326

id-SLDRBs-ToBeReleased-Item							ProtocolIE-ID ::= 327

id-SLDRBs-ToBeReleased-List							ProtocolIE-ID ::= 328

id-SLDRBs-ToBeSetup-Item							ProtocolIE-ID ::= 329

id-SLDRBs-ToBeSetup-List							ProtocolIE-ID ::= 330

id-SLDRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 331

id-SLDRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 332

id-SLDRBs-SetupMod-List								ProtocolIE-ID ::= 333

id-SLDRBs-FailedToBeSetupMod-List					ProtocolIE-ID ::= 334

id-SLDRBs-SetupMod-Item								ProtocolIE-ID ::= 335

id-SLDRBs-FailedToBeSetupMod-Item					ProtocolIE-ID ::= 336

id-SLDRBs-ModifiedConf-List							ProtocolIE-ID ::= 337

id-SLDRBs-ModifiedConf-Item							ProtocolIE-ID ::= 338

id-UEAssistanceInformationEUTRA						ProtocolIE-ID ::= 339

id-PC5LinkAMBR										ProtocolIE-ID ::= 340

id-SL-PHY-MAC-RLC-Config							ProtocolIE-ID ::= 341

id-SL-ConfigDedicatedEUTRA-Info						ProtocolIE-ID ::= 342

id-AlternativeQoSParaSetList						ProtocolIE-ID ::= 343

id-CurrentQoSParaSetIndex							ProtocolIE-ID ::= 344

id-gNBCUMeasurementID								ProtocolIE-ID ::= 345

id-gNBDUMeasurementID								ProtocolIE-ID ::= 346

id-RegistrationRequest								ProtocolIE-ID ::= 347

id-ReportCharacteristics							ProtocolIE-ID ::= 348

id-CellToReportList									ProtocolIE-ID ::= 349

id-CellMeasurementResultList						ProtocolIE-ID ::= 350

id-HardwareLoadIndicator							ProtocolIE-ID ::= 351

id-ReportingPeriodicity								ProtocolIE-ID ::= 352

id-TNLCapacityIndicator								ProtocolIE-ID ::= 353

id-CarrierList										ProtocolIE-ID ::= 354

id-ULCarrierList									ProtocolIE-ID ::= 355

id-FrequencyShift7p5khz								ProtocolIE-ID ::= 356

id-SSB-PositionsInBurst								ProtocolIE-ID ::= 357

id-NRPRACHConfig									ProtocolIE-ID ::= 358

id-RAReportList										ProtocolIE-ID ::= 359

id-RLFReportInformationList							ProtocolIE-ID ::= 360

id-TDD-UL-DLConfigCommonNR							ProtocolIE-ID ::= 361

id-CNPacketDelayBudgetDownlink						ProtocolIE-ID ::= 362

id-ExtendedPacketDelayBudget						ProtocolIE-ID ::= 363

id-TSCTrafficCharacteristics						ProtocolIE-ID ::= 364

id-ReportingRequestType								ProtocolIE-ID ::= 365

id-TimeReferenceInformation							ProtocolIE-ID ::= 366

id-CNPacketDelayBudgetUplink						ProtocolIE-ID ::= 369

id-AdditionalPDCPDuplicationTNL-List				ProtocolIE-ID ::= 370

id-RLCDuplicationInformation						ProtocolIE-ID ::= 371

id-AdditionalDuplicationIndication					ProtocolIE-ID ::= 372

id-ConditionalInterDUMobilityInformation			ProtocolIE-ID ::= 373

id-ConditionalIntraDUMobilityInformation			ProtocolIE-ID ::= 374

id-targetCellsToCancel								ProtocolIE-ID ::= 375

id-requestedTargetCellGlobalID						ProtocolIE-ID ::= 376

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 377

id-TraceCollectionEntityIPAddress 					ProtocolIE-ID ::= 378

id-PrivacyIndicator									ProtocolIE-ID ::= 379

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 380

id-mdtConfiguration									ProtocolIE-ID ::= 381

id-ServingNID										ProtocolIE-ID ::= 382

id-NPNBroadcastInformation							ProtocolIE-ID ::= 383

id-NPNSupportInfo									ProtocolIE-ID ::= 384

id-NID												ProtocolIE-ID ::= 385

id-AvailableSNPN-ID-List							ProtocolIE-ID ::= 386

id-SIB10-message									ProtocolIE-ID ::= 387

id-DLCarrierList									ProtocolIE-ID ::= 389

id-ExtendedTAISliceSupportList					ProtocolIE-ID ::= 390

id-RequestedSRSTransmissionCharacteristics			ProtocolIE-ID ::= 391

id-PosAssistance-Information						ProtocolIE-ID ::= 392

id-PosBroadcast										ProtocolIE-ID ::= 393

id-RoutingID										ProtocolIE-ID ::= 394

id-PosAssistanceInformationFailureList				ProtocolIE-ID ::= 395

id-PosMeasurementQuantities							ProtocolIE-ID ::= 396

id-PosMeasurementResultList							ProtocolIE-ID ::= 397

id-TRPInformationTypeListTRPReq						ProtocolIE-ID ::= 398

id-TRPInformationTypeItem							ProtocolIE-ID ::= 399

id-TRPInformationListTRPResp						ProtocolIE-ID ::= 400

id-TRPInformationItem								ProtocolIE-ID ::= 401

id-LMF-MeasurementID								ProtocolIE-ID ::= 402

id-SRSType											ProtocolIE-ID ::= 403

id-ActivationTime									ProtocolIE-ID ::= 404

id-AbortTransmission								ProtocolIE-ID ::= 405

id-PositioningBroadcastCells						ProtocolIE-ID ::= 406

id-SRSConfiguration									ProtocolIE-ID ::= 407

id-PosReportCharacteristics							ProtocolIE-ID ::= 408

id-PosMeasurementPeriodicity						ProtocolIE-ID ::= 409

id-TRPList											ProtocolIE-ID ::= 410

id-RAN-MeasurementID								ProtocolIE-ID ::= 411

id-LMF-UE-MeasurementID								ProtocolIE-ID ::= 412

id-RAN-UE-MeasurementID								ProtocolIE-ID ::= 413

id-E-CID-MeasurementQuantities						ProtocolIE-ID ::= 414

id-E-CID-MeasurementQuantities-Item					ProtocolIE-ID ::= 415

id-E-CID-MeasurementPeriodicity						ProtocolIE-ID ::= 416

id-E-CID-MeasurementResult							ProtocolIE-ID ::= 417

id-Cell-Portion-ID									ProtocolIE-ID ::= 418

id-SFNInitialisationTime							ProtocolIE-ID ::= 419

id-SystemFrameNumber								ProtocolIE-ID ::= 420

id-SlotNumber										ProtocolIE-ID ::= 421

id-TRP-MeasurementRequestList						ProtocolIE-ID ::= 422

id-MeasurementBeamInfoRequest						ProtocolIE-ID ::= 423

id-E-CID-ReportCharacteristics						ProtocolIE-ID ::= 424

id-ConfiguredTACIndication							ProtocolIE-ID ::= 425

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 426

id-Extended-GNB-DU-Name								ProtocolIE-ID ::= 427

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-PDCPTerminatingNodeDLTNLAddrInfo					ProtocolIE-ID ::= 436

id-ENBDLTNLAddress									ProtocolIE-ID ::= 437

id-PosMeasurementPeriodicityExtended				ProtocolIE-ID ::= 438

id-PRS-Resource-ID									ProtocolIE-ID ::= 439

id-LocationMeasurementInformation					ProtocolIE-ID ::= 440

id-SliceRadioResourceStatus							ProtocolIE-ID ::= 441

id-CompositeAvailableCapacity-SUL					ProtocolIE-ID ::= 442

id-SuccessfulHOReportInformationList				ProtocolIE-ID ::= 443

id-NR-U-Channel-List								ProtocolIE-ID ::= 444

id-NR-U												ProtocolIE-ID ::= 445

id-Coverage-Modification-Notification				ProtocolIE-ID ::= 446

id-CCO-Assistance-Information						ProtocolIE-ID ::= 447

id-ProtocolIE-ID-448-not-to-be-used				ProtocolIE-ID ::= 448

id-CellsForSON-List									ProtocolIE-ID ::= 449

id-MIMOPRBusageInformation							ProtocolIE-ID ::= 450

id-gNB-CU-MBS-F1AP-ID								ProtocolIE-ID ::= 451

id-gNB-DU-MBS-F1AP-ID								ProtocolIE-ID ::= 452

id-ProtocolIE-ID-453-not-to-be-used				ProtocolIE-ID ::= 453

id-MBS-CUtoDURRCInformation							ProtocolIE-ID ::= 454

id-MBS-Session-ID									ProtocolIE-ID ::= 455

id-SNSSAI											ProtocolIE-ID ::= 456

id-MBS-Broadcast-NeighbourCellList					ProtocolIE-ID ::= 457

id-BroadcastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 458

id-BroadcastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 459

id-BroadcastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 460

id-BroadcastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 461

id-BroadcastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 462

id-BroadcastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 463

id-BroadcastMRBs-Modified-List						ProtocolIE-ID ::= 464

id-BroadcastMRBs-Modified-Item						ProtocolIE-ID ::= 465

id-BroadcastMRBs-Setup-List							ProtocolIE-ID ::= 466

id-BroadcastMRBs-Setup-Item							ProtocolIE-ID ::= 467

id-BroadcastMRBs-SetupMod-List						ProtocolIE-ID ::= 468

id-BroadcastMRBs-SetupMod-Item						ProtocolIE-ID ::= 469

id-BroadcastMRBs-ToBeModified-List					ProtocolIE-ID ::= 470

id-BroadcastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 471

id-BroadcastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 472

id-BroadcastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 473

id-BroadcastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 474

id-BroadcastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 475

id-BroadcastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 476

id-BroadcastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 477

id-Supported-MBS-FSA-ID-List						ProtocolIE-ID ::= 478

id-UEIdentity-List-For-Paging-List 					ProtocolIE-ID ::= 479

id-UEIdentity-List-For-Paging-Item 					ProtocolIE-ID ::= 480

id-MBS-ServiceArea									ProtocolIE-ID ::= 481

id-MulticastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 482

id-MulticastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 483

id-MulticastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 484

id-MulticastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 485

id-MulticastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 486

id-MulticastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 487

id-MulticastMRBs-Modified-List						ProtocolIE-ID ::= 488

id-MulticastMRBs-Modified-Item						ProtocolIE-ID ::= 489

id-MulticastMRBs-Setup-List							ProtocolIE-ID ::= 490

id-MulticastMRBs-Setup-Item							ProtocolIE-ID ::= 491

id-MulticastMRBs-SetupMod-List						ProtocolIE-ID ::= 492

id-MulticastMRBs-SetupMod-Item						ProtocolIE-ID ::= 493

id-MulticastMRBs-ToBeModified-List					ProtocolIE-ID ::= 494

id-MulticastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 495

id-MulticastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 496

id-MulticastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 497

id-MulticastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 498

id-MulticastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 499

id-MulticastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 500

id-MulticastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 501

id-MBSMulticastF1UContextDescriptor					ProtocolIE-ID ::= 502

id-MulticastF1UContext-ToBeSetup-List				ProtocolIE-ID ::= 503

id-MulticastF1UContext-ToBeSetup-Item				ProtocolIE-ID ::= 504

id-MulticastF1UContext-Setup-List					ProtocolIE-ID ::= 505

id-MulticastF1UContext-Setup-Item					ProtocolIE-ID ::= 506

id-MulticastF1UContext-FailedToBeSetup-List			ProtocolIE-ID ::= 507

id-MulticastF1UContext-FailedToBeSetup-Item			ProtocolIE-ID ::= 508

id-IABCongestionIndication							ProtocolIE-ID ::= 509

id-IABConditionalRRCMessageDeliveryIndication		ProtocolIE-ID ::= 510

id-F1CTransferPathNRDC								ProtocolIE-ID ::= 511

id-BufferSizeThresh 								ProtocolIE-ID ::= 512

id-IAB-TNL-Addresses-Exception						ProtocolIE-ID ::= 513

id-BAP-Header-Rewriting-Added-List					ProtocolIE-ID ::= 514

id-BAP-Header-Rewriting-Added-List-Item				ProtocolIE-ID ::= 515

id-Re-routingEnableIndicator						ProtocolIE-ID ::= 516

id-NonF1terminatingTopologyIndicator				ProtocolIE-ID ::= 517

id-EgressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 518

id-IngressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 519

id-rBSetConfiguration 								ProtocolIE-ID ::= 520

id-frequency-Domain-HSNA-Configuration-List			ProtocolIE-ID ::= 521

id-child-IAB-Nodes-NA-Resource-List					ProtocolIE-ID ::= 522

id-Parent-IAB-Nodes-NA-Resource-Configuration-List	ProtocolIE-ID ::= 523

id-uL-FreqInfo										ProtocolIE-ID ::= 524

id-uL-Transmission-Bandwidth						ProtocolIE-ID ::= 525

id-dL-FreqInfo										ProtocolIE-ID ::= 526

id-dL-Transmission-Bandwidth						ProtocolIE-ID ::= 527

id-uL-NR-Carrier-List								ProtocolIE-ID ::= 528

id-dL-NR-Carrier-List								ProtocolIE-ID ::= 529

id-nRFreqInfo										ProtocolIE-ID ::= 530

id-transmission-Bandwidth							ProtocolIE-ID ::= 531

id-nR-Carrier-List									ProtocolIE-ID ::= 532

id-Neighbour-Node-Cells-List						ProtocolIE-ID ::= 533

id-Serving-Cells-List								ProtocolIE-ID ::= 534

id-permutation										ProtocolIE-ID ::= 535

id-MDTPollutedMeasurementIndicator					ProtocolIE-ID ::= 536

id-M5ReportAmount 									ProtocolIE-ID ::= 537

id-M6ReportAmount 									ProtocolIE-ID ::= 538

id-M7ReportAmount 									ProtocolIE-ID ::= 539

id-SurvivalTime										ProtocolIE-ID ::= 540

id-PDCMeasurementPeriodicity						ProtocolIE-ID ::= 541

id-PDCMeasurementQuantities							ProtocolIE-ID ::= 542

id-PDCMeasurementQuantities-Item					ProtocolIE-ID ::= 543

id-PDCMeasurementResult								ProtocolIE-ID ::= 544

id-PDCReportType									ProtocolIE-ID ::= 545

id-RAN-UE-PDC-MeasID								ProtocolIE-ID ::= 546

id-SCGActivationRequest								ProtocolIE-ID ::= 547

id-SCGActivationStatus								ProtocolIE-ID ::= 548

id-PRSTRPList										ProtocolIE-ID ::= 549

id-PRSTransmissionTRPList							ProtocolIE-ID ::= 550

id-OnDemandPRS										ProtocolIE-ID ::= 551

id-AoA-SearchWindow									ProtocolIE-ID ::= 552

id-TRP-MeasurementUpdateList						ProtocolIE-ID ::= 553

id-ZoAInformation									ProtocolIE-ID ::= 554

id-ResponseTime										ProtocolIE-ID ::= 555

id-ARPLocationInfo									ProtocolIE-ID ::= 556

id-ARP-ID											ProtocolIE-ID ::= 557

id-MultipleULAoA									ProtocolIE-ID ::= 558

id-UL-SRS-RSRPP										ProtocolIE-ID ::= 559

id-SRSResourcetype									ProtocolIE-ID ::= 560

id-ExtendedAdditionalPathList						ProtocolIE-ID ::= 561

id-LoS-NLoSInformation								ProtocolIE-ID ::= 562

id-NumberOfTRPRxTEG									ProtocolIE-ID ::= 564

id-NumberOfTRPRxTxTEG								ProtocolIE-ID ::= 565

id-TRPTxTEGAssociation								ProtocolIE-ID ::= 566

id-TRPTEGInformation								ProtocolIE-ID ::= 567

id-TRPRx-TEGInformation								ProtocolIE-ID ::= 568

id-TRP-PRS-Info-List								ProtocolIE-ID ::= 569

id-PRS-Measurement-Info-List						ProtocolIE-ID ::= 570

id-PRSConfigRequestType								ProtocolIE-ID ::= 571

id-MeasurementTimeOccasion							ProtocolIE-ID ::= 573

id-MeasurementCharacteristicsRequestIndicator		ProtocolIE-ID ::= 574

id-UEReportingInformation							ProtocolIE-ID ::= 575

id-PosContextRevIndication							ProtocolIE-ID ::= 576

id-TRPBeamAntennaInformation						ProtocolIE-ID ::= 577

id-NRRedCapUEIndication 							ProtocolIE-ID ::= 578

id-Redcap-Bcast-Information							ProtocolIE-ID ::= 579

id-RANUEPagingDRX									ProtocolIE-ID ::= 580

id-CNUEPagingDRX									ProtocolIE-ID ::= 581

id-NRPagingeDRXInformation							ProtocolIE-ID ::= 582

id-NRPagingeDRXInformationforRRCINACTIVE			ProtocolIE-ID ::= 583

id-NR-TADV											ProtocolIE-ID ::= 584

id-QoEInformation									ProtocolIE-ID ::= 585

id-CG-SDTQueryIndication							ProtocolIE-ID ::= 586

id-SDT-MAC-PHY-CG-Config							ProtocolIE-ID ::= 587

id-CG-SDTKeptIndicator								ProtocolIE-ID ::= 588

id-CG-SDTindicatorSetup								ProtocolIE-ID ::= 589

id-CG-SDTindicatorMod								ProtocolIE-ID ::= 590

id-CG-SDTSessionInfoOld								ProtocolIE-ID ::= 591

id-SDTInformation									ProtocolIE-ID ::= 592

id-SDTRLCBearerConfiguration						ProtocolIE-ID ::= 593

id-FiveG-ProSeAuthorized							ProtocolIE-ID ::= 594

id-FiveG-ProSeUEPC5AggregateMaximumBitrate			ProtocolIE-ID ::= 595

id-FiveG-ProSePC5LinkAMBR	 						ProtocolIE-ID ::= 596

id-SRBMappingInfo									ProtocolIE-ID ::= 597

id-DRBMappingInfo									ProtocolIE-ID ::= 598

id-UuRLCChannelToBeSetupList						ProtocolIE-ID ::= 599

id-UuRLCChannelToBeModifiedList						ProtocolIE-ID ::= 600

id-UuRLCChannelToBeReleasedList						ProtocolIE-ID ::= 601

id-UuRLCChannelSetupList							ProtocolIE-ID ::= 602

id-UuRLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 603

id-UuRLCChannelModifiedList							ProtocolIE-ID ::= 604

id-UuRLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 605

id-UuRLCChannelRequiredToBeModifiedList				ProtocolIE-ID ::= 606

id-UuRLCChannelRequiredToBeReleasedList				ProtocolIE-ID ::= 607

id-PC5RLCChannelToBeSetupList						ProtocolIE-ID ::= 608

id-PC5RLCChannelToBeModifiedList					ProtocolIE-ID ::= 609

id-PC5RLCChannelToBeReleasedList					ProtocolIE-ID ::= 610

id-PC5RLCChannelSetupList							ProtocolIE-ID ::= 611

id-PC5RLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 612

id-PC5RLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 613

id-PC5RLCChannelRequiredToBeModifiedList			ProtocolIE-ID ::= 614

id-PC5RLCChannelRequiredToBeReleasedList			ProtocolIE-ID ::= 615

id-PC5RLCChannelModifiedList						ProtocolIE-ID ::= 616

id-SidelinkRelayConfiguration						ProtocolIE-ID ::= 617

id-UpdatedRemoteUELocalID							ProtocolIE-ID ::= 618

id-PathSwitchConfiguration							ProtocolIE-ID ::= 619

id-PagingCause										ProtocolIE-ID ::= 620

id-MUSIM-GapConfig									ProtocolIE-ID ::= 621

id-PEIPSAssistanceInfo								ProtocolIE-ID ::= 622

id-UEPagingCapability								ProtocolIE-ID ::= 623

id-LastUsedCellIndication							ProtocolIE-ID ::= 624

id-SIB17-message									ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-SIB20-message									ProtocolIE-ID ::= 627

id-UE-MulticastMRBs-ToBeReleased-List				ProtocolIE-ID ::= 628

id-UE-MulticastMRBs-ToBeReleased-Item				ProtocolIE-ID ::= 629

id-UE-MulticastMRBs-ToBeSetup-List					ProtocolIE-ID ::= 630

id-UE-MulticastMRBs-ToBeSetup-Item					ProtocolIE-ID ::= 631

id-MulticastMBSSessionSetupList						ProtocolIE-ID ::= 632

id-MulticastMBSSessionRemoveList					ProtocolIE-ID ::= 633

id-PosMeasurementAmount								ProtocolIE-ID ::= 634

id-SDT-Termination-Request							ProtocolIE-ID ::= 635

id-pathPower										ProtocolIE-ID ::= 636

id-DU-RX-MT-RX-Extend								ProtocolIE-ID ::= 637

id-DU-TX-MT-TX-Extend								ProtocolIE-ID ::= 638

id-DU-RX-MT-TX-Extend								ProtocolIE-ID ::= 639

id-DU-TX-MT-RX-Extend								ProtocolIE-ID ::= 640

id-BAP-Header-Rewriting-Removed-List				ProtocolIE-ID ::= 641

id-BAP-Header-Rewriting-Removed-List-Item			ProtocolIE-ID ::= 642

id-SLDRXCycleList									ProtocolIE-ID ::= 643

id-TAINSAGSupportList								ProtocolIE-ID ::= 644

id-SL-RLC-ChannelToAddModList						ProtocolIE-ID ::= 645

id-BroadcastAreaScope								ProtocolIE-ID ::= 646

id-ManagementBasedMDTPLMNModificationList 			ProtocolIE-ID ::= 647

id-SIB15-message									ProtocolIE-ID ::= 648

id-ActivationRequestType							ProtocolIE-ID ::= 649

id-PosMeasGapPreConfigList							ProtocolIE-ID ::= 650

id-InterFrequencyConfig-NoGap						ProtocolIE-ID ::= 651

id-MBSInterestIndication							ProtocolIE-ID ::= 652

id-UE-MulticastMRBs-ConfirmedToBeModified-List		ProtocolIE-ID ::= 653

id-UE-MulticastMRBs-ConfirmedToBeModified-Item		ProtocolIE-ID ::= 654

id-UE-MulticastMRBs-RequiredToBeModified-List		ProtocolIE-ID ::= 655

id-UE-MulticastMRBs-RequiredToBeModified-Item		ProtocolIE-ID ::= 656

id-UE-MulticastMRBs-RequiredToBeReleased-List		ProtocolIE-ID ::= 657

id-UE-MulticastMRBs-RequiredToBeReleased-Item		ProtocolIE-ID ::= 658

id-L571Info											ProtocolIE-ID ::= 659

id-L1151Info										ProtocolIE-ID ::= 660

id-SCS-480											ProtocolIE-ID ::= 661

id-SCS-960											ProtocolIE-ID ::= 662

id-SRSPortIndex										ProtocolIE-ID ::= 663

id-PEISubgroupingSupportIndication					ProtocolIE-ID ::= 664

id-NeedForGapsInfoNR								ProtocolIE-ID ::= 665

id-NeedForGapNCSGInfoNR								ProtocolIE-ID ::= 666

id-NeedForGapNCSGInfoEUTRA							ProtocolIE-ID ::= 667

id-ProtocolIE-ID-668-not-to-be-used					ProtocolIE-ID ::= 668

id-ProtocolIE-ID-669-not-to-be-used					ProtocolIE-ID ::= 669

id-ProtocolIE-ID-670-not-to-be-used					ProtocolIE-ID ::= 670

id-Source-MRB-ID									ProtocolIE-ID ::= 671

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-RedCapIndication									ProtocolIE-ID ::= 673

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList									ProtocolIE-ID ::= 707

id-DedicatedSIDeliveryIndication					ProtocolIE-ID ::= 708

id-Configured-BWP-List								ProtocolIE-ID ::= 709

id-Preconfigured-measurement-GAP-Request			ProtocolIE-ID ::= 710

id-BWP-Id											ProtocolIE-ID ::= 711

id-NetworkControlledRepeaterAuthorized				ProtocolIE-ID ::= 712

id-MT-SDT-Information								ProtocolIE-ID ::= 713

id-ExtendedResourceSymbolOffset						ProtocolIE-ID ::= 714

id-NeedForInterruptionInfoNR						ProtocolIE-ID ::= 715

id-SDT-Volume-Threshold								ProtocolIE-ID ::= 716

id-SupportedUETypeList								ProtocolIE-ID ::= 717

id-MusimCapabilityRestrictionIndication				ProtocolIE-ID ::= 718

id-duplicationIndication							ProtocolIE-ID ::= 719

id-LTMInformation-Setup								ProtocolIE-ID ::= 720

id-LTMConfigurationIDMappingList 					ProtocolIE-ID ::= 721

id-LTMInformation-Modify							ProtocolIE-ID ::= 722

id-LTMCells-ToBeReleased-List						ProtocolIE-ID ::= 723

id-ProtocolIE-ID-724-not-to-be-used						ProtocolIE-ID ::= 724

id-LTMConfiguration									ProtocolIE-ID ::= 725

id-EarlySyncInformation-Request						ProtocolIE-ID ::= 726

id-EarlySyncInformation								ProtocolIE-ID ::= 727

id-EarlySyncCandidateCellInformation-List			ProtocolIE-ID ::= 728

id-LTMCellSwitchInformation							ProtocolIE-ID ::= 729

id-DUtoCUTAInformation-List							ProtocolIE-ID ::= 730

id-ProtocolIE-ID-731-not-to-be-used					ProtocolIE-ID ::= 731

id-dRB-List											ProtocolIE-ID ::= 732

id-DeactivationIndication							ProtocolIE-ID ::= 733

id-RAReportIndicationList							ProtocolIE-ID ::= 734

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= 735

id-SuccessfulPSCellChangeReportInformationList		ProtocolIE-ID ::= 736

id-RadioResourceStatusNR-U							ProtocolIE-ID ::= 737

id-FiveG-ProSeLayer2Multipath						ProtocolIE-ID ::= 738

id-FiveG-ProSeLayer2UEtoUERelay						ProtocolIE-ID ::= 739

id-FiveG-ProSeLayer2UEtoUERemote					ProtocolIE-ID ::= 740

id-PathAdditionInformation							ProtocolIE-ID ::= 741

id-Recommended-SSBs-List							ProtocolIE-ID ::= 742

id-Recommended-SSBs-for-Paging-List					ProtocolIE-ID ::= 743

id-SSBs-withinTheCell-tobe-Activated-List			ProtocolIE-ID ::= 744

id-Cells-With-SSBs-Activated-List					ProtocolIE-ID ::= 745

id-Cells-Allowed-to-be-Deactivated-List				ProtocolIE-ID ::= 746

id-Cells-Allowed-to-be-Deactivated-List-Item		ProtocolIE-ID ::= 747

id-Coverage-Modification-Cause						ProtocolIE-ID ::= 748

id-RANTSSRequestType								ProtocolIE-ID ::= 749

id-RANTimingSynchronisationStatusInfo				ProtocolIE-ID ::= 750

id-TSCTrafficCharacteristicsFeedback				ProtocolIE-ID ::= 751

id-RANfeedbacktype									ProtocolIE-ID ::= 752

id-Mobile-TRP-LocationInformation					ProtocolIE-ID ::= 753

id-Mobile-IAB-MT-UE-ID								ProtocolIE-ID ::= 754

id-Target-gNB-ID									ProtocolIE-ID ::= 755

id-Target-gNB-IP-address							ProtocolIE-ID ::= 756

id-Target-SeGW-IP-address							ProtocolIE-ID ::= 757

id-Activated-Cells-Mapping-List						ProtocolIE-ID ::= 758

id-Activated-Cells-Mapping-List-Item				ProtocolIE-ID ::= 759

id-F1SetupOutcome									ProtocolIE-ID ::= 760

id-RRC-Terminating-IAB-Donor-Related-Info			ProtocolIE-ID ::= 761

id-RRC-Terminating-IAB-Donor-gNB-ID					ProtocolIE-ID ::= 762

id-NCGI-to-be-Updated-List							ProtocolIE-ID ::= 763

id-NCGI-to-be-Updated-List-Item						ProtocolIE-ID ::= 764

id-Mobile-IAB-MTUserLocationInformation				ProtocolIE-ID ::= 765

id-MobileAccessPointLocation						ProtocolIE-ID ::= 766

id-AssociatedSessionID								ProtocolIE-ID ::= 767

id-IndicationMCInactiveReception					ProtocolIE-ID ::= 768

id-MulticastCU2DURRCInfo							ProtocolIE-ID ::= 769

id-MBSMulticastSessionReceptionState				ProtocolIE-ID ::= 770

id-F1UTunnelNotEstablished							ProtocolIE-ID ::= 771

id-MulticastDU2CURRCInfo							ProtocolIE-ID ::= 772

id-SIB24-message										ProtocolIE-ID ::= 773

id-MulticastCU2DUCommonRRCInfo						ProtocolIE-ID ::= 774

id-PDUSetQoSParameters								ProtocolIE-ID ::= 775

id-N6JitterInformation								ProtocolIE-ID ::= 776

id-ECNMarkingorCongestionInformationReportingRequest	ProtocolIE-ID ::= 777

id-ECNMarkingorCongestionInformationReportingStatus	ProtocolIE-ID ::= 778

id-NRA2XServicesAuthorized							ProtocolIE-ID ::= 779

id-LTEA2XServicesAuthorized							ProtocolIE-ID ::= 780

id-NRUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 781

id-LTEUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 782

id-NReRedCapUEIndication 							ProtocolIE-ID ::= 783

id-ERedcap-Bcast-Information 						ProtocolIE-ID ::= 784

id-NRPaginglongeDRXInformationforRRCINACTIVE		ProtocolIE-ID ::= 785

id-SCPAC-Request									ProtocolIE-ID ::= 786

id-Target-F1-Terminating-Donor-gNB-ID				ProtocolIE-ID ::= 787

id-MobileIAB-Barred									ProtocolIE-ID ::= 788

id-Broadcast-MRBs-Transport-Request-List 			ProtocolIE-ID ::= 789

id-Broadcast-MRBs-Transport-Request-Item			ProtocolIE-ID ::= 790

id-S-CPACLowerLayerReferenceConfigRequest			ProtocolIE-ID ::= 791

id-S-CPAC-Configuration								ProtocolIE-ID ::= 792

id-MusimCandidateBandList							ProtocolIE-ID ::= 793

id-DLLBTFailureInformationRequest					ProtocolIE-ID ::= 794

id-DLLBTFailureInformationList						ProtocolIE-ID ::= 795

id-PSIbasedSDUdiscardUL								ProtocolIE-ID ::= 796

id-SIB22-message 									ProtocolIE-ID ::= 797

id-CUtoDUTAInformation-List							ProtocolIE-ID ::= 798

id-U2URLCChannelQoS									ProtocolIE-ID ::= 799

id-SL-PHY-MAC-RLC-ConfigExt							ProtocolIE-ID ::= 800

id-SLPositioning-Ranging-Service-Info				ProtocolIE-ID ::= 801

id-TimeWindowInformation-SRS-List					ProtocolIE-ID ::= 802

id-TimeWindowInformation-Measurement-List			ProtocolIE-ID ::= 803

id-UL-RSCP											ProtocolIE-ID ::= 804

id-BW-Aggregation-Request-Indication				ProtocolIE-ID ::= 805

id-ReportingGranularitykminus1						ProtocolIE-ID ::= 806

id-ReportingGranularitykminus2						ProtocolIE-ID ::= 807

id-ReportingGranularitykminus1additionalpath		ProtocolIE-ID ::= 808

id-ReportingGranularitykminus2additionalpath		ProtocolIE-ID ::= 809

id-TimingReportingGranularityFactorExtended			ProtocolIE-ID ::= 810

id-SRSPosRRCInactiveValidityAreaConfig				ProtocolIE-ID ::= 811

id-PosValidityAreaCellList							ProtocolIE-ID ::= 812

id-SRSReservationType								ProtocolIE-ID ::= 813

id-SymbolIndex										ProtocolIE-ID ::= 814

id-PRSBandwidthAggregationRequestIndication			ProtocolIE-ID ::= 815

id-AggregatedPosSRSResourceIDList					ProtocolIE-ID ::= 816

id-AggregatedPRSResourceSetList						ProtocolIE-ID ::= 817

id-PhaseQuality										ProtocolIE-ID ::= 818

id-MeasuredFrequencyHops							ProtocolIE-ID ::= 819

id-TxHoppingConfiguration							ProtocolIE-ID ::= 820

id-ReportingGranularitykminus3						ProtocolIE-ID ::= 821

id-ReportingGranularitykminus4						ProtocolIE-ID ::= 822

id-ReportingGranularitykminus5						ProtocolIE-ID ::= 823

id-ReportingGranularitykminus6						ProtocolIE-ID ::= 824

id-ReportingGranularitykminus3additionalpath		ProtocolIE-ID ::= 825

id-ReportingGranularitykminus4additionalpath		ProtocolIE-ID ::= 826

id-ReportingGranularitykminus5additionalpath		ProtocolIE-ID ::= 827

id-ReportingGranularitykminus6additionalpath		ProtocolIE-ID ::= 828

id-AggregatedPosSRSResourceSetList					ProtocolIE-ID ::= 829

id-RequestedSRSPreconfigurationCharacteristics-List	ProtocolIE-ID ::= 830

id-SRSPreconfiguration-List							ProtocolIE-ID ::= 831

id-SRSInformation									ProtocolIE-ID ::= 832

id-ValidityAreaSpecificSRSInformation				ProtocolIE-ID ::= 833

id-E-CID-MeasuredResultsAssociatedInfoList			ProtocolIE-ID ::= 834

id-XR-Bcast-Information								ProtocolIE-ID ::= 835

id-MaxDataBurstVolume 								ProtocolIE-ID ::= 836

id-TAInformation-List								ProtocolIE-ID ::= 837

id-NonIntegerDRXCycle								ProtocolIE-ID ::= 838

id-PointA											ProtocolIE-ID ::= 839

id-SCS-SpecificCarrier								ProtocolIE-ID ::= 840

id-NR-PCI											ProtocolIE-ID ::= 841

id-PeerUE-ID										ProtocolIE-ID ::= 842

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-Future-Coverage-Modification-Notification		ProtocolIE-ID ::= xxx

id-Predicted-CCO-Assistance-Information				ProtocolIE-ID ::= yyy

id-PerformanceDelayMonitoring						ProtocolIE-ID ::= yy2

id-NodeAssociatedInfoResult							ProtocolIE-ID ::= zzz

id-NeighbourFutureCoverageModNotification			ProtocolIE-ID ::= yy1

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>