3GPP TSG-RAN WG3 Meeting #130	R3-258119

Dallas, US, 17th ~21th November 2025

Title:	Further consideration on RAN-CN interface principles and functions

Agenda Item:	10.3.1

Source:	Huawei

Document for:	Agreement

# 1	Introduction

In this contribution, we further discuss the principles and functions for the 6G RAN-CN interface, and provides the related pCR to the TR 38.760-3.

# 2	Discussion

## 2.1	Principles

In Oct meeting, the following RAN-CN interface principles were agreed:

- the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN; - the 6G RAN-CN interface supports control plane and user plane separation; - the 6G RAN-CN interface supports future enhancements; - the 6G RAN-CN interface supports all possible RAN deployment scenarios; - the 6G RAN-CN interface supports RAN sharing between multiple operators; - the 6G RAN-CN interface supports the operation of network slicing; - the 6G RAN-CN interface supports enhanced service awareness in RAN; - the 6G RAN-CN control plane interface supports reliable signalling transmission; - the 6G RAN-CN interface is designed with a clear functional split between RAN and CN.

While the following ones were not:

-	the 6G RAN-CN interface separates Radio Network Layer and Transport Network Layer;

-	the 6G RAN-CN interface, from logical standpoint, is a point-to-point interface between an RAN node and a CN node. A point-to-point logical interface is feasible even in the absence of a physical direct connection between the RAN node and CN node;

In legacy RAN-CN interfaces, the separation of Radio Network Layer and Transport Network Layer is supported. For Service-based interface, HTTP/2 protocol with JSON is used as the application layer serialization protocol, TCP is used as the transport protocol, therefore it also supports separation of Radio Network Layer and Transport Network Layer.

Proposal 1: the 6G RAN-CN interface supports Radio Network Layer and Transport Network Layer separation.

In order to get the interim-study-result about “RAN-CN interface: P2P vs SBI”, maybe RAN3 can discuss the RAN-CN interface by separating logical standpoint and transport standpoint.

The legacy NG-C interface and other legacy interfaces within RAN, are point-to-point interfaces from both logical and transport standpoints.

Within 5GC, although the “service-based interface” is used within 5GC Control Plane network functions, but the interactions between the network functions are actually described by point-to-point reference points as follows:

| Non-Roaming 5G System Architecture (from TS 23.501) |  |
| --- | --- |
| Service-based interfaces are used within the Control Plane of 5GC | various network functions interact with each other using the reference point representation. |

Considering of the agreed principle “the 6G RAN-CN interface is designed with a clear functional split between RAN and CN”, even in service-based interface solution, it is not needed for the RAN node to directly communicate with lots of core network functions, there should be limited one(s). And to support the communication between the RAN node and a core network function, from logical standpoint, point-to-point interactions/references will still be used.

Proposal 2: the 6G RAN-CN interface, is point-to-point from logical standpoint.

## 2.2	RAN-CN interface Functions

In Oct meeting, the following RAN-CN interface functions were agreed

- UE context management : The functionality to manage UE context between the RAN and CN; - Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress; - PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress.

Besides these agreed functions, the following RAN-CN interface functions should also be captured:

-	Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of interface between two RAN nodes) between two RAN nodes via the CN;

-	Warning Message Transmission: The functionality to transfer warning messages via RAN-CN interface or cancel ongoing broadcast of warning messages;

-	Paging: The functionality to send paging requests to the RAN nodes, subject to SA2 progress;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN, subject to SA2 and RAN2 progress.

For interface management procedure, we can discuss it after the interim-study-result about “RAN-CN interface: P2P vs SBI”.

For 6G new services/features, from our view, it is straight forward to introduce new service management functions, but the details are subject to further progress of the discussion of these new services/features.

Proposal 3: RAN-CN interface also supports the above-mentioned functions.

# 3	Conclusion

In this contribution, we further discussed the principles and functions for RAN-CN interface, and get the following proposals:

Proposal 1: the 6G RAN-CN interface supports Radio Network Layer and Transport Network Layer separation.

Proposal 2: the 6G RAN-CN interface, is point-to-point from logical standpoint.

Proposal 3: RAN-CN interface also supports the following functions:

- Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of interface between two RAN nodes) between two RAN nodes via the CN;

- Warning Message Transmission: The functionality to transfer warning messages via RAN-CN interface or cancel ongoing broadcast of warning messages;

- Paging: The functionality to send paging requests to the RAN nodes, subject to SA2 progress;

- UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN, subject to SA2 and RAN2 progress;

- New service management: The functionality to manage the new service between the RAN and CN, Details are FFS.

The corresponding pCR is provided in section 4.

# 4	pCR to the TR 38.760-3 V0.1.0

--------------Start of the change---------------

## 6.1		RAN-CN Interface

### 6.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN-CN Interface.

The general principles for the specification of the 6G RAN-CN interface are as follows:

-	the 6G RAN-CN interface supports the exchange of signalling information between the RAN and CN;

-	the 6G RAN-CN interface supports control plane and user plane separation;

-	the 6G RAN-CN interface supports future enhancements;

-	the 6G RAN-CN interface supports all possible RAN deployment scenarios;

-	the 6G RAN-CN interface supports RAN sharing between multiple operators;

-	the 6G RAN-CN interface supports the operation of network slicing;

-	the 6G RAN-CN interface supports enhanced service awareness in RAN;

-	the 6G RAN-CN control plane interface supports reliable signalling transmission;

-	the 6G RAN-CN interface is designed with a clear functional split between RAN and CN;

-	the 6G RAN-CN interface supports Radio Network Layer and Transport Network Layer separation;

-	the 6G RAN-CN interface, is point-to-point from logical standpoint.

### 6.1.2	RAN-CN Interface Functions

RAN-CN control plane interface supports following functions:

-	UE context management	: The functionality to manage UE context between the RAN and CN;

-	Transport of NAS messages: The functionality to transfer NAS messages between the CN and UE, subject to SA2 progress;

-	PDU Session Management: The functionality to manage PDU sessions and respective RAN resources, subject to SA2 progress;

-	Configuration Transfer: The functionality to transfer the RAN configuration information (e.g., transport layer addresses for establishment of interface between two RAN nodes) between two RAN nodes via the CN;

-	Warning Message Transmission: The functionality to transfer warning messages via RAN-CN interface or cancel ongoing broadcast of warning messages;

-	Paging: The functionality to send paging requests to the RAN nodes, subject to SA2 progress;

-	UE mobility management: The functionality to manage the UE mobility for connected mode between the RAN and CN, subject to SA2 and RAN2 progress;

-	New service management: The functionality to manage the new service between the RAN and CN, Details are FFS.

--------------End of the change---------------