3GPP TSG-RAN WG3 Meeting #130	R3-258147

Dallas, US, November 17-21, 2025

Agenda Item:	13.2

Source:	ZTE Corporation

Title:	(TP to pCR 38.765) Further discussion on Network Architecture for ISAC

Document for:	Discussion and Approval

- Introduction

In last RAN3 meeting, the network architecture for ISAC in R20 has been discussed and some conclusion has been reached as below[1].

Capture logical architecture for ISAC RAN3 focus on sensing protocol in coordination with SA2 To continue the discussion on protocol stacks, particularly the protocol stack for sensing reporting, to be aligned with SA2 conclusions.

In this contribution, we provide our further consideration on network architecture for ISAC in R20.

- Discussion

- 2.1 Logical architecture

In last RAN3 meeting, the logical architecture for ISAC in R20 has been agreed as below[2].

Figure 7.1 Logical architecture for ISAC

In SA2#171 meeting, to support KI#1 System Architecture to Support Sensing, the following principles are concluded as below[3]:

Principle 1: one new Network Function (i.e., Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF). NOTE 1: There is no standardized interface between the SCF and SPF in Rel-20 5G-A. NOTE 2: How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g., co-located or separated, can be discussed during the normative phase. NOTE 3: The final name and Acronym for Sensing Function may need further update if necessary.

Although the SF can be split into the SCF and SPF defined by SA2, from RAN’s prospective, the internal architecture of SF is transparent to RAN3, and RAN3 shall keep the logical architecture as above.

Observation: The internal architecture of the SF defined by SA2 is transparent to RAN3, RAN3 shall keep the logical architecture in the existing pCR to TR 38.765.

- 2.2 Protocol stack design

In R20, the basic scenario of ISAC and the sensing mode can be given as below. The sensing function should be supported in the gNB and the UAV is the sensing target in R20.

Figure 1: Sensing mode for ISAC in R20

Regarding the design of protocol stack in RAN3, there are three candidate options on the table.

Option 1: New interface solution

Option 2: Positioning like solution

Option 3: Ambient-IoT like solution

For Option 1, the gNB shall connect directly with the Sensing Function, and a new interface between the gNB and the Sensing Function needs to be defined. In this case, the new protocol stack should be introduced accordingly which could apply the similar design like the existing NG interface or new SBI interface. And the corresponding protocol stack for this option is give as below.

Figure 2: Potential protocol stack for new interface solution

For Option 2, the gNB shall connect with the Sensing Function like the way we designed for the Positioning feature. In this case, the existing NG interface can be reused, and a new application protocol, similar as the NRPPa, should be defined. Furthermore, the communication between the gNB and the Sensing Function can be achieved by the new application protocol on the top of NGAP. The ISAC specific information should be transferred transparently through the container, similar as the NRPPa-PDU over NG interface. In this option, the protocol stack can be designed with the new application protocol (e.g., NRSPa) on top over the NGAP. And the corresponding protocol stack for this option is give as below.

Figure 3: Potential protocol stack for Positioning like solution

For Option 3, the gNB shall connect with the Sensing Function like the way we designed for the Ambient-IoT feature. In this case, both of the existing NG interface and NG-C protocol stack can be reused, and the ISAC specific information should also be transferred transparently via the AMF. In this option, the AMF acts as a transparent node for ISAC specific information, both direct and indirect connection between RAN and SF can be supported. And the  protocol stack with direct connection for this option is give as below.

Figure 4: Potential protocol stack for A-IoT like solution

Among the three options, Option 1 introduces the most impact on the existing CN since a new interface and the corresponding protocol stacks should be defined. As R20 is the last release in 5G, it is more appropriate to specify the solution based on current 5GA architecture and minimize the standard impact as much as possible for rapid commercial deployment.

For Option 2 and Option 3, both of them introduce less impact on the existing CN than Option 1 since the NG interface can be reused. While, for Option 2, a new application protocol like NRPPa should be defined, which may introduce more standardization work than Option 3. For Option 3, only the NGAP signalling enhancement is needed to support the ISAC, which has the least impact on the existing RAN architecture.

|  | Impact on CN | New interface to be introduced? | New protocol to be studied? |
| --- | --- | --- | --- |
| Option 1 | High | Yes | Yes |
| Option 2 | Medium | No☺ | Yes |
| Option 3 | Low | No☺ | No☺ |

Table 1: Comparison among the candidate options

Proposal : The above candidate options need to be studied and evaluated in RAN3 for 5GA sensing. Option3 has less standard impact compared with other two options.

3	Summary

Observation: The internal architecture of the SF defined by SA2 is transparent to RAN3, RAN3 shall keep the logical architecture in the existing pCR to TR 38.765.

Candidate options for protocol stack of ISAC:

Option 1: New interface solution

Option 2: Positioning like solution

Option 3: Ambient-IoT like solution

Proposal : The above candidate options need to be studied and evaluated in RAN3 for 5GA sensing. Option3 has less standard impact compared with other two options.

4	Reference

- [1] RAN3#129bis Chairman Notes

- [2] R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR, China Telecom, Xiaomi

- [3] TR 23.700-14 v1.1.0

Annex		Text Proposal to BL pCR to TR 38.765

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

This clause identifies and describes the logical architecture to support the sensing in the overall 5G system architecture.

Figure 7.1 depicts a logical architecture for ISAC, where the Nx interface is between the gNB and the SF.

Figure 7.1	Logical architecture for ISAC

This logical architecture is independent of the transport, e.g. direct or via the AMF, between the gNB and the SF.

Editor’s Note:  The details of "Nx" interface need further discussion and decision.

- x	Solutions for sensing

7.x.1	General

To support the sensing, the following solutions are to be studied for the design of protocol stack:

-	New interface solution. With this solution, the gNB connects directly with the Sensing Function, and a new interface between the gNB and the Sensing Function needs to be defined.

-	Positioning like solution. With this solution, the gNB connects with the Sensing Function following the way of the Positioning feature. The existing NG interface can be reused, and a new application protocol, similar as the NRPPa, should be defined.

-	Ambient-IoT like solution. With this solution, the gNB connects with the Sensing Function following the way of the Ambient-IoT feature. Both of the existing NG interface and NG-C protocol stack can be reused.

Editor’s Note:  Whether to down-select the options in the SI phase is FFS.

7.x.2	New interface solution

In this solution, a new interface between the gNB and the SF should be defined, and the corresponding new protocol stack should be also defined.

Nx in the new interface, the ISAC specific information is transported over the NxAP.

Editor’s Note:  The naming of the new interface and the new protocol stack are FFS.

Figure 7.x shows the Protocol stack for New interface solution:

Figure 7.x. Protocol Stack for New interface solution

7.x.3	Positioning like solution

In this solution, a new application protocol like NRPPa (e.g., NRSPa) should be defined.

NLx in the new interface between AMF and SF, NRSPa is the new application protocol, and the ISAC specific information is transported over the NRSPa on the top of NGAP.

The NRSPa protocol is transparent to the AMF.

Editor’s Note:  The naming of the new application protocol is FFS.

Figure 7.y shows the Protocol stack for Positioning like solution:

Figure 7.y. Protocol Stack for Positioning like solution

7.x.4	Ambient-IoT like solution

The ISAC specific information is transported over the NGAP between gNB and SF, while both direct connection and in-direct connection between gNB and SF can be supported.

Figure 7.z shows the Protocol stack for Ambient-IoT like solution:

Figure 7.z. Protocol Stack for Ambient-IoT like solution

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>