3GPP TSG-RAN WG3 Meeting #130	R3-258168

Dallas, US, 17 – 21 November 2025

Agenda item:	10.4

Source:	Nokia, Orange

Title:	Considerations on the 6G RAN internal functional split and possible work plan for RAN3

Document for:	Discussion

# 1	Introduction

At RAN#108, the new SID on 6G RAN was agreed in [1].

The main RAN3 led objective to study is listed below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

As part of point c), RAN3 was tasked with a first milestone to achieve:

4.1.1	Interim Milestone

Interim results shall be delivered as per the milestones below, in coordination with the RAN Plenary 6G Study [RP-250810].

TSG#112 (June/2026):

RAN3 to provide interim study results to allow TSGs to make a decision on:

- RAN-CN interface: P2P vs SBI

- RAN internal interfaces: CU-DU split, CP-UP split.

This paper addresses the objective (c) above and describes our view on how we can proceed with the above RAN3 task and also highlights the main challenges with 5G RAN internal split and interfaces.

# 2	Work plan proposal for HLS discussion

At RAN3#129bis, the first discussion on the Higher Layer Split (HLS) took place. It was clear that many acknowledge the benefits of a split deployment of the gNB in 5G. However, minimal known standard-based implementations of the HLS led to the discussion whether defining the HLS in 3GPP standards offers any gains as compared to the implementation-based deployments.

Proposal 1: RAN3 to acknowledge that HLS may be beneficial. Further work is to focus on the question if the HLS shall be standardised and, if yes, how to define it to avoid problems that handicapped standardised 5G HLS.

It has thus become evident there is a need to attain clear justification and openness to study lessons learnt as well as benefits and drawbacks of 5G HLS. Also, the following FFS point was captured in the chair’s notes:

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

In our opinion, it is a good way forward and RAN3, having different opinions on the 5G HLS, should first focus on the pain points and benefits of the existing solution. Based on that, RAN3 may derive the requirements that need to be fulfilled by the 6G HLS. To achieve that, we propose the following work plan for the CU-DU split.

- RAN3 to list the pain points and benefits of 5G HLS in RAN3#130 and RAN3#131.

- RAN3 to send an LS to RAN plenary in March 2026 capturing pain points and benefits of 5G HLS and ask for further guidance/decision on whether to standardise the HLS in 6G or rather to keep it as an implementation option.

- RAN plenary to provide guidance to RAN3 on whether/how to continue the study on standardised HLS (i.e. RAN plenary to agree on existing 5G HLS or no HLS or provide guidance to RAN3 to study different HLS variants).

- Based on RAN plenary guidance, RAN3 to further study different HLS design for 6G and provide interim results to RAN plenary, as originally planned, in June 2026, based on the agreed requirements.

Proposal 2: RAN3 to first agree on the work plan for HLS design, as mentioned above.

# 3	Pain Points and Benefits of 5G HLS

## 3.1	Motivation and benefits of 5G HLS

In 5G, the CU-DU split (the F1 interface) was introduced to enable flexible and possibly multi-vendor disaggregated RAN deployments. In particular, the following benefits motivated specification of the HLS in 5G.

- Motivation # 1: Flexibility and scalability in deployment

Having a split architecture design definitely brings flexibility in deployment where in one deployment DU can be deployed closer to RU and CU can be centralized. Also, it is often claimed that such disaggregated deployment brings scalability since CU can be centralized and it can be connected to multiple DUs and hence support  large gNB deployments.

Benefit # 1: Disaggregated architecture allows flexibility and scalability in deployments.

In our view, we should separate the discussion on deployment architecture with standards. There is no denying of the flexibility and scalability that disaggregated and modular RAN design brings, and standards should help in achieving such flexibility and scalability, not limit it. RAN3 should strive to achieve full modularity and scalability in design and only provide specification for interfaces that need to be truly multi-vendor. In our view, the current 5G HLS is actually limiting the flexibility and scalability of RAN deployments.

Observation 1: 5G HLS design as specified by 3GPP limits the true flexibility and scalability which is necessary for cloud deployments.

- Motivation # 2: Multi-vendor Open RAN support

One of the envisioned benefits of interoperable CU-DU split was to provide openness in RAN so that it can function in a multi-vendor fashion. Although the vision is correct and supported by us, it is far from reality in actual commercial deployments. We have not seen many commercial deployment supporting multi-vendor CU-DU split where CU and DU are located at different sites. This is mainly because multi-vendor RAN support comes at a huge integration cost which cannot be neglected. Furthermore, it also limits the flexibility and scalability in the deployment choice which is not ignorable.

Benefit # 2: Specified HLS nurtures the environment of multi-vendor support.

However, we observe the following:

Observation 2: Multi-vendor disaggregated gNB deployment has not become the mainstream deployment in 5G networks.

## 3.2	Performance analysis of 5G HLS

One of the most mentioned benefits of CU-DU split in standardized manner is the possibility of centralization and leverage pooling gains. However, in this section, we will explain that the often-mentioned centralization gains are taken away due to additional overhead resulted from the way 5G HLS was designed.

For this purpose, we did some numerical analysis considering the following deployment scenarios.

Figure 1: Different deployment scenarios used for numerical analysis of pooling gains

Considering the above deployment scenarios, we try to understand the advantages of centralization focusing on CP pooling gains as CP higher-layer split was specified as an afterthought of doing UP split between RLC and PDCP.

Assumptions:

  - For deployment#1, we assume that ~1 CPU is needed per site to handle the peak traffic load.

  - For deployment#2 and #3/3a, some processing (~0.5 CPU) is always needed to handle the mean load and rest of the processing is pooled to tackle varying peaks.

Does pooling with standardized F1-C make sense?

Based on our analysis, we get the following results and observations.

Observation 3: Deployment option on far-edge without F1 (Option 2) shows around 30% pooling gains.

Observation 4: Deployment option on CU at Edge and with F1 (Option 3) shows similar pooling gains as deployment option with CU+DU at cell sites (Option 1).

Observation 5: Deployment option on CU at Edge and DU at far-edge with F1 (Option 3a) shows decreased pooling gains of around 40% as deployment option with CU+DU at far-edge without F1 (Option 2).

Based on the above observations, it can be said that standardized 5G HLS results in degradation in the pooling gains which can be achieved otherwise. From our perspective, the main reason for this is that CP HLS was standardized as an afterthought and resulted in sub-optimum split.

Is CP latency affected due to standardized 5G HLS?

Furthermore, we show the increased CP overhead (measured by the number of communicated messages) of some typical and most frequent RRC procedures (e.g. initial access and RRC reestablishment) based on TS 38.401.

| Common CP Procedures | No F1 | With F1 | Signaling overhead Increase (%) |
| --- | --- | --- | --- |
| Initial access | 10 messages | 18 messages | >40% |
| RRC reestablishment | 7 messages | 14 messages | >50% |

Observation 6: Increase in F1 messages causes additional signaling overhead and increased CP latency.

## 3.3	Challenges and pain points of 5G HLS

One of the reasons why CU-DU split has not become truly multivendor interface is due to the way 3GPP has standardized the interface in 5G which makes it a very complex interface. At the same time, the performance benefits of having a multivendor disaggregated RAN are not high which does not provide enough justification for multivendor deployments given the additional integration and inter-operable testing complexities. Additionally, the control plane performance degrades as compared to that of the monolithic RAN. Specifically, in 5G CP HLS, the RRC is terminated in the CU, but the DU owns the radio resources and their management. For example, the CU is responsible for cell selection (admission) and establishment of the RRC connection for the User Equipment (UE). The same holds for cell/gNB changes during handovers. However, the DU manages the cell resources. This results in the fact that during RRC setup, a UE context must be setup both in the CU and the DU, and the UE context data must be aligned/coordinated at both nodes. As a result, many control plane procedures (e.g. UE initial access procedure and handover procedure) contain significantly more messages as compared to the monolithic RAN. Similarly, functional split in RRM configuration management also leads to complicated and suboptimal UE capabilities coordination between CU and DU. Such suboptimal UE capability management at the network leads to suboptimal RRM configuration which affects the overall system performance. In many deployments, propriety fixes were done to avoid the performance loss.

Observation 2: Functional split in 5G between CU and DU is not optimal because the CU acts as the RRC anchor, while most of the RRC resources are owned by DU. This leads to significant increase in control signalling and complex procedures compared to monolithic RAN.

Observation 3: Functional split in RRM configuration management leads to complicated and suboptimal UE capabilities coordination between CU and DU.

In the following, we describe the challenges that appear because of functional split between CU and DU in 5G.

- Challenge # 1: CP Latency

As a result of 5G HLS, the F1-C signalling related to the RRC coordination results in significant control plane latency for the UE(s). For example, it is observed that RRC procedures become slower in disaggregated RAN deployments as compared to monolithic RAN deployments. One may argue that this is not the major challenge since CP procedures are not that frequent. However, in our view, procedures like UE initial access and RRC reestablishment are the most frequent CP procedures, and they can impact the call setup time and service continuity. In addition, for the support of low latency services in edge clouds, optimized RRC is one important factor. Therefore, these CP procedures should be as latency optimized as possible.

Pain point 1: CP latency of disaggregated RAN deployment is higher than monolithic RAN deployment.

- Challenge # 2: Unrealized pooling gains

One of the real motivations to do the CU-DU split is the centralization of CU at the (far-)edge sites which could provide significant pooling gains both in UP as well as CP. However, CP pooling gains get significantly impacted due to increased processing needs of F1 messages. Since, the existing 5G F1-C results in lots of ping-pong between CU and DU, it offsets the pooling gains one can expect due to centralization of CU.

Pain point 2: Increased CP overhead over F1-C significantly decreases the pooling gains.

- Challenge # 3: RRC optimizations

As mentioned above, the current 5G HLS resulted in a split in RRC where some parameters are handled by CU and some are handled by DU. With such split, it becomes even harder to handle possible optimizations of the RRC procedures that may be introduced in RAN2 (which tend to get more and more complex in every release and generation due to large number of supported devices and features). Having an architecture that can flexibly adapt to such optimizations in 6G RRC is instrumental to make the whole system future proof and implementation friendly, not only from network’s but also from device’s perspective. Therefore, possible split between CU and DU parameters must avoid splitting the RRC handling as much as possible. Currently, 5G HLS hinders such optimizations.

Pain point 3: 5G HLS complicates handling of possible RRC optimizations introduced in RAN2, which are instrumental for future proofness and supporting various device types/ UE capabilities.

- Challenge # 4: Implementation friendliness

By the time 6G networks will be commercialized, we expect that still both classic and cloud RAN deployments will coexist. This means that in order to cater for multiple services and their requirements (e.g. latency etc.) as well as reduce the total cost of ownership (TCO) for the 6G network, we will have to support all possible deployments where DU/CU may be present at the cell site in some cases but are centralized in the other cases. Therefore, we have to design the RAN architecture in such a way that it not only provides similar performance in all different deployment scenarios but also allows implementation friendliness. However, the way current 5G HLS is designed, it does not allow easy implementation to cater for different deployments.

Pain point 4: Current 5G HLS design does not allow the implementation to be adapted to the needs of particular deployment.

- Challenge # 5: Standardization complexity

Current 5G HLS went through multiple releases in 5G and, for every 5G feature, HLS procedures and optimizations were supported as an afterthought in the later release. Such procedures resulted in complex and sometimes incomplete F1 specifications. If specification is not ideal or complete, then network vendors may make their own "corrections" or additions to procedures/functionalities to make their networks operate properly. As a result, if different vendors have different implementations of CU and DU functionalities, then multi-vendor CU-DU operation becomes very difficult, which, in turn, jeopardises the main purpose of defining the interface (see above).

Pain point 5: In 5G, feature support with 5G disaggregated RAN is always done as an after-thought which results in either complex or incomplete specifications.

Furthermore, due to unnecessary complexity of RRC protocol and because of F1 originated CP latency, it is not able to handle its main task of controlling radio resources. In order to do the faster control of radio resources, a lot of control information is added in the MAC headers which resulted it additional MAC layer complexities. Going forward, in 6G, 3GPP should avoid such interlinks of functionalities and keep the design simple where right protocol is used for the right purpose.

Pain point 6: In 5G, due to unnecessary CP latency and complexity of RRC protocols, a lot of control information is added to MAC CEs which unnecessarily complicates the MAC layer.

- Challenge # 6: No standardized resiliency

Current 5G HLS does not support any standardized resiliency solution. The cardinalities are defined such that CU-CP becomes a single point of failure (e.g., a single CU connection per DU). Furthermore, true centralization gain of CUs comes when it is connected to multiple DUs and support large number of UEs. Therefore, it becomes very important that 3GPP defines an architecture where CU does not become a single point of failure to avoid service disruptions.

Pain point 7: In 5G, CU may become a single point of failure causing service disruptions especially in centralized RAN deployments.

## 3.4	Summary

In the following table, we summarize the benefits and pain points of the standardised 5G HLS and it is proposed to take the following as the starting point for RAN3 discussions and capture in the RAN3 TR.

| Benefits of 5G HLS | Pain points of 5G HLS |
| --- | --- |
| Disaggregated architecture allows flexibility and scalability in deployments. | CP latency of disaggregated RAN deployment is higher than monolithic RAN deployment. |
| Specified HLS nurtures the environment of multi-vendor support. | Increased CP overhead over F1-C significantly decreases the pooling gains that could be achieved theoretically. |
|  | 5G HLS complicates handling of possible RRC optimizations introduced in RAN2, which are instrumental for future proofness and supporting various device types/ UE capabilities. |
|  | Current 5G HLS design does not allow the implementation to be adapted to the needs of particular deployment. |
|  | In 5G, feature support with 5G disaggregated RAN is always done as an after-thought which results in either complex or incomplete specifications. |
|  | In 5G, due to unnecessary CP latency and complexity of RRC protocols, a lot of control information is added to MAC CEs which unnecessarily complicates the MAC layer. |
|  | In 5G, CU may become a single point of failure causing service disruptions especially in centralized RAN deployments. |

Proposal 3: Capture the above table (pain points vs benefits) in the TR.

# 5	Conclusion

In this paper, we started from considering the work plan for the study on the 6G HLS. We propose:

Proposal 1: RAN3 to acknowledge that HLS may be beneficial. Further work is to focus on the question if the HLS shall be standardised and, if yes, how to define it to avoid problems that handicapped standardised 5G HLS.

Proposal 2: RAN3 to first agree on the work plan for HLS design, as mentioned above.

Then we consider benefits of the 5G HLS that are relevant in the context of the study, some of which we have also analysed in the context of offered performance:

Observation 1: 5G HLS design as specified by 3GPP limits the true flexibility and scalability which is necessary for cloud deployments.

Observation 2: Multi-vendor disaggregated gNB deployment has not become the mainstream deployment in 5G networks.

Observation 3: Deployment option on far-edge without F1 (Option 2) shows around 30% pooling gains.

Observation 4: Deployment option on CU at Edge and with F1 (Option 3) shows similar pooling gains as deployment option with CU+DU at cell sites (Option 1).

Observation 5: Deployment option on CU at Edge and DU at far-edge with F1 (Option 3a) shows decreased pooling gains of around 40% as deployment option with CU+DU at far-edge without F1 (Option 2).

Observation 6: Increase in F1 messages causes additional signaling overhead and increased CP latency.

Further, we discuss known pain points that can be listed as follows:

Pain point 1: CP latency of disaggregated RAN deployment is higher than monolithic RAN deployment.

Pain point 2: Increased CP overhead over F1-C significantly decreases the pooling gains.

Pain point 3: 5G HLS complicates handling of possible RRC optimizations introduced in RAN2, which are instrumental for future proofness and supporting various device types/ UE capabilities.

Pain point 4: Current 5G HLS does not allow implementation friendly design.

Pain point 5: In 5G, feature support with 5G disaggregated RAN is always done as an after-thought which results in either complex or incomplete specifications.

Pain point 6: In 5G, due to unnecessary CP latency and complexity of RRC protocols, a lot of control information is added to MAC CEs which unnecessarily complicates the MAC layer.

Pain point 7: In 5G, CU may become a single point of failure causing service disruptions especially in centralized RAN deployments.

We propose to capture these considerations in the 6G study TR:

Proposal 3: Capture the above table (pain points vs benefits) in the TR.

We offer also a pCR for the TR 38.760-3, which is presented in the Annex 1 below.

# 4	References

- RP-252912, Revised SID: Study on 6G Radio.

# Annex 1:	pCR to the TR 38.760-3

The below text proposal is based on the TR 38.760-3 v 0.1.1.

First change, omitted text not changed

## 6.2	RAN Internal Interfaces

Editor’s Note: This chapter shall comprehensively address study on CU-DU and CP-UP aspects including functional split considering interim milestone TSG#112 (June/2026) to take a decision during TSG#115: March 2027

Editor’s Note: This chapter should also address the interfaces between 6G RAN nodes (equivalent to Xn).

### 6.2.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

RAN3 acknowledges usefulness of designing the 6G Node in a way that enables disaggregated deployment.

Editor’s note: it is FFS if the disaggregation option requires standardisation.

### 6.2.X	Analysis of the standardised 5G HLS

RAN3 has identified following benefits and pain points of the standardised 5G Higher Layer Split (HLS):

| Benefits of 5G HLS | Pain points of 5G HLS |
| --- | --- |
| Disaggregated architecture allows flexibility and scalability in deployments. | CP latency of disaggregated RAN deployment is higher than monolithic RAN deployment. |
| Specified HLS nurtures the environment of multi-vendor support. | Increased CP overhead over F1-C significantly decreases the pooling gains that could be achieved theoretically. |
|  | 5G HLS complicates handling of possible RRC optimizations introduced in RAN2, which are instrumental for future proofness and supporting various device types/ UE capabilities. |
|  | Current 5G HLS design does not allow the implementation to be adapted to the needs of particular deployment. |
|  | In 5G, feature support with 5G disaggregated RAN is always done as an after-thought which results in either complex or incomplete specifications. |
|  | In 5G, due to unnecessary CP latency and complexity of RRC protocols, a lot of control information is added to MAC CEs which unnecessarily complicates the MAC layer. |
|  | In 5G, CU may become a single point of failure causing service disruptions especially in centralized RAN deployments. |

Remaining text not changed