3GPP TSG-RAN WG3 Meeting #130	R3-258197

Dallas, US, 17th–21st  November, 2025

Agenda Item:	13.3

Source:	Tejas Networks

Title:	RAN-CN procedures and signaling for NR ISAC

Document Type:	other

# Introduction

A new study on integrated sensing and communications (ISAC) has been approved in RAN #108 Error! Reference source not found. and revised at RAN#109 (FS_Sensing_NR_bis) [2], where the objectives of the study for 5GA-based ISAC are as follows:

Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

In RAN3 #129Bis, it was agreed to capture Sensing Reporting procedure in TR as shown in Figure 1 and continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC

Figure 1 Message flow for sensing reporting

In this contribution, we discuss interface functions and further signaling procedures for ISAC.

# Discussion

- The sensing protocol provides the following functions:

- TRP Information Transfer. This function allows an SF to obtain TRP related information from an NG-RAN node

- Sensing Information Transfer. This function allows the SF to exchange sensing information with the NG-RAN node for the purpose of sensing.

- Sensing Measurement Transfer. This function allows the SF to exchange measurement information with the NG-RAN node for the purpose of sensing.

## 2.1 TRP Information Exchange

The purpose of the TRP Information Exchange procedure is to allow the SF to request gNB to provide detailed information for TRPs hosted by the gNB. The SF initiates the procedure by sending a TRP Information message with TRP list included. The gNB responds with a TRP Information Response message that contains the requested TRP information. This procedure is used to support sensing capabilities and sensing related configuration exchange such as TP and/or RP functionality, geographical coordinates of the TPs and RPs, and possibly other configuration information (e.g., beam information, sensing frequency layer, etc., dependent on RAN1). If the gNB cannot provide any of the requested information for any TRP, the gNB responds with a TRP Information Failure message.

Figure 2 Message flow for TRP Information Exchange Procedure

- Proposal 1: It is necessary to define a Class 1 procedure for TRP Information exchange. This procedure consists of a TRP Information Request (from the SF to the sensing entity i.e., gNB) and a TRP Information Response (from the gNB to the SF).

## Sensing Information Exchange

The Sensing Information Exchange procedure is initiated by the SF to request the gNB Sensing Reference Signal configuration. The SF initiates the procedure by sending a Sensing Information request message to the gNB. The request may include a desired sensing signal characteristic, such as desired bandwidth of the sensing signal. The SF may send a Sensing Information request message to the gNB to request a sensing signal configuration, which the gNB provides in a Sensing Information response message to the SF. If the gNB is unable to provide any of the requested information, the gNB shall respond with a Sensing Information Failure message with an appropriate cause value.

Figure 3 Message flow for Sensing Information Exchange Procedure

- Proposal 2: It is necessary to define a Class 1 procedure for Sensing Information exchange. This procedure consists of a Sensing Information Request (from the SF to the sensing entity i.e., gNB) and a Sensing Information Response (from the gNB to the SF).

## Sensing Measurement Abort

The purpose of the Sensing Measurement Abort Procedure is to enable the SF to abort an on-going measurement. The SF initiates the procedure by sending a Sensing Measurement Abort message.

Upon receiving this message, the gNB shall terminate the on-going measurement identified by the SF Measurement ID and may release any resources previously allocated for the same measurement.

gNB can also initiate Sensing Measurement Abort if needed.

Figure 4 Message flow for Sensing Measurement Abort Procedure

- Proposal 3: It is necessary to define a Class 2 procedure for Sensing Measurement Abort. This procedure consists of a Sensing Measurement Abort (from the SF to the sensing entity i.e., gNB).

- Using above procedures, a general call flow for sensing operation could be as shown in Figure 5.

Figure 5 Sensing procedure between gNB and SF

During RAN3 #129Bis discussions, the sensing measurement procedures were referred to as Sensing Request, Sensing Response and Sensing Report. While these names are functional, they may lead to ambiguity as the ISAC framework evolves to include multiple sensing-related procedures such as Sensing Information, Sensing Capability Exchange, and others.

To improve clarity and future scalability, it is proposed to rename the sensing reporting procedures as follows:

- Current Name: Sensing Request
Proposed Name: Sensing Measurement Request

- Current Name: Sensing Response
Proposed Name: Sensing Measurement Response

- Current Name: Sensing Report
Proposed Name: Sensing Measurement Report

These names explicitly indicate that the procedures pertain to the exchange of sensing measurement data, distinguishing them from other sensing-related procedures.

Proposal 4: Sensing Reporting procedure messages shall be renamed as shown above

# 3	Conclusion

To support gNB-based sensing, TRP Information exchange, Sensing Configuration exchange, Sensing Measurement procedures needs to be defined. we need a mechanism to transport NRSP messages between the gNB and the SF.

- Proposal 1: It is necessary to define a Class 1 procedure for TRP Information exchange. This procedure consists of a TRP Information Request (from the SF to the sensing entity i.e., gNB) and a TRP Information Response (from the gNB to the SF).

- Proposal 2: It is necessary to define a Class 1 procedure for Sensing Information exchange. This procedure consists of a Sensing Information Request (from the SF to the sensing entity i.e., gNB) and a Sensing Information Response (from the gNB to the SF).

- Proposal 3: It is necessary to define a Class 2 procedure for Sensing Measurement Abort. This procedure consists of a Sensing Measurement Abort (from the SF to the sensing entity i.e., gNB).

Proposal 4: Sensing Reporting procedure messages shall be renamed as shown above

# 4 Annex: TP for BL pCR of TR 38.765

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing

8.x	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting

1.	The SF sends sensing measurement request to the gNB.

2.	The gNB sends sensing measurement response to the SF.

3.	If requested in Step 1, the gNB sends sensing measurement report to the SF.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

# 5 References

- RP-251861, New SID: Study on Integrated Sensing And Communication (ISAC) for NR

- RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR

- TR 22.837: "Study on Integrated Sensing and Communication".

- TS 22.137: "Integrated Sensing and Communication".

- R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR, Prague, Czech Republic, 13 – 17 October 2025

- RAN3 #129bis Chair Notes