3GPP TSG-RAN WG3  Meeting #130	R3-258226

USA, Dallas 17th-21st November 2025

Source: 	Vodafone, Ericsson, Interdigital, Nokia, Telecom Italia

Title: 		ASN1 Usage in 6G AP

Document for: decision

Agenda Item: 10.2.1

Introduction

RAN WG2 agreed during the last RAN2#131bis meeting to continue to use ASN1 for RRC. From chairlady report of RAN2#131bis

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

Discussion

The basis for the discussion that led to this conclusion was the document R2-2507172: “Views on Control Plane for 6G” by Qualcomm Incorporated. In this document, the following points are presented as the rationale for continuing the use of ASN.1:

- ASN.1 (Abstract Syntax Notation One) and PER (Packed Encoding Rules) are specified by ITU-T in the ITU-T Recommendations X.680, X.681 and X.691 (which are also same as various ISO/IEC International Standards).

- So far 3GPP has been using ASN.1 UPER (Unaligned PER) for encoding of RRC messages. A pre-defined ASN.1 schema is used by both the transmitter and the receiver. The receiver can decode the ASN.1 signalling sent over the air interface using the pre-defined schema and the rules.

- We looked at other potential encoding methods for 6G. However, they have their merits and drawbacks. For example, JSON, XML are big, unoptimized/verbose strings. Also, they are not precise. Protobuf is not an international standard, but a serialization format developed by Google.

- ASN.1 is likely the most compact encoding method compared to the other options. Therefore, it seems reasonable to continue using ASN.1 for encoding of RRC signalling over the 6G air interface.

We believe the rationale outlined above is applicable to the 6G RAN-CN interface too. Regardless of the RAN-CN interface option 6G RAN-CN Application Protocol messages should use an efficient encoding method. ASN.1 has consistently proven to be a highly effective and compact encoding method over time, as also stated in R2-2507172.

Furthermore, it is important to highlight that future 6G RAN systems will likely need to support ASN.1 to ensure seamless connected mobility between

6G and 5G networks anyhow. This is because 5G RAN exclusively supports ASN.1 encoding and does not accommodate alternative formats. Assuming that 6G has to follow the well established "source adapts to target" principle, it is neither realistic nor acceptable that 5G RAN will adopt different encoding for interworking purposes.

Additionally, continuing with ASN.1 encoding would eliminate the need to introduce fundamentally different encoding for 6G RAN interfaces as compared to the encoding used in RRC.

This is relevant for example in cases where RRC IEs are signaled over e.g.NG or Xn-AP, and where the receiver of the AP message needs to understand the content of such RRC messages. For example, RAN3 introduced recently in 38.423 the SBFD Frequency Configuration IE, encoded as SBFD-Subband-Allocation IE, as defined in 38.331). Hence, this ensures consistency across interfaces and reduces complexity in protocol translation.

For RAN Internal Interfaces the same assumptions are applicable and therefore, we think that 6G RAN Internal Interface AP should also be encoded using ASN1.

Proposal:

It is proposed to capture that ASN.1 is used for encoding of 6G RAN-CN AP and 6G RAN Internal AP messages

---------------------------------------------------------------------------------------------------------------

# Annex

## pCR to TR 38.760-3 for section:

## 5.1	General Principles

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS]

ASN1 is used for encoding of 6G RAN-CN AP and 6G RAN Internal AP messages