3GPP TSG-RAN3 Meeting #130	R3-258281

Dallas, US, 17 - 21 November, 2025

Agenda item:	10.2.2

Source:	Xiaomi

Title:	RAN logical architecture and functions

Document for:	Discussion and Decision

1	Introduction

One of the objectives of the study in [1] is to progress Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc).

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] Overall RAN architecture aspects [RAN3, RAN2]

An agreed draft TR in [2] at RAN plenary #109 meeting has identified the following requirements for architecture and migration.

## 5.2 Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: - The 6G RAN architecture shall support standalone RAN architecture. - The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR. - The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR. - The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated. - The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs. - 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability. - The 6G RAN architecture shall allow for control plane and user plane separation. - The 6G RAN architecture shall support sharing of the RAN between multiple operators. - The 6G RAN architecture shall allow for the operation of network slicing. - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. - The 6G RAN architecture shall support enhanced service awareness in RAN. - The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable. - The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks. - The 6G RAN architecture shall allow non-public networks.

In last RAN3 meeting, general principles and deployment scenarios has been discussed and made the following progresses.

5.1 General Principles All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles. The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS] 5.2 Deployment Scenarios Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914. The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914. - FFS on the implications of this requirement on 6G RAN architecture. - FFS whether all deployment scenarios of TR 38.914 can be supported.

In this contribution, we’d like to discuss the RAN architecture and RAN functions according to the above background.

2	Discussion

2.1 	Standalone RAN Architecture

Based on the above requirements, a new Radio Access Technology (RAT), referred to as 6GR, will be introduced. While it remains uncertain whether a new 6G core network will be developed, support for new services will necessitate the introduction of new or enhanced core network functions. These aspects are subject to further discussion in the RAN plenary and SA groups. For the RAN3 architecture discussions, given the introduction of a new RAT, it is assumed that new interface(s) between the RAN and core network functions will be required.

Observation 1, For the RAN3 architecture discussions, with the introduction of a new RAT, it is assumed that new interface(s) between the RAN and core network function(s) will be required—regardless of whether a new core network or an enhanced existing core is adopted in SA2.

Lessons learnt from 5G

The following is the overall architecture description for 5G as defined in TS 38.300 [3]

An NG-RAN node is either: - a gNB, providing NR user plane and control plane protocol terminations towards the UE; or - an ng-eNB, providing E-UTRA user plane and control plane protocol terminations towards the UE. The gNBs and ng-eNBs are interconnected with each other by means of the Xn interface. The gNBs and ng-eNBs are also connected by means of the NG interfaces to the 5GC, more specifically to the AMF (Access and Mobility Management Function) by means of the NG-C interface and to the UPF (User Plane Function) by means of the NG-U interface (see TS 23.501 [3]).

- Experience from 5G shows that integrating both gNB and ng-eNB within NG-RAN significantly increased architectural complexity, operational costs, and slowed the adoption of NR-native capabilities. Maintaining dual protocol stacks and interworking mechanisms introduced signaling overhead and constrained innovation. For 6G, a clean-slate approach is essential: the 6G RAN should exclusively support the new 6G RAT only, while legacy inter-working should be handled at the core network level. This separation minimizes complexity, reduces cost, and accelerates the deployment of advanced 6G features such as AI-native optimization and integrated sensing, ensuring that backward compatibility does not hinder forward progress.

| - 5G | - 6G |
| --- | --- |

- Lesson learnt from 5G (1), Integrating gNB and ng-eNB in 5G NG-RAN led to increased complexity, higher costs, and hindered NR-native innovation due to dual protocol stacks and signaling overhead.

- Proposal 1, RAN3 starts standalone RAN logical architecture study as baseline.

2.2 	6G RAN functions

According to TS 38.300 [3], the RAN-CN function split is summarized on the figure below where yellow boxes depict the logical nodes and white boxes depict the main functions.

Functional Split between NG-RAN and 5GC

- While 5G primarily focused on communication aspects such as data, control, and mobility—with AI/ML and sensing treated as add-ons, 6G RAN–CN function split should build on 5G principles but also natively support for emerging functionalities like AI/ML and sensing.

- In general, the functionalities for AI and sensing may be potentially distributed between RAN and CN, so the following figure shows the main function split between 6G RAN and 6G CN.

- RAN3 can first focus on the RAN side functions and discuss the CN functions according to SA2 progress. The following table shows our views on 6G RAN potential functionalities comparing to 5G.

- Legacy RAN functions

- The following table presents an analysis of the applicability of 5G RAN functions to 6G RAN functions, based on the requirements defined for 6G.

| - Category | - 5G RAN functions | - Can be 6G RAN functions? | - Proposed 6G RAN functions |
| --- | --- | --- | --- |
| - Radio Resource Management | - Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling) | - Yes, as a foundational concept, but FFS on sidelink | - Radio Resource Management, details subject to RAN1/2 progress. |
| - Data Processing and Security | - IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data | - Depends on RAN2 progress | - FFS |
|  | - Encryption & Integrity Protection | - Depends on RAN2 progress | - FFS |
| - Connection and Session Management | - Connection Setup and Release | - Yes, as fundamental procedure for managing radio resources and UE connectivity. | - Connection Setup and Release |
|  | - Session Management | - Yes, as a foundational function to manage the PDU session for user plane data transmission | - Session Management |
|  | - Support of UEs in RRC_INACTIVE state | - Subject to RAN2 discussion on UE state | - FFS |
| - QoS & Network Slicing | - QoS Flow Management and Mapping to Data Radio Bearers | - Yes, but may be evolved, the requirement for enhanced service awareness implies a much richer, dynamic, and intelligent QoS framework, which is subject to SA2 | - QoS flow management and mapping to DRBs, subject to SA2 progress |
|  | - Support of Network Slicing | - Yes, as a core requirement in TR 38.914 [2] | - Support of Network Slicing |
| - Routing and selection | - AMF Selection at UE Attachment | - Yes, 6G RAN node may directly interface with multiple network functions (NFs). At this stage, using a generalized core network function name is preferred. | - NF selection |
|  | - User Plane Data Routing to UPF | - Yes, as a foundational function. | - User Plane Data Routing to UPF |
|  | - Control Plane Information Routing to AMF | - Yes,6G RAN node may directly interface with multiple network functions (NFs). At this stage, using a generalized core network function name is preferred. | - Control Plane Information Routing to NF |
|  | - Distribution function for NAS messages | - Yes, as a foundational function | - Distribution function for NAS messages |
| - System Information and Paging | - Scheduling and Transmission of Paging Messages | - Yes,as a foundational function, details are subject to RAN2 progress | - Scheduling and Transmission of Paging Messages, subject to RAN2 progress |
|  | - Scheduling and Transmission of System Broadcast Information | - Yes,as a foundational function, details are subject to RAN2 progress | - Scheduling and Transmission of System Broadcast Information, subject to RAN2 progress |
| - Multi-Operator Support | - Radio Access Network Sharing | - Yes, as a core requirement in TR 38.914 [2] | - Radio Access Network Sharing |

- Based on the above analysis, several foundational functions, such as RRM, paging, and connection setup/release, are considered applicable to 6G RAN. However, the specific implementation details depend on the progress of RAN1 and RAN2. Functions like session management and QoS fall under the scope of SA2, while others, such as data processing and security, remain uncertain from the RAN3 perspective and require input from other working groups. Overall, the following 6G RAN functions can be documented in the TR at this stage.

- Proposal 2, RAN3 agrees the following 6G RAN functions in the TR.

- - Radio Resource Management, details subject to RAN1/2 progress;

- - Connection Setup and Release;

- - Session Management, subject to SA2 progress;

- - QoS flow management and mapping to DRBs, subject to SA2 progress;

- - Support of Network Slicing;

- - NF selection;

- - User Plane Data Routing to UPF;

- - Control Plane Information Routing to NF;

- - Distribution function for NAS messages;

- - Scheduling and Transmission of Paging Messages, subject to RAN2 progress;

- - Scheduling and Transmission of System Broadcast Information, subject to RAN2 progress;

- - Radio Access Network Sharing.

- New functions for new services

- To enable intelligent and adaptive network operations in 6G, the introduction of a dedicated AI Control Function within the RAN is essential. This entity would manage the full LCM of AI/ML models at the air interface, including training, deployment, and updates, while also handling data collection configuration and reporting to ensure high-quality datasets. Furthermore, it would support efficient processing and transfer of model parameters and datasets, which are critical for real-time AI-driven decision-making.

- To support ISAC, the sensing related RAN functionalities should also be considered. It was agreed in RANP#109 meeting, sensing use cases are agreed in [2] on Requirements of new and existing services for 6G, and the sensing mode are agreed during the online discussion.

### 5.4.4 Sensing Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

- The sensing related functionalities at least includes sensing resource allocation, measurement configuration, sensing processing and report can be further considered. From RAN3 perspective, a general new function to support sensing should be considered, and the details sensing related functions split study should align and coordinate with other working groups (e.g., RAN2 and SA2).

- Proposal 3, RAN3 capture the general AI/ML and RAN sensing functions, and further study the details in coordination with other working groups (e.g., RAN1, RAN2, SA2).

- In the Annex, we provide pCR to 6G TR to capture the general 6G standalone architecture and RAN functions.

- Proposal 4, RAN3 agrees the pCR in the Annex.

3	Conclusion

In this contribution, we discussed the 6G general RAN architecture, 6G RAN functions, and have the following observations and proposals.

Standalone RAN Architecture

- Lesson learnt from 5G (1), Integrating gNB and ng-eNB in 5G NG-RAN led to increased complexity, higher costs, and hindered NR-native innovation due to dual protocol stacks and signaling overhead.

- Proposal 1, RAN3 starts standalone RAN logical architecture study as baseline.

6G RAN functions

- Proposal 2, RAN3 agrees the following 6G RAN functions in the TR.

- - Radio Resource Management, details subject to RAN1/2 progress;

- - Connection Setup and Release;

- - Session Management, subject to SA2 progress;

- - QoS flow management and mapping to DRBs, subject to SA2 progress;

- - Support of Network Slicing;

- - NF selection;

- - User Plane Data Routing to UPF;

- - Control Plane Information Routing to NF;

- - Distribution function for NAS messages;

- - Scheduling and Transmission of Paging Messages, subject to RAN2 progress;

- - Scheduling and Transmission of System Broadcast Information, subject to RAN2 progress;

- - Radio Access Network Sharing.

- Proposal 3, RAN3 capture the general AI/ML and RAN sensing functions, and further study the details in coordination with other working groups (e.g., RAN1, RAN2, SA2).

- Proposal 4, RAN3 agrees the pCR in the Annex.

4	Reference

[1]	RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, CMCC

[3] 3GPP TS 38.300, NR; NR and NG-RAN Overall Description, stage2.

5	Annex (pCR to 6G TR 38.760-3)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

5.3	Overall RAN Architecture

Editor’s note: The logical architecture can be captured here and should include the entire architecture including possible influence from existing and new services.

A 6G RAN node is providing 6GR user plane and control plane protocol terminations towards the UE.

The 6G RAN node is connected by means of the Nx interfaces to the NF(s) in 6GC.

Editor’s Note, the NF(s) could be 6G AMF, 6G UPF, other NFs, that are pending to further discussion.

The 6G RAN architecture is illustrated in Figure 5.3.x-1 below.

Figure 5.3.x1-1: Overall Architecture

5.3.1	RAN-CN Functional Split

Editor’s note: The aim of this section is to describe functions split between RAN and CN.

Figure 5.3.1-1: Functional Split between 6G RAN and CN

5.3.2	RAN Functions

Editor’s note: The aim of this section is to describe the functions supported in RAN.

6G RAN node hosts the following functions:

-  Radio Resource Management, details subject to RAN1/2 progress;

-  Connection Setup and Release;

-  Session Management, subject to SA2 progress;

-  QoS flow management and mapping to DRBs, subject to SA2 progress;

-  Support of Network Slicing;

-  NF selection;

-  User Plane Data Routing to UPF;

-  Control Plane Information Routing to NF;

-  Distribution function for NAS messages;

-  Scheduling and Transmission of Paging Messages, subject to RAN2 progress;

-  Scheduling and Transmission of System Broadcast Information, subject to RAN2 progress;

-  Radio Access Network Sharing;

- 	AI/ML, FFS on details;

-	Sensing, FFS on details;

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change ends>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>