3GPP TSG-RAN3 Meeting #130	R3-258402

Dallas, US, November 17 - 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1635 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Criticality in the OD-SIB1 Configuration Provision Status Update message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: | In the current TS 38.423 specification, the IE tabular of OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message is given as below. This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted OD-SIB1 configuration provision is being stopped. Direction: NG-RAN node2 ® NG-RAN node1. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; ignore. NES Cell ID; M; NR CGI 9.2.2.7; YES; reject. Cell A ID; O; NR CGI 9.2.2.7; YES; ignore. Provision Status; M; ENUMERATED (stopped, …); Indicates the status of the OD-SIB1 configuration transmission.; YES; reject. Interface Instance Indication; O; 9.2.2.39; YES; reject. After receiving the above message, even the NG-RAN node1 cannot comprehend the Provision Status IE, the whole procedure still works since this IE only has one status “stopped”. Therefore, the Assigned Criticality of the Provision Status IE in the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message should be changed as “ignore”. If there are multiple OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE messages with different NES Cell IDs in parallel and the NG-RAN node1 cannot comprehend the NES Cell ID IE, the NG-RAN node1 cannot differentiate the broadcasting of the OD-SIB1 configuration for which NES Cell has been stopped. Therefore, the Assigned Criticality of the NES Cell ID IE should be kept as “reject”. |
| Summary of change: | - In the IE tabular, change the Assigned Criticality of the Provision Status IE as “ignore” in the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message. - In the ASN.1, change the Criticality of the NES Cell ID IE as “reject” and the Criticality of the Cel Al ID IE as “ignore”in the OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) because it only impacts the OD-SIB1 function in Rel-19. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | - The OD-SIB1 Configuration Provision procedure is wrongly rejected if the NG-RAN node1 cannot comprehend the Provision Status IE. |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 9.1.3.33; 9.3.4 |  |  |  |  |  |  |  |  |  |
|  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.1.3.33	OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted OD-SIB1 configuration provision is being stopped.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NES Cell ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Cell A ID | O |  | NR CGI 9.2.2.7 |  | YES | ignore |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the OD-SIB1 configuration transmission. | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- OD-SIB1 CONFIGURATION PROVISION STATUS

--

-- **************************************************************

ODSIB1ConfigurationProvisionStatus ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ODSIB1ConfigurationProvisionStatus-IEs}},

...

}

ODSIB1ConfigurationProvisionStatus-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-NES-Cell-ID								CRITICALITY reject	TYPE NR-CGI										PRESENCE mandatory}|

{ ID id-Cell-A-ID								CRITICALITY ignore	TYPE NR-CGI										PRESENCE optional }|

{ ID id-ProvisionStatus							CRITICALITY ignore	TYPE ProvisionStatus							PRESENCE mandatory}|

{ ID id-InterfaceInstanceIndication				CRITICALITY reject	TYPE InterfaceInstanceIndication			PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<<END OF CHANGES>>>>>>>>>>>>>>>>>>>>