3GPP TSG-RAN WG3 Meeting #130                                                             R3-258445

Dallas, US, 17 – 21 November 2025

Agenda Item:	10.2.1

Source:	CATT,CBN

Title:	Discussion on Deployment Principles and Scenarios for 6G RAN

Document for:	Decision

1	Introduction

In the 6G WID [1], RAN3 is responsible for the Radio Access Network Architecture, interface protocols and procedures, including the Overall RAN architecture aspects, as highlighted below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In addition, some technical proposals  were agreed upon at the previous RAN3 meeting as follows:

The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914.

-  FFS on the implications of this requirement on 6G RAN architecture.

-  FFS whether all deployment scenarios of TR 38.914 can be supported.

In this contribution, we initially analyze basic principles for 6G RAN deployment from RAN3 perspective, and subsequently, based on these principles, we further provide specific proposals for 6G RAN deployment.

2	Discussion

2.1  Deployment Principles for 6G RAN

In addition to all the scenarios defined in TR 38.914, the RAN architecture also needs to take into account some typical scenarios from RAN3. For example, in 5G, TR 38.801 of RAN3 has studied several deployment scenarios, mainly including centralized deployment, non-centralized deployment, co-sited deployment, shared RAN deployment. In 6G, from the perspective of RAN architecture, these typical scenarios are also valuable.

Considering the six technical scenarios for 6G proposed by ITU, which basically cover both legacy 5G communication services and new services such as AI and ISAC. To better support these services, the design of 6G RAN deployment scenarios needs to first define certain principles. Specifically, RAN3 can consider the following aspects:

- Flexibility of Functions Deployment

There are significant differences in 6G service requirements (e.g,  from NTN and TN integration to ultra-low latency). Therefore, the architecture design needs to support more flexible functional splitting (such as the CU-DU separation like that in 5G), so as to avoid the single-node fixed mode like that in 4G networks.

- Coordination Across Nodes

The 6G network needs to manage multiple types of nodes such as indoor nodes, macro base stations, and satellites simultaneously. By implementing coordinated management of different site types, it is beneficial to achieve optimal network energy saving and resource coordination effects.

- Continuity of Service

It should be ensured that the existing 5G services (e.g, high-definition video, voice, etc) can be smoothly transitioned to 6G. Therefore, the network deployment should allow for co-site management for new and old network nodes to avoid service interruptions.

- Minimization of Commercial Costs

In 6G, operators should be able to minimize deployment costs by sharing network resources, and such deployment has already achieved actual commercial deployment in 5G networks.

- Simplification of Technical Design

For deployment scenarios that have been maturely used in 5G, 6G can continue to adopt such deployments and focus on more advanced technical applications (e.g, ultra-high frequency band, new services, etc.), which can further simplify the cost of technical evolution.

Proposal 1: RAN3 agrees with the above-mentioned basic principles to design various deployment types of 6G RAN.

2.2  Deployment Scenarios for 6G RAN

Based on the above deployment principles, we propose that 6G RAN should support the following six typical deployment scenarios:

1）Centralized Deployment

This is one of the typical RAN deployments in 5G. Through such a deployment, 6G RAN can obtain the following benefits:

- It allows centralized deployment of some functions, enabling full sharing of software/hardware resources and further improving resource utilization.

- In addition, it is helpful to realizing cloud-native deployment of 6G RAN.

- Finally, it enables unified management and coordination of radio resources, which helps simplify signaling and improve resource management efficiency.

The following figure shows an example of a typical 6G deployment scenario:

2）Non-centralized Deployment

This is another typical RAN deployment in 5G. Through such a deployment, 6G RAN can obtain the following benefits:

- By aggregating all functions within a single node, the transmission latency of signaling and user data can be further reduced.

- In addition, with the large-scale deployment of individual nodes, e.g, for high-frequency micro sites, the costs and workload of operation and maintenance can be significantly reduced.

- Finally, this deployment is also a widely commercially used scenario in current 5G networks.

The following figure shows an example of a typical 6G deployment scenario:

3）Co-sited Deployment

As 5G evolves to 6G, in order to support the design of 5G and 6G sharing spectrum resources, it is necessary to study MRSS technical solutions. In the early stage of 6G deployment, a typical deployment design should allow the co-site deployment of 5G and 6G radio access functions. The advantage of such a deployment is that it allows 6G RAN to realize dynamic spectrum sharing with 5G RAN by non-standardized way.

The following figure shows an example of a typical 6G deployment scenario:

4）Heterogeneous Deployment

In 5G, macro cell and small cell are deployed simultaneously, where a macro cell is used to provide coverage, while small cells are used to enhance hotspot capacity. Additionally, in 5G, technologies such as DC and NES have been specified between macro nodes and small nodes. For 6G RAN, these application scenarios still have potential space, especially in terms of network energy saving. For example, under the control of macro node, actively turning small cells on and off can effectively improve the energy efficiency of the entire network.

The following figure shows an example of a typical 6G deployment scenario:

5）Shared Deployment

Similar to 5G, 6G networks should also support shared RAN deployment to serve multiple hosted core network operators. A shared 6G RAN can cover vast geographical areas, such as in national-level or regional-level network sharing scenarios. The shared RAN should enable efficient interoperability with non-shared RAN. Mobility between non-shared RAN and shared RAN should be supported, and the level should be at least no lower than that of mobility support in 5G systems.

The following figure shows an example of a typical 6G deployment scenario:

Proposal 2：Capture the aforementioned typical deployment scenarios into the RAN3 TR.

3	Conclusion

Base on the discussion above, we provide the following proposals:

Proposal 1: RAN3 agrees with the following basic principles to design various deployment types of 6G RAN.

- Flexibility of Functions Deployment

- Coordination Across Nodes

- Continuity of Service

- Minimization of Commercial Costs

- Simplification of Technical Design

Proposal 2：Capture the aforementioned typical deployment scenarios into the RAN3 TR.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- TR 38.914 Study on 6G Scenarios and Requirements  v0.1.0

Annex1	Text Proposal

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

5.2	Deployment Scenarios

Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914.

The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914.

-  FFS on the implications of this requirement on 6G RAN architecture.

-  FFS whether all deployment scenarios of TR 38.914 can be supported.

The following example scenarios should be considered for support by the RAN architecture.

1）Centralized Deployment

6G RAN should support centralization of the upper layers of the 6G radio stacks.

Different protocol split options between Central Unit and lower layers of  6G NB nodes may be possible. The functional split between the Central Unit and lower layers of 6G NB nodes may depend on the transport layer:

2）Non-centralized Deployment

In this scenario, the full protocol stack is supported at the 6G NB e.g. in a macro deployment or indoor hotspot environment (could be public or enterprise). The 6G NB can be connected to “any” transport. It is assumed that the 6G NB is able to connect to other 6G NBs via a RAN interface:

3）Co-sited Deployment

In this scenario the 6G functionality is co-sited with NR functionality either as part of the same base station or as multiple base stations at the same site. Co-sited deployment can be applicable in all NR deployment scenarios e.g. Urban Macro:

4）Heterogeneous Deployment

In 6G, macro cell and small cell can be deployed simultaneously, where a macro cell is used to provide coverage, while small cells are used to enhance hotspot capacity. Additionally, in 6G, the technology similar as Network Energy Saving can be applied between macro nodes and small nodes:

5）Shared Deployment

6G should support shared RAN deployments, supporting multiple hosted Core Operators. The Shared RAN could cover large geographical areas, as in the case of national or regional network sharing. The Shared RAN coverage could also be heterogeneous, i.e. limited to few or many smaller areas, for example in the case of Shared in-building RANs. A shared RAN should be able to efficiently interoperate with a non-shared RAN.

Each Core Operator may have their own non-shared RAN serving areas adjacent to the Shared RAN. Mobility between the non-shared RAN and the Shared RAN shall be supported in a way at least as good as for 5G NR:

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>