3GPP TSG-RAN WG3 Meeting #130	R3-258448

Dallas, US, 17th – 21st November, 2025

Agenda Item:	10.3.2

Source:	CATT

Title:	Considerations on 6G RAN-CN interface options

Document for:	Decision

1	Introduction

In the 6G WID [1], RAN3 is responsible for the Radio Access Network Architecture, interface protocols and procedures, including the RAN-CN function split, interface, protocol stack and procedures, as highlighted below:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc.). [RAN3, RAN2]

- Overall RAN architecture aspects [RAN3, RAN2]

- RAN-CN functional split, interface, protocol stack and procedures [RAN3]

- RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we initially analyze the 6G RAN-CN interface options, considering different requirements from the legacy services and new services. Considering how to make support of legacy NG-like functions and how to make support of the new services e.g. AI, Sensing, over the 6G RAN-CN interface separately.

Base on the discussion, we will provide the observations and proposals accordingly.

2	Discussion

2.1 6G RAN-CN Interface Overview

6G RAN-CN interface shall support both legacy communication based functions and new service functions. In the last meeting, it’s agreed that “the 6G RAN-CN interface supports control plane and user plane separation”. We understand that at least this is applied to the legacy communication based functions, for the new services, this can be further studied, e.g. whether the same user plane is applied or new data plane is needed. Protocol stack of the control plane and user plane is provided in section 2.2.

For new services such as AI/ML and ISAC, there are distinct characteristics that differentiate them from traditional communication functions/services. For instance, sensing typically involves tracking targets not served by the cellular network. Similarly, AI/ML is generally aimed at optimizing overall system performance rather than serving a specific UE. In such cases, there is no need for AMF to be involved throughout the entire process. Therefore, establishing a direct interface between RAN and the core network for these new services appears to be a more efficient approach.

- For the new services, it’s feasible and more efficient to support direct connection between 6G RAN and new service function(s) in the CN.

If a new direct interface is established between the RAN and a new service function in 6G Core Network, it should adhere to a principle similar to the separation of control plane and user plane of NG interface. This is because signalling transmission and data transmission entail fundamentally different requirements in terms of latency, reliability, and data volume. Applying such a separation would facilitate independent scaling of resources, and contribute to more efficient end-to-end service delivery for emerging applications such as AI/ML and ISAC. Details on signalling and data transmission for the new services will be further discussed in section 2.3.

- For the new services, signalling and data transmission over the 6G RAN-CN interface should be considered separately.

Besides the new services mentioned above, we consider the potential optimization to the positioning service from architecture perspective.

In 5G, on support of the Positioning, NG-RAN connects to LMF via the AMF indirectly, as shown in the figure below:

Figure 1: UE Positioning Overall Architecture applicable to NG-RAN

Base on this architecture, positioning related procedures are exchanged between NG-RAN and LMF through AMF transparently. For 6G, we can further consider whether and how to simplify the Positioning architecture, e.g. support direct connection between 6G RAN and the Positioning function in the CN (LMF or LMF+), which could make the Positioning related procedures more straightforward and efficient. However, how to support UE-specific procedures between RAN and LMF need to be further investigated, SA2 involvement seems needed.

- For positioning service, further study the feasibility of direct connectivity between 6G RAN and related positioning function in the CN (e.g. LMF+).

As above, an example of 6G RAN-CN interface could be illustrated with the figure below:

Figure 2. Example of 6G RAN-CN interface

As shown in the figure:

- 6G RAN directly connect to AMF+ for legacy control plane functions;

- 6G RAN directly connect to UPF+  for legacy user plane data transmission;

- 6G RAN directly connect to Positioning function in the CN (maybe evolved LMF, maybe new function);

- 6G RAN which holds AI/ML function, directly connect to AI/ML function(s) in the CN;

- 6G RAN which holds Sensing function, directly connect to Sensing function(s) in the CN;

- For the other new services, if any, 6G RAN directly connect to the related new service functions in the CN.

NOTE: the direct interface between 6G RAN and new service functions in the CN support both signalling and data transmission.

2.2 Support of legacy functions

The legacy functions (NG-like functions) of the 6G RAN-CN interface (called “NG+” for short below), NG-like interface design could be taken as the baseline, e.g. control plane and user plane separation.

The NG+ control plane interface is defined between the 6G RAN and 6G CN entity. It’s assumed the control plane protocol stack of the NG+ interface could base on the NG-C (refer to [5]), the transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG+ Application Protocol.

Figure 3: NG+ Interface Control Plane

The SCTP layer provides the guaranteed delivery of application layer messages.

In the transport IP layer point-to-point transmission is used to deliver the signalling PDUs.

SCTP/IP could be reused as transport protocol for the control plane of NG+.

We see a few companies expect to change SCTP to QUIC, we need to further discuss the necessity of the change, and evaluate the potential risks, e.g. the complexity and cost of network deployment.

- For legacy functions, SCTP/IP could be reused as transport layer for Control plane of 6G RAN-CN interface (NG+). The application layer signalling protocol is referred to as NG+ Application Protocol.

The NG+ user plane interface is defined between the 6G RAN and the UPGW. The user plane interface provides non-guaranteed delivery of user plane PDUs between the 6G RAN and the UPGW.

The XG-U interface shall support per PDU Session tunnelling.

NOTE 1:	Support of other type tunnelling e.g. per node tunnelling may be decided at WI phase, and pending to the progress of SA2.

The protocol stack for NG+ user plane is shown in Figure below.

Figure 4: NG+ User Plane protocol structure

NOTE 2:	GTP-U is a baseline for NG-U, however, later some enhancements or introduction of a new alternative protocol is still possible if justified.

- For legacy functions, GTP-U/UDP/IP could be reused as transport protocol for the user plane of the 6G RAN-CN interface (similar to NG-U).

2.3 Support of new services

2.3.1 Signalling aspects

Unlike legacy functions, for 6G network, many new types of services need to be introduced, e.g., AI, Sensing, etc. To adapt to the new services scenarios, the requirements for new network interfaces and protocols should be elastic scalability, cloud-native, independence, and so on [2]. It’s assumed that the 6G CN will continue to adopt the SBI-based network architecture, if the new service types also take the SBI-based interface and protocol into account, it can further realize the unified architecture design of 6G Network in the field of new services.

For the SBI-based interface and protocol on which new services can rely, there are the following technical advantages:

- Extensibility of Network Functions

By defining NF Services (SBI APIs), modularization and extensibility can be realized. This is mainly due to service-based architecture principle: adding a new service to an existing Network Function (NF) or introducing a new NF with additional services has minimal impact on other NFs. Network Repository Functions provide registration and discovery services, enabling service providers to announce their new services (or service modifications) in near real-time, while service consumers can receive immediate notifications of such modifications.

- Cloud-Friendly Features

The architecture based on SBI can adapt to and leverage cloud-native technologies, enabling virtualization and containerization of NFs. This in turn brings advantages, e.g., flexible scaling, rapid iteration, improved resource utilization, and enhanced openness. The SBI-based architecture also introduces cloud-friendly protocols, applicable to subscription and notification mechanisms as well as authentication scenarios, e.g., protocols like HTTP/2, HTTP/3, TCP or QUIC.

- Independence of Network Functions

For legacy N2 protocol, all UE-related signalling and non-UE-related signalling are bound to SCTP associations. In contrast, an SBI-based architecture supports resource-level granularity: service operations and created resources can be addressed via URIs. This means message routing can be executed without parsing the message payload, thereby achieving decoupling between service functions and the transport layer. Additionally, the SBI-based architecture enhances flexibility in encryption deployment by providing a method to flexibly control encryption implementation (e.g., HTTP over TLS) on demand.

- Resilience of Network Functions

The service-based architecture natively supports auxiliary services, e.g., load balancing. Message destinations can be identified using the target URI in HTTP headers, and load balancing can be performed without parsing the message payload. This realizes system scalability without relying on specific application protocol. Furthermore, the definition of NF (service) sets provides increased resiliency, e.g., in case of overload or failure of NF instances.

- Efficiency of Transmission

For new types of services, direct communication between different NFs should be permitted. On one hand, this reduces transmission latency. On the other hand, it eliminates redundant binding relationships between different NFs. The SBI-based architecture adopts a bus-based topology design, which ensures that direct communication connections between different NFs can be established on demand. This simplifies the overall network design.

In summary, regarding new types of services, especially in terms of control signalling design, it is recommended to consider the technical advantages of interfaces and protocols based on SBI.

- For signalling transmission of the new services, consider the technical advantages SBI based interface.

2.3.2 Data Transmission

For data transmission of the new services, we understand that the traditional user plane based solution (as discussed in section 2.2) may be not entirely suitable with the following concerns:

- Different QoS requirements

For legacy 5G services, DRBs are used to carry user plane data, such as voice, video, and application data, which have diverse QoS profiles tailored to user services. DRBs are more flexible in quantity—multiple DRBs can be established for different services of a single UE, each with unique QoS configurations. For AI/ML data in different scenarios/applications, there may be significant differences in QoS requirements, e.g., training data is not sensitive to latency and allows packet loss; model data does not allow packet loss and has a large data volume, etc.

- Routing and Delay

The UP tunnel can only be established between UE and UPF, the fixed start/terminate node may not fulfil the data transmission requirements of diverse AI/ML and sensing services. For example, in some scenario, it is only necessary to exchange data between RAN and CN, without involvement of UE. We understand it’s beneficial to establish a direct data transmission connection between the RAN and the corresponding Core Network function for new services, rather than relying on the current user plane architecture, which always routes traffic through the UPF.

We understand that, on the detail design of the user plane/data plane for the new services, coordination with RAN2 and SA2 are needed.

- For data transmission of the new services, it’s preferred to establish a direct data transmission connection between RAN and the corresponding service function in the CN rather than route data traffic via UPF, coordination with RAN2 and SA2 are needed on the interface design.

As above, corresponding pCR is provided in the section 5, including considerations on RAN-CN interface options for legacy communication based functions and new services. RAN3 is requested to discuss and agree the pCR to TR 38.760-3.

- RAN3 is requested to discuss and agree the pCR to TR 38.760-3 on 6G RAN-CN interface aspects.

3	Conclusion

Base on the discussion above, we provide the following proposals:

- For the new services, it’s feasible and more efficient to support direct connection between 6G RAN and new service function(s) in the CN.

- For the new services, signalling and data transmission over the 6G RAN-CN interface should be considered separately.

- For positioning service, further study the feasibility of direct connectivity between 6G RAN and related positioning function in the CN (e.g. LMF+).

- For legacy functions, SCTP/IP could be reused as transport layer for Control plane of 6G RAN-CN interface (NG+). The application layer signalling protocol is referred to as NG+ Application Protocol.

- For legacy functions, GTP-U/UDP/IP could be reused as transport protocol for the user plane of the 6G RAN-CN interface (similar to NG-U).

- For signalling transmission of the new services, consider the technical advantages SBI based interface.

- For data transmission of the new services, it’s preferred to establish a direct data transmission connection between RAN and the corresponding service function in the CN rather than route data traffic via UPF, coordination with RAN2 and SA2 are needed on the interface design.

- RAN3 is requested to discuss and agree the pCR to TR 38.760-3 on 6G RAN-CN interface aspects.

4	Reference

- RP-251881 New SID: Study on 6G Radio

- 3GPP TR38.801 v14.0 Study on new radio access technology: Radio access architecture and interfaces;

- 3GPP TS38.410 v18.6.0 NG general aspects and principles

- 3GPP TS38.413 v18.6.0 NG Application Protocol (NGAP)

- 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

5	pCR to TR 38.760-3

<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

6.1.3	RAN-CN Interface Options

Editor’s note: This chapter includes description of RAN-CN interface options including protocol stacks, considering new and existing services.

6.1.3.1 6G RAN-CN interface for legacy functions

6.1.3.1.1 Control Plane

The control plane of 6G RAN-CN interface (called NG+ below) is defined between the 6G RAN and the CN entity. It’s assumed the control plane protocol stack of the NG+ interface could base on the NG-C (refer to [5]), the transport network layer is built on IP transport. For the reliable transport of signalling messages, SCTP is added on top of IP. The application layer signalling protocol is referred to as NG+ Application Protocol.

Figure 6.1.3-1: NG+ Interface Control Plane

The SCTP layer provides the guaranteed delivery of application layer messages.

In the transport IP layer point-to-point transmission is used to deliver the signalling PDUs.

SCTP/IP is could be reused as transport protocol for the control plane of NG+.

NOTE:  NG+ could be replaced, if the name of 6G RAN-CN interface is decided.

6.1.3.1.2 User Plane

The user plane of 6G RAN-CN interface is defined between the 6G RAN and the UPGW (FFS). The user plane interface provides non-guaranteed delivery of user plane PDUs between the 6G RAN and the UPGW.

The XG-U interface shall support per PDU Session tunnelling.

NOTE 1:	Support of other type tunnelling e.g. per node tunnelling may be decided at WI phase, and pending to the progress of SA2.

The protocol stack for NG+ user plane is shown in Figure below.

Figure 6.1.3-2: NG+ User Plane protocol structure

NOTE 2:	GTP-U is a baseline for NG-U, however, later some enhancements or introduction of a new alternative protocol is still possible if justified.

6.1.3.2 6G RAN-CN interface for new services

6.1.3.2.1 Control Plane

We should further evaluate P2P vs SBI for the control plane of the new services. The control plane of P2P interface is similar to the 6.1.3.1.1. While for the Service Based Interface, the following advantages could be taken into account:

- Extensibility of Network Functions

By defining NF Services (SBI APIs), modularization and extensibility can be realized. This is mainly due to service-based architecture principle: adding a new service to an existing Network Function (NF) or introducing a new NF with additional services has minimal impact on other NFs. Network Repository Functions provide registration and discovery services, enabling service providers to announce their new services (or service modifications) in near real-time, while service consumers can receive immediate notifications of such modifications.

- Cloud-Friendly Features

The architecture based on SBI can adapt to and leverage cloud-native technologies, enabling virtualization and containerization of NFs. This in turn brings advantages, e.g., flexible scaling, rapid iteration, improved resource utilization, and enhanced openness. The SBI-based architecture also introduces cloud-friendly protocols, applicable to subscription and notification mechanisms as well as authentication scenarios, e.g., protocols like HTTP/2, HTTP/3, TCP or QUIC.

- Independence of Network Functions

For legacy N2 protocol, all UE-related signalling and non-UE-related signalling are bound to SCTP associations. In contrast, an SBI-based architecture supports resource-level granularity: service operations and created resources can be addressed via URIs. This means message routing can be executed without parsing the message payload, thereby achieving decoupling between service functions and the transport layer. Additionally, the SBI-based architecture enhances flexibility in encryption deployment by providing a method to flexibly control encryption implementation (e.g., HTTP over TLS) on demand.

- Resilience of Network Functions

The service-based architecture natively supports auxiliary services, e.g., load balancing. Message destinations can be identified using the target URI in HTTP headers, and load balancing can be performed without parsing the message payload. This realizes system scalability without relying on specific application protocol. Furthermore, the definition of NF (service) sets provides increased resiliency, e.g., in case of overload or failure of NF instances.

- Efficiency of Transmission

For new types of services, direct communication between different NFs should be permitted. On one hand, this reduces transmission latency. On the other hand, it eliminates redundant binding relationships between different NFs. The SBI-based architecture adopts a bus-based topology design, which ensures that direct communication connections between different NFs can be established on demand. This simplifies the overall network design.

6.1.3.2.2 User/Data Plane

For data transmission of new services, e.g. AI/ML, Sensing, traditional user plane based solution is not suitable with the following concerns:

- Different QoS requirements

For legacy 5G services, DRBs are used to carry user plane data, such as voice, video, and application data, which have diverse QoS profiles tailored to user services. DRBs are more flexible in quantity—multiple DRBs can be established for different services of a single UE, each with unique QoS configurations.

For AI/ML data in different scenarios/applications, there may be significant differences in QoS requirements, e.g., training data is not sensitive to latency and allows packet loss; model data does not allow packet loss and has a large data volume, etc.

- Routing and Delay

The UP tunnel can only be established between UE and UPF, the fixed start/terminate node may not fulfil the data transmission requirements of diverse AI/ML and sensing services. For example, in some scenario, it is only necessary to exchange data between RAN and CN, without involvement of UE. We understand it’s beneficial to establish a direct data transmission connection between the RAN and the corresponding Core Network function for new services, rather than relying on the current user plane architecture, which always routes traffic through the UPF.

NOTE: Coordination with RAN2 and SA2 are needed on the interface design of the user plane/data plane for the new services.

<<<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>>>>>>>>

6.1.4	Evaluation of RAN-CN Interface Options

Editor’s note: This chapter includes evaluation and comparison of RAN-CN interface options described in clause 6.1.3.

For the control plane for the new services, further evaluation and comparison on P2P interface vs SBI based interface should consider but not limit to the following aspects:

- Reliability

- Resilience

- Extendibility

- Efficiency

- Security

- Evolution

NOTE: RAN3 should first discuss and decide how to evaluate and compare the P2P vs SBI interface.

<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>